3GPP TSG RAN WG1 #116	 R1-2400527

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.6.1

Source:	Honor

Title:	Discussion on LP-WUS and LP-SS design

Document for:	Discussion and Decision

Introduction

In RAN#102, a new Rel-19 WID “Low-power wake-up signal and receiver for NR (LP-WUS/WUR)” has been approved with the following objectives [1]:

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - This objective is to be further refined in RAN#103

In this contribution, we provide input for the first objective. The following aspects of LPWUS design will be discussed: waveform generation, overlaid OFDM sequences, detection scheme and content.

- Discussion

  - Waveform

    - Waveform generation

Currently, there are only two generation methods of LPWUS waveforms: OOK-1 and OOK-4. As described in [2]:

-	Option OOK-1: Single-bit in 1 OFDM symbol, SCs of LP-WUS are

-	OOK=1 means all SCs are modulated

-	OOK=0 means all SCs are zero power (from base-band point of view)

- Option OOK-4: Transform M-bit OOK in time domain

-	N SCs of OOK-1 are generated by a transformation (DFT/Least square)

-	N’ samples are generated from M-bits

-	signal modification may or may NOT be used

-	truncation or other additional modification may or may NOT be used, if not used, N is the same as N’

-	N’ can be the same as K

It can be seen from the waveform generation processes of OOK-1 and OOK-4 that the OOK-1 process is more direct, while OOK-4 requires more signal generation processes.

Observation 1: OOK-4 is more complex to implement than OOK-1.

For performance, the following results can be observed in [2]:

- Single frequency segment OOK-1 and OOK-4 are the most robust waveforms to frequency errors.

- OOK-4 will be less robust to timing errors than OOK-1 as M increases but could be addressed by using a sliding window at the receiver or by pulse shaping in time domain.

- OOK-4 with variable M can provide a flexible range of spectral efficiencies for a fixed LP-WUS resource.

Observation 2: The performance of OOK-1 and OOK-4 is equivalent in both frequency domain and time domain, but when M increases, OOK-4's robustness to timing errors decreases.

Observation 3: The advantage of OOK-4 is that it can provide a flexible range of spectral efficiencies, but it needs to increase the implementation complexity to ensure performance.

The primary design target of LP-WUS is to simplify the implementation and to reduce power consumption of the receiver, rather than optimizing the design for spectral efficiency. The design can even be simplified by sacrificing a bit of spectral efficiency. Therefore, it is recommended that OOK-1 be given higher priority than OOK-4.

Proposal 1: Prioritize OOK-1.

    - Overlaid OFDM sequences

According to the evaluation results of R18 LPWUS SI, OOK-1 and OOK-4 can improve detection performance by using overlaid OFDM sequences. On the one hand, the OFDM sequence can make the spectrum of the OOK waveform more uniform in the frequency domain. On the other hand, the receiving side can use known sequence information to improve detection performance. It can be seen from the existing evaluation results that the receiver using sequences for correlation is simpler and has lower power consumption than the OFDM receiver with the I/Q branches.

Observation 4: Using the correlation of sequences, the implementation complexity and power consumption of the receiver can be reduced.

When designing the NR PSS, the performance of the M sequence and the ZC sequence was evaluated, and the M sequence was finally selected based on the performance advantage of anti-frequency offset. Under LP-WUS, some terminals will still use PSS/SSS for synchronization. Therefore, it can be considered to use the same sequence for LP-WUS and PSS/SSS to reduce the complexity of the receiver. In addition, it is best to use the same sequence type for the design of LP-SS, which not only ensures performance but also enables unified design.

Observation 5: The M sequence not only has better anti-frequency offset performance but also can reduce the complexity of the receiving side.

Proposal 2: Prioritize M sequence.

  - Detection scheme

RAN1 has studied two schemes: duty-cycled monitoring and continuous monitoring. It can be seen from the intuitive analysis and evaluation results that duty-cycle detection has better power saving effect than continuous detection. However, an important research motivation for this topic is to solve the delay problem in current UE power saving methods. For example, in fire detection and extinguishment use case, fire shutters shall be closed, and fire sprinklers shall be turned on by the actuators within 1 to 2 seconds from the time the fire is detected by sensors.

Observation 6: In actual use cases, there are scenarios where delay is more important than power saving.

SI includes the following use cases:

-	IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc., including the following characteristics,

-	Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc.,

-	eMBB cases including e.g., XR/smart glasses, smart phones and etc.,

In the usage scenarios of IOT and wearable devices, there are scenarios where latency is more important. Therefore, it is recommended to support both detection schemes.

Proposal 3: Support both duty-cycled monitoring and continuous monitoring.

  - Content

If the OFDM sequence is only used for performance improvement, the content information will only be sent through OOK modulated information. However, the objective of WI includes the possibility of OFDM sequences carrying information. Therefore, it is necessary to discuss how LP-WUS information is carried through OOK information and OFDM sequences. The design of information carried by OFDM sequences will lead to different requirements for UE capabilities. For example, if the OFDM sequence carries mandatory information, then all terminals are required to demodulate the OFDM sequence to obtain the information.

Observation 7: The method by which OFDM sequences carry information will affect the UE capability requirements.

The methods of carrying LPWUS information through OOK modulated information and OFDM sequence information can be roughly divided into the following three types:

- Separate carrying: Information is carried simultaneously by OOK modulated information and OFDM sequences, which can be original bits or encoded bits.

- Priority carrying: OOK modulated information and OFDM sequence information have different priorities. For example, OOK information contains main information, while OFDM sequence information is only supplementary information and is only demodulated for devices with strong capabilities.

- Repeated carrying: OOK modulated information and OFDM sequence information contain the same information to improve the reliability of information transmission.

The requirements for the UE capabilities of the three methods are from high to low, so the spectrum efficiency of transmission is also from high to low. The final trade-off and selection need to be made based on the purpose and requirements of LP-WUS signal design.

Proposal 4: Discuss the schemes for OOK and OFDM sequences to carry LP-WUS information: separate carrying, priority carrying and repeated carrying.

- Conclusions

In this contribution, we provide our views on waveform, detection scheme, and information carrying. The following observations and proposals are given:

Observation 1: OOK-4 is more complex to implement than OOK-1.

Observation 2: The performance of OOK-1 and OOK-4 is equivalent in both frequency domain and time domain, but when M increases, OOK-4's robustness to timing errors decreases.

Observation 3: The advantage of OOK-4 is that it can provide a flexible range of spectral efficiencies, but it needs to increase the implementation complexity to ensure performance.

Observation 4: Using the correlation of sequences, the implementation complexity and power consumption of the receiver can be reduced.

Observation 5: The M sequence not only has better anti-frequency offset performance but also can reduce the complexity of the receiving side.

Observation 6: In actual use cases, there are scenarios where delay is more important than power saving.

Observation 7: The method by which OFDM sequences carry information will affect the UE capability requirements.

Proposal 1: Prioritize OOK-1.

Proposal 2: Prioritize M sequence.

Proposal 3: Support both duty-cycled monitoring and continuous monitoring.

Proposal 4: Discuss the schemes for OOK and OFDM sequences to carry LP-WUS information: separate carrying, priority carrying and repeated carrying.

References

- RP-234056, “Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, RAN#102, Dec. 11 - 15, 2023.

- 3GPP TR 38.869