3GPP TSG RAN WG1 #116		R1-2400528

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.11.1

Source:	Honor

Title:	Discussion on NR-NTN downlink coverage enhancement

Document for:	Discussion and Decision

Introduction

In RAN#102, a new Rel-19 WID “Non-Terrestrial Networks (NTN) for NR Phase 3” has been approved with the following objective [1]:

- 4 Objective - 4.1 Objective of SI or Core part WI or Testing part WI - Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Notes for this objective: - SSB channel enhancement is not considered - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

Discussion on power sharing among satellite beams

The Rel-18 work on coverage enhancements has been focused on UL enhancements, but it was found that even DL coverage could be a bottleneck in some situations. When addressing handset terminals (including smartphones with -5.5 dBi antenna gain) w.r.t. downlink coverage considering the NTN deployment constraints such as payload power limitation, large satellite foot print and limited feeder link bandwidth. DL coverage enhancements are needed to accommodate satellite payload constraints which may be unable to have all its beams active with the nominal EIRP density per beam (see Section 6.1.1 in TR 38.821) at a given time due to limited power and limited feeder link bandwidth, while maximizing the number of beams that can be activated simultaneously, and ensuring that all user terminals can be served across the satellite foot print while maximizing the overall satellite throughput and ensuring that all satellite’s radio cells are kept alive even without traffic but allowing new users to join or preventing impact on end-user QoS.

Figure 1 Illustration of the beam vs. footprint

As shown in the Figure 1, the system level study should be triggered with consideration on several aspects, e.g., satellite capability on the number of simultaneous beams vs. number of footprints in target area, required EIRP density for one beam vs. total power of satellite and service continuity for each UE vs. service availability within one footprint. For example, a total number of beams = 1200 may be assumed for NGSO operating in FR1 band. This would correspond to the number of beams necessary to serve a satellite footprint at 30° min elevation with ~50 km diameter beam size. Reasonably, 56 dBW of satellite aggregated power should be considered, leading to a maximum number of 30 beams to be assumed with the nominal EIRP density. Satellite parameters in TR 38.821 should be reused as much as possible, except for additional considering satellite aggregated power and simultaneous illuminated beam number.

Proposal 1: Satellite parameters in TR 38.821 should be reused as much as possible, additional parameter configurations including total satellite power constraint and number of simultaneous satellite beams should be defined for downlink coverage enhancement evaluations.

In current NR specs, the downlink SS/PBCH SSS EPRE can be derived from the SS/PBCH downlink transmit power given by the parameter ss-PBCH-BlockPower provided by L3 signaling. The downlink CSI-RS EPRE can be derived from the SS/PBCH block downlink transmit power given by the parameter ss-PBCH-BlockPower and CSI-RS power offset given by the parameter powerControlOffsetSS provided by L3 signaling.

For the purpose of SS-RSRP, SS-RSRQ and SS-SINR measurements, the UE may assume downlink EPRE is constant across the bandwidth. For the purpose of SS-RSRP, SS-RSRQ and SS-SINR measurements, the UE may assume downlink EPRE is constant over SSS carried in different SS/PBCH blocks.

For the purpose of CSI-RSRP, CSI-RSRQ and CSI-SINR measurements, the UE may assume downlink EPRE of a port of CSI-RS resource configuration is constant across the configured downlink bandwidth and constant across all configured OFDM symbols.

If dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size is introduced, L1/L2 signaling used for downlink power allocation should be studied. The above assumptions used by the UE when making measurements may be re-evaluated.

Proposal 2: The L1/L2 signaling to support dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size should be studied.

Proposal 3: Impacts on measurements should be studied when dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size is introduced in NTN system.

Discussion on PDCCH/PDSCH coverage enhancement

Rel-18 NTN started to consider practical parameters used for commercial smartphones with   -5.5 dBi antenna gain. However, there was no time to look into the impact on DL coverage from the above parameter in Rel-18. During Rel-18 NR NTN study phase for coverage enhancement, the link level evaluation identified UL coverage as the bottleneck. However, the evaluation assumptions did not take into account DL satellite power split among multiple DL satellite beams. A satellite supporting more simultaneous active satellite beams can provide service in larger area. On the contrary, it also leads to more reduction for the transmission power over a single beam, naturally yielding more reduced per-beam signal strength. Therefore, when the power split is considered, there is a need also for DL coverage enhancement. Considering the above realistic assumptions, the DL channels including PDCCH and PDSCH should be evaluated.

Link-level analysis with realistic assumptions , e.g., UE antenna gain set to -5.5dBi, 2Rx in UE side, DL power/EIRP density of satellite, is needed to identify the bottleneck channel. Additionally, regarding the target for improvement, considering the back-forward compatibility, the required SINR of SSB can be taken as the target for enhancement.

PDCCH reliability enhancements are introduced in Rel-17 via PDCCH repetitions that have different TCI states in multi-TRP scenario. This mechanism can be applied on a single TRP, i.e. corresponding to a satellite, by network implementation. The PDCCH repetition scheme introduced in Rel-17 should be starting point for coverage enhancement for PDCCH in NTN scenario.

Proposal 4: The coverage performance of broadcast PDCCH and unicast PDCCH should be evaluated to specify whether coverage enhancements are needed for them.

In Rel-15, PDSCH transmission can be repeated in a semi-static manner, when the UE is configured by the higher layer parameter AggregationFactor. In Rel-16, additional columns can be configured in the PDSCH-TimeDomainResourceAllocation table to provide repetitionNumber information. PDSCH transmission can be repeated in a dynamic manner, via DCI indicating the time-domain resource allocation table including repetitionNumber information. The coverage performance of PDSCH can be improved by PDSCH repetition supported by current spec in NTN scenario.

Proposal 5: Deprioritize the coverage enhancements for PDSCH in Rel-19 NTN work item.

In order to improve the precision of channel estimation at low SNR, DM-RS bundling is applied to PUSCH and/or PUCCH repetitions and/or transport-block processing over multiple slots has been introduced in Rel-17. Similarly DM-RS bundling applied to PDCCH/PDSCH can be considered in Rel-19 NTN coverage enhancements.

Proposal 6: DM-RS bundling applied to PDCCH/PDSCH repetitions should be considered in Rel-19 NTN.

Conclusion

- In this contribution, we provide our views on NR-NTN downlink coverage enhancement with the following proposals.

Proposal 1: Satellite parameters in TR 38.821 should be reused as much as possible, additional parameter configurations including total satellite power constraint and number of simultaneous satellite beams should be defined for downlink coverage enhancement evaluations.

Proposal 2: The L1/L2 signaling to support dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size should be studied.

Proposal 3: Impacts on measurements should be studied when dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size is introduced in NTN system.

Proposal 4: The coverage performance of broadcast PDCCH and unicast PDCCH should be evaluated to specify whether coverage enhancements are needed for them.

Proposal 5: Deprioritize the coverage enhancements for PDSCH in Rel-19 NTN work item.

Proposal 6: DM-RS bundling applied to PDCCH/PDSCH repetitions should be considered in Rel-19 NTN.

- References

- RP-234078	New WID: Non-Terrestrial Networks (NTN) for NR Phase 3

- RP-231565	Proposed RAN1/2/3 led potential topics for Rel-19 NTN evolution  THALES

- TR 38.821	Solutions for NR to support non-terrestrial networks (NTN) 	V16.2.0

- TS 38.214	Physical layer procedures for data		V18.1.0