3GPP TSG-RAN WG1 Meeting #116	R1-2400529

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.7.1

Source:	Ericsson

Title:	Discussion on ISAC Deployment Scenarios

Document for:	Discussion, Decision

# Introduction

The objective of the study on channel modelling for Integrated Sensing And Communication (ISAC) for NR [1] includes the following:

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects:

- UAVs

- Humans indoors and outdoors

- Automotive vehicles (at least outdoors)

- Automated guided vehicles (e.g. in indoor factories)

- Objects creating hazards on roads/railways, with a minimum size dependent on frequency

All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic).

For the above use cases, sensing modes and frequencies:

- Identify details of the deployment scenarios corresponding to the above use cases.

In this contribution, we discuss new sensing scenarios, corresponding stochastic parameters, and how the sensing scenarios are paired with existing scenarios in TR 38.901 for a sensing target.

# Discussion

  - New sensing scenarios

The deployments used in conjunction with the channel model in TR 38.901, some of them being Urban Macro (UMa), Urban Micro (UMi), Rural Macro (RMa), etc., specify how the locations of user terminals (UTs), also known at user equipments (UEs), and base stations (BSs) are generated. When studying sensing, a scenario should specify, in addition to the UE and BS deployment, the relevant passive objects that affect the sensing outcome. The objects are passive in the sense that they themselves do not transmit signals and just reflect any incoming RF signal. The sensing objects include unwanted objects, termed clutters, and desired objects, termed targets.

There are two types of sensing objects: “targets” denotes objects that are intended to be detected and sensed, and “clutters” denotes objects that are interfering with the sensing.

Sensing objects are characterized by their location, how they move during the studied scenario, a type that denotes the physical object that they represent (e.g., pedestrian, car, drone), radar cross-sections, and so on. Several types of targets are to be considered in the study item [1], while the types of clutters can be diverse and need to be studied. For example, if the sensing targets are automotive vehicles, the possible clutters could be buildings and walls.

- To model sensing objects, study parameters of their locations, trajectories, radar cross-sections, and clutter types.

A question is whether the sensing objects and related parameters are to be added into existing scenarios in TR 38.901 or new sensing scenarios. In our view, it is better to consider sensing as an add-on to communication. Given sensing is an optional feature, simulations of communication can be realized with the new channel model, which is the output of the Rel-19 SI, without sensing additions. Another reason for sensing as an add-on to communication is to study the impact of overhead of sensing signal on communication systems, namely, to compare the communication performance with and without sensing. It should be ensured that the sensing-related model additions to TR 38.901, when enabled, do not have an undue effect on perceived communication quality and potential conclusions thereof. Therefore, new sensing scenarios can be defined.

Since sensing is an optional feature, it can be regarded as an add-on to communication.

- RAN1 to define new sensing scenarios and ensure that the sensing-related model additions to TR 38.901, when enabled, do not have an undue effect on perceived communication quality and potential conclusions thereof.

For sensing, a sensing transmitter sends out a signal, which is reflected by a target and/or by a clutter and then received by a sensing receiver. For a sensing target, there are two target-related sensing links: from sensing transmitter to target and from target to sensing receiver. The direct propagation path between sensing transmitter and sensing receiver has a significantly larger gain compared to the propagation paths scattered off the target and therefore needs to be modelled in a sensing channel model for all sensing modes.

The link between sensing transmitter and sensing receiver often contributes significantly to the channel gain and needs to be modelled in a sensing channel model for all sensing modes.

It is worth noting that in TR 38.901 up to Rel-18, gNB-UE bistatic communication channel mode is the only supported mode. For TRP-UE bistatic and UE-TRP bistatic sensing modes, the direct propagation path between sensing transmitter and sensing receiver can be referred to the existing gNB-UE communication model. However, for TRP-TRP bistatic, TRP monostatic, UE-UE bistatic, and UE monostatic sensing modes, the path between sensing transmitter and sensing receiver is not defined in TR 38.901 and needs to be studied.

- For TRP-UE bistatic and UE-TRP bistatic sensing modes, the direct propagation path between sensing transmitter and sensing receiver can be referred to the existing gNB-UE communication channel model.

- For TRP-TRP bistatic, TRP monostatic, UE monostatic, and UE-UE bistatic sensing modes, the direct propagation path between sensing transmitter and sensing receiver is not defined in TR 38.901 and needs to be studied.

  - Pairing between sensing scenarios and existing deployment scenarios

Sensing as an add-on to communication mode can be realized by pairing sensing scenarios with the existing deployment scenarios to define the geometry of the simulation setup, both for BSs and UEs, and for sensing objects. Considering the similarity between communication model and sensing model, we discuss how to pair the two kinds of scenarios.

Each of the existing deployment scenarios in TR 38.901, such as UMa and Indoor Office, models either indoor gNB or outdoor gNB deployments. Likewise, a sensing scenario can be characterized by one type of sensing target, either indoor or outdoor. In this way, there is no need to model mixed types of targets in a sensing scenario.

- A sensing scenario models only one type of sensing target, either indoor or outdoor.

Outdoor-to-outdoor (O2O), indoor-to-indoor (I2I), and outdoor-to-indoor (O2I) communication channel models are supported in TR 38.901, where both gNB and UE are outdoor, both of them indoor, or gNB is outdoor and UE is indoor. If sensing targets are added to the O2O and I2I communication models, it is reasonable to consider that in an outdoor environment, all of the sensing transmitters, sensing receivers, and the targets are outdoor, and all of them are indoor in an indoor environment. The rationale applies to all sensing modes. For example, the sensing scenario for automotive vehicles can be paired with UMa. UMi, or RMa. Outdoor gNBs and outdoor UEs can act as sensing transmitters/receivers for the sensing modes. Note that the UMa and UMi deployment scenarios models both indoor and outdoor UEs simultaneously according to a distribution ratio, and outdoor UEs are suitable to be the sensing transmitters/receivers to sense outdoor targets, compared with indoor UEs.

Moreover, despite O2I penetration loss in the outdoor gNB-to-indoor UE communication model, from channel modelling perspective, it is worth studying the O2I sensing link. For example, outdoor gNBs are used as sensing transmitters/receivers to sense indoor targets.

- Support the following scenarios:

- All of the sensing transmitters, sensing receivers, and targets are outdoor.

- All of the sensing transmitters, sensing receivers, and targets are indoor.

- If outdoor gNBs are the sensing transmitters/receivers, the targets are indoor.

We now clarify the indoor and outdoor UAVs as the sensing targets. Five kinds of targets are listed in [1], which clearly states each kind of targets is indoor, outdoor, or both, except for UAVs which needs to be clarified.

It is unclear if UAVs as sensing targets considered in Rel-19 ISAC Channel Modelling SI are outdoor or indoor.

Nevertheless, Objective in [1] states the following.

“The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137).”

In TS 22.137 [2], Table 6.2-1 specifies performance requirements for 5G Wireless sensing. Both indoor UAVs and outdoor UAVs are listed as sensing targets for object detection and object tracking. Moreover, UAVs are commonly used in warehouses and factories. Therefore, in our view, both indoor and outdoor UAVs are in the scope of Rel-19 ISAC channel modelling SI.

- Both indoor and outdoor UAVs as sensing target are in the scope of Rel-19 ISAC channel modelling SI.

Based on the previous discussion, the pairing between sensing scenarios and existing deployment scenarios is summarized in Table 1.

Table 1: Pairing between sensing scenarios and existing deployment scenarios

| Sensing scenarios | Compatible deployment scenarios from Section 7.2 in TR38.901 |
| --- | --- |
| Outdoor UAV | UMa, RMa, UMi |
| Indoor UAV | Indoor Factory |
| Outdoor human | UMa, UMi, RMa |
| Indoor human | UMa, UMi, Indoor Office, Indoor Factory |
| Automotive vehicles | UMa, RMa, UMi |
| Automated guided vehicles | Indoor Factory |
| Objects creating hazards on roads | UMa, RMa |
| Objects creating hazards on railways | UMa, RMa |

We propose the following:

- The pairing between sensing scenario and existing deployment scenarios in TR 38.901, is defined according to Table 1.

# Conclusion

In the previous sections we made the following observations:

Observation 1	There are two types of sensing objects: “targets” denotes objects that are intended to be detected and sensed, and “clutters” denotes objects that are interfering with the sensing.

Observation 2	Since sensing is an optional feature, it can be regarded as an add-on to communication.

Observation 3	The link between sensing transmitter and sensing receiver often contributes significantly to the channel gain and needs to be modelled in a sensing channel model for all sensing modes.

Observation 4	It is unclear if UAVs as sensing targets considered in Rel-19 ISAC Channel Modelling SI are outdoor or indoor.

Based on the discussion in the previous sections we propose the following:

Proposal 1	To model sensing objects, study parameters of their locations, trajectories, radar cross-sections, and clutter types.

Proposal 2	RAN1 to define new sensing scenarios and ensure that the sensing-related model additions to TR 38.901, when enabled, do not have an undue effect on perceived communication quality and potential conclusions thereof.

Proposal 3	For TRP-UE bistatic and UE-TRP bistatic sensing modes, the direct propagation path between sensing transmitter and sensing receiver can be referred to the existing gNB-UE communication channel model.

Proposal 4	For TRP-TRP bistatic, TRP monostatic, UE monostatic, and UE-UE bistatic sensing modes, the direct propagation path between sensing transmitter and sensing receiver is not defined in TR 38.901 and needs to be studied.

Proposal 5	A sensing scenario models only one type of sensing target, either indoor or outdoor.

Proposal 6	Support the following scenarios:

	All of the sensing transmitters, sensing receivers, and targets are outdoor.

	All of the sensing transmitters, sensing receivers, and targets are indoor.

	If outdoor gNBs are the sensing transmitters/receivers, the targets are indoor.

Proposal 7	Both indoor and outdoor UAVs as sensing target are in the scope of Rel-19 ISAC channel modelling SI.

Proposal 8	The pairing between sensing scenario and existing deployment scenarios in TR 38.901, is defined according to Table 1.

# References

RP-234016, “New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR), Nokia, 3GPP TSG RAN Meeting #102, December 2023

TS 22.137, Service requirements for Integrated Sensing and Communication, v19.0.0, 3GPP TSG SA Meeting #102, December 2023