3GPP TSG RAN WG1 #116                                       	      R1-2400531

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	8.1

Source:	Xiaomi

Title:                     Maintenance on NR MIMO Evolution for Downlink and Uplink

Document for:	Discussion and Decision

Introduction

The WI for R18 MIMO was approved in RAN-94 e-meeting, and the detailed objectives can be seen in [1]. While in this contribution, we will discuss the remaining issues on NR MIMO Evolution for Downlink and Uplink.

Unified TCI framework extension for multi-TRP

Unified TCI state indication in S-DCI case

For TCI state indication for S-DCI based M-TRP, the agreement in RAN1-111 e-meeting [2].

Agreement

On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list

- FFS: Increase on the size of the TCI field

- Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS

For TCI state indication for S-DCI based M-TRP, the agreement in RAN1-112b e-meeting [3].

Conclusion

On unified TCI framework extension for S-DCI based MTRP operation, there is no consensus to support dynamic switching between single-TRP operation and multi-TRP operation for channels/signals based on the number of TCI states mapped to the received TCI codepoint in DCI format 1_1/1_2

- FFS: How to switch between Rel-17 sTRP operation and Rel-18 mTRP operation

Agreement

On unified TCI framework extension for S-DCI based MTRP operation, support the followings:

- For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {first joint TCI state, second joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE)

- For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE)

- TCI state activation command (MAC-CE) should indicate that each joint/DL/UL TCI state mapped to a TCI codepoint is the first or second joint/DL/UL TCI state (detail on how to indicate above is up to RAN2 design)

- The first/second indicated joint/DL/UL TCI state(s) is updated according to the corresponding first/second joint/DL/UL TCI state(s) mapped to the TCI codepoint received by the UE

  - If the UE receives a TCI codepoint mapped with a sub-set of {first joint TCI state, second joint TCI state} or {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}, the UE shall update the first/second indicated joint/DL/UL TCI state(s) according to the first/second joint/DL/UL TCI state(s) in the subset and keep other indicated first/second joint/DL/UL TCI state(s) that is not updated by the received TCI codepoint

According to the agreement, dynamic switching between S-TRP operation and S-DCI based MTRP operation by legacy TCI field is not supported. It means that the S-TRP operation or S-DCI based TRP operation will be decided by RRC or MAC CE. In our point of view, we prefer to decide it by MAC CE. If there is at least one codepoint maps to more than one indicated joint/DL/UL TCI state activated by MAC CE, it means the S-DCI based MTRP operation. And in order to avoid the case that there is no other TCI state to keep when a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 indicates only one indicated joint/DL/UL TCI state, we suggest the default rule that UE applies the indicated joint/DL/UL TCI states of the lowest codepoint among codepoints mapping with more than one indicated joint/DL/UL TCI state activated by MAC CE after the application time of the MAC CE activation command.

Proposal 2-1: UE applies the indicated joint/DL/UL TCI states of the lowest codepoint among codepoints mapping with more than one indicated joint/DL/UL TCI state activated by MAC CE after the application time of the MAC CE activation command which activate more than one indicated joint/DL/UL TCI state for at least one codepoint.

Default TCI state for AP CSI-RS resource

As for the default TCI state for AP CSI-RS resource in S-DCI based MTRP scenario, the agreements were archived in RAN1-114 meeting [4] and RAN1-114bis meeting [5].

Agreement

On unified TCI framework extension for S-DCI based MTRP, if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of AP CSI-RS resources in an AP CSI-RS resource set for BM/CSI is smaller than a threshold for AP CSI-RS reception:

- FFS: If there is any other DL signal in the same symbols as the AP CSI-RS

- If there is no DL signal in the same symbols as the AP CSI-RS:

  - If the UE is in FR1 or the UE supports the capability of two default beams for S-DCI based MTRP in FR2, the UE shall apply the first or the second indicated joint/DL TCI state to the AP CSI-RS according to the RRC configuration(s) provided to the AP CSI-RS resource or AP CSI-RS resource set

    - Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold.

  - Otherwise, the UE shall apply the first indicated joint/DL TCI state to the AP CSI-RS.

- FFS: The definition of other DL signals

Note: Whether to reuse the legacy UE capability (beamSwitchTiming/beamSwitchTiming-r16) as the threshold for AP CSI-RS reception is discussed in Rel-18 UE feature AI

Agreement

On unified TCI framework extension for M-DCI based MTRP, if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of AP CSI-RS resources in an AP CSI-RS resource set for BM/CSI is smaller than a threshold for AP CSI-RS reception:

- If there is no other DL signal in the same symbols as the AP CSI-RS:

  - If the UE is in FR1 or the UE supports the capability of default beam per coresetPoolIndex for M-DCI based MTRP in FR2:

    - Alt1: The UE shall apply the first or the second indicated joint/DL TCI state to the AP CSI-RS according to the RRC configuration(s) provided to the AP CSI-RS resources or AP CSI-RS resource set

    - Note: If the UE supports the capability of two default beam per coresetPoolIndex for M-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold.

  - Otherwise, the UE shall apply the indicated joint/DL TCI state specific to coresetPoolIndex value 0 to the AP CSI-RS resource set.

- FFS: If there is any other DL signal in the same symbols as the AP CSI-RS

- FFS: The definition of other DL signals

- Note: Whether to reuse the legacy UE capability (beamSwitchTiming/beamSwitchTiming-r16) as the threshold for AP CSI-RS reception is discussed in Rel-18 UE feature AI

Agreement

On unified TCI framework extension, if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of AP CSI-RS for BM/CSI is smaller than a threshold for AP CSI-RS reception:

  - If there is other DL signal in the same symbols as the AP CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the AP CSI-RS

  - For S-DCI based MTRP operation, if there is a PDSCH applying two indicated TCI states in the same symbols as the AP CSI-RS, the UE applies the first indicated TCI state of the two indicated TCI states when receiving the AP CSI-RS the first or the second indicated joint/DL TCI state to the AP CSI-RS according to the higher layer configuration(s) provided to the AP CSI-RS resource or to the aperiodic CSI-RS resource set.

  - Rel-17 definitions of “other DL signal” specified in TS 38.214 for AP CSI-RS reception in S-DCI based MTRP operations and M-DCI based MTPR operation are retained

For S-DCI based MTRP, the above agreement only covers the case that AP CSI-RS was configured of followunifiedTCI-state. But for AP CSI-RS not configured of followunifiedTCI-state, a default rule is also needed to define the default beam. We propose to apply the first indicated joint/DL TCI state to the AP CSI-RS.

Proposal 2-2: On unified TCI framework extension, if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of AP CSI-RS for BM/CSI is smaller than a threshold for AP CSI-RS reception:

  - If there is other DL signal in the same symbols as the AP CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the AP CSI-RS

  - For S-DCI based MTRP operation, if there is a PDSCH applying two indicated TCI states in the same symbols as the AP CSI-RS, the UE applies the first indicated TCI state of the two indicated joint/DL TCI states when receiving the AP CSI-RS not configured of followunifiedTCI-state. And the following TP#1 should be supported.

TP#1 to TS 38.214 V18.1.0 Section 5.2.1.5.1: for S-DCI based MTRP

- 5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology

<omitted text>

When a UE is configured with dl-OrJointTCI-StateList and is having two indicated TCI states and if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is smaller than a threshold:

-	If there is no DL signal in the same symbols as the aperiodic CSI-RS

-	if the UE is in frequency range 1, or the UE reports its capability of [two default beams for S-DCI based MTRP] in frequency range 2, the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS according to the higher layer configuration(s) provided to the aperiodic CSI-RS resource or to the aperiodic CSI-RS resource set, if provided.

-	Otherwise, the UE shall apply the first indicated joint/DL TCI state to the aperiodic CSI-RS.

-	else if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], periodic CSI-RS, semi-persistent CSI-RS, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48}∙2max(0, μCSIRS-3) and when enableBeamSwitchTiming is not provided or the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter trs-Info, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'off' or configured without the higher layer parameters repetition and trs-Info scheduled with offset larger than or equal to 48∙2max(0,μCSIRS-3) when the UE provides beamSwitchTiming-r16 and enableBeamSwitchTiming is provided, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on' scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming-r16 and enableBeamSwitchTiming is provided. If there is a PDSCH applying two indicated joint/DL TCI states in the same symbols as the AP CSI-RS, the UE applies the first or the second indicated joint/DL TCI state to the AP CSI-RS according to the higher layer configuration(s) provided to the AP CSI-RS resource or to the aperiodic CSI-RS resource set, if provided; otherwise, the UE applies the first indicated joint/DL TCI state to the AP CSI-RS.

<omitted text>

For M-DCI based MTRP, it is also possible that the other DL signals contain PDSCHs scheduled by different CORESETs with different CORESETPoolIndex if UE support fully/partially-overlapped PDSCHs in time domain. In this case, UE will apply the QCL assumption of which other DL signal is a problem. In our understanding, if the AP CSI-RS is configured of followunifiedTCI-state, the UE applies the first or the second indicated joint/DL TCI state to the AP CSI-RS according to the higher layer configuration(s) provided to the AP CSI-RS resource or to the aperiodic CSI-RS resource set if a UE is configured with enableDefaultTCI-StatePerCoresetPoolIndex. Else, the UE applies the first indicated joint/DL TCI state to the AP CSI-RS.

If a UE is not configured with enableDefaultTCI-StatePerCoresetPoolIndex, it needs to restrict that the other DL signal should be the DL signal scheduled by a PDCCH associated with the same coresetPoolIndex as the PDCCH triggering the aperiodic CSI-RS. Thus the following TP should be supported based on the latest version of TS 38.214 i10 in [6].

Proposal 2-3: On unified TCI framework extension, if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of AP CSI-RS for BM/CSI is smaller than a threshold for AP CSI-RS reception:

  - If there is other DL signal in the same symbols as the AP CSI-RS,

  - For M-DCI based MTRP operation, the following TP#2 should be supported.

    - if a UE is configured with enableDefaultTCI-StatePerCoresetPoolIndex or if the other DL signals refer to two DL signals scheduled by PDCCH associated to different CORESETs having different values of CORESETPoolIndex, the UE applies the first or the second indicated joint/DL TCI state to the AP CSI-RS configured of followunifiedTCI-state according to the higher layer configuration(s) provided to the AP CSI-RS resource or to the aperiodic CSI-RS resource set, if provided. otherwise, the UE applies the first indicated joint/DL TCI state to the AP CSI-RS.

    - Else, the other DL signal refers to PDSCH scheduled by a PDCCH associated with the same coresetPoolIndex as the PDCCH triggering the aperiodic CSI-RS.

TP#2 to TS 38.214 V18.1.0 Section 5.2.1.5.1: for M-DCI based MTRP

- 5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology

<omitted text>

When a UE is configured with dl-OrJointTCI-StateList, is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in different ControlResourceSets, is having two indicated TCI states and if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is smaller than a threshold:

-	If there is no DL signal in the same symbols as the aperiodic CSI-RS

-	if the UE is in frequency range 1, or the UE reports its capability of [default beam per coresetPoolIndex for M-DCI based MTRP] in frequency range 2, the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS according to the higher layer configuration(s) provided to the aperiodic CSI-RS resource or aperiodic CSI-RS resource set, if provided.

-	otherwise, the UE shall apply the indicated joint/DL TCI state specific to coresetPoolIndex value 0 to the aperiodic CSI-RS resource set

-	else if there is any other DL signal(s) with an indicated TCI state in the same symbols as the CSI-RS,

-	if a UE is configured with enableDefaultTCI-StatePerCoresetPoolIndex, or if the other DL signal(s) refer to two DL signals scheduled by PDCCH associated to different CORESETs having different values of CORESETPoolIndex, the UE applies the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS according to the higher layer configuration(s) provided to the aperiodic CSI-RS resource or aperiodic CSI-RS resource set, if provided; otherwise, the UE applies the indicated joint/DL TCI state specific to coresetPoolIndex value 0 to the aperiodic CSI-RS resource. The other DL signal(s) refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], periodic CSI-RS, semi-persistent CSI-RS, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48}∙2max(0,μCSIRS-3) and when enableBeamSwitchTiming is not provided or the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter trs-Info, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'off' or configured without the higher layer parameters repetition and trs-Info scheduled with offset larger than or equal to 48∙2max(0,μCSIRS-3) when the UE provides beamSwitchTiming-r16 and enableBeamSwitchTiming is provided, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on' scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming-r16 and enableBeamSwitchTiming is provided.

-	else the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled by a PDCCH associated with the same coresetPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS triggered by a PDCCH associated with the same coresetPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48}∙2max(0, μCSIRS-3) and enableBeamSwitchTiming is not provided, aperiodic CSI-RS triggered by a PDCCH associated with the same coresetPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to 48∙2max(0, μCSIRS-3) when the reported value of beamSwitchTiming-r16 is one of the values {224, 336}∙2max(0, μCSIRS-3) and enableBeamSwitchTiming is provided, periodic CSI-RS, semi-persistent CSI-RS.

<omitted text>

CSI enhancement

For enhanced Type II codebook for CJT, UE could be configured with NTRP∈{1,2,3,4} CSI-RS resources in a resource set for channel measurement. Each CSI-RS resource includes  PCSI-RS=2N1N2 ports. The values of N1, N2 are the same for all NTRP CSI-RS resources, and they are configured by the higher layer parameter CBSR-r18 in TS 38.331 [7].

Regarding CBSR, RAN1 has agreed that one of the NTRP configured CSI-RS resources must be configured with CBSR, while the remaining (NTRP –1) configured CSI-RS resources can be optionally configured. If the  NTRP TRPs for CJT is not quasi co-location, the configured CBSRs should be different. In TS 38.331 [7], the higher parameter CBSR-r18 is configured per CSI-RS resource, and CBSR list is used to configure the CBSRs of multiple CSI-RS resources, as shown in the following with highlight. However, such configuration method leads UE does not know the configured CBSR corresponding to which CSI-RS resource.

CodebookConfig-r18  ::=                   SEQUENCE {

codebookType                              CHOICE {

type2                                     SEQUENCE {

typeII-CJT-r18                            SEQUENCE {

n1-n2-codebookSubsetRestrictionList-r18   SEQUENCE (SIZE (1..4)) OF CBSR-r18,

paramCombination-CJT-r18                  INTEGER (1..7),

paramCombination-CJT-L-r18                SEQUENCE (SIZE (1..4)) OF INTEGER (1..5)     OPTIONAL,  -- Need R

restrictedCMR-Selection-r18               ENUMERATED {enable}                          OPTIONAL,  -- Need R

valueOfO3-r18                             ENUMERATED {n1, n4}                          OPTIONAL,  -- Need R

numberOfPMI-SubbandsPerCQI-Subband-r18    INTEGER(1..2)                                OPTIONAL,  -- Need R

typeII-RI-Restriction-r18                 BIT STRING (SIZE (4)),

codebookMode-r18                          INTEGER (1..2)                               OPTIONAL   -- Need R

},

CBSR-r18 ::=    CHOICE {

two-one                                BIT STRING (SIZE (8)),

two-two                                BIT STRING (SIZE (27)),

four-one                               BIT STRING (SIZE (16)),

three-two                              BIT STRING (SIZE (35)),

six-one                                BIT STRING (SIZE (24)),

four-two                               BIT STRING (SIZE (43)),

eight-one                              BIT STRING (SIZE (32)),

four-three                             BIT STRING (SIZE (59)),

six-two                                BIT STRING (SIZE (59)),

twelve-one                             BIT STRING (SIZE (48)),

four-four                              BIT STRING (SIZE (75)),

eight-two                              BIT STRING (SIZE (75)),

sixteen-one                            BIT STRING (SIZE (64))

}

Observation 3-1: When different CBSRs are configured via a list, UE does not know the configured CBSRs corresponding to which CSI-RS resources.

Assume each resource is configured with CBSR. We can define that the CBSRs are configured according to the order of CSI-RS resource ID.  For example, the smallest CSI-RS resource ID corresponds to the first CBSR, the second smallest CSI-RS corresponds to the second CBSR, and so forth.  Figure 1 shows the illustration of mapping between CBSRs and CSI-RS resources. In the figure, NTRP=4 CSI-RS resources and NCBSR= 4 CBSRs are respectively configured by higher layer parameter. The first CBSR to the fourth CBSR are mapped from least significant bit (LSB) to most significant bit (MSB). Such definition method does not require additional signaling overhead to indicate the mapping between CBSR and CSI-RS resource.

Figure 1: Illustration of mapping between CBSRs and CSI-RS resources

If the number of CBSRs is less than that of CSI-RS resources, i.e., NCBSR<NTRP, a bitmap with size NTRP bits could indicate which CSI-RS resources are configured with CBSRs. E.g., the bit ‘0’ in the bitmap indicates the corresponding resource is not configured with CBSR. Otherwise, the corresponding resource is configured with CBSR.

Proposal 3-1: When the number of configurated CBSRs is less than that of CSI-RS resources, a bitmap with size NTRP should be adopted to indicate which CSI-RS resources configured with CBSRs. If each CSI-RS resource is configured with CBSR, such bitmap could be omitted by defining that CBSRs are configured in order of CSI-RS resource ID.

The following agreement on Type II codebook for CJT has been achieved in [8]. This agreement has been captured in enhanced Type II codebook for CJT. But it has not been captured in further enhanced Type II port selection codebook for CJT.

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP with NTRP>1 TRP/TRP-groups, the following is supported:

- The CMR comprises K>1 NZP CSI-RS resources, where one resource corresponds to one TRP/TRP-group (i.e. K=NTRP)

  - Each of the CSI-RS resources has a same number of CSI-RS ports

- Note: The terms TRP and TRP-group are used for discussion purposes only (no spec impact is implied).

In addition, for enhanced Type II codebook for CJT, it does not provide which higher layer parameter is adopted to configure N1 and N2 in current specification. Note that  O1 and O2 does not need to be configured by the higher layer parameter. They depend on the configuration of N1 and N2.

In TS 38.331 [7], the higher layer parameter for configuration of (N1, N2) is named CBSR-r18. However, the higher layer parameter n1-n2‑codebookSubsetRestriction-Doppler-r18 is still adopted in enhanced Type II codebook for predicted PMI.

Proposal 3-2: Adopt the following TPs on enhanced Type II codebook for CJT, further enhanced Type II port selection codebook for CJT and enhanced Type II codebook for predicted PMI to TS 38.214.

- Reason for change: For enhanced type II codebook for CJT, it  should clarify that only the values of N1, N2 needs to be configured by higher layer parameter while O1 and O2 do not require configuration. For further enhanced Type II port selection codebook for CJT, we have agreed that  each of the CSI-RS resources has a same number of CSI-RS ports. However, such agreement has not captured in current specification. For enhanced Type II codebook for predicted PMI, the higher layer parameter on the configuration of  N1,N2 is not consistent with the description in TS 38.331.

- Summary of change: Clarify that the values of N1 and N2  needs to be configured by higher layer parameter CBSR-r18, and each of the CSI-RS resources should have a same number of CSI-RS ports.

- Consequences if not approved: The current description makes people misunderstand the values of  O1, O2 should be configured by higher layer parameter. It is not clear whether each of the CSI-RS resources should have a same number of CSI-RS ports. The values of N1 and N2 could not be  configured with higher layer parameter CBSR-r18.

- 5.2.2.2.8 Enhanced Type II codebook for CJT ------------------------------------------Start of Text Proposal ---------------------------------- <Unchanged text is omitted> For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031} per CSI-RS resource, and the UE configured with NTRP∈{1,2,3,4} CSI-RS resources in a resource set for channel measurement and with higher layer parameter codebookType set to 'typeII-CJT-r18' -the values of N1, N2 and O1, O2 are the same for all NTRP CSI-RS resources and the values of N1, N2 are configured with the higher layer parameter CBSR-r18. The supported configurations of N1,N2 for a given number of CSI-RS ports and the corresponding values of O1,O2 are given in Table 5.2.2.2.1-2. The number of CSI-RS ports, PCSI-RS, is 2N1N2 for each of the NTRP CSI-RS resources. < Unchanged parts are omitted > The bitmap parameter CBSR-r18 is configured per CSI-RS resource and for at least one of the NTRP CSI-RS resources, and it is configured as described in Clause 5.2.2.2.5, where only the bit values '00' or '11' of Table 5.2.2.2.5-6 are configurable. If parameter CBSR-r18 is not configured for a CSI-RS resource, no restriction is applied to the selection of vectors vm1,ji,m2,j(i) corresponding to that resource. < Unchanged parts are omitted > - 5.2.2.2.9 Further enhanced Type II port selection codebook for CJT For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031} per CSI-RS resource, the UE configured with NTRP∈{1,2,3,4} CSI-RS resources in a resource set for channel measurement and with higher layer parameter codebookType set to 'typeII-CJT-PortSelection-r18' - the number of CSI-RS ports for each CSI-RS resource, PCSI-RS, is configured as in clause 5.2.2.2.4, each of the CSI-RS resources has a same number of CSI-RS ports. < Unchanged parts are omitted > - 5.2.2.2.10 Enhanced Type II codebook for predicted PMI For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031} per CSI-RS resource, the UE configured with K∈{4,8,12} aperiodic CSI-RS resources or with a single periodic or semi-persistent CSI-RS resource in the resource set for channel measurement and with codebookType set to 'typeII-Doppler-r18' - The values of N1 and N2 are configured with the higher layer parameter CBSR-r18. The supported configurations of N1,N2 for a given number of CSI-RS ports and the corresponding values of O1,O2 are given in Table 5.2.2.2.1-2. The number of CSI-RS ports, PCSI-RS, is 2N1N2. < Unchanged parts are omitted > The bitmap parameter CBSR-r18 is configured as described in Clause 5.2.2.2.5, where only the bit values '00' or '11' of Table 5.2.2.2.5-6 are configurable. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

In [9], the following agreement on TDCP reporting was achieved.

Agreement

For the Rel-18 TRS-based TDCP reporting, adopt the following TP for TS 38.214:

Reason for change: There is no agreement to support KTRS=1 with aperiodic TRS regardless of the interpretation of the previous conclusions (below)

[113] Conclusion

For the Rel-18 TRS-based TDCP reporting, for TDCP measurement and calculation, there is no consensus on supporting the following: joint use of P and AP-TRS resource sets for TDCP measurement and calculation is supported at least for Y=1 as a UE-optional feature

[114] Conclusion:

For the Rel-18 TRS-based TDCP reporting, there is no consensus on supporting the following proposals:

- additional D value(s)

TRS resource configuration where all the configured KTRS resource sets are aperiodic

The agreement shows that aperiodic TRS resource set is not supported for TDCP reporting. However, in current specification, “A UE does not expect to be configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to other than 'tdcp' or 'none' for aperiodic NZP CSI-RS resource set configured with trs-Info” is described for TDCP reporting. This implies that a UE can still expect to be configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'tdcp' for aperiodic TRS resource set, which against the above agreement.

Proposal 3-3: Adopt the following TP on TDCP reporting for aperiodic TRS resource set in section 5.1.6.1.1 of TS 38.214.

- Reason for change:  TDCP reporting is not supported for aperiodic TRS resources set.  A UE  should not expect to be configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'tdcp' for aperiodic NZP CSI-RS resource set configured with trs-Info.

- Summary of change: Clarify the CSI-ReportConfig with the higher layer parameter reportQuantity set to 'tdcp' is not supported for aperiodic TRS resource set.

- Consequences if not approved: The agreement on TDCP reporting is not correctly captured in current specification.

- 5.1.6.1.1 CSI-RS for tracking ------------------------------------------Start of Text Proposal ---------------------------------- <Unchanged text is omitted> A UE in RRC connected mode is expected to receive the higher layer UE specific configuration of a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. For a NZP-CSI-RS-ResourceSet configured with the higher layer parameter trs-Info, the UE shall assume the antenna port with the same port index of the configured NZP CSI-RS resources in the NZP-CSI-RS-ResourceSet is the same. - For frequency range 1, the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. If no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated, then the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot. - For frequency range 2 the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of two periodic CSI-RS resources in one slot or with a NZP-CSI-RS-ResourceSet of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. A UE configured with NZP-CSI-RS-ResourceSet(s) configured with higher layer parameter trs-Info may have the CSI-RS resources configured as: - Periodic, with the CSI-RS resources in the NZP-CSI-RS-ResourceSet configured with same periodicity, bandwidth and subcarrier location. - Periodic CSI-RS resource in one set and aperiodic CSI-RS resources in a second set, with the aperiodic CSI-RS and periodic CSI-RS resource having the same bandwidth (with same RB location) and the aperiodic CSI-RS being configured with qcl-Type set to 'typeA' and 'typeD', where applicable, with the periodic CSI-RS resources. For frequency range 2, the UE does not expect that the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is smaller than beamSwitchTiming + d ∙2μCSIRS/2μPDCCH in CSI-RS symbols, where beamSwitchTiming is UE reported value defined in [13, TS 38.306], the reported value is one of the values of {14, 28, 48}∙2max(0, μCSIRS-3), and the beam switching timing delay d is defined in Table 5.2.1.5.1a-1 if µPDCCH < µCSIRS , else d is zero. The UE shall expect that the periodic CSI-RS resource set and aperiodic CSI-RS resource set are configured with the same number of CSI-RS resources and with the same number of CSI-RS resources in a slot. For the aperiodic CSI-RS resource set if triggered, and if the associated periodic CSI-RS resource set is configured with four periodic CSI-RS resources with two consecutive slots with two periodic CSI-RS resources in each slot, the higher layer parameter aperiodicTriggeringOffset indicates the triggering offset for the first slot for the first two CSI-RS resources in the set. A UE does not expect to be configured with a CSI-ReportConfig that is linked to a CSI-ResourceConfig containing an NZP-CSI-RS-ResourceSet configured with trs-Info and with the CSI-ReportConfig configured with the higher layer parameter timeRestrictionForChannelMeasurements set to 'configured'. A UE does not expect to be configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to other than 'none' for aperiodic NZP CSI-RS resource set configured with trs-Info. -< Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Conclusion

In this contribution, we discuss about the remaining issues on NR MIMO Evolution for Downlink and Uplink. Based on above discusses, we provide the following proposals.

Unified TCI framework extension for multi-TRP

Proposal 2-1: UE applies the indicated joint/DL/UL TCI states of the lowest codepoint among codepoints mapping with more than one indicated joint/DL/UL TCI state activated by MAC CE after the application time of the MAC CE activation command which activate more than one indicated joint/DL/UL TCI state for at least one codepoint.

Proposal 2-2: On unified TCI framework extension, if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of AP CSI-RS for BM/CSI is smaller than a threshold for AP CSI-RS reception:

  - If there is other DL signal in the same symbols as the AP CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the AP CSI-RS

  - For S-DCI based MTRP operation, if there is a PDSCH applying two indicated TCI states in the same symbols as the AP CSI-RS, the UE applies the first indicated TCI state of the two indicated joint/DL TCI states when receiving the AP CSI-RS not configured of followunifiedTCI-state. And the following TP#1 should be supported.

TP#1 to TS 38.214 V18.1.0 Section 5.2.1.5.1: for S-DCI based MTRP

- 5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology

<omitted text>

When a UE is configured with dl-OrJointTCI-StateList and is having two indicated TCI states and if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is smaller than a threshold:

-	If there is no DL signal in the same symbols as the aperiodic CSI-RS

-	if the UE is in frequency range 1, or the UE reports its capability of [two default beams for S-DCI based MTRP] in frequency range 2, the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS according to the higher layer configuration(s) provided to the aperiodic CSI-RS resource or to the aperiodic CSI-RS resource set, if provided.

-	Otherwise, the UE shall apply the first indicated joint/DL TCI state to the aperiodic CSI-RS.

-	else if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], periodic CSI-RS, semi-persistent CSI-RS, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48}∙2max(0, μCSIRS-3) and when enableBeamSwitchTiming is not provided or the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter trs-Info, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'off' or configured without the higher layer parameters repetition and trs-Info scheduled with offset larger than or equal to 48∙2max(0,μCSIRS-3) when the UE provides beamSwitchTiming-r16 and enableBeamSwitchTiming is provided, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on' scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming-r16 and enableBeamSwitchTiming is provided. If there is a PDSCH applying two indicated joint/DL TCI states in the same symbols as the AP CSI-RS, the UE applies the first or the second indicated joint/DL TCI state to the AP CSI-RS according to the higher layer configuration(s) provided to the AP CSI-RS resource or to the aperiodic CSI-RS resource set, if provided; otherwise, the UE applies the first indicated joint/DL TCI state to the AP CSI-RS.

<omitted text>

Proposal 2-3: On unified TCI framework extension, if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of AP CSI-RS for BM/CSI is smaller than a threshold for AP CSI-RS reception:

  - If there is other DL signal in the same symbols as the AP CSI-RS,

  - For M-DCI based MTRP operation, the following TP#2 should be supported.

    - if a UE is configured with enableDefaultTCI-StatePerCoresetPoolIndex or if the other DL signals refer to two DL signals scheduled by PDCCH associated to different CORESETs having different values of CORESETPoolIndex, the UE applies the first or the second indicated joint/DL TCI state to the AP CSI-RS configured of followunifiedTCI-state according to the higher layer configuration(s) provided to the AP CSI-RS resource or to the aperiodic CSI-RS resource set, if provided. otherwise, the UE applies the first indicated joint/DL TCI state to the AP CSI-RS.

    - Else, the other DL signal refers to PDSCH scheduled by a PDCCH associated with the same coresetPoolIndex as the PDCCH triggering the aperiodic CSI-RS.

TP#2 to TS 38.214 V18.1.0 Section 5.2.1.5.1: for M-DCI based MTRP

- 5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology

<omitted text>

When a UE is configured with dl-OrJointTCI-StateList, is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in different ControlResourceSets, is having two indicated TCI states and if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is smaller than a threshold:

-	If there is no DL signal in the same symbols as the aperiodic CSI-RS

-	if the UE is in frequency range 1, or the UE reports its capability of [default beam per coresetPoolIndex for M-DCI based MTRP] in frequency range 2, the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS according to the higher layer configuration(s) provided to the aperiodic CSI-RS resource or aperiodic CSI-RS resource set, if provided.

-	otherwise, the UE shall apply the indicated joint/DL TCI state specific to coresetPoolIndex value 0 to the aperiodic CSI-RS resource set

-	else if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS,

-	if a UE is configured with enableDefaultTCI-StatePerCoresetPoolIndex, or if the other DL signal(s) refer to two DL signals scheduled by PDCCH associated to different CORESETs having different values of CORESETPoolIndex, the UE applies the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS according to the higher layer configuration(s) provided to the aperiodic CSI-RS resource or aperiodic CSI-RS resource set, if provided; otherwise, the UE applies the indicated joint/DL TCI state specific to coresetPoolIndex value 0 to the aperiodic CSI-RS resource. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], periodic CSI-RS, semi-persistent CSI-RS, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48}∙2max(0,μCSIRS-3) and when enableBeamSwitchTiming is not provided or the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter trs-Info, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'off' or configured without the higher layer parameters repetition and trs-Info scheduled with offset larger than or equal to 48∙2max(0,μCSIRS-3) when the UE provides beamSwitchTiming-r16 and enableBeamSwitchTiming is provided, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on' scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming-r16 and enableBeamSwitchTiming is provided.

-	else the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled by a PDCCH associated with the same coresetPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS triggered by a PDCCH associated with the same coresetPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48}∙2max(0, μCSIRS-3) and enableBeamSwitchTiming is not provided, aperiodic CSI-RS triggered by a PDCCH associated with the same coresetPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to 48∙2max(0, μCSIRS-3) when the reported value of beamSwitchTiming-r16 is one of the values {224, 336}∙2max(0, μCSIRS-3) and enableBeamSwitchTiming is provided, periodic CSI-RS, semi-persistent CSI-RS.

<omitted text>

CSI enhancement

Observation 3-1: When different CBSRs are configured via a list, UE does not know the configured CBSRs corresponding to which CSI-RS resources.

Proposal 3-1: When the number of configurated CBSRs is less than that of CSI-RS resources, a bitmap with size NTRP should be adopted to indicate which CSI-RS resources configured with CBSRs. If each CSI-RS resource is configured with CBSR, such bitmap could be omitted by defining that CBSRs are configured in order of CSI-RS resource ID.

Proposal 3-2: Adopt the following TPs on enhanced Type II codebook for CJT, further enhanced Type II port selection codebook for CJT and enhanced Type II codebook for predicted PMI to TS 38.214.

- Reason for change: For enhanced type II codebook for CJT, it  should clarify that only the values of N1, N2 needs to be configured by higher layer parameter while O1 and O2 do not require configuration. For further enhanced Type II port selection codebook for CJT, we have agreed that  each of the CSI-RS resources has a same number of CSI-RS ports. However, such agreement has not captured in current specification. For enhanced Type II codebook for predicted PMI, the higher layer parameter on the configuration of  N1,N2 is not consistent with the description in TS 38.331.

- Summary of change: Clarify that the values of N1 and N2  needs to be configured by higher layer parameter CBSR-r18, and each of the CSI-RS resources should have a same number of CSI-RS ports.

- Consequences if not approved: The current description makes people misunderstand the values of  O1, O2 should be configured by higher layer parameter. It is not clear whether each of the CSI-RS resources should have a same number of CSI-RS ports. The values of N1 and N2 could not be  configured with higher layer parameter CBSR-r18.

- 5.2.2.2.8 Enhanced Type II codebook for CJT ------------------------------------------Start of Text Proposal ---------------------------------- <Unchanged text is omitted> For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031} per CSI-RS resource, and the UE configured with NTRP∈{1,2,3,4} CSI-RS resources in a resource set for channel measurement and with higher layer parameter codebookType set to 'typeII-CJT-r18' -the values of N1, N2 and O1, O2 are the same for all NTRP CSI-RS resources and the values of N1, N2 are configured with the higher layer parameter CBSR-r18. The supported configurations of N1,N2 for a given number of CSI-RS ports and the corresponding values of O1,O2 are given in Table 5.2.2.2.1-2. The number of CSI-RS ports, PCSI-RS, is 2N1N2 for each of the NTRP CSI-RS resources. < Unchanged parts are omitted > The bitmap parameter CBSR-r18 is configured per CSI-RS resource and for at least one of the NTRP CSI-RS resources, and it is configured as described in Clause 5.2.2.2.5, where only the bit values '00' or '11' of Table 5.2.2.2.5-6 are configurable. If parameter CBSR-r18 is not configured for a CSI-RS resource, no restriction is applied to the selection of vectors vm1,ji,m2,j(i) corresponding to that resource. < Unchanged parts are omitted > - 5.2.2.2.9 Further enhanced Type II port selection codebook for CJT For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031} per CSI-RS resource, the UE configured with NTRP∈{1,2,3,4} CSI-RS resources in a resource set for channel measurement and with higher layer parameter codebookType set to 'typeII-CJT-PortSelection-r18' - the number of CSI-RS ports for each CSI-RS resource, PCSI-RS, is configured as in clause 5.2.2.2.4, each of the CSI-RS resources has a same number of CSI-RS ports. < Unchanged parts are omitted > - 5.2.2.2.10 Enhanced Type II codebook for predicted PMI For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031} per CSI-RS resource, the UE configured with K∈{4,8,12} aperiodic CSI-RS resources or with a single periodic or semi-persistent CSI-RS resource in the resource set for channel measurement and with codebookType set to 'typeII-Doppler-r18' - The values of N1 and N2 are configured with the higher layer parameter CBSR-r18. The supported configurations of N1,N2 for a given number of CSI-RS ports and the corresponding values of O1,O2 are given in Table 5.2.2.2.1-2. The number of CSI-RS ports, PCSI-RS, is 2N1N2. < Unchanged parts are omitted > The bitmap parameter CBSR-r18 is configured as described in Clause 5.2.2.2.5, where only the bit values '00' or '11' of Table 5.2.2.2.5-6 are configurable. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Proposal 3-3: Adopt the following TP on TDCP reporting for aperiodic TRS resource set in section 5.1.6.1.1 of TS 38.214.

- Reason for change:  TDCP reporting is not supported for aperiodic TRS resources set.  A UE  should not expect to be configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'tdcp' for aperiodic NZP CSI-RS resource set configured with trs-Info.

- Summary of change: Clarify the CSI-ReportConfig with the higher layer parameter reportQuantity set to 'tdcp' is not supported for aperiodic TRS resource set.

- Consequences if not approved: The agreement on TDCP reporting is not correctly captured in current specification.

- 5.1.6.1.1 CSI-RS for tracking ------------------------------------------Start of Text Proposal ---------------------------------- <Unchanged text is omitted> A UE in RRC connected mode is expected to receive the higher layer UE specific configuration of a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. For a NZP-CSI-RS-ResourceSet configured with the higher layer parameter trs-Info, the UE shall assume the antenna port with the same port index of the configured NZP CSI-RS resources in the NZP-CSI-RS-ResourceSet is the same. - For frequency range 1, the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. If no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated, then the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot. - For frequency range 2 the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of two periodic CSI-RS resources in one slot or with a NZP-CSI-RS-ResourceSet of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. A UE configured with NZP-CSI-RS-ResourceSet(s) configured with higher layer parameter trs-Info may have the CSI-RS resources configured as: - Periodic, with the CSI-RS resources in the NZP-CSI-RS-ResourceSet configured with same periodicity, bandwidth and subcarrier location. - Periodic CSI-RS resource in one set and aperiodic CSI-RS resources in a second set, with the aperiodic CSI-RS and periodic CSI-RS resource having the same bandwidth (with same RB location) and the aperiodic CSI-RS being configured with qcl-Type set to 'typeA' and 'typeD', where applicable, with the periodic CSI-RS resources. For frequency range 2, the UE does not expect that the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is smaller than beamSwitchTiming + d ∙2μCSIRS/2μPDCCH in CSI-RS symbols, where beamSwitchTiming is UE reported value defined in [13, TS 38.306], the reported value is one of the values of {14, 28, 48}∙2max(0, μCSIRS-3), and the beam switching timing delay d is defined in Table 5.2.1.5.1a-1 if µPDCCH < µCSIRS , else d is zero. The UE shall expect that the periodic CSI-RS resource set and aperiodic CSI-RS resource set are configured with the same number of CSI-RS resources and with the same number of CSI-RS resources in a slot. For the aperiodic CSI-RS resource set if triggered, and if the associated periodic CSI-RS resource set is configured with four periodic CSI-RS resources with two consecutive slots with two periodic CSI-RS resources in each slot, the higher layer parameter aperiodicTriggeringOffset indicates the triggering offset for the first slot for the first two CSI-RS resources in the set. A UE does not expect to be configured with a CSI-ReportConfig that is linked to a CSI-ResourceConfig containing an NZP-CSI-RS-ResourceSet configured with trs-Info and with the CSI-ReportConfig configured with the higher layer parameter timeRestrictionForChannelMeasurements set to 'configured'. A UE does not expect to be configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to other than 'none' for aperiodic NZP CSI-RS resource set configured with trs-Info. -< Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink

- RAN1-111 meeting Chairman notes.

- RAN1-112b e-meeting Chairman notes.

- RAN1-114 meeting Chairman notes.

- RAN1-114bis meeting Chairman notes.

- 3GPP TS 38.214 V18.1.0 (2023-12)

- 3GPP TS 38.331 V18.0.0 (2023-12)

- RAN1-110 meeting Chairman notes.

- RAN1-115 meeting Chairman notes.