3GPP TSG RAN WG1 #116														   R1-2400532

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	8.10

Source:	           Xiaomi

Title:	Discussion on remaining issues in NR NTN enhancements

Document for:	Decision

- Introduction

In this contribution, we present our views on the remaining issues in NR NTN enhancements.

- Common PUCCH repetition during handover and TA timer expiry.

- DMRS bundling related issues, e.g., how to capture the phase pre-compensation into specification.

- Discussion

  - PUCCH repetition

In RAN1#115 meeting[1], the following TP was agreed for PUCCH repetition to allow the same repetition number can be applied to any PUCCH transmission before dedicated PUCCH resource configuration is provided.

Agreement

The TP below is endorsed for TS38.213

- Reason for change: It was agreed that this WI can support PUCCH repetition not only for Msg4 HARQ-ACK but also for subsequent PUCCH e.g., to report HARQ-ACK corresponding to a PDSCH conveying RRC configuration parameters.

- Summary of change: It is clarified that the indicated repetition factor is applied to any PUCCH transmission by using common PUCCH resource.

- Consequences if not approved: It is unclear which repetition factor is applied to PUCCH transmissions after Msg4 HARQ-ACK transmission when dedicated PUCCH resource configuration is not provided.

9.2.6 PUCCH repetition procedure A UE that does not have dedicated PUCCH resource configuration and indicates a capability to transmit with repetitions a PUCCH with HARQ-ACK information [11, TS 38.321], determines a number of NPUCCHrepeat slots for repetitions of a PUCCH transmission with HARQ-ACK information based on an indication by numberOfPUCCHforMsg4HARQACK-RepetitionsList. If numberOfPUCCHforMsg4HARQACK-RepetitionsList provides more than one values, the DAI field in a DCI format 1_0 with CRC scrambled by a TC-RNTI scheduling a PDSCH reception that includes a UE contention resolution identity indicates NPUCCHrepeat from the more than one values. The number of NPUCCHrepeat repetitions is applied to any PUCCH transmission before dedicated PUCCH resource configuration is provided. The UE transmits each repetition of the PUCCH using frequency hopping as described in Clause 9.2.1. In the remaining of this clause, a UE without dedicated PUCCH resource configuration determines a value of a parameter, if applicable, according to Table 9.2.1-1 and/or as specified above in this clause for a PUCCH transmission with repetitions from the UE. *** Unchanged parts are omitted ***

However, there are special cases for common PUCCH repetition, which can’t be covered by the current specification.

- Case 1: If the UE is not configured with a dedicated preamble when accessing the target cell during the handover procedure, a CBRA will be triggered in which case. The DCI scheduling Msg4 is scrambled by a C-RNTI.

- Case 2: If TA timer expiry, the UE will clear all the dedicated PUCCH configuration and initiate CBRA with C-RNTI and the DCI scheduling Msg4 is scrambled by a C-RNTI.

The DCI is scrambled by C-RNTI for contention resolution in the above two cases, while the current specification only captures the case of DCI scrambled by TC-RNTI. It is not feasible for UE to reuse DAI field for repetition factor indication as DAI field is not reserved in DCI format 1_0 scrambled by C-RNTI.

Some possible solutions were proposed during last meeting for example,

- Re-interpret of the existing field, e.g., MCS field

- Always using the largest repetition factor configured via SIB

- Reusing the repetition factor indicated by DCI format scrambled by TC-RNTI

For option (1), it will reduce the scheduling flexibility, e.g., limited MCS index can be indicated if the MCS field is re-interpreted. For option (2) it may cause redundant PUCCH transmission as the largest repetition factor may not necessary. For option (3) the repetition factor may out-of-date and not appropriate for the new channel condition. Therefore, we propose a new solution that reuse the repetition factor of the last dedicated PUCCH with same PUCCH format as Msg.4 HARQ-ACK for the special cases. In that sense, no existing DCI field is re-interpreted and the repetition factor is appropriate for the current channel condition.

Proposal 1: For Msg4 HARQ-ACK scheduled by DCI format 1_0 scrambled by C-RNTI, reuse the repetition factor of the last dedicated PUCCH with the same PUCCH format as Msg4 HARQ-ACK scheduled by DCI format 1_0 scrambled by C-RNTI.

Proposal 2: The repetition factor is applied to any PUCCH transmission before dedicated PUCCH resource configuration is provided.

  - DMRS bundling

In RAN#115 meeting, the following TP for TS 38.300 was agreed:

Text Proposal for TS38.300:

<UNCHANGED TEXT IS OMITTED>

---------------------- TEXT PROPOSAL BEGIN ----------------------

16.14.X	 Support for NR NTN coverage enhancements

- NTN-specific PUSCH DMRS bundling enhancement that enables DMRS bundling in presence of timing drift, whereby UE maintains phase continuity considering effects of transmission delay variation between UE and uplink time synchronization reference point to enable improved channel estimation.

One remaining issue is how to address the UE behaviour on phase rotation compensation in RAN1 specification. As captured in TS 38.300, it is consensus that UE maintains phase continuity considering effects of transmission delay variation between UE and uplink time synchronization reference point. The following TP#1 is provided to clarify UE behaviour on phase rotation compensation in TS 38.214.

Table 1. TP#1 for clarify UE behaviour on phase rotation compensation

| Reason for change: | It is not specified that UE supporting NTN DMRS bundling enhancements for PUSCH shall maintain phase continuity taking into account phase rotation due to timing drift. |
| --- | --- |
| Summary of change: | It is specified that UE supporting NTN DMRS bundling enhancements for PUSCH shall maintain phase continuity taking into account phase rotation due to timing drift between the UE and the uplink time synchronization reference point. |
| Consequences if not approved: | The meaning of maintaining phase continuity remains unclear for UE supporting NTN DMRS bundling enhancements. |
| ----------------------------------------------------Start of TP#1 of 38.214 V18.1.0------------------------------------------ ### 6.1.7 UE procedure for determining time domain windows for bundling DM-RS < Unchanged parts are omitted > The UE shall maintain power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to frequency hopping, or in response to the use of a different SRS resource set association for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, or in response to the use of different spatial relations or different power control parameters for the two PUCCH transmissions of PUCCH repetition, or in response to any event not triggered by DCI or MAC-CE. The UE maintains power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to an event triggered by DCI other than frequency hopping or the use of a different SRS resource set association for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, or the use of different spatial relations or different power control parameters for the two PUCCH transmissions of PUCCH repetition, or in response to an event triggered by MAC-CE, subject to UE capability. of dmrs-BundlingRestart [13, TS 38.306] and when pusch-WindowRestart or pucch-WindowRestart is enabled. For a UE that supports NTN DMRS bundling enhancements for PUSCH, phase continuity shall be maintained taking into account phase rotation due to timing drift between the UE and the uplink time synchronization reference point [6, TS 38.213]. < Unchanged parts are omitted > ----------------------------------------------------End of TP#1 of 38.214 V18.1.0------------------------------------------ |  |

UE behaviour on TA compensation within actual TDW

The following working assumption were made in previous meeting [2]:

Working assumption For NTN-specific PUSCH DMRS bundling, to satisfy the phase difference limit without causing phase discontinuity, it is assumed that pre-compensation to keep phase rotation due to timing drift within the phase difference limit can be performed at UE side. - UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit. - FFS: how to determine the actual TDW - FFS: specification impact - Send an LS to RAN4

Whether UE can perform TA update within a TDW and how to capture the above working assumption into the spec. was heated discussed in last meeting. In our understanding, the TA update will cause phase change and may break the phase continuity. And based on the working assumption, the UE shall not perform TA pre-compensation update within an actual TDW if it cause phase discontinuity that may violate the phase difference limit. Therefore, the following TP#2 is provided to clarify UE behaviour on TA compensation within actual TDW.

Table 2. TP#2 for clarify UE behaviour on TA compensation within actual TDW

| Reason for change: | It is not specified that UE supporting NTN DMRS bundling enhancements for PUSCH shall avoid TA update within an actual TDW if it cause phase discontinuity that may violate the phase difference limit. |
| --- | --- |
| Summary of change: | It is specified that UE supporting NTN DMRS bundling enhancements for PUSCH shall avoid TA update within an actual TDW if it cause phase discontinuity that may violate the phase difference limit. |
| Consequences if not approved: | The UE behaviour on TA compensation within actual TDW remains unclear for UE supporting NTN DMRS bundling enhancements. |
| ----------------------------------------------------Start of TP#2 of 38.213 V18.1.0------------------------------------------ ### 4.2 Transmission timing adjustments < Unchanged parts are omitted > Using higher-layer ephemeris parameters for a serving satellite, if provided, a UE pre-compensates the two-way transmission delay on the service link based on NTA,adjUE that the UE determines using the serving satellite position and its own position. To pre-compensate the two-way transmission delay between the uplink time synchronization reference point and the serving satellite, the UE determines NTA,adjcommon [4, TS 38.211] based on one-way propagation delay Delaycommont that the UE determines as: Delaycommont= TACommon2+ TACommonDrift2×t-tepoch+TACommonDriftVariant2×t-tepoch2 where TACommon, TACommonDrift, and TACommonDriftVariant are respectively provided by ta-Common, ta-CommonDrift, and ta-CommonDriftVariant and tepoch is provided by epochTime which is the epoch time of ta-Common, ta-CommonDrift, and ta-CommonDriftVariant [12, TS 38.331]. Delaycommon(t) provides a distance at time t between the serving satellite and the uplink time synchronization reference point divided by the speed of light. The uplink time synchronization reference point is the point where DL and UL are frame aligned with an offset given by NTA,offset. The UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference if applicable according to clause 6.1.7 of [6, 38.214]. < Unchanged parts are omitted > ----------------------------------------------------End of TP#2 of 38.213 V18.1.0------------------------------------------ |  |

Proposal 3: Adopt the TP#1 to clarify UE behaviour on phase rotation compensation in TS 38.214.

Proposal 4: Adopt the TP#2 to clarify UE behaviour on TA compensation within actual TDW in TS 38.213.

- Conclusions

In this contribution, we present views on the remaining issues of NTN, we have the following observations and proposals:

Proposal 1: For Msg4 HARQ-ACK scheduled by DCI format 1_0 scrambled by C-RNTI, reuse the repetition factor of the last dedicated PUCCH with the same PUCCH format as Msg4 HARQ-ACK scheduled by DCI format 1_0 scrambled by C-RNTI.

Proposal 2: The repetition factor is applied to any PUCCH transmission before dedicated PUCCH resource configuration is provided.

Proposal 3: Adopt the TP#1 to clarify UE behaviour on phase rotation compensation in TS 38.214.

Proposal 4: Adopt the TP#2 to clarify UE behaviour on TA compensation within actual TDW in TS 38.213.

- References

[1] R1-2312510, Session notes for 8.13 Maintenance on NTN (Non-Terrestrial Networks) Enhancements, 3GPP TSG RAN WG1 #115.

[2] RAN1#112bis Chairman’s Notes