3GPP TSG RAN WG1 #116                                                       R1-2400534

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	8.12.1

Source:	Xiaomi

Title:	Discussion on UE features for NR MIMO evolution

Document for:	Discussion

- Introduction

In RAN1#115 meeting, the UE features for NR MIMO evolution were discussed and summarized in R1-2312572 [1]. In this contribution, we provide our further views on some remaining issues for Rel-18 UE features.

- Unified TCI framework extension for multi-TRP

According to the agreement in R1-2312572 [1], the following FGs/rows are agreed as baseline for further discussions on unified TCI framework extension for multi-TRP.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-1-1 | Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC | 1. Maximum number of configured joint TCI states per CC per BWP 2. Maximum number of activated joint TCI states across all CCs | FFS 23-1-1, 16-2b-1 or 16-2b-2 or 16-2b-3 or 16-2b-4 or 16-2b-5 or 23-2-1 or 23-3-1 or 23-3-2 or 23-3-3 or 23-6-1 or 23-6-2 or 23-6-1-1 or 23-6-1b or 23-6-2b | yes | n/a | Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported | Per band | n/a | n/a | n/a | Note: FG 16-2b-0 can be used to indicate support of two default beams | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-1b | TRP-specific BFR with unified TCI framework with Unified TCI | Support for TRP-specific BFR with unified TCI framework with Unified TCI | FFS 23-5-2, At least one of {40-1-1/1a/2/2a/7/7a/9/9a} | yes | n/a | TRP-specific BFR with unified TCI framework with Unified TCI is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-2a | Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC | 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) with DL assignment b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) without DL assignment 2. M aximum number of activated DL TCI states across all CCs 3. Maximum number of activated UL TCI states across all CCs | FFS 23-10-1b, 40-1-2 | yes | n/a | Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: {2,4,8,16} Component 3 candidate values: {2,4,8,16} Note: FG 16-2b-0 can be used to indicate support of two default beams | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-7 | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC | 1. Support of mTRP operation for M-DCI with joint TCI state 3. Maximum number of configured joint TCI states per BWP per CC 4. Maximum number of activated joint TCI states across all CCs per ‘coresetPoolIndex’ value 5. One MAC-CE activates one joint TCI-states per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value | 23-1-1, [16-2a, 23-4, 40-6-1, 40-6-2, 40-6-3a, 40-6-3b, 40-6-4] | yes | n/a | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values {intra-cell, intra-cell and inter-cell} Component 3 candidate values: {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate values: {1, 2, 4, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions Note: FG 16-2a-6 can be used to indicate support of two default beams | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-7a | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC | 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) with DL assignment b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) without DL assignment 2. Maximum number of MAC-CE activated joint TCI states per CC per coresetpoolindex | 40-1-7 23-1-1b | yes | n/a | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: {2,3,4,5,6,7,8) | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-9 | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC | 1. Maximum number of configured DL TCI states per BWP per CC 2. Maximum number of configured UL TCI states per BWP per CC 3. Maximum number of activated DL TCI states across all CC 4. Maximum number of activated UL TCI states across all CC 5. One MAC-CE activated DL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. 6. One MAC-CE activated UL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. | 23-10-1, 16-2a | yes | n/a | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported | Per band | n/a | n/a | n/a | Component 1 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate value {8, 12, 16, 24, 32, 48, 64} Component 3 candidate values: {1, 2, 4, 8, 16} Component 4 candidate values: {1, 2, 4, 8, 16} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-9a | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC | 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 without DL assignment) 2. maximum number of activated DL TCI states per CORESETPoolIndex per BWP per CC 3. maximum number of activated UL TCI states per CORESETPoolIndex per BWP per CC | 23-10-1b, 16-2a, 40-1-9 | yes | n/a | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8,} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-12 | Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP | 1. Support of common multi-CC TCI state ID update and activation for single-DCI based multi-TRP 2. Maximum number of CC list(s) | [40-1-1, 40-1-1a, 40-1-2, 40-1-2a] | yes | n/a | Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-13 | Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP | 1. Support of common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP 2. Maximum number of CC list(s) | [40-1-7, 40-1-7a, 40-1-9, 40-1-9a] | yes | n/a | Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-14a | Dynamic switching - scheme A | Support of dynamic switching between single-TRP and PDSCH SFN scheme A by TCI selection field in DCI formats 1_1, 1_2 | 40-1-4, 23-6-1a | Yes |  |  |  |  |  |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-14b | Dynamic switching - scheme B | Support of dynamic switching between single-TRP and PDSCH SFN scheme B by TCI selection field in DCI formats 1_1, 1_2 | 40-1-4, 23-6-2a | Yes |  |  |  |  |  |  |  | Optional with capability signaling |

Refer to TS 38.306[2], the following two UE feature were defined that UE need to indicate whether the UE supports dynamic switching between single-TRP and PDSCH SFN scheme A/B by TCI state field in DCI formats 1_1 and 1_2. But in Rel-18, a new DCI field, i.e., TCI selection field will be introduced into DCI formats 1_1 and 1_2 to indicate the TCI state for PDSCH. Thus it is necessary to define a new UE capability to indicate whether the UE supports dynamic switching between single-TRP and PDSCH SFN scheme A/B by TCI selection field in DCI formats 1_1 and 1_2.

| sfn-SchemeA-DynamicSwitching-r17 Indicates whether the UE supports dynamic switching between single-TRP and PDSCH SFN scheme A by TCI state field in DCI formats 1_1 and 1_2. The UE supporting this feature shall indicate sfn-SchemeA-r17 or sfn-SchemeA-PDSCH-only-r17. | FS | No |  | N/A | N/A |
| --- | --- | --- | --- | --- | --- |
| sfn-SchemeB-DynamicSwitching-r17 Indicates whether the UE supports dynamic switching between single-TRP and PDSCH SFN scheme B by TCI state field in DCI formats 1_1 and 1_2. The UE supporting this feature shall indicate sfn-schemeB-r17 or sfn-schemeB-PDSCH-only-r17. | FS | No |  | N/A | N/A |

Proposal 2-1:   Define a new UE capability to indicate whether the UE supports dynamic switching between single-TRP and PDSCH SFN scheme A/B by TCI selection field in DCI formats 1_1 and 1_2.

As for the ‘Prerequisite feature groups’, please see the inputs in the above table.

- Two TAs for multi-DCI

The following is proposed as the outcome of RAN1#115 meeting for the two TAs for multi-DCI [1]. Based on the agreements achived in the RAN1 meetings, our ideas on two TAs for multi-DCI are shown as follows.

For FG 40-2-1, support to add the supported maximum number of n-TimingAdvanceOffset value per serving cell, and the candidate values for the maximum number of n-TimingAdvanceOffset value per serving cell is {1,2}.

For FG 40-2-3, 40-2-4, 40-2-4a, 40-2-6, 40-2-7, 40-2-8, and 40-2-9, the prerequisite feature groups can be set as FG 40-2-1 and 40-2-2.

We thus support the following proposal.

Proposal 3-1: Adopt the following for two TAs for multi-DCI UE features.

- Support to add the supported maximum number of n-TimingAdvanceOffset value per serving cell for FG 40-2-1, and the candidate values for the maximum number of n-TimingAdvanceOffset value per serving cell is {1,2}.

- Support the prerequisite feature groups FG 40-2-1 and 40-2-2 for FG 40-2-3, 40-2-4, 40-2-4a, 40-2-6, 40-2-7, 40-2-8, and 40-2-9.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-2-1 | Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement | 1. Support of two TA enhancement for multi-DCI based intra-cell Multi-TRP operation 2. Maximum number of n-TimingAdvanceOffset value per serving cell | 16-2a | yes | n/a | Two TA enhancement for multi-DCI based intra-cell Multi-TRP operation is not supported | Per FSPC | n/a | n/a | n/a | Component 2 candidate values: {1,2} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-2 | Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement | 1. Support of two TA enhancement for multi-DCI based inter-cell Multi-TRP operation 2. Maximum number of n-TimingAdvanceOffset value per serving cell | 16-2a | yes | n/a | Two TA enhancement for multi-DCI based inter-cell Multi-TRP operation is not supported | Per FSPC | n/a | n/a | n/a | Component 2 candidate values: {1,2} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-3 | TAG ID indication via absolute TA command MAC CE | Support indicating one of two TAG IDs configured in the SpCell via absolute TA command MAC CE. | 40-2-1, 40-2-2 | yes | n/a | TAG ID indication via absolute TA command MAC CE is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-4 | PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA for inter-cell | Support of cross-TRP PDCCH order based on CFRA for inter-cell multi-DCI based mTRP | 40-2-1, 40-2-2 | yes | N/A | Inter-cell cross-TRP PDCCH ordered PRACH transmission is not supported | Per band | No | No | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-2-4a | PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA for intra-cell | Support of cross-TRP PDCCH order based on CFRA for intra-cell multi-DCI based mTRP | 40-2-1, 40-2-2 | yes | N/A | Intra-cell cross-TRP PDCCH ordered PRACH transmission is not supported | Per band | No | No | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-2-6 | Rx timing difference larger than CP length | 1. Support of the Rx timing difference between the two DL reference timings is larger than CP length | 40-2-1, 40-2-2 | yes | N/A | Rx timing difference larger than CP is not supported | Per FSPC | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-2-7 | Two TAs for multi-DCI STxMP PUSCH+PUSCH | Support of two TAs for multi-DCI STxMP PUSCH+PUSCH | [40-2-1 or 40-2-2, FFS more] | yes | n/a | Two TAs for multi-DCI STxMP PUSCH+PUSCH is not supported | Per FSPC | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-8 | Maximum number of TAGs across all CCs | Maximum number of TAGs across all CCs | FFS40-2-1, 40-2-2 | yes | n/a | Maximum number of TAGs across all CCs is unknown when UE supports two TAGs per CC | Per BC | n/a | n/a | n/a | Component candidate values: {2,3,4} Note: UE only supports the configuration where all UL CCs of the same frequency band are configured with up to 2 Timing Advance Group ID Note: The same description of “supportedNumberTAG” in 38.306 applies to this FG as well | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-2-9 | Overlapping UL transmission reduction | Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is not configured with UL STx2P for multi-DCI based multi-TRP operation with two TA enhancement | 40-2-1, 40-2-2 | Yes | N/A | Reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions is not supported | Per band | N/A | N/A | N/A | Note: If UE does not support this feature, UE does not expect the two UL transmissions to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions) | Optional with capability signalling |

- CSI enhancement

CSI enhancement on m-TRP CJT

According to the agreement in R1-2312572 [1], the following FGs/rows are agreed as baseline for further discussions on CSI for CJT:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-1 | Basic feature for Rel-16-based CJT type-II codebook | Support of N=N_TRP only Support of N_L=1 only 1. Support of mode 2 for Rel-16 eType-II codebook refinement for multi-TRP CJT 2. Support for PMI subband R=1. 3. Support of parameter combinations with L=2,4 4. Support of rank 1,2 5. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs 7. Scaling factor X for CPU occupation counting for Rel-16-based CJT type-II codebook 8. Maximum number of NZP CSI-RS resources in one NZP CSI-RS resource set associated with multi-TRP CJT | [2-35], 16-3a | Yes | N/A | Mode 2 for Rel-16-based CJT type-II codebook is not supported | Per band and Per BC | No | N/A | N/A | Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Component 7 candidate values: {1, 1.5, 2} Component 8 candidate values: {2,3,4} Note: When NTRP=1 TRP is configured, OCPU =1. When NTRP>1 TRPS are configured, OCPU = ceil(X * NTRP) Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel. 16 based type-II CJT must support this FG | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-3 | Support R=2 for Rel-16-based CJT codebook | 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subbands R=2 2. {Max # of Tx ports in one resource set, Max # of resource sets, total # of Tx ports}, [across all CCs] simultaneously, with R=2 | 40-3-1-1 | Yes | N/A | R=2 for Rel-16-based CJT codebook is not supported | Per band Per BC | N/A | N/A | N/A | Component 2 candidate values: a) {4,8,12,16,24,32} b) {2 to 64} c) {4 to 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-5 | Basic feature for Rel-17-based CJT type-II codebook | Support of N=N_TRP only Support of N_L=1 only 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT 2. Support of PMI subband R=1. 3. Support of parameter combinations with M=1 4. Support of rank 1,2 5. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs 7. Scaling factor X for CPU occupation counting for Rel-17-based CJT type-II codebook 8. Maximum number of NZP CSI-RS resources in one NZP CSI-RS resource set associated with multi-TRP CJT | [2-35], 23-9-1 | Yes | N/A | Mode 2 for Rel-17-based CJT type-II codebook is not supported | Per band and Per BC | No | N/A | N/A | Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Component 7 candidate values: {1, 1.5, 2} Component 8 candidate values: {2,3,4} Note: When NTRP=1 TRP is configured, OCPU =1. When NTRP>1 TRPS are configured, OCPU = ceil(X * NTRP) Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel 17 based type-II CJT must support this FG | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-24 | The maxmum total number of supported SD basis for M-TRP CJT | The maxmum total number of supported SD basis for M-TRP CJT |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-25 | The supported subset of linkages | The supported subset of linkages |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-26 | The supported value of Z/Z’ for NTRP>1 | The supported value of Z/Z’ for NTRP>1 |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-27 | The supported value of CPU occupation for NTRP>1 | The supported value of CPU occupation for NTRP>1 |  |  |  |  |  |  |  |  | {1, 1.5, 2} |  |

According to the following agreement, the number of supported SD basis combination can be reported based on UE capability.

Agreement (RAN1-112 meeting)

On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {Ln} for the higher-layer-configured value of NTRP (FFS by RAN1#112: whether the bracketed permutations are also supported):

- FFS by RAN1#112: whether other combinations can be supported

FFS (by RAN1#112bis-e): Whether/how the supported combinations of {n} for Rel-17-based refinement are derived from the supported combinations of {Ln} for Rel-16-based refinement

FFS: Whether the total number of Ln is a UE capability

| NTRP | {Ln} combination |
| --- | --- |
| 1 | {2} |
|  | {4} |
|  | {6} (analogous to legacy, only for total # ports =32, rank 1-2, R=1 |
| 2 | {2,2} |
|  | {2,4}, [{4,2}] |
|  | {4,4} |
| 3 | {2,2,2} |
|  | {2,2,4} [and its other permutations] |
|  | {4,4,4} |
| 4 | {2,2,2,2} |
|  | {2,2,2,4} [and its other permutations] |
|  | {2,2,4,4} [and its other permutations] |
|  | {4,4,4,4} |

Proposal 4-1: define a UE capability to indicate the total number of supported SD basis combinations.

According to the following agreement, the supported subset of linkages can be reported based on UE capability.

Agreement (RAN1-112b e-meeting)

On the Parameter Combination of Type-II codebook refinement for CJT mTRP, only the following linkages are supported (marked ‘x’), for Rel-16 eType-II based

- For NTRP =1,

  - fully reuse seven out of the eight Parameter Combinations from Rel-16 eType-II as indicated in the table below

    - FFS (by RAN1#112bis-e): whether to add one more Parameter Combination for L=4 based on the legacy Rel-16 eType-II FD combo {½, ½, ¼, ¼; ½} or the agreed FD combo {½, ½, ½, ½; ½}, or not to add from the indicated seven below

- For NTRP >1, only the following linkages are supported (marked ‘x’)

- Note: Configured linkage(s) are associated with the configured value of NTRP, regardless whether the dynamic TRP selection (the dynamic change of N given NTRP) is configured. Also, the configured linkage(s) are valid for any dynamically selected SD basis and/or any dynamically selected CSI-RS resource (TRP).

- FFS: UE feature/capability to support only a subset of linkages

| NTRP | SD combo | FD combo {pv}, |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | {1/8, 1/8, 1/16, 1/16}, ¼ | {1/8, 1/8, 1/16, 1/16}, ½ | {1/4, ¼, 1/8, 1/8}, ¼ | {1/4, ¼, 1/8, 1/8}, ½ | {1/4, ¼, ¼, ¼}, ¾ | {1/2, ½, ½, ½}, ½ |
| 1 | 2 |  |  | x | x |  |  |
|  | 4 |  |  | x | x | x |  |
|  | 6 w/ restriction |  |  |  | x | x |  |
| 2 | {2,2} | x |  |  |  |  |  |
|  | {2,4} {4,2} | x |  |  |  |  |  |
|  | {4,4} |  | x |  | x |  | x |
| 3 | {2,2,2} | x | x |  |  |  |  |
|  | {2,2,4} {2,4,2} {4,2,2} | x | x |  |  |  |  |
|  | {4,4,4} | x | x | x | x | x | x |
| 4 | {2,2,2,2} | x |  |  |  |  | N/A |
|  | {2,2,2,4} | x |  |  |  |  | N/A |
|  | {2,2,4,4} |  |  |  | x | x | N/A |
|  | {4,4,4,4} |  | x |  | x | x | N/A |

Proposal 4-2: define a UE capability to indicate the supported subset of linkages.

According to the following agreement, the supported value of Z/Z’ can be reported based on UE capability for NTRP>1.

Agreement(RAN1-113 meeting)

For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’:

- For NTRP=1: reuse legacy Z/Z’ values

- For NTRP>1, introduce two UE capabilities:

  - Capability 1: Reuse legacy Z/Z’ values

  - Capability 2: Legacy Z/Z’ values + r

    - The value(s) of r>0 can depend on the configured NTRP value

    - FFS: exact value(s) of r

Note: Since this pertains Type-II, the relevant values are Z2/Z2’

Proposal 4-3: define a UE capability to indicate the supported value of Z/Z’ for NTRP>1.

Agreement(RAN1-113 meeting)

For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation: OCPU = X.NTRP where

- X≥1 when NTRP>1, is defined based on UE capabilities and determined by the UE

- FFS: Whether the supported value(s) of X are common or can depend on the value of NTRP, NL, total sum of {Ln}, and/or other CJT features (e.g. dynamic TRP selection)

- The legacy specification on CPU pools is fully reused

- Note: When NTRP=1 is configured, legacy OCPU applies, i.e. OCPU =1

Agreement(RAN1-114 meeting)

For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation,

- X is a “common value” and not dependent on NTRP or any other parameter value, reported by the UE from a set of candidate values {1, 1.5, 2} according to UE capability

According to the following agreement, the supported value of CPU oocupation can be reported based on UE capability for NTRP>1.

Proposal 4-4: define a UE capability to indicate the supported value of CPU occupation for NTRP>1.

As for the ‘Prerequisite feature groups’, please see the inputs in the above table.

CSI enhancement on high/medium velocities

For index 40-3-2-1a of UE feature, i.e., support of Rel-16-based doppler measurement with N4>1, the maximum N4 should be larger than 1 when N4>1. Hence, the component 2 candidate value 1 should be removed.

Proposal 4-5: For index 40-3-2-1a, the candidate value 1 should be removed in component 2 when N4>1 .

For index 40-3-2-6 of UE feature, i.e., Support R=2 for Rel-17-based doppler codebook, the component 2 does not include parameter d. The parmeter d should be removed in the component.

Proposal 4-6: For index 40-3-2-6, the parmeter d should be removed in the component 2.

According to the following agreement, L=6 should be as a additional UE feature/capability, which is similar with UE feature of Rel-16 Type II codebook.

Agreement

For the Type-II codebook refinement for high/medium velocities based on Rel-16 eType-II regular codebook, at least the following Parameter Combinations are supported

| L | pυ |  | β |
| --- | --- | --- | --- |
|  | υ ∈1,2 | υ ∈3,4 |  |
| 4 | 1/4 | 1/4 | 1/4 |
| 4 | 1/4 | 1/4 | 1/2 |
| 4 (*) | 1/2 | 1/4 | 1/2 |
| 4 (*) | 1/4 | 1/4 | 3/4 |
| 6 (*) | 1/4 | -- | 1/2 |
| 6 (*) | 1/4 | -- | 3/4 |

(*) Note: From legacy. For L=6, the same restriction and UE optionality as legacy apply

Proposal 4-6: L=6 should be as an additional UE feature.

Acorrding to the following agreement, the supported value of w is a UE feature and needs to be discussed in this part.

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z for Capability 2 associated with P/SP-CSI-RS, decide, in RAN1#114, based on the following alternatives:

- Alt1: w=14.(KP–1).d or 14. KP.d, where d denotes the CSI-RS periodicity

- Alt2: w=14 or 28 (fixed)

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z associated with P/SP-CSI-RS,

- w (unit of symbols) is reported by UE

  - To be finalized as part of Rel-18 UE feature discussions

In our view, the value of w should be assocated with the periodic of CSI-RS, since the CSI processing time needs to increase with the periodic increases. Therefore, Alt1, i.e.,w=14.(KP–1).d or 14. KP.d is preferred.

Proposal 4-7: The value of w should be as an additional UE feature and Alt1, i.e.,w=14.(KP–1).d or 14. KP.d is preferred.

Based on above discussion, the following proposal is given.

Proposal 4-8: The following tables on UE feature updated with red color for CSI enhancement on high/medium velocities can be considered.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-1 | Support of Rel-16-based doppler CSI | 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 2. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 3. Support parameter combinations with L=2,4 4. Support for rank = 1,2 5. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously 76. Value of Y for CPU occupation (OCPU = Y.N4), when P/SP-CSI-RS is configured for CMR 87. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 98. Support for the size of DD-basis, N4=1 109. Scaling factor for active resource counting Kp | 2-35 | Yes | N/A | Rel-16 based Type II doppler codebook is not supported | Per band and Per BC | N/A | N/A | N/A | Component 5 candidate values a. {4,8,12,16,24,32} b. {2,3,4 … 64} c. {4, …, 256} Component 76 candidate values: {1, 2, 3} Component 87 candidate values: {1, 2, 3} Component 109 candidate values: {1, 2, 4} Note: When N4=1, OCPU =4 Note: OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 16 based type-II doppler must support this FG | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-1a | Support of Rel-16-based doppler measurement with N4>1 | 1. Support for the size of DD-basis, N4>1 2. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR | 40-3-2-1 | Yes | N/A | Rel-16 based Type II doppler codebook with N4>1 is not supported | Per band and Per BC | N/A | N/A | N/A | Component 2 candidate values a. {1,2,4,8} b. {4,8,12,16,24,32} c. {2,3,4 … 64} d. {4, …, 256} Component 3 Candidate values a. {1,2,4,8} b. {4,8,12,16,24,32} c. {4,8,12} d.{4, …, 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-1a-1 | DD unit size when A-CSI-RS is configured for CMR N4>1 | Value of d=1 for the DD unit size when A-CSI-RS is configured for CMR | 40-3-2-1a | Yes | N/A | DD unit size d=1 is not supported when A-CSI-RS is configured for CMR N4>1 | Per-band and Per-BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL |  | Support parameter combinations with L=6 |  | 40-3-2-1 | Yes | N/A | parameter combinations with L=6 is not supported | Per-band and Per-BC | N/A | N/A | N/A |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL |  | The supported value of w |  | 40-3-2-1 | Yes | N/A | The value of w is not reported. | Per-band and Per-BC | N/A | N/A | N/A | - Alt1: w=14.(KP–1).d or 14. KP.d, where d denotes the CSI-RS periodicity | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-2 | Support R=2 for Rel-16-based doppler codebook | A list of supported combinations {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}, across all CCs simultaneously, with R=2 | 40-3-2-1 | Yes | N/A | R=2 for Rel-16-based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-3 | Support X=1 based on first and last slot of WCSI, for Rel-16-based doppler codebook | Support of TDCQI = ‘1-2’ | 40-3-2-1 | Yes | N/A | X=1 based on first and last slot of WCSI, for Rel-16-based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-3a | Support X=2 CQI based on 2 slots for Rel-16-based doppler codebook | Support of TDCQI = ‘2’ | 40-3-2-1 | Yes | N/A | X=2 CQI based on 2 slots for Rel-16-based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-4 | Support of Rel-17-based doppler | 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI 2. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subband R=1 3. Support of parameter combinations with M = 1 4. Support for rank = 1,2 5: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement 76. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 87. Support for N4=1 98. Scaling factor for active resource counting Kp | 40-3-2-1, 2-35 | Yes | N/A | Rel-17 based Type II doppler codebook is not supported | Per-band and Per-BC | N/A | N/A | N/A | Component 5 candidate values a. {4,8,12,16,24,32} b. {1, 2,3,4 … 64} c. {4, …, 256} Component 76 candidate values: {1, 2, 3} Component 98 candidate values: {1, 2, 4} Note: OCPU = 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 17-based type-2 doppler must support this FG | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-5 | Support of M=2 and R=1 for Rel-17-based doppler codebook | 1. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with M=2 and PMI subband R=1 2. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource b) Maximum total number of NZP CSI-RS resource - c) Maximum total number of Tx ports of NZP CSI-RS resources | 40-3-2-4 | Yes | N/A | M=2 and R=1 for Rel-17-based doppler codebook is not supported | Per-band and Per-BC | N/A | N/A | N/A | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-6 | Support R=2 for Rel-17-based doppler codebook | 1. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subbands R=2 2. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource b) Maximum total number of NZP CSI-RS resource c) Maximum total number of Tx ports of NZP CSI-RS resources | 40-3-2-4 | Yes | N/A | R=2 for Rel-17-based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {2,3,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-7 | support of l = (n – nCSI,ref ) for CSI reference slot for Rel-16 based doppler codebook | 1. Support of l = (n – nCSI,ref ) for CSI reference slot when N4=1 and d>1 2. Support of l = (n – nCSI,ref ) for CSI reference slot when N4>1 | 40-3-2-1 | Yes | N/A | l = (n – nCSI,ref ) for CSI reference slot for Rel-16 based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-7a | Support of l = (n – nCSI,ref ) for CSI reference slot for Rel-17 based doppler codebook | Support of l = (n – nCSI,ref ) for CSI reference slot when N4=1 and d>1 | 40-3-2-4 | Yes | N/A | l = (n – nCSI,ref ) for CSI reference slot for Rel-17 based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |

CSI enhancement on TDCP reporting

Accordinng to the following agreement, only when D=10, it requires that the SCS should be larger than 30 kHz. For D=6, there is no any additional resturiction.

Agreement

For the Rel-18 TRS-based TDCP reporting, regarding the value of parameter D,

- Dbasic = 1 slot

- Confirm the working assumption on the support for D=6

- Confirm the working assumption on the support for D=10 (only for >=30kHz SCS)

Proposal 4-9: The following tables on UE feature updated with red color for TDCP reporting can be considered.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-1 | TDCP (Time Domain Channel Properties) report | 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report 4. Value of X for CPU occupation (OCPU=(Y+1).X) 5. Support to configure KTRS = 1 TRS resource set 6. Maximum number of simultaneously active CSI-RS resources for TDCP across all CCs | [2-35] | Yes | N/A | Time Domain Channel Properties report is not supported | Per band and Per BC | N/A | N/A | N/A | Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214 | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-2 | Number of delay values | Number Y>1 of delay values for which TDCP is reported | 40-3-3-1 | Yes | N/A | TDCP is not reported for more than 1 delay value | Per FS | N/A | N/A | N/A | Candidate values: {2,3,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-1a | Supported maximum delay value larger than D_basic | Support of maximum delay value larger than D_basic =1 slot | 40-3-3-1 | Yes | N/A | delay value(s) larger than D_basic are not supported | Per FS | N/A | N/A | N/A | Candidate values: {2 slots, 3 slots, 4 slots, 5 slots, 6 slots, 10 slots} [Note: 10 slots is only applicable for SCS >= 30 kHz, and 6 slots is maximum for SCS = 15 kHz] | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-4 | Phase report | Support of phase report for Y>=1 | 40-3-3-1 | Yes | N/A | Phase report for Y>=1 is not supported | Per FS | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-5 | Number of CSI-RS resources for TDCP | 1. Maximum number of configured CSI-RS resources for TDCP per CC 2. Maximum number of configured CSI-RS resources for TDCP across all CCs 3. Maximum number of simultaneously active CSI-RS resources for TDCP per CC Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214 | 40-3-3-1 | Yes | N/A | Number of CSI-RS resources for TDCP is not reported | Per band and Per BC | N/A | N/A | N/A | Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214 | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-6 | Maximum number of TRS resource sets in a report configuration | Max number of TRS resource sets in a single CSI-RS resource setting | 40-3-3-1 | Yes | N/A | More than 1 TRS resource set in a report configuration is not supported | Per FS | N/A | N/A | N/A | Candidate values: {2,3} | Optional with capability signaling |

- Increased number of orthogonal DMRS ports

DMRS enhancement for 8Tx

According to the proposed FGs as the outcome of RAN1 #115 for this UL SRS enhancements for 8Tx transmission [1], the FFS needs to be discussed further.

- Add FG2-47 as prerequisite FG for FG 40-4-1i/ FG40-4-6g/FG40-4-6h;

- Add FG2-48 as prerequisite FG for FG40-4-6i/FG40-4-6j;

Based on the discussions above, we propose:

Proposal 5-1: Adopt the following for DMRS enhancements for 8TX operation UE features, and consider the following updated table.

- Add FG2-47 as prerequisite FG for FG 40-4-1i/ FG40-4-6g/FG40-4-6h

- Add FG2-48 as prerequisite FG for FG40-4-6i/FG40-4-6j;

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-4-1i | 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 | Support of 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 | 40-4-1, 2-47 | Yes | n/a | 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6g | 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | 40-4-6, 2-47 | Yes | n/a | 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6h | 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | 40-4-6, 2-47 | Yes | n/a | 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6i | 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | 40-4-6, 2-48 | Yes | n/a | 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6j | 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | 40-4-6, 2-48 | Yes | n/a | 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-10 | DMRS port configuration for PUSCH with rank 5-8/ with 8Tx | DMRS port configuration for PUSCH with rank 5-8 / with 8Tx for Rel 15 and Rel. 18 | 40-4-6 | Yes | n/a | Port configuration for PUSCH with rank 5-8 / with 8Tx is not supported | Per FS | No | No | n/a | Candidate values: {Rel. 15 DMRS, Rel. 15 DMRS and Rel. 18 DMRS} [Note: A UE supporting 8 Tx must support this FG] | Optional with capability signaling |

- SRS enhancements targeting TDD CJT and 8Tx operation

SRS enhancement for 8Tx

According to the proposed FGs as the outcome of RAN1 #115 for this UL SRS enhancements for 8Tx transmission [1], the FFS needs to be discussed further.

Proposal 6-1: Adopt the following FG for SRS enhancements for 8TX operation UE features

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-5-4 | SRS 8 Tx ports—antenna switching | 1. Support of 8T8R for antenna switching 2. Downgrade antenna switching configurations 3. Report the entry number of the first-listed band with UL in the band combination that affects this DL 4. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL | 2-53，2-55，23-8-5，23-8-8 | Yes | n/a | SRS with 8 Tx ports—antenna switching is not supported | Per FS | n/a | n/a | n/a | Component 1 candidate values: {noTDM, TDM and noTDM} Component 2 candidate value: combination (including empty) of {1T1R, 1T2R, 1T4R, 1T6R, 1T8R, 2T2R, 2T4R, 2T6R, 2T8R, 4T4R, 4T8R} Component 3 candidate value: {1,2,…,32} Component 4 candidate value: {1,2,…,32} Note: UE reports support of SRS with 8 Tx ports and Comb8 mapping —antenna switching via FG 23-8-8 | Optional with capability signaling |

SRS enhancement for CJT

For index 40-5-2a, 40-5-1c and 40-5-2c of UE feature, the given information in the note column should be written in the consequence column if the feature is not supported. The following agreement shows that smaller cyclic shift granularity for cyclic shift hopping is not supported when a subset for cyclic shift is configured. This information should be givne in the note column.

Agreement

For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., αi=2πnSRScs,inSRScs,max+2πnSRScs,offsetK×nSRScs,max, where nSRScs,offset can be randomly chosen from 0,1,…K×nSRScs,max-1 at each SRS transmission.

- Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled.

If a subset for cyclic shifts is configured, this feature cannot be configured.

Above is a UE optional feature.

Proposal 6-2: The following table on UE feature updated with red color for SRS enhancement for CJT can be considered.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-5-1 | SRS comb offset hopping | Support of SRS comb offset hopping | FFS 2-53 | Yes | n/a | SRS comb offset hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-2 | SRS cyclic shift hopping | Support of SRS cyclic shift hopping | [2-53] | Yes | n/a | SRS cyclic shift hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-2a | Smaller cyclic shift granularity for cyclic shift hopping | Support configuration of cyclic shift hopping with smaller granularity (with factor K=2) | 40-5-2 | Yes | N/A | Configuration of cyclic shift hopping with smaller granularity (with factor K=2) is not supported | Per band | n/a | n/a | n/a | Configuration of cyclic shift hopping with smaller granularity (with factor K=2) is not supported If a subset for cyclic shifts is configured, this feature cannot be supported. | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-1c | Comb offset hopping within a subset | Support configuration of subset of comb offsets for comb offset hopping | 40-5-1 | Yes | N/A | Configuration of subset of comb offsets for comb offset hopping is not supported | Per band | n/a | n/a | n/a | Configuration of subset of comb offsets for comb offset hopping is not supported | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-2c | Cyclic shift hopping within a subset | Support configuration of subset of cyclic shifts for cyclic shift hopping | 40-5-2 | Yes | N/A | Configuration of subset of cyclic shifts for cyclic shift hopping is not supported | Per band | n/a | n/a | n/a | Configuration of subset of cyclic shifts for cyclic shift hopping is not supported | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-1b | SRS comb offset hopping combined with legacy group/sequence hopping | Support of SRS comb offset hopping combined with legacy group/sequence hopping | 40-5-1 | Yes | n/a | SRS comb offset hopping combined with legacy group/sequence hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-2b | SRS cyclic shift hopping combined with legacy group/sequence hopping | Support of SRS cyclic shift hopping combined with legacy group/sequence hopping | 40-5-2 | Yes | n/a | SRS cyclic shift hopping combined with legacy group/sequence hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-3 | SRS cyclic shift hopping combined with SRS comb offset hopping | Support of SRS cyclic shift hopping combined SRS comb offset hopping | 40-5-1, 40-5-2 | Yes | n/a | SRS cyclic shift hopping combined with SRS comb offset hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-1a | Comb offset hopping time-domain behavior when repetition factor R>1 | Supported comb offset hopping granularity in time when repetition factor R>1 is configured | 40-5-1 | Yes | n/a | Comb offset hopping time-domain behavior when repetition factor R>1 is not supported | Per band | n/a | n/a | n/a | Component candidate values: {‘per SRS symbol’,’per R SRS symbols’, ‘both’} | Optional with capability signaling |

- Simultaneous multi-panel transmission

According to the proposed FGs as the outcome of RAN1 #115 for this UL simultaneous multi-panel transmission [1], the FFS needs to be discussed further.

Proposal 7-1: Adopt the following FGs for STxMP enhancements UE features

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-6-3h | Codebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG | Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH | 40-6-3a | Yes | N/A | Multi-DCI based STx2P for CG+CG is not supported for codebook | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3i | Codebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG | Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH | 40-6-3a | Yes | N/A | Multi-DCI based STx2P for DG+CG is not supported for codebook | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3o | Noncodebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG | Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH for noncodebook | 40-6-3a | Yes | N/A | Multi-DCI based STx2P for CG+CG is not supported for noncodebook | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3p | Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG | Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH for noncodebook | 40-6-3a | Yes | N/A | Multi-DCI based STx2P for DG+CG is not supported for noncodebook | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3b-1 | Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH | 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for multi-DCI non-codebook based STxMP scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously | 2-15a, 40-6-3b | Yes | n/a | Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported | Per Band | n/a | FR2 only | n/a | Component 2 candidate values: {1 to 8} Component 3 candidate values: {1 to 8} Component 4 candidate values: {0 to 8} Component 5 candidate values: {1 to 16} Component 6 candidate values: {1 to 2} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-5 | Support grouped-based beam reporting for STx2P | 1. Support group based L1-RSRP reporting for STxMP based transmission 2. Max number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets 3. Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs 4. Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs | 23-5-1 | Yes | n/a | Grouped-based beam reporting for STx2P is not supported | Per Band | n/a | FR2 only | n/a | Component 1 candidate values: {JointULandDL, ULOnly, both} Component 2 candidate values: {1,2,3,4} Component 3 candidate values: {2,3,4,8,16,32,64} Component 4 candidate values: {8, 16, 32, 64, 128} Note: components 3 and 4 are also counted in FG 16-1g, 16-1g-1, and 23-5-1 | Optional with capability signaling |

- SRI/TPMI for 8Tx UL transmission

The following is proposed as the outcome of RAN1#115 for the SRI/TPMI for 8Tx UL transmission [1]. Based on the agreements achived in the RAN1 meetings, our ideas on UL 8Tx are shown as follows.

For FG 40-7-1, support that the candidate values of Component 1 can be set as {1,2,3,4,5,6,7,8}.

For FG 40-7-1a, support that the candidate values of Component 1 can be set as {(4,1), (2,2)}.

For FG 40-7-1g-1, support that the candidate values of Component 1 can be set as {1,2,4,8}.

For FG 40-7-1, 40-7-1e, 40-7-1f, 40-7-1g, 40-7-2, 40-7-2a, and 40-7-3, support that the prerequisite feature groups can be set as FG 2-14, 16-5a, 16-5b, 16-5c, 2-15, 2-15a, and {40-7-1, 40-7-2}, respectively.

We thus support the following proposal.

Proposal 8-1: Adopt the following for 8Tx UL UE features.

- For FG 40-7-1, support that the candidate values of Component 1 can be set as {1,2,3,4,5,6,7,8}.

- For FG 40-7-1a, support that the candidate values of Component 1 can be set as {(4,1), (2,2)}.

- For FG 40-7-1g-1, support that the candidate values of Component 1 can be set as {1,2,4,8}.

- For FG 40-7-1, 40-7-1e, 40-7-1f, 40-7-1g, 40-7-2, 40-7-2a, and 40-7-3, support that the prerequisite feature groups can be set as FG 2-14, 16-5a, 16-5b, 16-5c, 2-15, 2-15a, and {40-7-1, 40-7-2}, respectively.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-7-1 | Basic features for Codebook-based 8Tx PUSCH | 1. Maximum number of PUSCH MIMO layers for codebook based PUSCH 2. Maximum number of 8 port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 8Tx PUSCH 3. SRS 8 Tx ports—codebook | FFS2-14 | Yes | n/a | Codebook-based 8Tx PUSCH is not supported | Per FSPC | No | No | No | Component 1 candidate values: {1,2 [,3],4 [,5,6,7],8} Component 2 candidate values: {1,2} Component 3 candidate values: {noTDM, TDM and noTDM} A UE that supprts FG 40-7-1 must support at least one of FGs 40-7-1a/b/c/d | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1a | Codebook-based 8Tx PUSCH—codebook1 | 1. Support of codebook-based 8Tx PUSCH—codebook1 2. Support of (N1, N2) for codebook-based 8Tx PUSCH—codebook1 | 40-7-1 | Yes | n/a | Codebook-based 8Tx PUSCH—codebook1 is not supported | Per FSPC | No | No | No | 2. Component candidate values: {(4,1), (2,2), both} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1b | Codebook-based 8Tx PUSCH—codebook2 | Support of codebook-based 8Tx PUSCH—codebook2 | 40-7-1 | Yes | n/a | Codebook-based 8Tx PUSCH—codebook2 is not supported | Per FSPC | No | No | No |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1c | Codebook-based 8Tx PUSCH—codebook3 | Support of codebook-based 8Tx PUSCH—codebook3 | 40-7-1 | Yes | n/a | Codebook-based 8Tx PUSCH—codebook3 is not supported | Per FSPC | No | No | No |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1d | Codebook-based 8Tx PUSCH—codebook4 | Support of codebook-based 8Tx PUSCH—codebook4 | 40-7-1 | Yes | n/a | Codebook-based 8Tx PUSCH—codebook4 is not supported | Per FSPC | No | No | No |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1e | UL full power transmission mode 0 | Support of UL full power transmission mode of fullpower when UE is capable of 8 Tx codebook based PUSCH operation | FFS16-5a | yes | n/a | UL full power transmission mode 0 is not supported | Per FSPC | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1f | UL full power transmission mode 1 | Support of UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation | FFS16-5b | yes | n/a | UL full power transmission mode 1 is not supported | Per FSPC | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1g | UL full power transmission mode 2 with 1/2/4 resources | 1. Support of UL full power transmission mode of fullpowerMode2 when UE is capable of 8 Tx codebook based PUSCH operation 2. Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2 | FFS16-5c | yes | n/a | UL full power transmission mode 2 is not supported | Per FSPC | n/a | n/a | n/a | Component 2 candidate values: {1, 2, 4} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1g-1 | SRS resources for UL full power transmission mode 2 | 1. SRS configurations with different number of antenna ports per SRS respource for mode 2 | 40-7-1g | yes | n/a | SRS resources for UL full power transmission mode 2 cannot be signaled | Per FSPC | n/a | n/a | n/a | Component 1 candidate values: FFS{1,2,4,8} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-7-2 | Basic features for Non-Codebook-based 8Tx PUSCH | 1. Supported maximal PUSCH MIMO layers for non-codebook based PUSCH 2. Supported maximum number of SRS resources per SRS resource set with usage set to 'nonCodebook’ 3. Maximum number of simultaneous transmitted SRS resources at one symbol | FFS2-15 | Yes | n/a | Non-codebook based 8Tx PUSCH is not supported | Per FSPC | No | No | No |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-2a | Association between CSI-RS and SRS for non-codebook case | 1. Support association between NZP-CSI-RS and SRS resource set via RRC parameter "SRS-ResourceSet" for noncodebook 8Tx PUSCH operation 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} across all CCs simultaneously | FFS2-15a | yes | n/a | Association between CSI-RS and SRS for non-codebook case is not supported | Per FSPC | No | No | No | Component 2 candidate value: Maximum size of the list is 16. The candidate values for the max # of Tx port in one resource is [{2, 4, 8, 12, 16, 24, 32}] The candidate value set of the max # of resources is: [{1 to 64}] The candidate value set of total # of ports is: [{2 to 256}] | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-7-3 | CBG based 2 CWs PUSCH with rank >4 | Support CBG based transmission for 2 CWs PUSCH | FFS40-7-1, 40-7-2 | yes | n/a | CBG based transmission for 2 CWs PUSCH is not supported | Per FSPC | No | No | No |  | Optional with capability signalling |

9 References

- R1-2312572, updated RAN1 UE features list for Rel-18 NR after RAN1_115, Noverber 2023.

- 3GPP TS 38.306.