3GPP TSG RAN WG1 #116			R1-2400537

Athens, Greece, February 26th – March 1st, 2024

Source: 	     Xiaomi

Title:                  Discussion on UE features for MC enhancement

Agenda item:    8.12.4

Document for:  Discussion

Introduction

In RAN1#115 meeting, UE features for MC enhancements were heatedly discussed and very good progress was achieved. The detail agreements can be found in session notes for AI 8.16.12. [1]

In this contribution, we provided our views on remaining issues.

Discussion

FG 49-1

Note: One note is currently capture in FG49-1 as below:

[Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell]

It is well known that 1) SCell scheduling PCell/PSCell is excluded for Rel-18 MC 2) SCell can have only single scheduling cell in the specification. Hence, the above restriction should be applicable to any scheduling cell despite of it is inside or outside the set of cells. The note should be updated to more generic version. We prefer to keep this note and replace it with ‘Note: UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell’.

Proposal 1: Update the note in FG 49-1 as below:

- Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell.

FG 49-2

Component 9) is captured as below:

9) The number of unicast UL DCIs to process per slot of scheduling cell for a set of cells configured for multi-cell PUSCH scheduling by DCI format 0_3 - For FDD scheduling cell - [Up to] one DCI format 0_3 for the set of cells and, - [Up to] one unicast UL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells - For a cell in a set of cells, no more than one DCI scheduling PUSCH for the cell - For TDD scheduling cell - [Up to] two DCI format 0_3 for the set of cells and, - [Up to] two unicast UL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells - For a cell in a set of cells, no more than two DCI scheduling PUSCH for the cell

In FG 3-1, the following components related to unicast DCI processing are captured.

5) Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot per scheduled CC for FDD 6) Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot per scheduled CC for TDD

The spirit of FG 3-1 can be reused for FG 49-2. To be specific, the following principle should be respected:

- For FDD, one unicast DCI scheduling PUSCH is supported, despite of DCI format.

- For TDD, two unicast DCI scheduling PUSCH are supported, despite of DCI format.

Accordingly, the current version of component 9) is sufficient.

Proposal 2: Adopt the current version of component 9) for FG 49-2 with removing the bracket.

Note: One note is currently capture in FG49-2as below:

[Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell]

Same as FG 49-1, we think the note should be updated to a more generic version.

Proposal 3: Update the note in FG 49-2 as below:

- Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell.

FG 49-2b

Same as FG 49-2, the bracket in component 9) should be removed.

Proposal 4: Adopt the current version of component 9) for FG 49-2b with removing the bracket.

FG 49-3x & FG49-3y

Basically, we agree with introducing FG49-3x and FG49-3y which precisely reflect the following agreement achieved in previous meeting.

Agreement - Following component is introduced in FG 49-1 - The number of unicast DL DCI to process [for a set of cells] configured for multi-cell PDSCH scheduling by a DCI format 1_3 - One unicast DCI per slot of scheduling cell [for the set of cells] for FDD/TDD scheduling cell - FFS whether to count DCI format 1_3 only or both legacy DCI formats and DCI format 1_3 - Introduce advanced UE capability for larger number of unicast DL DCI, details FFS - Following component is introduced in FG 49-2 - The number of unicast UL DCI to process [for a set of cells] configured for multi-cell PUSCH scheduling by a DCI format 0_3 - One unicast DCI per slot of scheduling cell [for the set of cells] for FDD scheduling cell - Two unicast DCI per slot of scheduling cell [for the set of cells] for TDD scheduling cell - FFS whether to count DCI format 0_3 only or both legacy DCI formats and DCI format 0_3 - Introduce advanced UE capability for larger number of unicast UL DCI, details FFS

Regarding the details, the following principles should be satisfied:

- The advanced UE capability should be more aggressive than basic DCI processing capability

- Report UE capability for different SCS combinations between scheduling cell and scheduled cell

- Report UE capability for different duplex scheme, e.g. TDD and FDD

Based on the above principles, we propose to define the component as below:

| 49.NR_MC_enh | 49-3x | Advanced UE capability for larger number of unicast DL DCI | The number of unicast DL DCI to process for a set of cells configured for multi-cell PDSCH scheduling by a DCI format 1_3 and single cell PDSCH scheduling by a DCI format 1_0, DCI format 1_1, and/or DCI format 1_2 - For same SCS between scheduling cell and scheduled cell(s), two unicast DCIs per slot of scheduling cell for each cell of the set of cells for FDD/TDD scheduling cell - For different SCS between scheduling cell and scheduled cell(s), - For low-to-high SCS, two unicast DCIs per slot of scheduling cell for each cell of the set of cells for FDD/TDD scheduling cell - For high-to-low SCS, two unicast DCIs per N consecutive slots of scheduling cell for each cell of the set of cells for FDD scheduling cell, where N=2 for (30,15), (60,30), (120,60) and N=4 for (60,5), (120,30), N = 8 for (120,15) |
| --- | --- | --- | --- |
| 49.NR_MC_enh | 49-3y | Advanced UE capability for larger number of unicast UL DCI | The number of unicast UL DCI to process for a set of cells configured for multi-cell PDSCH scheduling by a DCI format 0_3 and single cell PDSCH scheduling by a DCI format 0_0, DCI format 0_1, and/or DCI format 0_2 - For same SCS between scheduling cell and scheduled cell(s), - two unicast DCIs per slot of scheduling cell for each cell of the set of cells for FDD scheduling cell - four unicast DCIs per slot of scheduling cell for each cell of the set of cells for TDD scheduling cell - For different SCS between scheduling cell and scheduled cell(s), - For low-to-high SCS, a) two unicast DCIs per slot of scheduling cell for each cell of the set of cells for FDD scheduling cell b) four unicast DCIs per slot of each cell for the set of cells for TDD scheduling cell - For high-to-low SCS, a) two unicast DCIs per N consecutive slots of scheduling cell for each cell of the set of cells for FDD scheduling cell, where N=2 for (30,15), (60,30), (120,60) and N=4 for (60,5), (120,30), N = 8 for (120,15) b) four unicast DCIs per N consecutive slots of scheduling cell for each cell of the set of cells for TDD scheduling cell, where N=2 for (30,15), (60,30), (120,60) and N=4 for (60,5), (120,30), N = 8 for (120,15) |

Accordingly, we have the following proposal:

Proposal 5: Define FG 49-3x and FG 49-3y as below:

| 49.NR_MC_enh | 49-3x | Advanced UE capability for larger number of unicast DL DCI | The number of unicast DL DCI to process for a set of cells configured for multi-cell PDSCH scheduling by a DCI format 1_3 and single cell PDSCH scheduling by a DCI format 1_0, DCI format 1_1, and/or DCI format 1_2 - For same SCS between scheduling cell and scheduled cell(s), two unicast DCIs per slot of scheduling cell for each cell of the set of cells for FDD/TDD scheduling cell - For different SCS between scheduling cell and scheduled cell(s), - For low-to-high SCS, two unicast DCIs per slot of scheduling cell for each cell of the set of cells for FDD/TDD scheduling cell - For high-to-low SCS, two unicast DCIs per N consecutive slots of scheduling cell for each cell of the set of cells for FDD scheduling cell, where N=2 for (30,15), (60,30), (120,60) and N=4 for (60,5), (120,30), N = 8 for (120,15) |
| --- | --- | --- | --- |
| 49.NR_MC_enh | 49-3y | Advanced UE capability for larger number of unicast UL DCI | The number of unicast UL DCI to process for a set of cells configured for multi-cell PDSCH scheduling by a DCI format 0_3 and single cell PDSCH scheduling by a DCI format 0_0, DCI format 0_1, and/or DCI format 0_2 - For same SCS between scheduling cell and scheduled cell(s), - two unicast DCIs per slot of scheduling cell for each cell of the set of cells for FDD scheduling cell - four unicast DCIs per slot of scheduling cell for each cell of the set of cells for TDD scheduling cell - For different SCS between scheduling cell and scheduled cell(s), - For low-to-high SCS, a) two unicast DCIs per slot of scheduling cell for each cell of the set of cells for FDD scheduling cell b) four unicast DCIs per slot of each cell for the set of cells for TDD scheduling cell - For high-to-low SCS, a) two unicast DCIs per N consecutive slots of scheduling cell for each cell of the set of cells for FDD scheduling cell, where N=2 for (30,15), (60,30), (120,60) and N=4 for (60,5), (120,30), N = 8 for (120,15) b) four unicast DCIs per N consecutive slots of scheduling cell for each cell of the set of cells for TDD scheduling cell, where N=2 for (30,15), (60,30), (120,60) and N=4 for (60,5), (120,30), N = 8 for (120,15) |

FG 49-5a

In the latest UE feature list, the following components are captured for FG 49-5a, which needs further discussion.

1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_3 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_3 without scheduling a PDSCH using a reserved FDRA value

Basically, the above components are borrowed from FG 10-16 but replacing DCI format 1_1 with DCI format 1_3. There are two issues:

- For the first component, it is too restrictive. DCI format 1_3 can be used to schedule multiple PDSCH on multiple CCs. Hence, component 1) should be updated as ‘Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_3 scheduling at least a PDSCH’.

- For the second component, we think it is too aggressive and need a RAN1 conclusion. Different from DCI format 1_1, the payload size of DCI format 1_3 is very large as it contains scheduling information for a cell set. It doesn’t make sense to use a DCI format 1_3 without scheduling any PDSCH, especially UE can use a DCI format 1_1 without scheduling a PDSCH to trigger type 3 HARQ-ACK codebook.

Proposal 6:  Update the components of FG 49-5a as below:

1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_3 scheduling at least a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_3 without scheduling a PDSCH using a reserved FDRA value

FG 49-5b

In the latest UE feature list, the following components are captured for FG 49-5b, which needs further discussion.

1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_3 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_3 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) 5. Supported maximum number of actual PUCCH transmissions for type 3 or enhanced type 3 HARQ-ACK codebook feedback within a slot

Basically, the above components are borrowed from FG 25-6 but replacing DCI format 1_1 and 1_2 with DCI format 1_3. Although DCI format 1_3 is introduced for MC scheduling, the functionality of enhanced type 3 HARQ-ACK codebook is not changed. Hence, we think the current version for FG 49-5a is sufficient.

Proposal 7:  Confirm the current version of FG 49-5b captured in R1-2312705.

Conclusion

In this contribution, we provide our views on UE features for MC. We have the following proposals:

Proposal 1: Update the note in FG 49-1 as below:

- Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell.

Proposal 2: Adopt the current version of component 9) for FG 49-2 with removing the bracket.

Proposal 3: Update the note in FG 49-2 as below:

- Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell.

Proposal 4: Adopt the current version of component 9) for FG 49-2b with removing the bracket.

Proposal 5: Define FG 49-3x and FG 49-3y as below:

| 49.NR_MC_enh | 49-3x | Advanced UE capability for larger number of unicast DL DCI | The number of unicast DL DCI to process for a set of cells configured for multi-cell PDSCH scheduling by a DCI format 1_3 and single cell PDSCH scheduling by a DCI format 1_0, DCI format 1_1, and/or DCI format 1_2 - For same SCS between scheduling cell and scheduled cell(s), two unicast DCIs per slot of scheduling cell for each cell of the set of cells for FDD/TDD scheduling cell - For different SCS between scheduling cell and scheduled cell(s), - For low-to-high SCS, two unicast DCIs per slot of scheduling cell for each cell of the set of cells for FDD/TDD scheduling cell - For high-to-low SCS, two unicast DCIs per N consecutive slots of scheduling cell for each cell of the set of cells for FDD scheduling cell, where N=2 for (30,15), (60,30), (120,60) and N=4 for (60,5), (120,30), N = 8 for (120,15) |
| --- | --- | --- | --- |
| 49.NR_MC_enh | 49-3y | Advanced UE capability for larger number of unicast UL DCI | The number of unicast UL DCI to process for a set of cells configured for multi-cell PDSCH scheduling by a DCI format 0_3 and single cell PDSCH scheduling by a DCI format 0_0, DCI format 0_1, and/or DCI format 0_2 - For same SCS between scheduling cell and scheduled cell(s), - two unicast DCIs per slot of scheduling cell for each cell of the set of cells for FDD scheduling cell - four unicast DCIs per slot of scheduling cell for each cell of the set of cells for TDD scheduling cell - For different SCS between scheduling cell and scheduled cell(s), - For low-to-high SCS, a) two unicast DCIs per slot of scheduling cell for each cell of the set of cells for FDD scheduling cell b) four unicast DCIs per slot of each cell for the set of cells for TDD scheduling cell - For high-to-low SCS, a) two unicast DCIs per N consecutive slots of scheduling cell for each cell of the set of cells for FDD scheduling cell, where N=2 for (30,15), (60,30), (120,60) and N=4 for (60,5), (120,30), N = 8 for (120,15) b) four unicast DCIs per N consecutive slots of scheduling cell for each cell of the set of cells for TDD scheduling cell, where N=2 for (30,15), (60,30), (120,60) and N=4 for (60,5), (120,30), N = 8 for (120,15) |

Proposal 6:  Update the components of FG 49-5a as below:

1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_3 scheduling at least a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_3 without scheduling a PDSCH using a reserved FDRA value

Proposal 7:  Confirm the current version of FG 49-5b captured in R1-2312705.

Reference

  - R1-2312581, Session Notes for 8.16.12, Ad-Hoc Chair (NTT DOCOMO), RAN1#115 meeting, November 13th – November 17th, 2023

  - R1-2312705, Corrected RAN1 UE features list for Rel-18 NR after RAN1#115, Moderators (AT&T, NTT DOCOMO, INC.), November 13th – November 17th, 2023

  - R1-2312589, Summary on UE features for MC enhancements, Moderator (NTT DOCOMO, INC.), RAN1#115 meeting, November 13th – November 17th, 2023