3GPP TSG RAN WG1 #116 	                                                                                                R1-2400546

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	9.1.3.2

Source:	            		Xiaomi

Title:	Discussion on two-sided AI/ML model based CSI compression

Document for:	Discussion and Decision

Introduction

In the RAN#102 meeting, the following objectives on AI/ML based CSI feedback were achieved [1]. CSI compression with two-sided AI/ML model needs to further study performance gain or reduce computation complexity and feedback overhead.

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain). - Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

In this contribution, performance evaluation and specification impact of CSI compression feedback with two-sided model are respectively discussed.

Discussion on performance evaluation of two-sided AI/ML based CSI compression feedback

Evaluation cases discussion

For CSI compression, multiple solutions are proposed to improve trade-off between performance and complexity/overhead including extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction.

Spatial/temporal/frequency compression

Based on the analysis on the dataset, we found that the CSI/eigenvector of target slot has high relativity with the CSI/eigenvector of neighbor history slots. We calculated the SGCS of eigenvector of slot#50 with the eigenvector of slot#1~slot#49, as Figure 1 shown, most of the UEs have similar eigenvector from slot#40 with eigenvector of slot#50, which implies that the CSI/eigenvector can be further compressed in time domain. The spatial/temporal/frequency compression is expected to have higher performance gain over spatial/frequency compression.

Further evaluations such as model generalization, scalability and separate training can be considered if better performance is observed on spatial/temporal/frequency compression.

Proposal 1: Companies are encouraged to provide intermediate results and UPT results of SFT compression

Proposal 2: Companies are encouraged to provide intermediate results of generalization/scalability performance for SFT compression.

Proposal 3: Companies are encouraged to provide intermediate results of different training types of SFT compression.

Figure 1: SGCS of eigenvector of target slot and neighbor history eigenvector

Cell/site specific models

From our understanding, the cell/site specific models is useful in reducing the model complexity. However, based on the evaluation assumption agreed in Rel-18, it is hard to generate the cell/site specific dataset assuming the scenario and other network architecture parameters are same. And it is hard to differentiate the models without cell/site specific dataset.

Observation 1: It is not clear how to generate the cell/site specific dataset based on current evaluation assumption.

CSI compression plus prediction

It is expected that CSI predication plus CSI compression can achieve higher performance gain than CSI compression only. However, it is not sure whether CSI prediction is better than Rel-18 non-AI/ML based approach, e.g., Rel-18 MIMO codebook. If no performance gain over Rel-18 non-AI/ML based approach is observed on CSI prediction, it is less likely that the performance of CSI prediction plus CSI compression is better than CSI compression only. Therefore, we propose to study AL/ML based CSI prediction only first, if AL/ML based CSI prediction performance gain over Rel-18 non-AI/ML based approach is observed, further study the performance of CSI prediction plus CSI compression.

Proposal 4: If AI/ML based CSI prediction performance gain over Rel-18 non-AI/ML based approach is observed, further study the performance of CSI prediction plus CSI compression.

SFT CSI compression

In this section, we provide our evaluation methodology and initial evaluation results on spatial/temporal/frequency compression.

AI model description

As Figure 2 shown, the AI model is LSTM and Transformer based. Different with AI model for spatial/frequency compression, the input of AI model for spatial/temporal/frequency compression includes both the CSI/eigenvector of current slot and accumulated information from historic slot. And the output of encoder is the compressed CSI/eigenvector after quantization, while an intermediate output of LSTM part of encoder is the accumulated information, which would be used as input for the next slot. Similar as the encoder part, the decoder needs to recovery the CSI/eigenvector based on both compressed CSI/eigenvector and the historic accumulated information calculated in the decoder side.

Figure 2: Structure of AI model for spatial/temporal/frequency compression

The FLOPs and the number of parameters are used for complexity and storage comparison between spatial/frequency compression and spatial/temporal/frequency compression. Table 1 shows the complexity and parameter numbers of different AI models. It is observed that the complexity of SFT compression is higher than SF compression, while the number of parameters of SFT compression is less than SF compression. Table 2 shows the parameters for AI model training.

Table 1: Complexity and memory storage of AI model

|  |  | CSI generation part | CSI reconstruction part |
| --- | --- | --- | --- |
| SF | FLOPs | 279 M | 279 M |
|  | Number of AI/ML model parameters | 10.79 M | 10.79 M |
| SFT | FLOPs | 754 M | 754 M |
|  | Number of AI/ML model parameters | 6.89 M | 6.89 M |

Table 2：Parameter for AI model training

| AI training parameter | Value |
| --- | --- |
| Quantization | Scalar quantization |
| Loss function | SGCS |
| Learning rate | 0.0001 |
| Optimizer | Adam |
| Epoch | 200 |
| Batch size | 128 |

Evaluation assumption and results

The evaluation assumption for dataset generation is listed in Table 3 and the SGCS results for SF and SFT CSI compression is shown is Table 4. Taking eTypeII codebook as baseline, when the feedback payload is less than 80bits, the performance gain of SF compression is around 4% while the performance gain of SFT compression is around 5% for layer 1, the performance gain of SF compression is around 12% while the performance gain of SFT compression is around 15% for layer 2. When the feedback payload is around 120 bits, the performance gain of SF compression is around 2.7% while the performance gain of SFT compression is around 3.8% for layer 1, the performance gain of SF compression is around 3.7% while the performance gain of SFT compression is around 6.9% for layer 2. When the feedback payload is larger than 230bits, the performance gain of SF compression is around 0.7% while the performance gain of SFT compression is around 1.6% for layer 1, the performance gain of SF compression is around 0.4% while the performance gain of SFT compression is around 2.8% for layer 2.

Generally, it is observed that the SFT compression has higher performance gain than SF compression, and the gap is larger in layer2.

Observation 2: The SFT compression has higher performance gain than SF compression, and the gap between SFT compression and SF compression is larger in layer 2.

Table 3： Simulation assumption for dataset generation

| Parameter | Value |
| --- | --- |
| Scenario | UMa |
| Frequency Range | 2GHz |
| Bandwidth | 10MHz |
| Subcarrier spacing | 15KHz |
| Antenna setup and port layouts at gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| Dataset type | Eigenvector (layer1, layer2) |
| Channel estimation | Ideal |
| UE distribution | 80% indoor (3km/h), 20% outdoor (30km/h) |

Table 4：SGCS results for SF and SFT CSI compression

| Payload | Case | SGCS |  |
| --- | --- | --- | --- |
|  |  | Layer 1 | Layer 2 |
| X: <=80bits | eTypeII | 0.7049 | 0.5392 |
|  | SF | 0.7342 | 0.6041 |
|  | SF Gain | 0.0293(4.16%) | 0.0649(12.04%) |
|  | SFT | 0.7402 | 0.6212 |
|  | SFT Gain | 0.0353(5.01%) | 0.082(15.21%) |
| Y: 100bits-140bits | eTypeII | 0.7801 | 0.6574 |
|  | SF | 0.8010 | 0.6815 |
|  | SF Gain | 0.0209(2.68%) | 0.0241(3.67%) |
|  | SFT | 0.8097 | 0.7025 |
|  | SFT Gain | 0.0296(3.79%) | 0.0451(6.86%) |
| Z: >=230bits | eTypeII | 0.8682 | 0.7835 |
|  | SF | 0.8746 | 0.7868 |
|  | SF Gain | 0.0064(0.74%) | 0.0033(0.42%) |
|  | SFT | 0.8821 | 0.8054 |
|  | SFT Gain | 0.0139(1.60%) | 0.0219(2.80%) |

Different with SF compression, SFT compression introduced the time domain information into the compression. Therefore, we further evaluated the impact of time domain length in compression. Keep the training time step number unchanged as 20 CSI, which means that the 20th CSI is compressed using the historic CSIs from the 1st CSI to the 19th CSI during the training. While in the test, different testing time steps are evaluated including time step equals to 1,2,10,20,50,100 and 200 CSIs. When the testing time step is X, the Mth CSI is compressed using the historic CSIs from the 1st CSI to the M-1th CSI, wherein M<=X.

As Figure 3 shown, the performance of SF compression is not impacted by the time step, while the performance of SFT compression is increased with the time step length, and keep unchanged after reached the peak when the time step length is 100. When the time step is not larger than 2, the performance of SFT compression is worse than SF compression, which indicates that the historic CSI information does not have a positive effect in the compression when the historic CSI information is very limited. The performance of SFT compression becomes better with the larger time step, which indicates that the historic CSI information have a positive effect in the compression when the historic CSI information is sufficient.

Observation 3: The historic CSI information does not have a positive effect in SFT compression when the historic CSI information is very limited.

Observation 4: The historic CSI information have a positive effect in the compression when the historic CSI information is sufficient, the performance of SFT compression increased with larger historic CSI information.

Figure 3: SGCS results for different testing time steps

Specification impact for CSI compression feedback with two-sided AI/ML model

Discussion on inter-vendor training for two-sided model

In the RAN1#115 meeting, companies have achieved the consensus for almost all pros and cons of training Type 1, Type 2 and Type 3. However, there are still remained issues to be further discussed. In this section, we provide our view on these remained issues.

Discussion on remained issues for Type 1

- Extendibility: To train new UE-side model compatible with NW-side model in use for Type 1 at UE side

Before discussing its feasibility, it should clarify whether the UE has transmitted or delivered encoder (UE-side model) to NW. Assume that the user has trained encoder and decoder (NW-side model), and the trained decoder is transmitted to NW by UE. Then, it is feasible that the user to train new encoder which is compatible with decoder in use, since the decoder could be freezed during training the new encoder at UE side. Hence, the extendibility could be supported if the user has the NW-side model in use. But, if the UE is a new user in one cell, the user does not own the decoder in use at NW side. It is obvious that UE could not train new encoder which is compatible with decoder.

- Extendibility: To train new NW-side model compatible with UE-side model in use for Type 1 at NW side

Similarly, if the network has the UE-side model, NW could also train the new NW-side model which is compatible with UE-side model in use by freezing the UE-side model during training. Therefore, the extendibility could be supported if the NW has the UE-side model in use.

Proposal 5: In CSI compression using two-sided model use case, the following Table 5 captures the pros/cons of extendibility for training collaboration Type 1:

Table 5: Pros and Cons of training collaboration Type 1

| Characteristics \ Training Types | Type 1: NW side |  | Type 1: UE side |  |
| --- | --- | --- | --- | --- |
|  | Unknown model structure at UE | Known model structure at UE | Unknown model structure at UE | Known model structure at UE |
| Whether model can be kept proprietary | No | No | No | No |
| Whether require privacy-sensitive dataset sharing | No (Note 1) | No (Note 1) | No (Note 1) | No (Note 1) |
| Flexibility to support cell/site/scenario/configuration specific model | Flexible except for UE defined scenarios. Not flexible for UE defined scenarios unless UE assistance information is supported and available. (Note 6) | Flexible except for UE defined scenarios. Not flexible for UE defined scenarios unless UE assistance information is supported and available. (Note 6) | Flexible except for NW defined scenarios. Not flexible for NW defined scenarios unless NW assistance information is supported and available. (Note 6) | Flexible except for NW defined scenarios. Not flexible for NW defined scenarios unless NW assistance information is supported and available. (Note 6) |
| Whether gNB/device specific optimization is allowed | gNB: Yes UE: No | gNB: Yes UE: less flexible compared to UE side | gNB: No UE: Yes | gNB: less flexible compared to NW side UE: Yes |
| Model update flexibility after deployment | Flexible only if UE supports the new structure | Flexible for parameter update | Flexible, less flexible than Type 1 NW side | Flexible for parameter update, less flexible than Type 1 NW side |
| Feasibility of allowing UE side and NW side to develop/update models separately | gNB: Feasible UE: Not feasible due to Type 1 definition. | gNB: Feasible with restriction for CSI reconstruction model. UE: Not feasible due to Type 1 definition. | gNB: Not feasible due to Type 1 definition. UE: Feasible. | gNB: Not feasible due to Type 1 definition. UE: Feasible with restriction for CSI generation model. |
| Whether gNB can maintain/store a single/unified CSI reconstruction model over different UEs (Note 2) | Yes | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” of clause 6.2.2.4 (Note 4) | No | No |
| Whether UE device can maintain/store a single/unified CSI generation model over different NW vendors (Note 3) | No | No | Yes | Yes. Performance refers to observations in “1 UE part model to N>1 NW part models” of clause 6.2.2.4 (Note 4) |
| Extendibility: to train new UE-side model compatible with NW-side model in use; (Note 5) | Yes | Yes | No consensus Yes (Note x1) | No consensus Yes (Note x1) |
| Extendibility: To train new NW-side model compatible with UE-side model in use; (Note 5) | No consensus Yes (Note x2) | No consensus Yes (Note x2) | Yes | Yes |
| Whether training data distribution can match the inference device | Limited | Limited | Yes | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | No for UE | Yes | No for NW | Yes |
| Model performance | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 |

Note 1: Assume precoding matrix is not privacy sensitive data. FFS: other information such as channel matrix and assisted information.

Note 2: Whether gNB/UE needs to maintain/store multiple CSI generation/reconstruction models respectively, is not discussed.

Note 3: For model inference, UE does not need to use multiple models from different NW vendors per cell.

Note 4: One to many joint trainings is assumed.

Note 5: The performance of the new model is similar to the performance of sequential training when training Type 1 support freezing a part of two sided model.

Note 6: For this table, NW defined scenarios are scenarios with NW defined dataset categorization. UE defined scenarios are scenarios with UE defined dataset categorization.

Note x1: The user which is to train new UE-side model has the NW-side model in use.

Note x2: The network which is to train new NW-side model has the UE-side model in use.

Discussion on remained issues for Type 2 and Type 3

- Flexibility to support cell/site/scenario/configuration specific model

For Type 2, UE-side model and NW-side model are respectively trained at UE side and NW side. To support cell or configuration specific model, a lot of UEs needs to be involved in the model training. This requires significant cross-vendor co-engineering efforts compared with Type 1 or Type 3. From this perspective, it is not flexibility for Type 2 to support cell/site/scenario/configuration specific model. Different cells or configurations correspond to different dataset. For Type 3, the training dataset could be transmitted to opposite side. In addition, specific dataset corresponding to a cell or configuration could be defined or chartered by using the indicated assistance information, such that the trained UE-side or NW-side model to flexibly support different cells or configurations. The co-engineering efforts is limited. Hence, it is flexible to support cell/site/scenario/configuration specific model for Type 3.

- Model update flexibility after deployment (Note 4)

For sequential method of Type 2, the procedure is similar with Type 3 with NW-first. If model at NW side is updated, then corresponding UE-side model should be updated as well. This implies that a lot of users need to be involved in the model training. Hence, significant co-engineering efforts are required. Compared with Type 1, it should be semi-flexible for Type 2 with sequential training.

- Feasibility of allowing UE side and NW side to develop/update models separately

Different from Type 3, developing or updating a UE-side or NW-side model requires the opposite side to be involved in model training for Type 2, since the FP gradient or BP gradient should be delivered the opposite side. Hence, it is infeasible to independently develop or update UE-side or NW-side model.

- Extendibility: To train new UE-side model compatible with NW-side model in use for Type 3 with UE-first; Extendibility: To train new NW-side model compatible with UE-side model in use for Type 3 with NW-first.

The discussion is similar with that of the extendibility for Type 1. It should also firstly clarify whether the UE has transmitted or delivered encoder (UE-side model) to NW. If the user has trained encoder and decoder, it is feasible that the user to train new encoder which is compatible with decoder in use at NW side. Accordingly, if the network has trained encoder and decoder, it is feasible that the network to train new decoder which is compatible with encoder in use at UE side.

- Whether training data distribution can match the inference device

For Type 2 with simultaneous training, the training dataset may be collected from different inference devices. The training data distribution can match the devices only if the training dataset is from those inference devices. Hence, it is limit to training data distribution can match the inference device.

Proposal 6: In CSI compression using two-sided model use case, the following Table 6 captures the pros/cons of training collaboration Type 2 and Type 3:

Table 6: Pros and Cons of training collaboration Type 2 and Type 3

| Characteristics \ Training Types | Type 2 |  | Type 3 |  |
| --- | --- | --- | --- | --- |
|  | Simultaneous | Sequential | NW first | UE first |
| Whether model can be kept proprietary | Yes (Note 1) | Yes (Note 1) | Yes (Note 1) | Yes (Note 1) |
| Whether requires privacy-sensitive dataset sharing | No (Note 2) | No (Note 2) | No (Note 2) | No (Note 2) |
| Flexibility to support cell/site/scenario/configuration specific model | No consensus Not flexible | No consensus Not flexible | [Semi] flexible except for UE defined scenarios. (Note 3) [Semi] flexible for UE defined scenarios if UE assistance information is supported and available. | [Semi] flexible except for NW defined scenarios (Note 3). [Semi] flexible for NW defined scenarios if NW assistance information is supported and available. |
| Whether gNB/device specific optimization is allowed | Yes | Yes | Yes | Yes |
| Model update flexibility after deployment (Note 4) | Not flexible | No consensus Semi-flexible | Semi-flexible | Semi-flexible |
| Feasibility of allowing UE side and NW side to develop/update models separately | Infeasible | No consensus Infeasible | Feasible | Feasible |
| Whether gNB can maintain/store a single/unified model over different UE vendors (Note 5) | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” and “1 UE part model to N>1 NW part models” of clause 6.2.2.4. | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” of clause 6.2.2.4. | Yes. Performance refers to observations in “NW first training, 1 NW part model to 1 UE part model, same backbone” and “NW first training, 1 NW part model to 1 UE part model, different backbones” of clause 6.2.2.5. | Yes. Performance refers to observations in “UE first training, M>1 UE part models to 1 NW part model” of clause 6.2.2.5. |
| Whether UE device can maintain/store a single/unified CSI generation model over different NW vendors (Note 6) | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” and “1 UE part model to N>1 NW part models” of clause 6.2.2.4. | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” of clause 6.2.2.4. | Yes. Performance refers to observations in “NW first training, 1 UE part model to N>1 NW part models” of clause 6.2.2.5. | Yes. Performance refers to observations in “UE first training, 1 NW part model to 1 UE part model, same backbone”. And “UE first training, 1 NW part model to 1 UE part model, different backbones” of clause 6.2.2.5. |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Not support | Support | Support | No consensus Support (Note x1) |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Not support | Not support | No consensus Support (Note x2) | Support |
| Whether training data distribution can match the inference device | No consensus Limited | Yes for UE-part model, limited for NW-part model | Limited | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | Compatible | Compatible | Compatible | Compatible |
| Model performance | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 |

Note 1: Assume information on model structure disclosed in training collaboration does not reveal proprietary information.

Note 2: Assume precoding matrix is not privacy sensitive data. FFS: other information such as channel matrix and assisted information.

Note 3: For this table, NW defined scenarios are scenarios with NW defined dataset categorization. UE defined scenarios are scenarios with UE defined dataset categorization. [Semi] means no consensus for including “semi”.

Note 4: Flexibility after deployment is evaluated by the amount of offline cross-vendor co-engineering effort. Flexible indicates minimum additional co-engineering between vendors, semi-flexible indicates additional co-engineering effort between vendors.

Note 5: Whether gNB/UE needs to maintain/store multiple CSI generation/reconstruction models respectively, is not discussed.

Note 6: For model inference, UE does not need to use multiple models from different NW vendors per cell.

Note x1: The user which is to train new UE-side model has the NW-side model in use.

Note x2: The network which is to train new NW-side model has the UE-side model in use.

If AI/ML model is jointly trained by using Type 1 at UE side or Type 3 with UE-first, NW-side will store different AI/ML models for different UE vendors. Even for the same UE vendor, it may train different two-sided models if a generalized or scalability two-sided model cannot be trained for different scenarios or configurations. This implies that NW-sided will store and manage a lot of NW-side and UE-side models, which will be a challenge task for network. For Type 2, it has agreed that it should be study with low priority. Therefore, Type 1 at NW side and Type 3 with NW-first are suggest to study with higher priority.

Observation 5: NW may store and manage a lot of two-sided models for training collaboration Type 1 at UE side and Type 3 with UE-first.

Proposal 7: Both training Type 1 at NW side and Type 3 with NW-first are suggested to study with higher priority.

Discussion on the specification impact for two-sided model based CSI compression feedback

According to discussion in Rel-18, the conclusion on SF CSI compression were given in section 8 of TR 38.843 [2].

Potential specification impact on NW side/UE side data collection, dataset delivery, quantization alignment between CSI generation part at the UE and CSI reconstruction part at the NW, CSI report configuration, CSI report format, pairing information/procedure and monitoring approach were investigated but not all aspects were identified.

Both NW side and UE side performance monitoring were studied, some but not all aspects were concluded.

Although data collection, performance monitoring, CSI report configuration, model pairing, etc, have been investigated, some remained issues for them need to further study. In this section, these remained issues are respectively discussed.

Data collection

The following agreements on data collection were achieved [3]-[4]. Dataset for training or performance monitoring could be collected at UE or NW side. The necessity or feasibility needs to further study.

Agreement UE side data collection: In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact of UE side data collection enhancement including at least - Enhancement of CSI-RS configuration to enable higher accuracy measurement. - Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc. - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side. - Signaling for triggering the data collection NW side data collection: In CSI compression using two-sided model use case, further discuss the necessity, feasibility, and potential specification impact for NW side data collection including at least: - Enhancement of SRS and/or CSI-RS measurement and/or CSI reporting to enable higher accuracy measurement. - Contents of the ground-truth CSI including: - Data sample type, e.g., precoding matrix, channel matrix etc. - Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like). - Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator) - Latency requirement for data collection - Signaling for triggering the data collection Agreement In CSI compression using two-sided model use case, further study the necessity, complexity, overhead, latency and potential specification impact on ground truth CSI report for NW side data collection for model performance monitoring, including: - Scalar quantization for ground-truth CSI - FFS: any processing applied to the ground-truth CSI before scalar quantization - Codebook-based quantization for ground-truth CSI - FFS: Parameter set enhancement of existing eType II codebook, based on evaluation results in 9.2.2.1 - RRC signaling and/or L1 signaling procedure to enable fast identification of AI/ML model performance - Aperiodic/semi-persistent or periodic ground-truth CSI report.

According to above discussion in section 3.1, two-sided AI/ML model could be trained by using training collaboration Type 1, Type 2 and Type 3. Type 1 at NW side and Type 3 with NW-first are suggest to study with high priority. For Type 1 at NW side and Type 3 with NW-first, it is necessary to collect dataset for model training or performance monitoring at NW side. In FDD systems, ground truth CSI for mode training is estimated by using downlink pilot signal at UE side. The obtained ground truth CSI needs to be quantized and reported to NW.  Both scalar quantization and codebook-based quantization could be used for ground-truth CSI. Compared with scalar quantization, codebook-based quantization could save a lot of overhead. At the same time, there is no much performance loss by using enhanced eType II codebook parameters, as given in TR 38.843 [2].

In current specification, CSI-RS resource is UE-specific reference signal. This means that CSI-RS resource is configured for each user. For network side data collection, this will consume a lot of signalling overhead. In addition, the configured CSI-RS resource per UE may be different. In order to avoid additional pre-processing of training data, dimension of ground truth CSI should be same as far as possible. E.g., the number of CSI-RS resource ports is same for all users. The straightforward way is that CSI-RS resource is configured per cell for all users. It can not only reduce signalling overhead, but also make the dimension of ground truth CSI be same. Since two-sided model could be trained via offline, there is no strict requirement on latency of data collection. The ground truth CSI could be aperiodically /semi-persistently or periodically reported by using L1 or RRC signalling. Even if the collected data is used for performance monitoring, the latency of data collection could still be accepted. Hence, it is feasible that data is collected at NW side.

Proposal 8: At least for training Type 1 at NW side and Type 3 with NW-first, it is necessary and feasibility for NW side data collection considering the following aspects:

- Significant feedback overhead reduction by using codebook-based quantization.

- Signalling overhead reduction by using cell-specific CSI-RS resource configuration.

- No much strict latency requirement for data collection for model training or performance monitoring.

Performance monitoring

In [3]-[4], the following agreements on NW-side or UE-side performance monitoring were identified. The necessity and feasibility for performance monitoring by using intermediated KPIs or an existing CSI feedback scheme as the reference need to further study.

Agreement In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact for intermediate KPIs based monitoring including at least: - NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side. - UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - UE-side monitoring based on the output of the CSI reconstruction model at the UE-side - Note: CSI reconstruction model at the UE-side can be the same or different comparing to the actual CSI reconstruction model used at the NW-side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - FFS: Other solutions, e.g., UE-side uses a model that directly outputs intermediate KPI. Network-side monitoring based on target CSI measured via SRS from the UE. Note: Monitoring approaches not based on intermediate KPI are not precluded Note: the study of intermediate KPIs based monitoring should take into account the monitoring reliability (accuracy), overhead, complexity, and latency. Agreement In CSI compression using two-sided model use case, for NW-side monitoring, further study the necessity, feasibility and potential specification impact to enable performance monitoring using an existing CSI feedback scheme as the reference. - The association between AI/ML scheme and existing CSI feedback scheme for monitoring - Note: The metric for monitoring and comparison includes intermediate KPI and eventual KPI. - Other aspects are not precluded.

Considering the UE experienced channel variation, the two-sided AI/ML in use may result significant performance loss, which leads the current two-sided model is not applicable. In such case, NW needs to deactivate the model or switch the other two-sided model. Hence, it is necessary to monitor performance of two-sided model. According to above agreement on performance monitoring, both UE and NW could monitor performance based on intermediate KPIs. For UE-side model performance monitoring, the output of the CSI reconstruction model at NW side needs to transfer to UE if there is no CSI reconstruction model of NW at UE side. Accordingly, the target CSI needs to report to NW for NW-side model performance monitoring. Note that the target CSI or output of CSI reconstruction model could be quantized by using enhanced eType II codebook parameters, which could save significant feedback overhead. Since the procedure of codebook-based quantization is similar with that of PMI calculation, the quantization complexity for target CSI or output of CSI reconstruction model is affordable for UE.

The performance degradation may not be incurred by two-sided AI/ML model. The channel deterioration of the user experience could also result in performance loss. NW may not provide reliable decision just based on the reported target CSI. In this way, existing CSI feedback, e.g., eType II codebook feedback, could be utilized to enable the performance monitoring robustness. In current specification, eType II codebook feedback has been supported. Hence, it is necessary and feasible to monitor performance by using an existing CSI feedback scheme as the reference.

Proposal 9: It is necessary and feasible that performance monitoring by using intermediated KPIs or an existing CSI feedback scheme as the reference considering the following aspects:

- Significant overhead reduction for quantization of the target CSI or output of CSI reconstruction via enhanced eType II codebook parameters

- Affordable complexity for quantization of the target CSI or output of CSI reconstruction, which is similar to that of legacy eType II codebook.

- Ensuring the robust of monitoring performance by using an existing CSI feedback scheme as the reference

CSI reporting

CSI configuration and reporting related agreements are summarized in TR 38.843 [2]. The remained issues on CSI reporting enhancement, e.g., CSI priority or CSI omission need to further study. According to the agreed SID in [1], extending the SF domain compression to SFT domain compression is provided to further improve system performance. The potential spec impact of two-sided model inference for CSI reporting after introducing time domain information for CSI compression is discussed as well.

Potential specification enhancement on CSI feedback enhancement summarized in TR 38.843. - CSI-RS configurations (not including CSI-RS pattern design enhancements) - CSI configuration - For network to indicate CSI reporting related information, e.g., gNB indication to the UE of one or more of following: - Information indicating CSI payload size - Information indicating quantization method/granularity - Rank restriction - Other payload related aspects - CSI reporting configurations - For UE determination/reporting of the actual CSI payload size, UE reports related information as configured by the NW - CSI report UCI mapping/priority/omission - CSI processing procedures

CSI priority

When multiple CSI reports are sent to gNB at same time, CSI collision may occur, which results partial CSI reports are dropped. In current specification, CSI reporting is dropped through the predefined priority rule, and the rule is defined through the expression PriiCSIy,k,c,s=2∙Ncells∙Ms∙y+Ncells∙Ms∙k+Ms∙c+s. Each CSI report is associated with a priority value PriiCSIy,k,c,s. The smaller value PriiCSIy,k,c,s, the higher priority CSI reports.

The functionality of AI/ML-based CSI compress feedback is same with that of legacy CSI feedback. From this perspective, it is straightforward to legacy priority rule is reused for the priority of CSI reporting based on two-sided AI/ML model. However, once legacy CSI reporting and AI/ML-based CSI reporting may be transmitted to gNB through different CSI reports at the same time. For such case, how to drop them needs to be addressed. In our view, the following two methods can be considered to determine the priority of AI/ML-based CSI feedback.

- Alt1: A priority value PriiCSIy,k,c,s with new parameter value

- Alt2:  Introducing new parameter PriiCSIy',y,k,c,s

For Alt1, it is straightforward to determine the priority of AI/ML-based CSI feedback. For example, k=0 and k=1 respectively denotes L1-RSRP or L1-SINR reporting and other than L1-RSRP or L1-SINR reporting. In order to denote the priority of AI/ML-based CSI compression feedback, k can set to 0, 1 and 2, where k=0 still denotes L1-RSRP or L1-SINR reporting, let k=1 denote the AI/ML-based CSI compression reporting, and k=2 denote other than L1-RSRP or L1-SINR reporting and AI/ML-based CSI compression reporting.

For Alt2, a new parameter y' is introduced to denotes AI/ML-based CSI compression reporting. E.g., PriiCSIy,k',k,c,s= α∙Ncells∙Ms∙My∙y+β∙Ncells∙Ms∙My'∙y'+γ∙Ncells∙Ms∙k+Ms∙c+s. My' denotes the number of AI/ML-based CSI reporting, y'=0 denotes the AI/ML-based beam reporting, y'=1 denotes AI/ML based CSI reporting other than AI/ML-based beam reporting. Both alternatives can be considered to determine the priority of AI/ML-based CSI reporting.

Proposal 10: The legacy priority rule can be reused to define the priority the AI/ML based CSI reporting, and a priority value PriiCSIy,k,c,sPriiCSIy,k,c,s PriiCSIy,k,c,swith new parameter value or introducing new parameter  PriiCSIy',y,k,c,s is used to indicate the priority of AI/ML based CSI reporting.

CSI omission

When there is no enough uplink resource for transmitting all contents in a CSI reporting, CSI omission will occur. Then, partial contents in the CSI reporting need to be dropped. However, if some important CSI parts are dropped, it will result in significant performance loss. In order to address this issue, CSI Part 2 for eType II codebook is divided into three groups with different omission priority. It has agreed that CSI reporting based on AI/ML model also includes two parts as a starting point. In our view, the CSI Part 2 based on AI/ML model should also be divided into 1<N groups for CSI omission. The question is how to divide the compressed CSI into N groups.

Different from eType II codebook, AI/ML-based CSI compression feedback may not include indication information of SD basis, FD basis and non-zero coefficients. The legacy group method for eType II codebook cannot be directly adopted. Before providing the group method, it should firstly discuss the payload structure of compressed CSI. For the two-sided AI/ML model, it includes layer-common model, layer-specific model, rank-common model and rank-specific. For different types of two-sided model, the payload structures of CSI compression information may be different as well. In our view, the payload structure of layer-specific and layer-common is similar, while the structure of rank-specific and rank-common may be same. For layer-specific and layer-common, the payload of CSI part 2 could be arranged in layer by layer. E.g., the first layer belongs to the first group, the second layer belongs to the second group, and so on. For rank-specific and rank-common, the payload of CSI part 2 could be divided into N parts. Each part is equal to L/N, where L is the length of CSI part 2. Then, the first part belongs to the first group, the second part belongs to the second group, and so forth.

Proposal 11: The compressed CSI part 2 should be divided into 1<N groups for CSI omission. How to divide compressed CSI part 2 into N groups needs to further study.

Two-sided model inference for CSI reporting

For SF domain CSI compression feedback at time t, the input of encoder only needs the estimated CSI. However, in addition to the estimated CSI, the historic CSI should be as an additional input of encoder for SFT domain CSI compression. Figure 4 shows the illustration of SFT domain CSI compression procedure. In the figure, vt and vt' respectively denote the channel eigenvector and the recovered channel eigenvector by decoder for l-th data transmission layer at time t. Both at and bt respectively denote the output of historic CSI for encoder and decoder at time t corresponding to the l-th layer. Then, the output of historic CSI is applied to the input of encoder or decoder at time t+T, such that historic time domain channel information is utilized for improving system performance. At time t, we can observe that both encoder and decoder also need the input of historic CSI which corresponding to the output of historic CSI at time t-T.

Figure 4: The illustration of SFT domain CSI compression procedure

Assume the designed encoder and decoder are layer common AI/ML model, i.e., the same two-sided AI/ML model is applicable to compress the channel vectors for each layer. At time t-T, if UE only compresses the channel vector of the 1st layer, it leads that there is no output of historic CSI for 1<l-th layer. As a result, both encoder and decoder could not utilize the historic CSI for 1<l-th layer at time t. The system performance may be degraded due to the lost historic CSI. Therefore, it is necessary to study how to design the input of historic CSI if there is no output of historic CSI at the previous instance. One simple way is that the input of historic CSI is reset as the initial value or reuses the latest output of historic CSI before time t. Other methods could be further studied.

Proposal 12: If there is no output of historic CSI at the previous instance, how to design the current input of historic CSI for two-sided AI/ML model needs to study.

Two-sided AI/ML model identification

During Rel-18 discussion on general framework of AI/ML model, functionality-based LCM and model-ID-based LCM are provided in [2]. It has not agreed that which type of LCM is used for two-sided AI/ML model.

In functionality-based LCM, functionality needs to be identified between UE and network side. For functionality identification of two-sided model, UE could indicate its supported functionalities via capability reporting. One functionality could include one or multiple AI/ML models. For two-side AI/ML model, one decoder (encoder) may be paired with multiple encoder (decoder). However, different encoders may correspond to different functionalities. It is not flexible to regard the multiple encoders as one functionality identification.

In model-ID-based LCM, model needs to be identified between UE and network side via model ID. UE can indicate its supported AI/ML model by reporting the corresponding model ID. For two-side AI/ML model, if one decoder is paired with multiple encoders at UE side, UE could indicate these encoders to network by reporting encoder ID. Compared with functionality-based LCM, model-ID-based LCM is more simple and straightforward. Hence, at least model-ID-based LCM for two-side AI/ML model should be supported.

Proposal 13: At least model-ID-based LCM for two-sided AI/ML model should be supported.

Different from one side AI/ML model, two-part AI/ML model needs to respectively deploy at UE and NW side for two-side model, and the two-part AI/ML model should be compatible or paired. During Rel-18 discussion on specification impact of two-sided AI/ML model, different pairing information is provided for selecting compatible encoder or decoder. The pairing information could be encoder/decoder ID, encoder and decoder pair ID, dataset ID or session ID. It has also agreed that pairing information can be established based on model identification. However, how to identify two-sided AI/ML for UE and NW side has not been agreed.

The following agreements on model identification have been achieved during discussion on general framework of AI/ML model.

Agreement

Type A: Model is identified to NW (if applicable) and UE (if applicable) without over-the-air signalling

-	The model may be assigned with a model ID during the model identification, which may be referred/used in over-the-air signalling after model identification.

-	Type B: Model is identified via over-the-air signalling,

-	Type B1:

-	Model identification initiated by the UE, and NW assists the remaining steps (if any) of the model identification

-	the model may be assigned with a model ID during the model identification

-	Type B2:

-	Model identification initiated by the NW, and UE responds (if applicable) for the remaining steps (if any) of the model identification

-	the model may be assigned with a model ID during the model identification

-	Note:  This study does not imply that model identification is necessary.

According to discussion on training method of two-sided model, one decoder (encoder) may be paired with multiple encoder (decoder). This implies that there are still multiple two-sided AI/ML models even though the functionality for them is same.

Type A could be used to identify two-side model between UE and network without over-the-air signalling. There is no specification impact from RAN1 perspective. However, there are two questions for Type A.

- Q1: For Type A, all of two-sided models for different scenarios need to identify. In fact, for a scenario, it is not necessary to identify the two-sided models which belongs to other scenarios for.

- Q2: If two-sided model is updated via online, the associated signalling, dataset or model transfer need to transmit over-the air.  Hence, it is infeasible to use Type A for model identification after model update.

Both Type B1 and Type B2 could be considered to address above questions. According to discussion on training collaboration types in section 3.1, Type 1, Type 2 and Type 3 could be used to train the two-side AI/ML model via offline training. Type A could be used to identify the two-sided AI/ML model during model training. The trained two-sided AI/ML model may be assigned with an encoder/decoder ID, dataset ID, session ID or pair ID for model identification. In addition, each AI/ML model may be associated with some specifical conditions or configurations. The condition could be scenario, e.g., UMa or indoor. Table 7 shows an example of two-side AI/ML model mapping and associated condition after model identification by using Type A.

Table 7:  An example of Two-side AI/ML model pairing and associated condition

| UMa |  | Indoor |  |
| --- | --- | --- | --- |
| Pair/ Dataset/Session ID 1 | Pair/ Dataset/Session ID 2 | Pair/ Dataset/Session ID3 | Pair/ Dataset/Session ID4 |
| Decoder ID 1 |  | Decoder ID 2 |  |
| Encoder ID 1 | Encoder ID 2 | Encoder ID 3 | Encoder ID 4 |

We can observe that there are four two-sided AI/ML models in the Table, and each decoder is compatible with two encoders. The scenario of the user is fixed after the user access into a cell. Those AI/ML models which is not related with the scenario is not necessary to identify. This could save signalling overhead or make model identification be easy. In such case, Type B2 is a straightforward method for model identification, since NW can indicate the scenario information to UE. Then, UE will only report the two-sided AI/ML models associated with the scenario.

Based on performance monitoring result, UE or NW could update encoder/decoder by using training collaboration Type 3. Model may be identified between UE and network according to the training dataset for Type 3. Training dataset collection could be initiated by UE or NW. After updating encoder or decoder, UE or network could make a decision whether identify the updated encoder or decoder. Hence, identifying the updated two-side AI/ML model should adopt Type B1 or Type B2. The detail mechanisms or procedures for model identification need to further study.

Proposal 14: Type B1 or Type B2 could be considered to identify two-sided AI/ML model between UE and network. The mechanisms or procedure for model identification of two-sided AI/ML model by using Type B1 or Type B2 needs to study.

Conclusions

In this contribution, the proposals and observations on CSI compression feedback are summarised as follows:

Performance evaluation of two-sided AI/ML model

Observation 1: it is not clear how to generate the cell/site specific dataset based on current evaluation assumption.

Observation 2: The SFT compression has higher performance gain than SF compression, and the gap between SFT compression and SF compression is larger in layer 2.

Observation 3: The historic CSI information does not have a positive effect in SFT compression when the historic CSI information is very limited.

Observation 4: The historic CSI information have a positive effect in the compression when the historic CSI information is sufficient, the performance of SFT compression increased with larger historic CSI information.

Proposal 1: Companies are encouraged to provide intermediate results and UPT results of SFT compression

Proposal 2: Companies are encouraged to provide intermediate results of generalization/scalability performance for SFT compression.

Proposal 3: Companies are encouraged to provide intermediate results of different training types of SFT compression.

Proposal 4: If AI/ML based CSI prediction performance gain over Rel-18 non-AI/ML based approach is observed, further study the performance of CSI prediction plus CSI compression.

Specification impact of two-sided AI/ML model

Observation 5: NW may store and manage a lot of two-sided models for training collaboration Type 1 at UE side and Type 3 with UE-first.

Proposal 5: In CSI compression using two-sided model use case, the following Table 5 captures the pros/cons of extendibility for training collaboration Type 1:

Table 5: Pros and Cons of training collaboration Type 1

| Characteristics \ Training Types | Type 1: NW side |  | Type 1: UE side |  |
| --- | --- | --- | --- | --- |
|  | Unknown model structure at UE | Known model structure at UE | Unknown model structure at UE | Known model structure at UE |
| Whether model can be kept proprietary | No | No | No | No |
| Whether require privacy-sensitive dataset sharing | No (Note 1) | No (Note 1) | No (Note 1) | No (Note 1) |
| Flexibility to support cell/site/scenario/configuration specific model | Flexible except for UE defined scenarios. Not flexible for UE defined scenarios unless UE assistance information is supported and available. (Note 6) | Flexible except for UE defined scenarios. Not flexible for UE defined scenarios unless UE assistance information is supported and available. (Note 6) | Flexible except for NW defined scenarios. Not flexible for NW defined scenarios unless NW assistance information is supported and available. (Note 6) | Flexible except for NW defined scenarios. Not flexible for NW defined scenarios unless NW assistance information is supported and available. (Note 6) |
| Whether gNB/device specific optimization is allowed | gNB: Yes UE: No | gNB: Yes UE: less flexible compared to UE side | gNB: No UE: Yes | gNB: less flexible compared to NW side UE: Yes |
| Model update flexibility after deployment | Flexible only if UE supports the new structure | Flexible for parameter update | Flexible, less flexible than Type 1 NW side | Flexible for parameter update, less flexible than Type 1 NW side |
| Feasibility of allowing UE side and NW side to develop/update models separately | gNB: Feasible UE: Not feasible due to Type 1 definition. | gNB: Feasible with restriction for CSI reconstruction model. UE: Not feasible due to Type 1 definition. | gNB: Not feasible due to Type 1 definition. UE: Feasible. | gNB: Not feasible due to Type 1 definition. UE: Feasible with restriction for CSI generation model. |
| Whether gNB can maintain/store a single/unified CSI reconstruction model over different UEs (Note 2) | Yes | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” of clause 6.2.2.4 (Note 4) | No | No |
| Whether UE device can maintain/store a single/unified CSI generation model over different NW vendors (Note 3) | No | No | Yes | Yes. Performance refers to observations in “1 UE part model to N>1 NW part models” of clause 6.2.2.4 (Note 4) |
| Extendibility: to train new UE-side model compatible with NW-side model in use; (Note 5) | Yes | Yes | No consensus Yes (Note x1) | No consensus Yes (Note x1) |
| Extendibility: To train new NW-side model compatible with UE-side model in use; (Note 5) | No consensus Yes (Note x2) | No consensus Yes (Note x2) | Yes | Yes |
| Whether training data distribution can match the inference device | Limited | Limited | Yes | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | No for UE | Yes | No for NW | Yes |
| Model performance | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 |

Note x1: The user which is to train new UE-side model has the NW-side model in use.

Note x2: The network which is to train new NW-side model has the UE-side model in use.

Proposal 6: In CSI compression using two-sided model use case, the following Table 6 captures the pros/cons of training collaboration Type 2 and Type 3:

Table 6: Pros and Cons of training collaboration Type 2 and Type 3

| Characteristics \ Training Types | Type 2 |  | Type 3 |  |
| --- | --- | --- | --- | --- |
|  | Simultaneous | Sequential | NW first | UE first |
| Whether model can be kept proprietary | Yes (Note 1) | Yes (Note 1) | Yes (Note 1) | Yes (Note 1) |
| Whether requires privacy-sensitive dataset sharing | No (Note 2) | No (Note 2) | No (Note 2) | No (Note 2) |
| Flexibility to support cell/site/scenario/configuration specific model | No consensus Not flexible | No consensus Not flexible | [Semi] flexible except for UE defined scenarios. (Note 3) [Semi] flexible for UE defined scenarios if UE assistance information is supported and available. | [Semi] flexible except for NW defined scenarios (Note 3). [Semi] flexible for NW defined scenarios if NW assistance information is supported and available. |
| Whether gNB/device specific optimization is allowed | Yes | Yes | Yes | Yes |
| Model update flexibility after deployment (Note 4) | Not flexible | No consensus Semi-flexible | Semi-flexible | Semi-flexible |
| Feasibility of allowing UE side and NW side to develop/update models separately | Infeasible | No consensus Infeasible | Feasible | Feasible |
| Whether gNB can maintain/store a single/unified model over different UE vendors (Note 5) | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” and “1 UE part model to N>1 NW part models” of clause 6.2.2.4. | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” of clause 6.2.2.4. | Yes. Performance refers to observations in “NW first training, 1 NW part model to 1 UE part model, same backbone” and “NW first training, 1 NW part model to 1 UE part model, different backbones” of clause 6.2.2.5. | Yes. Performance refers to observations in “UE first training, M>1 UE part models to 1 NW part model” of clause 6.2.2.5. |
| Whether UE device can maintain/store a single/unified CSI generation model over different NW vendors (Note 6) | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” and “1 UE part model to N>1 NW part models” of clause 6.2.2.4. | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” of clause 6.2.2.4. | Yes. Performance refers to observations in “NW first training, 1 UE part model to N>1 NW part models” of clause 6.2.2.5. | Yes. Performance refers to observations in “UE first training, 1 NW part model to 1 UE part model, same backbone”. And “UE first training, 1 NW part model to 1 UE part model, different backbones” of clause 6.2.2.5. |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Not support | Support | Support | No consensus Support (Note x1) |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Not support | Not support | No consensus Support (Note x2) | Support |
| Whether training data distribution can match the inference device | No consensus Limited | Yes for UE-part model, limited for NW-part model | Limited | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | Compatible | Compatible | Compatible | Compatible |
| Model performance | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 | Performance refers to clause 6.2.2 |

Note x1: The user which is to train new UE-side model has the NW-side model in use.

Note x2: The network which is to train new NW-side model has the UE-side model in use.

Proposal 7: Both training Type 1 at NW side and Type 3 with NW-first are suggested to study with higher priority.

Proposal 8: At least for training Type 1 at NW side and Type 3 with NW-first, it is necessary and feasibility for NW side data collection considering the following aspects:

- Significant feedback overhead reduction by using codebook-based quantization.

- Signalling overhead reduction by using cell-specific CSI-RS resource configuration.

- No much strict latency requirement for data collection for model training or performance monitoring.

Proposal 9: It is necessary and feasible that performance monitoring by using intermediated KPIs or an existing CSI feedback scheme as the reference considering the following aspects:

- Significant overhead reduction for quantization of the target CSI or output of CSI reconstruction via enhanced eType II codebook parameters

- Affordable complexity for quantization of the target CSI or output of CSI reconstruction, which is similar to that of legacy eType II codebook.

- Ensuring the robust of monitoring performance by using an existing CSI feedback scheme as the reference

Proposal 10: The legacy priority rule can be reused to define the priority the AI/ML based CSI reporting, and a priority value PriiCSIy,k,c,sPriiCSIy,k,c,s PriiCSIy,k,c,swith new parameter value or introducing new parameter  PriiCSIy',y,k,c,s is used to indicate the priority of AI/ML based CSI reporting.

Proposal 11: The compressed CSI part 2 should be divided into 1<N groups for CSI omission. How to divide compressed CSI part 2 into N groups needs to further study.

Proposal 12: If there is no output of historic CSI at the previous instance, how to design the current input of historic CSI for two-sided AI/ML model needs to study.

Proposal 13: At least model-ID-based LCM for two-sided AI/ML model should be supported.

Proposal 14: Type B1 or Type B2 could be considered to identify two-sided AI/ML model between UE and network. The mechanisms or procedure for model identification of two-sided AI/ML model by using Type B1 or Type B2 needs to study.

References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, Dec. 11th – 15th, 2023.

- TR 38.843, V1.3.0 (2023.11).

- 3GPP RAN1#112 meeting, Chairman’s notes.

- 3GPP RAN1#113 meeting, Chairman’s notes.