3GPP TSG RAN WG1 #116	R1-2400548

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.10.1

Source:	Xiaomi

Title:	Discussion on enabling TXRX for XR during RRM measurements

Document for:	Discussion and Decision

- Introduction

The Rel-19 work item on XR has been approved in RAN#102 meeting [1], which includes the following objectives.

The Rel-19 XR ph3 objectives are as follows: - Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. - Note: Check in RAN#105 (check also other WG involvement if needed). - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. - Note: LCP implementation complexity need to be taken into account when evaluating solutions. - Note: Check in RAN#105 - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - If justified, define a mechanism for transmitter to inform the receiver of SN gap (or missing SNs) in PDCP. - Specify Core requirements related to the above objectives as necessary [RAN4] Note: Whether / to what extent network exposure / RAN awareness / e.g. RAN involved rate control, possibly additional info for DL scheduling, parallel with SA2 work, shall be covered in this WI is TBD.

In this contribution, we discuss potential design on enabling TX/RX for XR during RRM measurements and provide our views.

- Discussion

Currently, when the UE performs RRM measurement on a band, the normal data transmission of the UE is temporarily interrupted. However, for UEs with higher capabilities, they can complete the RRM measurement in advance. As shown in Fig.1, this leads to the phenomenon that part of the time of an RRM measurement GAP will be used by the UE for RRM measurement, and the other part will be idle. A UE waiting in idle period is practically meaningless.

Figure 1: RRM measurement gap

In scenarios where network resource occupancy rate is high, gNB has the flexibility to reallocate resources originally assigned to these UEs during their RRM measurement to other UEs without significantly impacting overall system performance metrics such as throughput. Conversely, if the resource occupancy rate is low across the entire communication system, these UEs will experience idle periods that result in wastage of allocated resources. Additionally, the presence of idle time may result in delayed scheduling of critical data packets, thereby impacting the service experience.

Observation1：The scheduling of UEs with higher capabilities can be expedited as they are able to complete their RRM measurements earlier.

Observation2：The enhancement of system scheduling efficiency can be achieved by effectively reducing the idle time of UEs caused by RRM measurement, particularly in scenarios with low system resource occupancy rate.

Proposal 1：Relevant reporting and processing mechanisms for mitigating UEs idle time caused by RRM measurements could be deliberated in RAN1, if deemed necessary.

To improve efficiency in Rel-18, a MAC CE named "positioning measurement gap activation/deactivation command" was introduced. A UE can be preconfigured with one or more measurement gaps to measure the DL PRS resource, and the gNB can activate or deactivate measurement gaps using the MAC CE command. A similar mechanism could be discussed in RAN1, enabling coordination between RRM measurements and service scheduling even for UEs with urgent service requirements. By doing so, when transmitting highly important data, the UE can temporarily suspend RRM measurements, complete the relevant transmission, and then resume RRM measurements. As depicted in Figure 2, when there is no immediate scheduled service to transmit, the UE needs to perform RRM measurement based on the pre-configured RRM MG(s). In case of a conflict between the resources allocated by the UE and those specified by the RRM MG, these resources are considered invalid during this period. If an emergency service scheduling occurs, it is necessary to deactivate the RRM MG. The activation and deactivation mechanisms of the RRM MG can be further discussed in RAN1.

Figure 2: RRM MG affected by emergency operations

Observation3：The activation and deactivation of the configured RRM measurement gap can provide protection for urgently scheduled services.

Proposal 2：If necessary, the activation and deactivation of the RRM measurement gap could be discussed in RAN1.

- Conclusion

In this contribution, we provide the following proposals and observations.

Observation1：The scheduling of UEs with higher capabilities can be expedited as they are able to complete their RRM measurements earlier.

Observation2：The enhancement of system scheduling efficiency can be achieved by effectively reducing the idle time of UEs caused by RRM measurement, particularly in scenarios with low system resource occupancy rate.

Observation3：The activation and deactivation of the configured RRM measurement gap can provide protection for urgently scheduled services.

Proposal 1：Relevant reporting and processing mechanisms for mitigating UEs idle time caused by RRM measurements could be deliberated in RAN1, if deemed necessary.

Proposal 2：If necessary, the activation and deactivation timing of the RRM measurement gap could be discussed in RAN1.

- References

- RP-234057, New WID: XR (eXtended Reality) for NR Phase 3, RAN#102, December 11-15, 2023.