3GPP TSG RAN WG1 #116														   R1-2400549

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	9.11.1

Source:	           Xiaomi

Title:	Discussion on NR-NTN downlink coverage enhancement

Document for:	Decision

- Introduction

In RAN#102 meeting, a new WID [1] as Non-Terrestrial Networks (NTN) for NR Phase 3 was approved. One of the objectives is to study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN. The details are the following:

- Justification: - Offer optimized performance especially when addressing handset terminals (including smartphones with -5.5 dBi antenna gain) w.r.t. downlink coverage considering the NTN deployment constraints such as payload power limitation, large satellite foot print and limited feeder link bandwidth. DL coverage enhancements are needed to accommodate satellite payload constraints which may be unable to have all its beams active with the « nominal » EIRP density per beam (see Section 6.1.1 in TR 38.821) at a given time due to limited power and limited feeder link bandwidth, while maximizing the number of beams that can be activated simultaneously, and ensuring that all user terminals can be served across the satellite foot print while maximizing the overall satellite throughput and ensuring that all satellite’s radio cells are kept alive even without traffic but allowing new users to join or preventing impact on end-user QoS. DL coverage enhancements can be considered at both - Link level to improve the link margin of selected physical channels in order to accommodate the EIRP reduction in FR1-NTN. A link margin improvement for physical channels (e.g. PDSCH and PDCCH) may be considered without impact on SSB design. - System level to support an efficient dynamic and flexible power sharing between beams or different beam pattern/size (i.e., wide or narrow) across the satellite foot print for FR1-NTN and FR2-NTN. The objectives of the work item are the following: - Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Notes for this objective: - SSB channel enhancement is not considered - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

In this contribution, we present our views on NR-NTN downlink coverage enhancement.

- General assumption on the beam deployment

In Rel-18, the nominal EIRP density were assumed per beam. In Rel-19, to ensure that all UEs can be served across the satellite foot print while maximizing the overall satellite throughput, the nominal EIRP density may separate by several beams as multiple beams are activated simultaneously.

Taking the following Figure 1 as example, there are 32 beam positions in total to cover the earth, wherein Satellite#1 is used to cover the Northern Hemisphere including beam position 1~16. In Fig.1 (a), there is only one satellite beam activated at a given time, while in Fig.1 (b), there are four satellite beams activated at a given time. Based on the parameters provided in TR 38.821 [2], the satellite EIRP for LEO-600km in S band is 41 dBW if the bandwidth per beam is 5 MHz, which is suitable for case shown in Fig.1(a). In case of Fig.1(b), the satellite EIRP density should be divided into four parts, and the EIRP is 35 dBW per beam with bandwidth per beam is 5 MHz assuming the total ERIP unchanged. If the EIRP density kept unchanged, the total EIRP should be 47dBW.

Fig.1(a) one beam activated 						Fig.1(b) four beams activated

Fig.1 Satellite beam position

How many beams can be activated simultaneously should be evaluated from many aspects, such as payload power limitation, satellite foot print and feeder link bandwidth. Assuming 5MHz is allocated to each beam, the system SCS is 15KHz, and the system band are n255 and n256, the beam number N should satisfy the equation in Step 1 and the DL CNR can be calculated based on the following steps.

- Step 1: Based on TR 38.821[2] Table 7.1-1 shown in Annex part, the typical Min / Max NTN beam foot print diameter is 50 km/1000 km. Assuming that satellite foot print diameter is 1000km, then the required beam number is 397 when the beam diameter is 50 km. Based on current NR spec., the Lmax for SSB number is 4 when the carrier <3GHz and SCS is 15KHz. Then the number of beams that activated simultaneously is N = 397/4 ~=100.

  - Based on the Note1 in TR 38.821[2], the beam foot print diameter are indicative. The diameter depends on the orbit, earth latitude, antenna design, and radio resource management strategy in a given system. Therefore, it is expected that the satellite network vendors and/or operators can provide a reference value of N.

- Step 2: Assuming the EIRP density is 34 dBW/MHz, considering that n255 and n256 band are agreed as NTN band, the total bandwidth is 64MHz, then the total satellite ERIP should be ERIP density * bandwidth = 52 dBW.

  - Similar with the consideration on the value of N, it is expected that satellite network vendors or operators can provide a reference value of the maximum ERIP for NTN.

- Step 3: Calculate the ERIP for each beam ERIP_per beam = 10^(52/10)/N, N =100, then ERIP_per beam = 32dBW

- Step 4: In case of LEO-600km, the evaluation angle is 30 degree, assuming the smartphones with -5.5 dBi antenna gain, the pathloss is 42.9 dB for 5MHz. The CNR= ERIP_per beam – pathloss= -10.9 dB.

Based on the assumption and calculation above, it is observed that the number of beams that can be activated simultaneously is around 100. The CNR is around -10.9dB assuming the total 100 beams are activated simultaneously. Based on the calculation results, the DL coverage can be enhanced from two aspects, one is link level enhancement to ensure that the SNR requirements for DL channels are lower than the calculated CNR, the other one is system level enhancement to increase the ERIP per beam, which can increase the CNR and relax the SNR requirement for DL channels.

Observation 1: The number of beams that activated simultaneously is around 100.

Observation 2: The CNR is around -10.9dB assuming the total 100 beams are activated simultaneously.

Observation 3: DL coverage can be enhanced from two aspects, one is link level enhancement to ensure that the SNR requirements for DL channels are lower than the calculated CNR, the other one is system level enhancement to increase the ERIP per beam, which can increase the CNR and relax the SNR requirement for DL channels.

- DL Channel coverage enhancement

  - Link level enhancement

Base on the analysis in section 2, the bottleneck channels can be found by checking the SNR requirements for each DL channels. We present our link level evaluation results in Table 1. As Table 1 shows, the SNR requirement of SSB and Msg2 PDSCH with scaling factor equals to 0.25 are lower than the CNR. With the repetition mechanism, the SNR requirement of PDSCH for low data rate traffic and VoIP traffic are also lower than the CNR. While, the PDCCH and Msg4 PDSCH are the bottleneck channels which need to be enhanced. The gap for PDCCH and Msg4 PDSCH are around 2dB and 3dB respectively.

We further evaluated the SNR requirement for different configurations of PDCCH, for example, increasing the repetition number of PDCCH, reducing the payload and increasing the aggregation level. The detailed the evaluation assumption and results are shown in Table 2 and Table 3. Based on the evaluation results, the possible solutions for PDCCH coverage enhancement can be repetition, payload reduction and increasing AL. For Msg4 PDSCH coverage enhancement, a straightforward solution can be repetition.

Observation 4: Msg4 PDSCH and PDCCH are bottleneck channels, the gap is around 2dB and 3dB respectively.

Observation 5: For PDCCH coverage enhancement, the possible solutions can be repetition, payload reduction and AL increasing. For Msg4 PDSCH coverage enhancement, a straightforward solution is repetition.

Table 1. The SNR requirements for different DL channels

| Configuration | SNR (dB) | BLER |
| --- | --- | --- |
| SSB | -12.7 | 0.01 |
| PDCCH | -9 | 0.01 |
| Msg2 PDSCH(6RB, scaling factor=0.25) | -13.4 | 0.1 |
| Msg4 PDSCH(36 RB) | -8.3 | 0.1 |
| PDSCH Low data rate (rep=4) | -12 | 0.1 |
| PDSCH VoIP (rep=4) | -11.8 | 0.02 |

Table 2. PDCCH coverage enhancement cases

| Case | Payload | Repetition | Aggregation level |
| --- | --- | --- | --- |
| 0 (Benchmark) | 40 | 1 | 16 |
| 1 | 25 | 1 | 16 |
| 2 | 40 | 2,4,8 | 16 |
| 3 | 40 | 1 | 24, 32 |

Table 3. PDCCH evaluation results of different cases

| Configuration | SNR (dB) | BLER |
| --- | --- | --- |
| Case 0 | -9 | 0.01 |
| Case 1 | -9.9 |  |
| Case 2, rep = 2 | -11 |  |
| Case 2, rep = 4 | -12.9 |  |
| Case 2, rep = 8 | -14.6 |  |
| Case3, AL=24 | -10.3 |  |
| Case3, AL=32 | -11 |  |

  - System level enhancement

System level solutions are used to support an efficient dynamic and flexible power sharing between beams or different beam pattern/size (i.e., wide or narrow) across the satellite foot print for FR1-NTN and FR2-NTN.

For beams with different evaluation angels, the path loss is different. As Fig.2 shows, the center of beam#1 is the nadir and the elevation of beam#2 is alpha, the path loss for UEs in beam #2 is larger than UEs in beam#1. To ensure that UEs in different locations can have similar service quality, the network can allocate different power for beam#1 and beam#2.

Besides the power/EIRP density, the allocated bandwidth also impacts the total power/ERIP, as the total power/ERIP equals to power/EIRP density multiplies bandwidth. For beams without traffic, the network can activate part bandwidth which can at least ensure new UE access. The network can allocate more bandwidth only if UEs need more bandwidth for data traffic. Similarly, the periodic channels/signals can be deactivated in case of no traffic, and the network can activate the power consuming periodic channel/signal only if UEs need those channel/signal for further communication. The solutions discussed in NES such as on-demand SIB1 can be considered in NTN.

Fig.2. Diagram for satellite beams with different elevations

Based on the analysis in section 3 and section 4, there are two alternatives for solution discussion in NTN DL coverage enhancement from our understanding.

•	Alt1: Increasing PDCCH and Msg.4 PDSCH performance by link level enhancement, e.g., repetition. System level enhancement is not considered or not specified.

•	Alt2: Increasing PDCCH and Msg.4 PDSCH performance partly by system level enhancement, e.g., periodic channel/signal deactivation. The remaining gap of Msg.4 PDSCH is increased by link level enhancement, e.g., repetition.

Proposal 1: Consider the following alternatives for solution discussion in NTN DL coverage enhancement.

•	Alt1: Increasing PDCCH and Msg.4 PDSCH performance by link level enhancement, e.g., repetition. System level enhancement is not considered or not specified.

•	Alt2: Increasing PDCCH and Msg.4 PDSCH performance partly by system level enhancement, e.g., periodic channel/signal deactivation. The remaining gap of Msg.4 PDSCH is increased by link level enhancement, e.g., repetition.

- Scope of DL coverage enhancement

One issue that need to be clarified is whether the Redcap device is in the scope of DL coverage enhancement. Different antenna gains are assumed for the Redcap device and the handheld device in TN, as a 3dB loss of antenna gain due to small form factor for the Redcap device should be assumed in the link budget calculation based on the agreements achieved in RAN1#102e-meeting (as shown in the table) [3] compared to that for the handheld device. This will have impact on the design target of the coverage enhancement, and thus, should be clarified.

Agreements: - The impact of small form factor is considered for all the uplink and downlink channels - A 3dB loss of antenna gain is included in link budget calculation for FR1 - FFS on the application to both FDD and TDD bands or only FDD bands

Proposal 2: Whether the Redcap device is considered in the DL CE working scope should be clarified. The additional loss of antenna gain for Redcap device compared that for the handheld device should be discussed if Redcap device is within the DL CE working scope.

- Conclusions

In this contribution, we present views on NR-NTN DL coverage enhancement, we have the following observations and proposals:

Observation 1: The number of beams that activated simultaneously is around 100.

Observation 2: The CNR is around -10.9dB assuming the total 100 beams are activated simultaneously.

Observation 3: DL coverage can be enhanced from two aspects, one is link level enhancement to ensure that the SNR requirements for DL channels are lower than the calculated CNR, the other one is system level enhancement to increase the ERIP per beam, which can increase the CNR and relax the SNR requirement for DL channels.

Observation 4: Msg4 PDSCH and PDCCH are bottleneck channels, the gap is around 2dB and 3dB respectively.

Observation 5: For PDCCH coverage enhancement, the possible solutions can be repetition, payload reduction and AL increasing. For Msg4 PDSCH coverage enhancement, a straightforward solution is repetition.

Proposal 1: Consider the following alternatives for solution discussion in NTN DL coverage enhancement.

•	Alt1: Increasing PDCCH and Msg.4 PDSCH performance by link level enhancement, e.g., repetition. System level enhancement is not considered or not specified.

•	Alt2: Increasing PDCCH and Msg.4 PDSCH performance partly by system level enhancement, e.g., periodic channel/signal deactivation. The remaining gap of Msg.4 PDSCH is increased by link level enhancement, e.g., repetition.

Proposal 2: Whether the Redcap device is considered in the DL CE working scope should be clarified. The additional loss of antenna gain for Redcap device compared that for the handheld device should be discussed if Redcap device is within the DL CE working scope.

- Annex

Table 7.1-1: NTN scenarios versus delay constraints [2]

| NTN scenarios | A | B | C1 | C2 | D1 | D2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO transparent payload | GEO regenerative payload | LEO transparent payload |  | LEO regenerative payload |  |
| Satellite altitude | 35786 km |  | 600 km |  |  |  |
| Relative speed of Satellite with respect to earth | negligible |  | 7.56 km per second |  |  |  |
| Min elevation for both feeder and service links | 10° for service link and 10° for feeder link |  |  |  |  |  |
| Typical Min / Max NTN beam foot print diameter (note 1) | 100 km / 3500 km |  | 50 km / 1000 km |  |  |  |
| Maximum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 541.46 ms (Worst case) | 270.73 ms | 25.77 ms |  | 12.89 ms |  |
| Minimum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 477.48 ms | 238.74 ms | 8 ms |  | 4 ms |  |
| Maximum Round Trip Delay variation as seen by the UE (note 2) | Negligible |  | Up to +/- 93.0 µs/sec (Worst case) |  | Up to +/- 47.6 µs/sec |  |
| NOTE 1: The beam foot print diameter are indicative. The diameter depends on the orbit, earth latitude, antenna design, and radio resource management strategy in a given system. NOTE 2: The delay variation measures how fast the round trip delay (function of UE-satellite-NTN gateway distance) varies over time. The worst case is when the satellite moves towards/away from the UE at 10⁰ elevation angle, assuming the UE speed is 1200 km/h. It is expressed in µs/s and is negligible for GEO scenario. NOTE 3: Void NOTE 4: Speed of light used for delay calculation is 299792458 m/s. |  |  |  |  |  |  |

- References

[1] RP-234078, 3GPP TSG RAN Meeting #102, Huawei (Moderator, RAN1 Vice-Chair)

[2] TR 38.821 V16.1.0 (2021-05)

[3] Chairman’s Notes RAN1#102