3GPP TSG RAN WG1 #116			R1-2400557

Athens, Greece, February 26th – March 1st, 2024

Source: 	     Xiaomi

Title:                  Discussion on reception and transmission procedure for SBFD operation

Agenda item:    9.3.1

Document for:  Discussion

Introduction

In RAN#102 meeting, a new WID for evolution of NR duplex operation was approved. [1]  The following objectives are agreed and need further specification work in Rel-19:

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC CONNECTED mode [RAN1, RAN2] - Study and specify, if justified, SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access [RAN1, RAN2] - RAN#104 to check whether to proceed normative work - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators - Specify enhancements for CLI handling [RAN1, RAN2, RAN3]: - Support gNB-to-gNB co-channel CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117) - Support UE-to-UE co-channel CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117) - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for co-channel CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In this contribution, we focus on the SBFD UE procedures related to RX/TX/measurement.

Discussion

Time domain resource allocation for UL subband

In study item of Rel-18 duplex evolution, it was agreed that explicit configuration of SBFD subband time locations within a period is the baseline. [2] Accordingly, we need to clarify how to determine the period. A straightforward direction is to configure SBFD subband time location within a period equal to one or multiple slot configuration periods determined by TDD UL DL configuration. Basically, there are different cases of TDD frame structure determination, which are summarized as below:

- Case#1: TDD frame structure with single periodicity is configured by tdd-UL-DL-ConfigurationCommon only

- Case#2: TDD frame structure with two periodicities is configured by tdd-UL-DL-ConfigurationCommon only

- Case#3: TDD frame structure is configured by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated additionally

  - Case#3-1: single periodicity

  - Case#3-2: dual periodicity

- Case#4: Network doesn’t provide any TDD UL-DL configuration, i.e., all the symbols are regarded as semi-static flexible symbol

For case#1, gNB provides UE a slot configuration within a periodicity, i.e., P milliseconds, via SIB1.  Generally speaking, the first N consecutive slot within period P are DL slots and the last M consecutive slots within period P are UL slots. Additionally, several DL symbols can be configured immediately after N DL slots and several UL symbols can be configured before the M UL slots. The left symbols are semi-static flexible symbols. In this case, the period of configuring SBFD subband time location can be equal to TDD periodicity. For example, a SLIV-like mechanism can be used to indicate time location of SBFD subband, i.e., indicate the starting position and the length.

Figure 1 Example of SBFD subband time location based on single TDD periodicity

For case#2, gNB configures dual TDD periodicity which are denoted as P and P2. For each periodicity, the configuration is same as the single periodicity illustrated in case#1. Any {P, P2} combination satisfies the condition that P+P2 divides 20 msec is supported. In the other words, the TDD frame structure corresponding to each period can be totally different from each other. Consequently, it is quite difficult to use single SLIV indication to indicate the time location of SBFD subband. For example, assuming the reference SCS=15 kHz with P=3 msec and P2=2 msec. The specific TDD frame structure is shown in Figure 2.  Obviously, the situation is much more complicated compared with case#1.  Here we still take SLIV-like mechanism as example. It is not clear whether a time domain indication is applied to two periodicity or it is applied to each periodicity respectively. If we follow the former understanding, a single SLIV is not sufficient to indicate time location of a UL subband in two TDD periodicities as the DL regions across two periodicities are discrete. On the other hand, the latter understanding bring restriction on the time domain location, as gNB has to guarantee the indicated time domain location does not exceed each TDD periodicity.

Figure 2 Example of SBFD subband time location based on dual TDD periodicity

For case#3, a slot level TDD structure can be indicated by tdd-UL-DL-ConfigurationDedicated on top of tdd-UL-DL-ConfigurationCommon. The UE-dedicated signalling is used to configure the slot structure of each semi-static flexible slot. One example is shown in Figure 3, where two switching points are configured. Similar issues as case#2, a SLIV-like method is incapable to indicate the discrete time domain resources for subband.

Figure 3 Example of SBFD subband time location based on common TDD+UE-dedicated TDD

Case#4 is a special case wherein all the slots are semi-static flexible. There is no reference TDD periodicity. Therefore, gNB needs to configure a period within which SBFD subband is configured. On the other hand, gNB has to configure at least one TDD period once it intends to configure UL subband, in which case case#4 is automatically converted to the other cases.

Observation 1: There are several mechanisms to realize TDD UL-DL configuration which leads to different TDD periodicity:

- Single TDD periodicity provided by tdd-UL-DL-ConfigurationCommon

- Dual TDD periodicity provided by tdd-UL-DL-ConfigurationCommon

- Additional TDD UL-DL configuration provided by tdd-UL-DL-ConfigurationDedicated on top of common configuration

- No TDD periodicity is provided by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated

Based on the aforementioned observation, there can be single TDD periodicity, double TDD periodicity or no TDD periodicity. A unified solution to indicate the SBFD subband time domain location should be pursued in order to simplify specification and gNB operation. Otherwise, gNB has to use different mechanisms to configure SBFD subband time domain location for different TDD UL-DL configurations.

Observation 2: A unified solution to indicate the SBFD subband time domain location should be employed in order to simplify specification and gNB operation.

Regarding to how to indicate SBFD subband time domain location within a time period, there are several candidate solutions:

- Option 1: gNB jointly indicates the starting position and duration of SBFD subband time domain location, i.e., SLIV-like mechanism.

- Option 2: gNB indicates SBFD subband time domain location via a bitmap, e.g., 1 denotes SBFD subband occupies the related time unit while 0 denotes no SBFD subband exists on the related time unit.

- Option 3: gNB indicates SBFD subband time domain location via a subband-specific TDD UL-DL configuration.

First of all, it can be observed that discrete DL region in time domain is a typical case, i.e., TDD frame structure with two periodicity or UE-dedicated configuration. SLIV-like mechanism is sufficient for indicating SBFD time location across discrete DL region only if additional mechanism is introduced to address the issue that UL subband is allocated on UL symbols. For example, it can be simply mentioned in the specification that UL subband configuration cannot be applied to UL symbols.

Observation 3: SLIV-like method cannot be directly used for indicating SBFD time location for discrete DL region.

Bitmap based method is sufficient to indicate SBFD subband time domain location, despite of consecutive time domain resources or discrete time domain resources. Accordingly, we need to further discuss the granularity in time domain for indication. For example, 1 bit contained by the bitmap can be used to indicate the occurrence of SBFD subband on a slot, several OFDM symbols or even single OFDM symbol. From our perspective, a time unit as small as single OFDM symbol is not necessary which will bring huge signalling overhead. Slot level granularity is sufficient which provides good trade-off between overhead and flexibility. As aforementioned, TDD frame structure is divergent and there may be single, double or no TDD periodicity is configured. Hence, time period during which the bitmap is applicable needs to be discussed. One straightforward way is to use individual bitmap to indicate the SBFD subband time domain location for each TDD periodicity. Single bitmap is configured if there is only one TDD periodicity. If two TDD patterns are configured, two bitmaps are needed in order to indicate the SBFD subband time domain location for each TDD periodicity. There is a possible case that neither common TDD configuration nor UE-dedicated TDD configuration is configured. Theoretically, every OFDM symbol is regarded as semi-static flexible. There is no TDD periodicity which can be directly used as time period for configuring SBFD subband time domain location. In order to resolve the ambiguity on time period, a default time period can be adopted once no TDD configuration is provided by network, e.g., 20 ms.  It may be also sufficient to leave it to gNB implementation to avoid such case, i.e., gNB has to provide at least one TDD configuration if it wants to configure UL subband.

Observation 4: Bitmap based method can be used to indicate SBFD time location within a time period.

- The time period is equal to TDD periodicity provided by tdd-UL-DL-ConfigurationCommon

- FFS: the time unit of bitmap

- FFS: how to determine time period if gNB doesn’t provide any TDD configuration, e.g., define a default period or leave it to gNB implementation

Both bitmap-based indication and SLIV-based indication can sufficiently provide the time domain location of SBFD subband. One example is shown in Figure 4 as below. In theory, guard period is needed between DL transmission and UL transmission in order to avoid inter-slot interference. Otherwise, UE-to-UE CLI will jeopardize the DL transmission for both SBFD aware UE and none SBFD aware UE which receives DL transmission on the previous DL slot. The Rx-to-Tx switching at UE side and Rx-to-Tx switching at gNB side also need guard period. Therefore, it needs attention on how to define or indicate guard period between DL symbol and the first OFDM symbol occupied by UL subband.

Figure 4 Example of SBFD subband time location based on bitmap indication

Observation 5: If bitmap-based mechanism or SLIV-based mechanism is used to indicate SBFD subband time domain location, how to define guard period between DL symbol and first symbol occupied by UL subband needs further study.

Option 3 provide a TDD UL-DL configuration for SBFD subband so as to indicate the time domain location. Current TDD UL-DL configuration can be fully reused for subband TDD structure configuration, i.e., common TDD configuration or UE dedicated TDD configuration. Take UE-dedicated TDD configuration as example, assuming TDD UL-DL configuration of the carrier is DDDDDDFFUU, gNB configures a subband-specific TDD UL-DL configuration for UL subband as DFUUUUUFU. Accordingly, the SBFD subband time domain allocation is shown in Figure 6. In this example, TDD periodicity is equal to the one provided by tdd-UL-DL-ConfigurationCommon for a carrier. Exactly same signalling structure as tdd-UL-DL-ConfigurationCommon is reused to indicate the subband-specific TDD structure. As gNB has full power to determine the TDD structure for each slot, the guard period between DL symbol and first OFDM symbol occupied by SBFD subband can be easily configured. Furthermore, it is also friendly to the scenario with UE-dedicated TDD frame structure for a carrier.

Figure 5 Example of SBFD subband time location based on SBFD-specific TDD UL-DL configuration

Observation 6: A subband-specific TDD UL-DL configuration can be used to indicate SBFD subband time domain location.

Based on the previous analyses, we think either bitmap-based/SLIV-based solution or TDD UL-DL configuration alike solution can be employed to indicate SBFD subband time domain location. Hence, we have the following proposal:

Proposal 1: For semi-static configuration of subband time location for SBFD operation, the following mechanisms can be considered:

- gNB indicates SBFD subband time domain location via a bitmap.

- gNB indicates SBFD subband time domain location via SLIV indication with assuming UL subband cannot be located on UL symbols.

- gNB indicates SBFD subband time domain location via a subband-specific TDD UL-DL configuration.

Basically, each bit in the bitmap indicates whether UL subband exists in the related time unit. For example, 1 means UL subband is present in the time unit and absent otherwise. If slot is adopted as the time unit, there is a case that semi-static UL symbols is contained in the slot. In this case, it should be clarified whether UL subband is present on UL symbols. Considering it was agreed that UL transmission is only allowed within UL subband, it will reduce the UL capacity if UL subband can be configured on the UL symbol. From this perspective, we don’t think UL subband can be configured on UL symbol.

Proposal 2: UL subband cannot be applicable to UL symbols, including semi-static UL symbol and dynamic UL symbol indicated via SFI.

- If gNB indicates UL subband is present on UL symbols, UE ignores the UL subband configuration on UL symbols and transmits uplink on UL symbol as normal.

In generic, gNB can indicate SBFD subband time location via either cell-specific signalling or UE-dedicated signalling. For cell-specific signalling, all SBFD capable UEs belonging to the same cell has unified subband in time domain. It is friendly to configuration from network perspective as gNB only needs to deliver same indication to all the UEs. However, it loses flexibility to some extent. Given different UE may be interested in different traffic, SBFD UE may have different requirement on subband. UE-dedicated signalling is expected to carter to UE-specific requirement. However, it may not end up as what is expected to. gNB cannot receive and transmit simultaneous on the same T/F resources, which will significantly complicate scheduling strategy as it has to consider all the other UEs when it wants to schedule a UE transmit or receive something in order to avoid any potential collision between any two UEs.

On the other hand, even SBFD subband is configured via cell-specific signalling, UE can still receive DL transmission within the DL subband on SBFD slots for sure. In this sense, the bonus from UE-dedicated subband configuration might be trivial compared with the penalty of more complicated configuration coming along.

Proposal 3: Cell-specific signalling is sufficient to configure SBFD subband time location.

No matter which mechanism is used to indicate SBFD subband time domain location, SBFD subband can be configured on semi-static flexible symbol. It is well known that gNB can adjust the transmission direction of semi-static flexible symbol according to traffic load at least. Assuming the TDD UL-DL configuration for carrier is DFFFU configured via tdd-UL-DL-ConfigurationCommon, gNB configures SBFD subband time location via a cell-specific signalling, either via a bitmap or subband-specific TDD UL-DL configuration. Here we assume the UL subband occupies slot#1, slot#2 and slot#3 in time domain, which is shown in Figure 7. As slot#1, slot#2 and slot#3 are semi-static flexible slots configured via cell-specific signalling, gNB can further indicate the slot direction via UE-dedicated signalling or SFI. To be specific, the flexible symbols can be either indicated as DL symbol or UL symbol. It can be summarized as below:

- If semi-flexible symbol is indicated as DL symbol on which UL subband is configured, what is the expected behaviour for SBFD aware UE?

- If semi-flexible symbol is indicated as UL symbol on which UL subband is configured, what is the expected behaviour for SBFD aware UE?

- If semi-flexible symbol is indicated as SFI flexible symbol on which UL subband is configured, what is the expected behaviour for SBFD aware UE?

Figure 6 Example of SBFD subband locates on semi-static flexible slots

Proposal 4: If UL subband is configured on semi-static flexible symbol and gNB further adjust direction for the symbols, clarify behaviour of SBFD aware UE at least for the following cases:

- Semi-flexible symbol is indicated as DL symbol on which UL subband is configured

- Semi-flexible symbol is indicated as UL symbol on which UL subband is configured

- Semi-flexible symbol is indicated as SFI flexible symbol on which UL subband is configured

In RAN1#112bis e-meeting, RAN1 draw the following conclusion on whether or not a slot containing both SBFD slot and non-SBFD slot is allowed:

Conclusion The following RAN1 observation is made: One motivation for allowing that a slot can consist of both SBFD and non-SBFD symbols is for compatibility with symbol-level TDD UL/DL configuration. Frequent switching between SBFD and non-SBFD symbols may increase the implementation complexity and interruptions of transmissions/receptions during transition. - Further study whether limitation(s) on the maximum number of switching points between SBFD and non-SBFD symbols within a slot, a TDD UL/DL pattern period, and/or semi-static SBFD configuration period (if different from TDD UL/DL pattern period) are needed - Further study scenarios a guard period between SBFD and non-SBFD symbols is required/not required and the length of the guard period if required Note: Whether or not a physical channel/signal occasion is mapped to both SBFD and non-SBFD symbols within a slot is a separate discussion.

First of all, it should be clear that non-SBFD symbol means an OFDM symbol without UL subband configuration, i.e., UL symbol, DL symbol and flexible symbol.  Hence, we need to take a look on the slot format, i.e., whether it is a typical case that a slot consists of different symbol types. Based on the previous discussion, TDD UL-DL configuration can be provided by several methods, including cell-specific signalling and UE-dedicated signalling. No matter which mechanism is adopted, flexible symbol is needed once DL-to-UL switching point occurs in a slot. Particularly, UE-dedicated TDD configuration can provide slot-based TDD structure for each slot, in order to satisfy diverse UE requirement and realize self-contained feedback. To be specific, it is typical that a slot contains DL symbol, flexible symbol and UL symbol. As UL subband can be only configured on DL symbol or flexible symbol, once gNB needs to configure UL subband on such slots, it is natural that it contains SBFD symbol and non-SBFD symbol. On the other hand, if only SBFD symbol is allowed for a SBFD slot which is self-contained, it means UL subband can be configured on UL symbols. Considering UL transmission is only allowed within UL subband, UL resource utilization is jeopardized significantly.

Figure 7  Example of slot constituting SBFD symbol and non-SBFD symbol

Observation 7: It is a typical case for a self-contained slot that it consists of both SBFD and non-SBFD symbols.

As shown above, allowing SBFD and non-SBFD mixed slot is friendly to increase flexibility in terms of scheduling and configuration. Regarding implementation complexity, we don’t see additional difficulties from both gNB side and UE side. From our understanding, same configuration and scheduling mechanisms as those of full-SBFD slot can be reused. Although a slot containing both SBFD symbol and non-SBFD symbol is typical, it doesn’t mean a UL transmission or DL reception across SBFD symbol and non-SBFD symbol within a slot is supported.

Proposal 5: A slot can consist of both SBFD symbols and non-SBFD symbols.

The major concern is that frequent switching between SBFD and non-SBFD symbols may increase the implementation complexity and interruptions of transmissions/receptions during transition. It is true that UE may need to adjust its RF parameters, power parameters and spatial parameters for the transmission on SBFD symbol and non-SBFD symbols. In order to provide sufficient period for accommodating operation on different symbol type, one or several symbols in-between may be needed. However, switching between SBFD symbols and non-SBFD symbols is a generic issue which needs to be studied for SBFD operation. In the other words, it is not specific for a slot consisting of both SBFD symbols and non-SBFD symbols. Therefore, we should consider the switching point not only for SBFD symbol and non-SBFD symbol, but also for SBFD symbol and SBFD symbol with different transmission direction.

Observation 8: The maximum number of transition point between DL/UL to UL/DL is a generic issue for SBFD operation.

It would be quite helpful if the maximum number of transition point can be limited within a window for both gNB and UE. Directly, the maximum number can be applicable to the configuration period of UL subband.

Proposal 6: Study the maximum number of transition point between DL/UL and UL/DL within consecutive SBFD slots.

For a slot consisting of both SBFD symbols and non-SBFD symbols, it can directly reuse the mechanisms as self-contained slot structure, i.e., only one switching point is allowed within the slot. The flexible symbols between DL symbols and UL symbols can be used as switching period for UE transition operation between SBFD symbols and non-SBFD symbols. Regarding which scenarios a guard period between SBFD and non-SBFD symbols is required/not required and how to determine the length of guard period, we think it is irrelevant to SBFD subband time domain configuration.

Proposal 7: Only one transition point is allowed within a slot consisting of both SBFD symbols and non-SBFD symbols.

- Flexible symbols between DL symbol and UL symbol can be used as guard period for switching between SBFD symbol and non-SBFD symbol.

Frequency domain resource allocation for UL subband

In RAN1#112bis e-meeting, the following agreement was achieved.

Agreement At least for semi-static SBFD, the following two options are viable solutions for frequency location configuration of DL subband(s) and guardband(s) if any. - Option 1: Frequency locations of DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s). - Option 2: The number of RBs for guardband(s), if any, is explicitly configured. DL subband(s) are implicitly derived as RBs which are not within UL subband or guardband(s).

Furthermore, it was agreed that frequency location of UL/DL subband is with reference to CRB grid in RAN1#111 meeting, which is shown as below. In this section, we further discuss the remaining issues on determining the frequency location of UL/DL subband.

Agreement For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid.

There are three types of frequency resource in SBFD symbol, i.e., UL subband, DL subband and guard band. gNB at most needs to indicate or determine two out of three subbands. The frequency location of the third subband can be automatically determined. For example, DL subbands within an active DL BWP can be derived from the information of UL subband and guard band. Hence, we can focus on one issue: whether explicit configuration/indication is needed for determining guard band. If the answer is yes, gNB needs to explicitly indicate either DL subband(s) or guard band.  In RAN1#112bis e-meeting, the following two options were regarded as viable solutions for frequency location configuration of DL subband(s) and guardband(s) if any:

- Option 1: Frequency locations of DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

- Option 2: The number of RBs for guardband(s), if any, is explicitly configured. DL subband(s) are implicitly derived as RBs which are not within UL subband or guardband(s).

Either option 1 or option 2 is workable. Considering DL subband typically contains much more RBs than that of guard band, more indication bits are needed for DL subband configuration. Hence, we slightly prefer option 2.

Proposal 8: Frequency location of guard subband(s) are explicitly configured if any. DL subband(s) are implicitly derived as the RBs which are not within UL subband or guard subband(s).

Currently, the frequency resources of UL BWP and DL BWP is indicated via SLIV, i.e., the starting RB index and BWP size provided by locationAndBandwidth. In the other words, gNB indicates the starting RB index and the number of RBs of a BWP within a carrier. The reference point is CRB#0 wherein the offset between the reference point and starting RB of BWP is indicated via offsetToCarrier. Considering the frequency location of SBFD subband is contained within a carrier, it is straightforward to reuse the same mechanism indicating frequency resource of a BWP.

On the other hand, it may be unnecessary to configure a UL subband with different sizes from that of active UL BWP. For example, the frequency location of UL subband can be exactly same as one of UL BWP. It can significantly reduce complexity of RRC signalling on frequency location configuration, i.e., gNB doesn’t need to additionally indicate the frequency location of a UL subband. However, gNB has to guarantee all of SBFD aware UE has same UL BWP in order to achieve a common UL subband configuration with same frequency location. Considering UE supports up to 4 UL BWPs within a carrier, the UL BWP with the smallest index can be used to determine the frequency location of UL subband if gNB doesn’t provide frequency location configuration for UL subband. Furthermore, if a cell-specific UL subband configuration carried by SIB1 is adopted, the frequency location of initial UL BWP can be used for UL subband determination in frequency domain by default.

Proposal 9: For configuring frequency location of UL subband, the following mechanism can be considered:

- Reuse the same mechanism of configuring BWP to configure the frequency locations of subband for SBFD operation, i.e., the starting position and length are indicated.

- If gNB doesn’t provide any configuration for UL subband frequency location, a default BWP is used to determine the frequency location of the UL subband.

Similar to time domain location indication, there are two directions for semi-static configuration of subband frequency locations for SBFD operation, i.e., via cell-specific signalling or via UE-dedicated signalling. If UL subband is configured via UE-dedicated RRC signalling, unaligned UL subband is inevitable across different gNB and across different UE. It is against the agreement that we should consider same subband frequency resources across different SBFD symbols as baseline. [2] On the other hand, unaligned UL subband brings intra-subband UE-to-UE CLI and gNB-to-gNB CLI, which bring additional difficulties for gNB coordination and certainly degrades system performance.

Proposal 10: Cell-specific RRC signalling is used to configure SBFD subband frequency location as baseline.

DL resource allocation in frequency domain on SBFD symbols

From our understanding, there is no restriction on frequency location of UL subband. Therefore, either DUD or DU are possible UL subband configuration. In Figure 8, we provide four cases considering the location of UL subband and whether there is guard band. No matter which case is configured by gNB, the available DL RB for DL transmission on SBFD slot is different from that on non-SBFD slot.

Figure 8 Example of frequency resources allocated to subband in SBFD slot

However, FDRA indication and frequency granularity are determined according to active DL BWP. Therefore, there are two issues needs to be studied:

- Issue#1: Following current specification, resource allocation for downlink channel and signal may be across DL subband and other unavailable subband. For example, the current DL resource allocation mechanism may lead to the DL channel or DL signal across two DL subbands.

- Issue#2: For the frequency unit at the boundary of DL subband, it may not be aligned with DL subband boundary so that partial RB within the frequency unit overlaps with guard band and/or UL subband.

Resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands

In the existing specification, the frequency allocation for PDSCH and CSI-RS resource is pretty flexible.

- For PDSCH, there are two RA types, i.e., RA type 0 and RA type 1. RA type 0 indicates RBG used for PDSCH transmission with a bitmap indication. Hence, indicated RBG can be contiguous or discrete.  It should be noted that the RBG is determined according to active DL BWP. RA type 1 indicates contiguous VRB or VRB group for PDSCH with a SLIV indication. The mapping between VRB to PRB can be either non-interleaved mapping or interleaved mapping. Despite of RA type 0 and RA type 1, it is sufficient to indicate frequency resources for PDSCH transmission on SBFD symbols.

- For CSI-RS resource configuration, it has three restrictions for a CSI-RS resource, i.e., 1) the number of RBs constituting a CSI-RS resource has to be multiple of 4 RBs, 2) the total number of RBs constituting a CSI-RS resource has to be equal to or larger than 24 RBs, 3) only contiguous RBs can be allocated to a CSI-RS resource. Taking those restrictions into account, existing mechanism of configurating CSI-RS resource is sufficient to indicate frequency resources for PDSCH transmission on SBFD symbols.

As analysed above, existing mechanisms of allocating frequency resources for PDSCH and CSI-RS can be used to indicate target resources in SBFD symbols. On the other hand, the indicated frequency resources may overlap with unavailable resources for DL transmission.

For PDSCH RA type 0, gNB can avoid the case that an entire RBG is allocated outside DL subband. Therefore, it can be boiled down to the case of unaligned boundaries between DL subband and RBG.

For PDSCH RA type 1, if non-interleaved VRB-to-PRB mapping is indicated, gNB can guarantee that the PDSCH is only allocated within one out of two DL subbands. If interleaved VRB-to-PRB mapping is indicated, the relevant PRB is distributed across entire active DL BWP. It is very hard to guarantee each PRB is contained by DL subband. On the other hand, it may be easily addressed following existing mapping rules. As the description excerpted below, the block of complex-valued symbols is only mapped to the VRBs which the corresponding physical resource blocks are declared as available for PDSCH. In SBFD symbols, only the RBs contained by DL subband(s) are available for PDSCH transmission. Hence, we only need to capture the definition of DL subband in SBFD symbols are available resources for DL transmission in TS 38.214. With this update, everything works well according to existing specification.

TS38.211, v18.1.0 - 7.3.1.5 Mapping to virtual resource blocks The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by TRS-ResourceSet IE, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource; - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. The mapping to resource elements (k',l)p,μ allocated for PDSCH according to [6, TS 38.214] and not reserved for other purposes shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index l.

Actually, if capturing only RBs contained by DL subbands are available for PDSCH transmission, it is also helpful for PDSCH RA type 1 with non-interleaving VRB-to-PRB mapping. There will be no issue even if gNB indicate contiguous RBs across two DL subbands.

Proposal 11: Capture it in TS38.214 that only RBs contained in DL subband are available for PDSCH.

A CSI-RS resource has to contain multiple of 4 RBs. The smallest configurable number is the minimum of 24 RB and the width of the associated BWP. Hence, we need to consider the impacts on CSI-RS resource configuration if it is associated with a SBFD symbol. For UL subband with {DU} or {UD} pattern, we believe there is no issue as the frequency resources of DL subband is consecutive and the boundaries of DL subband and CSI-RS subband can be aligned with each other. For {DUD} SBFD subband pattern, following options have been raised so far:

- Option 1: Two CSI-RS resources are configured for each DL subband respectively

- Option 2: Non-contiguous CSI-RS resource across two DL subbands via explicit or implicit mechanism is allowed

- Option 3: CSI-RS resource is allowed to overlap with UL subband and guardband(s), the corresponding CSI reporting is not required

In RAN1#112 meeting, RAN1 further discussed the mechanisms on CSI-RS resource determination across different DL subbands.

Agreement: Study the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs considering the following options: - Option 1: Two contiguous CSI-RS resources that are linked - Option 2: One CSI-RS resource - Option 2-1: Non-contiguous CSI-RS resource allocation - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

In RAN1#112bis e-meeting, the following conclusion was agreed:

Conclusion For the options agreed to study in RAN1#112 for frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, the following observations are agreed. - For all the options, there is no impact on CSI-RS sequence generation. - Option 1 requires additional signalling to link two CSI-RS resources in two DL subbands. - Option 2-1 requires new RRC structure to configure non-contiguous RBs for one CSI-RS resource, which may require additional signalling overhead. - Option 2-2 can reuse the existing signalling design for CSI-RS resource configuration. Option 2-2 can be used to resolve the potential unaligned boundaries between CSI-RS resource configuration and SBFD subbands - Further discussion is required on the UE complexity due to: - UE capability of maximum number of configured CSI-RS resources - Processing non-contiguous CSI-RS

Based on the above information, both option 1 and option 2-2 are potential mechanisms for configuring CSI-RS resource, which are existing mechanism. From this point of view, it is obvious that existing configuration for CSI-RS resource on SBFD symbols is workable.

Proposal 12: The existing mechanisms to indicate frequency resources for DL transmission is sufficient for SBFD symbols, including PDSCH and CSI-RS.

Considering multiple CSI-RS resource can be connected to the same CSI reporting, option 1 is actually already supported by the current specification. Subband or wideband CSI reporting can be used depending on the report quantity and indicated granularity if applicable. However, Option 1 consumes the limited number of CSI-RS resources, which may have negative impacts on the CSI-RS measurement and reporting on legacy slot. Furthermore, we need to further study how to address the issue that CSI-RS resource may partially overlap with UL subband. Option 2 makes it possible for CSI measurement on SBFD slot. It can also resolve the problem that partial RBs cannot be used at DL subband boundary. Hence, we slightly prefer option 2.

Furthermore, we achieve the following agreements in RAN1#112bis e-meeting:

Agreement - For semi-static SBFD, for a CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS resources within DL subband(s) are valid for SBFD-aware UE. - For semi-static SBFD, for a CSI reporting subband which overlaps with SBFD subband boundaries, CSI report is derived based on CSI-RS resources excluding CSI-RS resources outside DL subband(s).

The first bullet implies that a CSI-RS resource can be across DL subband boundary. If happens, only CSI-RS resources within DL subband(s) are valid for SBFD-aware UE. From this perspective, option 2-2 is already supported from the above agreement.

Proposal 13: For CSI-RS resource configuration on SBFD slot, non-contiguous CSI-RS resource configuration via implicit mechanism is supported on SBFD slot, i.e., option 2-2.

On the other hand, lots of report quantity require wideband CSI reporting, for example, reportQuantity is set to 'cri-RI-i1', or 'cri-RSRP' or 'ssb-Index-RSRP' or 'cri-SINR', or 'ssb-Index-SINR'. The impacts on CSI reporting accuracy because of discrete CSI-RS resources across two DL subbands should also be studied.

Handling of unaligned boundaries between SBFD subband(s) and frequency unit

As aforementioned, the frequency granularity of DL PDSCH is determined according to active DL BWP and the frequency granularity of CSI-RS resource is predefined according to specification. On the hand, the configuration of subband is pretty flexible and it is very difficult to guarantee boundaries between DL subband and any DL frequency unit are aligned. One example is shown in Figure 9. It can be left to gNB implementation to avoid any frequency resource allocation resulting into an edge RBG across DL subband boundary. However, it would restrict scheduling flexibility and degrade resource utilization. For example, assuming a RBG contains 32 RBs while the subband-edge RBG has one RB which is outside DL subband. In this example, 31 RB is not available for scheduling, which bring significantly negative impacts on scheduling and system performance.

Figure 9 Example of unaligned boudaries between RBG and DL subband

The unaligned boundaries issue is generic for all DL resource unit, including CSI reporting subband, CSI-RS resource and PRG. Hence, a unified solution should be pursued for all the aforementioned channel and signal. One straightforward and universal mechanism is that the available resources for DL are determined implicitly via excluding unavailable RBs outside DL sub-band(s). Accordingly, gNB can allocate DL resources for PDSCH and CSI-RS in SBFD symbols as usual. Once the subband-edge frequency unit, e.g., RBG, PRG, CSI-RS resource, partially overlaps with unavailable resources, UE only takes available RBs into consideration.

Proposal 14: For a subband-edge frequency unit, i.e., RBG, PRG and CSI-RS resource, only the RBs contained by DL subband are used for DL transmission.

Physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots

In RAN1#111 meeting, we discussed the potential issues on transmission/reception across non-SBFD symbol and SBFD symbol. The key point is that the available frequency resources in different slot type, i.e., non-SBFD symbol and SBFD symbol, are different. As a starting point, the following agreement was achieved:

Agreement Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following: - PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols - Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols - Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition - Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols - Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered. Examples of potential enhancements include: - Resource allocation in frequency domain including frequency hopping - Resource allocation in time domain - Power domain - Spatial domain FFS: If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot if configured.

In RAN1#112 meeting, we clarified that each transmission/reception within a slot has either all SBFD or all non-SBFD symbols.  Basically, two directions were identified, i.e., whether transmissions/receptions are allowed to be across different types of symbols, which are addressed by the following proposal.

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) - Study the following options for SBFD-aware UEs: - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following: - PDSCH/PUSCH/PUCCH repetitions - SPS PDSCH/CG PUSCH - TBoMS - Multi-PUSCH/PDSCH scheduled by a single DCI - Periodic/semi-persistent SRS/CSI-RS/PUCCH - PDCCH

Regarding the option 1 and option 2, they are high-level guidance which we need to achieve a common understanding. For example, option 1 may be unnecessary if resource allocation doesn’t exceed UL/DL subband in SBFD slot. On the other hand, always assuming the transmissions/receptions are in SBFD symbols and non-SBFD symbols may be too aggressive if there is no restriction on the frequency resource allocation. Besides, the issues caused by UL subband is quite different for different channels or signals. For example, discontinuous frequency resource is the major challenge for DL transmission while different available frequency range is the essential issue for uplink.  To be specific, the key issue for uplink transmission is the different number of available resources across SBFD slot and non-SBFD slot. Furthermore, option 1 is a sub-case of option 2 from our understanding. Hence, option 2 provides more flexibility and possibility to better handle SBFD operation across SBFD symbols and non-SBFD symbols.

Proposal 15: For UL transmission and DL reception, it can be across SBFD symbols and non-SBFD symbols in different slots.

- It is possible that the transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only.

On top of the aforementioned principle, the following agreement was achieved in RAN1#112bis meeting:

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), if the transmissions/receptions can be in SBFD symbols and non-SBFD symbols with different available resources, study at least the following frequency resource allocation options for PDSCH, CSI-RS, PUSCH, PUCCH, SRS for SBFD-aware UE: - Option 1: Separate FDRA determination for SBFD slots and non-SBFD slots. - Option 1-1: Separate FDRA configurations/indications for SBFD slots and non-SBFD slots - Option 1-2: Separate frequency resources determined for SBFD slots and non-SBFD slots based on single FDRA configuration/indication - Option 1-3: single FDRA configuration/indication and RB offset(s) - Option 2: Perform rate matching or puncturing on the RBs outside DL/UL subbands for DL/UL channels/signals. - Option 3: A DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot is dropped or postponed. Note: Different options can be studied for different signals/channels.

The above options target on the case wherein UL transmissions and DL receptions are across SBFD symbols and non-SBFD symbols in different slots.  For option 2, it leads to different coding rate for repetitions across different slots, which may be problematic for combination. Especially, it is not workable when polar coding is applied. Hence, we slightly prefer further study option 1 and option 3.

Proposal 16: For UL transmission and DL reception across SBFD symbols and non-SBFD symbols in different slots, further study option1 and option 3.

The frequency allocation mechanism highly depends on the physical structure of channel/signal, configuration, etc. In the following section, we provide our analyses for different channel/signal one by one.

PUSCH transmission on multiple slots

For PUSCH, RBs belonging to active UL BWP are available in UL symbol while only RBs belonging to UL subband are available in SBFD symbol. Different from DL resources, UL resources either on UL symbol or SBFD symbol are continuous. The issue comes from the number of available UL RBs on different OFDM symbol is different. Consequently, single FDRA indication may not be suitable once PUSCH is allocated with UL symbol and SBFD symbol. If frequency hopping is enabled, the issue is even more severe as PUSCH hop may exceed UL subband due to the more restricted available UL resources.

Figure 10 Examples for PUSCH transmission across SBFD symbol and non-SBFD symbol

PUSCH can be scheduled by UL grant, or be activated by an activation DCI, or be configured via CG configuration. No matter how a PUSCH is scheduled, we believe current specification is sufficient to avoid the case where a PUSCH is across SBFD symbol and non-SBFD symbol if repetition is not enabled.

Regarding to PUSCH repetition without frequency hopping, same resource allocation in frequency domain for different repetitions can be achieved up to implementation. At least the following several options can be considered:

- PUSCH repetition is scheduled on UL slot only, if its frequency allocation exceeds UL subband. One possible mechanism is gNB should guarantee the first PUSCH transmission locates on UL slot or flexible slot once PUSCH repetition is configured or indicated. For the left repetitions, they are transmitted on UL slot or flexible slots. In the other words, SBFD slot is regarded as unavailable slots. It is similar to option 3.

- PUSCH repetition is scheduled on SBFD slot only if its frequency allocation exceeds UL BWP. One example is the first PUSCH transmission locates on SBFD slot while UL subband is partial overlapped with UL BWP. However, it can be treated as error case considering we already agreed that UE can only transmit uplink within active UL BWP.

- PUSCH repetition is scheduled on UL slot and SBFD slot, with confining frequency resources in UL subband.

- PUSCH repetition is scheduled on UL slot and SBFD slot, the frequency allocation on UL slot and SBFD slot can be different. It is option 1.

For the last mechanism, enhanced FDRA for PUSCH is required. However, the motivation and benefits may need further justification.

Regarding to PUSCH repetition with frequency hopping, it is too restrictive if gNB has to always guarantee two PUSCH hops are located within UL subband. Currently, a frequency hopping offset list is configured for determining each PUSCH hop. In order to avoid PUSCH hop is out of UL subband, conservative scheduling decision is the only choice. It obviously degrades potential performance gain harvested from frequency hopping. Similar to PUSCH repetition without frequency hopping, the following several options can be considered:

- PUSCH repetition is scheduled on UL slot only if one of the hops exceeds UL subband. One possible mechanism is gNB should guarantee the first PUSCH transmission locates on UL slot or flexible slot once PUSCH repetition and frequency hopping are configured or indicated. For the left repetitions, they are transmitted on UL slot or flexible slots. In the other words, SBFD slot is regarded as unavailable slots.

- PUSCH repetition is scheduled on SBFD slot only if one of the hops exceeds UL BWP. One example is the first PUSCH transmission locates on SBFD slot while UL subband is partially overlapped with UL BWP. However, it can be treated as error case considering we already agreed that UE can only transmit uplink within active UL BWP.

- PUSCH repetition is scheduled on UL slot and SBFD slot, and gNB guarantees PUSCH hop on SBFD slot doesn’t exceed UL subband with the same frequency hopping offset list.

- PUSCH repetition is scheduled on UL slot and SBFD slot, separate frequency offset lists are applied to UL slot and SBFD slot respectively.

For the last mechanism, additional consideration on how to use single FH offset indication to indicate proper FH offset for UL slot and SBFD slot respectively is needed.  Furthermore, we think frequency hopping is not covered by any of frequency resource allocation options as aforementioned.

Proposal 17: For PUSCH repetition without frequency hopping, the following mechanisms can be considered for PUSCH across SBFD slot and non-SBFD slot:

- Option1: PUSCH repetition is mapped on non-SBFD slot only if PUSCH resource exceeds UL subband, otherwise PUSCH repetition can be mapped on non-SBFD slot and SBFD slot

- Option2: PUSCH repetition is scheduled on non-SBFD slot and SBFD slot with confining frequency resources in UL subband

- Option3: PUSCH repetition is scheduled on non-SBFD slot and SBFD slot, the frequency allocation on UL slot and SBFD slot can be different

Proposal 18: For PUSCH repetition with frequency hopping, the following mechanisms can be considered for PUSCH across SBFD slot and non-SBFD slot:

- Option1: PUSCH repetition is mapped on non-SBFD slot only if one of the hops exceeds UL subband, otherwise PUSCH repetition can be mapped on non-SBFD slot and SBFD slot

- Option2: PUSCH repetition is scheduled on non-SBFD slot and SBFD slot, same frequency hopping offset list is applied

- Option3: PUSCH repetition is scheduled on non-SBFD slot and SBFD slot, separate frequency offsets list are applied to UL slot and SBFD slot respectively

PUCCH can be transmitted with periodic manner or aperiodic manner. The time and frequency resource for PUCCH transmission is indicated by PUCCH resource indicator and/or PUCCH-config. Furthermore, PUCCH repetition is supported via either DCI indication or RRC configuration.

For a DCI indicated PUCCH transmission without repetition, gNB can indicate a PUCCH resource anywhere within UL subband based on pre-configured PUCCH resource list. Considering the available bandwidth for uplink transmission is quite different on UL slot and SBFD slot, gNB has to configure at least one PUCCH resource which fits into UL subband. As there are at least 8 PUCCH resources can be configured for a PUCCH resource set, it should be sufficient to reuse current mechanism to indicate the PUCCH resource on UL slot or on SBFD slot.

For a periodic PUCCH transmission, its time/frequency resource is totally configured by RRC signalling. The minimum periodicity is 4 slots and the offset within periodicity can also be configured. Theoretically, periodic PUCCH transmission can be always allocated on UL slot or SBFD slot. Even if the periodic PUCCH are located on both SBFD slot and UL slot, there is no issue if a proper configuration is provided, e.g., the assigned PUCCH resource is within UL subband in frequency domain. From this perspective, the current mechanism to configure periodic PUCCH transmission can be reused when UL subband is configured on some OFDM symbols.

Observation 9: For PUCCH without repetition, current specification is sufficient to avoid PUCCH transmission across SBFD symbols and non-SBFD symbols.

For PUCCH transmission with repetition, it is quite similar to PUSCH. Accordingly, we have the following proposal:

Proposal 19: For PUCCH repetition without frequency hopping, the following mechanisms can be considered for PUCCH across SBFD slot and non-SBFD slot:

- Option1: PUCCH repetition is mapped on non-SBFD slot only if PUCCH resource exceeds UL subband, otherwise PUCCH repetition can be mapped on non-SBFD slot and SBFD slot.

- Option2: PUCCH repetition is indicated on non-SBFD slot and SBFD slot with confining frequency resources in UL subband.

- Option3: PUCCH repetition is indicated on non-SBFD slot and SBFD slot, the frequency allocation on UL slot and SBFD slot can be different.

For the last mechanism, i.e., different PUCCH resources for UL slot and SBFD slot, we need to further study the mechanisms of how to indicate two different PUCCH resources. For example, whether to reuse current BWP-specific PUCCH resource set, and whether to introduce a joint indication for PUCCH resource determination, etc.

For PUCCH with repetition and inter-slot frequency hopping, we have the following proposal:

Proposal 20: For PUCCH repetition with frequency hopping, the following mechanisms can be considered for PUCCH across SBFD slot and non-SBFD slot

- Option1: PUCCH repetition is mapped on non-SBFD slot only if one of the hops exceeds UL subband, otherwise PUCCH repetition can be mapped on non-SBFD slot and SBFD slot

- Option2: PUCCH repetition is indicated on non-SBFD slot and SBFD slot, same frequency hopping offset list is applied

- Option3: PUCCH repetition is indicated on non-SBFD slot and SBFD slot, separate frequency offsets list are applied to UL slot and SBFD slot respectively

If TBoMS is enabled for PUSCH transmission, it will be transmitted on N available slots as defined in TS38.214. However, different from PUSCH repetition, a single TB is transmitted across N slots. The TBS is determined not only by the time/frequency within a slot but also the number of N slots. The procedure of determining NRE for a PUSCH with TBoMS is excerpted as below. [4].  Obviously, the allocated RB across different slot should be same. For TBoMS PUSCH across SBFD slot and non-SBFD slot, gNB should guarantee the same time/frequency resource allocation is applied.

TS38.214, section 6.1.4.2 - A UE determines the total number of REs allocated for PUSCH as follows - For TB processing over multiple slots, NRE=N*min156,NRE'⋅nPRB where is the total number of allocated PRBs for the UE and N is the number of slots used for TBS determination indicated by numberOfSlotsTBoMS. - Otherwise, NRE=min156,NRE'⋅nPRB.

Proposal 21: For PUSCH with TBoMS, the following mechanisms can be considered for PUSCH across SBFD slot and non-SBFD slot:

- Option1: PUSCH with TBoMS is mapped on UL slot only if resource allocation of PUSCH exceeds UL subband,  otherwise PUSCH with TBoMS can be mapped on non-SBFD slot and SBFD slot

- Option2: PUSCH with TBoMS is mapped on non-SBFD slot and SBFD slot with confining frequency resources in UL subband

For multi-PUSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols, same frequency resources are indicated for different PUSCH transmission on different slots. Although the TBs carried by different PUSCH are different, the resource allocation is quite similar to the case of PUSCH repetition. Accordingly, the same mechanisms as PUSCH without inter-slot frequency hopping can be considered. For multi-PDSCH scheduled by a single DCI, it is similar to multi-PUSCH excepting DL subband cannot exceed DL BWP. We have the following proposals for multi-PUSCH/multi-PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols respectively.

Proposal 22: For multi-PUSCH scheduled by a single DCI, the following mechanisms can be considered for PUSCH across SBFD slot and non-SBFD slot:

- Option 1: All PUSCHs are mapped on non-SBFD slot only if resource allocation of PUSCH exceeds UL subband, otherwise PUSCHs are mapped on both non-SBFD slot and SBFD slot.

- Option 2: Multi-PUSCH is scheduled on non-SBFD slot and SBFD slot with confining frequency resources in UL subband.

- Option 3: Multi-PUSCH is scheduled on non-SBFD slot and SBFD slot, the frequency allocation on UL slot and SBFD slot can be different.

Proposal 23: For multi-PDSCH scheduled by a single DCI, the following mechanisms can be considered for PDSCH across SBFD slot and non-SBFD slot:

- Option 1: All PDSCHs are mapped on non-SBFD slot only if resource allocation of PDSCH exceeds DL subband, otherwise PDSCHs are mapped on non-SBFD slot and SBFD slot.

- Option 2: Multi-PDSCH is scheduled on non-SBFD slot and SBFD slot with confining frequency resources in DL subband.

- Option 3: Multi-PDSCH is scheduled on non-SBFD slot and SBFD slot, the frequency allocation on DL slot and SBFD slot can be different.

PDCCH search space is configured in a semi-static manner and transmitted periodically. It is possible that search space occurs on SBFD symbols or non-SBFD symbols on different monitoring occasion. As the frequency resource of CORESET is configured via a bitmap with a granularity of 6 RBs, gNB can avoid collision between UL subband and CORESET in frequency domain with proper configuration.

Proposal 24: PDCCH monitoring occasions across SBFD slot and non-SBFD slot are allowed

- For PDCCH, it is up to gNB to avoid CORESET is assigned on unavailable resources in frequency domain.

For SPS PDSCH and Type-1/Type-2 CG PUSCH, they are transmitted in a periodic manner. The periodicity and starting position in time domain is determined by gNB configuration and DCI if applicable, i.e., SPS PDSCH and Type-2 CG PUSCH. Hence, gNB has full power to guarantee a particular SPS PDSCH configuration or CG PUSCH configuration is located on either SBFD slot or non-SBFD slot. One concern is that if SPS PDSCH and CG PUSCH is restricted on SBFD slot only or non-SBFD slot only, it may degrade the flexibility and performance. However, considering up to 8 SPS configuration and 12 CG configuration are supported, gNB can configure multiple SPS or CG in order to cover SBFD slot and non-SBFD slot. From this perspective, we don’t see motivation to further enhance SPS PDSCH and CG PUSCH across SBFD slot and non-SBFD slots.

Proposal 25:  Current mechanism is sufficient for SPS PDSCH and CG PUSCH across SBFD slot and non-SBFD slot if repetition is not configured.

- For a SPS configuration or CG configuration, it is only located on non-SBFD slots if the particular transmission exceeds DL subband or UL subband.

- Otherwise, SPS PDSCH or CG PUSCH can locates on both SBFD slot and non-SBFD slot.

For PDSCH with repetition, the situation is quite similar as that of PUSCH with repetitions when they are transmitted on both SBFD slots and non-SBFD slots. Resource allocation on DL slot may be not suitable for SBFD slot considering UL subband is configured. Hence, similar solutions as PUSCH with repetition can be considered. It should be noted that DL subband cannot exceed DL BWP as it is determined within DL BWP. We don’t need to consider the case wherein PDSCH repetition is transmitted on SBFD slot only.

Proposal 26: For PDSCH repetition, the following mechanisms can be considered for PDSCH across SBFD slot and non-SBFD slot:

- PDSCH repetition is mapped on non-SBFD slot only if PDSCH resource exceeds DL subband, otherwise PDSCH repetitions can be mapped on non-SBFD slot and SBFD slot.

- PDSCH repetition is scheduled on non-SBFD slot and SBFD slot with confining frequency resources in DL subband

- PDSCH repetition is scheduled on non-SBFD slot and SBFD slot, the frequency allocation on DL slot and SBFD slot can be different

CSI reporting configuration

In RAN1#112bis e-meeting, we discussed how a SBFD-aware UE reports CSI if the relevant CSI-RS resources locates on both SBFD slot and non-SBFD slot. The following agreement was made:

Agreement For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both SBFD and non-SBFD symbols: - Option 1: two CSI-ReportConfigs, where one is associated with SBFD symbols and the other is associated with non-SBFD symbols - Option 1-1: One CSI-ReportConfig is associated with a CSI-RS restricted to SBFD symbols only and the second CSI-ReportConfig is associated with a second CSI-RS restricted to non-SBFD symbols only; - Option 1-2: Both CSI-ReportConfigs are associated with the same CSI-RS. The CSI report associated with one CSI-ReportConfig is derived based on CSI-RS instances in SBFD symbols only. The CSI report associated with the second CSI-ReportConfig is derived based on CSI-RS instances in non-SBFD symbols only. - Option 2: one CSI-ReportConfig associated with both SBFD symbols and non-SBFD symbols - Option 2-1: One CSI-ReportConfig is associated with two CSI-RSs which are restricted to SBFD symbols and non-SBFD symbols respectively. Separate CSI measurements are derived based on the first and second CSI-RSs respectively. - Option 2-2: One CSI-ReportConfig is associated with one CSI-RS. The CSI report is derived based on CSI-RS which can be in SBFD symbols or non-SBFD symbols in different time instances. - FFS impact on UE CSI processing and reporting timeline Note: Whether the CSI-RS resource can be used for SBFD and non-SBFD symbols may depend on, e.g., gNB implementation of same/different antenna configuration in both symbols. Option 1-1 can be supported according to existing specification by gNB configuration of appropriate periodicities to ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. But it may restrict the gNB configuration flexibility and enhancements can be considered by additional indication or rules to determine the CSI-RS is valid within one symbol type and is invalid in the other symbol type. Option 2-2 can be supported according to existing specification to configure measurement restriction so that UE would not average CSI measurements across SBFD and non-SBFD symbols.

CSI reporting is carried by PUSCH or PUCCH, i.e., aperiodic CSI report is carried by PUSCH which is scheduled by DCI and periodic CSI report is carried by PUCCH which is configured by PUCCH. The resources carrying CSI reporting should be discussed under the umbrella of PUCCH and PUSCH.

As discussed in study item, there are several manners of implementation on antenna architecture for SBFD slot and non-SBFD slot, carter to the requirement on duplex technology. In RAN1#110 meeting, three options were raised:

- SBFD antenna configuration option-1 (same as Opt 1 in RAN1#109 agreement): The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD.

- SBFD antenna configuration option-2 (same as Opt 2 in RAN1#109 agreement): The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD.

- SBFD antenna configuration option-3 (new): The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is half of the total number of TxRUs of the antenna array for legacy TDD.

It is obviously the channel state measured on different slots assuming different antenna architecture is quite different. From this perspective, CSI reporting associated with same CSI-RS resource which locates on both SBFD slot and non-SBFD slot may be not capable to reflect the realistic channel condition. On the other hand, antenna architecture is totally up to gNB implementation. To be specific, gNB A may employ option 1 resulting in different capabilities on different slots. In this case, separate CSI reporting for SBFD symbols and non-SBFD symbols should be supported. On the other hand, gNB B may employ option 2 wherein the available TxRU and antenna element for SBFD slot and non-SBFD slot are same. In this case, same CSI reporting for SBFD symbols and non-SBFD symbols is sufficient and can reduce the consumption on CSI reporting configuration. Although the CSI RS located on non-SBFD slot suffers inter-subband UE-UE CLI, we believe it may be trivial in some cases and can be handled properly by gNB.

From our understanding, option 1-1 and option 2-2 are sufficient for accurate channel estimation/measurement. On the other hand, option 1-2 requires more specification change while the benefit is unclear. Furthermore, considering both option 1-1 and option 2-2 are already supported since Rel-15, there is no reason to preclude one of them. Regarding to option 2-1, it is quite similar to the mechanism introduced in Rel-18 NES spatial domain enhancement. To be specific, a CSI report consists of several sub-report, wherein each sub-report associates with different CSI-RS resource. Different from NES, we may further clarify that each sub-report corresponds to a CSI-RS resource located on specific symbol type.

Proposal 27: For SBFD-aware UEs, both option 1-1 and option 2-2 are supported for CSI report associated with periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both SBFD and non-SBFD symbols.

- FFS whether and how to support option 2-1

Conflict of transmission direction on SFBD symbols

SBFD aware UE is only capable of half duplex, i.e., it supports either uplink transmission or downlink reception at one time. Accordingly, gNB can schedule/configure a SBFD aware UE to transmit uplink in UL subband or receive DL in DL subband. Considering UE can only transmit uplink or receive downlink at one time, we should consider how to handle conflicts between UL and DL. There are several potential conflicting cases on SBFD slot for a SBFD aware UE, which are listed as below:

- Type 1: Dynamic DL on SBFD symbol conflicts with Dynamic UL on the UL subband.

- Type 2: Semi-static DL on SBFD symbol conflicts with Dynamic UL on the UL subband.

  - The semi-static DL includes periodic reference signal, semi-persistent reference signal, SPS PDSCH without DL assignment, etc.

- Type 3: Broadcast DL on SBFD symbol conflicts with UL on the UL subband.

  - The broadcast DL includes Type0/0A/1/2-PDCCH CSS and SSB

- Type 4: Dynamic DL on SBFD symbol conflicts with semi-static UL on the UL subband, including CG PUSCH, periodic uplink reference signal and periodic PUCCH.

- Type 5: Semi-static DL on SBFD symbol conflicts with semi-static UL on the UL subband.

Theoretically, gNB has full power to avoid collision between dynamic DL and dynamic UL on the same symbol. Therefore, it may be a reasonable hypothesis that UE doesn’t expect gNB dynamically schedules uplink and downlink at the same time. On the other hand, dynamic downlink channel and dynamic uplink channel may have different priorities, i.e., one is associated with eMBB while the other is associated with URLLC. gNB may firstly schedule a PUSCH or PDSCH carrying eMBB traffic and later schedule a PDSCH or PUSCH carrying URLLC on the same symbol. In this sense, it makes sense to consider collision between dynamic downlink and dynamic uplink with different priority respectively.

On the other hand, we briefly discussed whether it is a valid case wherein eMBB traffic and URLLC traffic with different direction colliding with each other on the same OFDM symbols. Considering URLLC traffic typically has sporadic characteristic, it is very hard for network to make a precise prediction. Once URLLC packet arrives, network should guarantee it is transmitted as soon as possible in order to satisfy the requirement of low latency. Hence, it is inevitable URLLC traffic collides with eMBB traffic. More importantly, 3GPP already designs and specifies the mechanism to handle the collision between URLLC and eMBB, for example:

- In Rel-15, pre-emption indication was specified in order to improve the performance of eMBB PDSCH which is interrupted by URLLC PDSCH.

- In Rel-16, uplink cancellation was specified in order to improve the performance of URLLC PUSCH which is interfered by eMBB PUSCH.

All in all, we should pay careful attention on the case of DG DL collides with DG DL if they have different priorities.

Observation 10: The conflicted dynamic downlink and dynamic uplink on the same SBFD symbol may have different priorities.

For type 2 collision, gNB may transmit semi-static downlink channel or reference signal on the DL subband(s) whilst schedule the same SBFD aware UE to transmit uplink channel or reference signal on the UL subband. One straightforward mechanism is to assume dynamic scheduling takes higher priority, which is the generic principle since Rel-15. However, the aforementioned dynamic channel and the semi-static channel may have different priorities. For example, gNB schedules a PUSCH with R=4 while the SPS periodicity equates two, which can be explained in Figure 14. On slot#0, SPS PDSCH collides with PUSCH repetition. If SPS PDSCH is configured with a high priority, i.e., its harq-CodebookID points to the HARQ codebook with high priority, it is hard to say the dynamic PUSCH should be prioritized.

Figure 13 Example of collision between semi-static DL and dynamic UL

Observation 11: Semi-static DL and dynamic UL may collide on SBFD symbol.

For type 3 collision, gNB transmits broadcast channels, i.e., type 0/0A/0B/1/2-PDCCH CSS and SSB, while UE is required to transmit uplink channel or signal on the same SBFD symbol. Both type 0/0A/0B/1/2-PDCCH CSS and SSB target for group of UEs, including SBFD aware UE and non-SBFD aware UE. Their configuration and transmission should not be impacted by SBFD operation. On the other hand, gNB may or may not schedule UL transmission within UL subband in a SBFD slot containing aforementioned DL broadcast channels. There is a concern that intra-cell UE-to-UE CLI may jeopardize DL performance, particularly SSB is fundamental for system performance. Actually, SSB is quite robust in nature, with low coding rate, robust physical structure, special RS design, etc. Even in the legacy network, SSB suffers intra-subband gNB-to-UE interference from neighbouring cell. Intra-gNB UE-to-UE CLI may be not as significant as legacy gNB-to-UE interference. Furthermore, frequency gap between SSB and UL transmission within UL subband can provide additional protection for SSB.

Observation 12: The impacts on SSB/CSS from UL transmission within UL subband may not always be an issue.

For type 4 collision, it is similar to type 2 collision, i.e., gNB may transmit dynamic downlink channel or reference signal on the DL subband(s) whilst indicates the same SBFD aware UE to transmit semi-static uplink channel or reference signal on the UL subband.

For type 5 collision, we need to discuss the semi-static DL transmission and semi-static UL transmission case by case. We summarize the procedure of how to determine the time location of each semi-static channel or reference signal as below:

Table 1  Determination of time domain resource for semi-static channel and reference signal

| Semi-static channel/signal | Procedure of determining time location for each transmission |
| --- | --- |
| Type-1 CG PUSCH | The periodicity, time reference SFN, time offset and time domain resource allocation are configured by RRC signalling. |
| Type-2 CG PUSCH | The periodicity of CG PUSCH occasion is provided by RRC signalling. The time domain resource allocation within a slot is provided via activation DCI. |
| Periodic PUCCH | PUCCH resource, periodicity and offset are configured by RRC signalling. |
| Periodic SRS | SRS resource, periodicity and offset are configured by RRC signalling. |
| Semi-persistent SRS | SP SRS is triggered via MAC CE. Its periodicity and offset are configured by RRC signalling. |
| SPS PDSCH | SPS PDSCH is activated via DCI format with CRC scrambled by CS-RNTI. The periodicity is configured via RRC signalling. |
| Periodic CSI RS | The periodicity and offset for NZP CSI-RS and ZP CSI-RS are configured via RRC signalling. |
| Semi-persistent CSI RS | Both SP ZP CSI-RS and SP NZP CSI-RS are triggered by MAC CE, i.e. with individual MAC CE respectively. The periodicity and offset are configured via RRC signalling. |

Generally speaking, gNB can avoid collision between semi-static uplink and semi-static downlink via proper configuration. On the other hand, the additional consideration on avoiding collision between DL and UL may degrade the performance for one or another. For example, Type-1 CG PUSCH carrying URLLC traffic with high priority and SPS PDSCH are configured for UE simultaneous. If gNB has to guarantee there is no overlapping between SPS PDSCH and CG PUSCH in time domain, it introduces restrictions on the configuration for both of them, which is not expected.

Observation 13: The conflict between semi-static DL and semi-static UL may occur.

Based on the aforementioned analyses, we have the following proposal:

Proposal 28: The following conflicting cases of DL reception and UL transmission are identified:

- Type 1: Dynamic DL on SBFD symbol conflicts with Dynamic UL on the UL subband.

- Type 2: Semi-static DL on SBFD symbol conflicts with Dynamic UL on the UL subband.

- Type 3: Broadcast DL on SBFD symbol conflicts with UL on the UL subband.

- Type 4: Dynamic DL on SBFD symbol conflicts with semi-static UL on the UL subband, including CG PUSCH, periodic uplink reference signal and periodic PUCCH.

- Type 5: Semi-static DL on SBFD symbol conflicts with semi-static UL on the UL subband.

DL-to-UL switching period between SBFD symbol and non-SBFD symbol

For unpaired spectrum, guard period is needed between DL region and UL region. Basically, the guard period is expected to provide sufficient time between DL region and UL region for DL-to-UL switching and ICI interference avoidance. For example, UE needs time to conduct DL-to-UL switching and TA adjustment. Currently the guard period is covered by flexible slot and the length of guard period is up to implementation, i.e. a value not smaller than NTA+NTA,offset. Similarly, guard period is also needed between the last OFDM symbol in DL region and the first OFDM symbol in UL subband. One example is shown in Figure 17, wherein DL-to-UL switching point between DL slot and UL subband is needed in addition to the legacy guard period. The additional DL-to-UL switching point can be configured explicitly or implicitly. However, the overhead should also be carefully evaluated as the guard period cannot be used for transmission and reception. Additional DL-to-UL switching point would reduce the spectrum utilization.

Figure 14 Examples of DL to UL switching point between DL and UL subband

For legacy TDD, uplink frame number  for transmission from the UE shall start TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc before the start of the corresponding downlink frame at the UE side. The purpose is to compensate propagation delay and provide sufficient time for Rx-to-Tx switching at gNB side.[5] The Uplink-downlink time relation can be explained by Figure 18. As there is always flexible symbol between DL symbol and UL symbol, inter-slot interference caused by time advance at UE side can be avoided and Rx-to-Tx switching gap NRx-TxTc can be guaranteed.

Figure 15 Uplink-downlink timing relation

For SBFD aware UE, UL subband is configured in DL slots. An example is shown in Figure 19, wherein assuming UL subband is configured in DL slot#2 to DL slot#5. There is a back-to-back transmission between DL slot#1 and UL subband in slot#2. As a timing advance is applied to UE, i.e., TTA defined in TS38.211, UE needs to transmit uplink in advance with TTA relative to its DL timing. As there is no guard period between DL symbol and UL subband, the following issues are observed:

- Issue#1: A SBFD aware UE needs to transmit uplink on the resources potentially scheduled with downlink, i.e., there is collision between DL and UL for the UE. Considering UE only half duplex capability, gNB has to avoid such kind of collision via scheduling. In the other words, some resources are unavailable for DL reception or UL transmission for this UE, which degrades its performance.

- Issue#2: If gNB schedules uplink transmission on the conflicting symbols, gNB suffers intra-subband self-interference as gNB needs to schedule DL for the other UEs. This case is out of scope as Rel-18 only study SBFD operation, instead of SSFD operation.

- Issue#3: The uplink transmission on the conflicting resources introduces intra-subband UE-UE CLI for the other UEs belonging to the same serving cell. It will significantly degrade legacy UE’s DL performance, which is not preferred.

Figure 16 Timing alignment due to back-to-back DL and UL

Currently, flexible symbols need to be configured between DL region and UL region. Thanks to the flexible symbols in between, NTA,offset and NTA can be employed to guarantee aligned timing at gNB side and provide sufficient gap for Rx-to-Tx switching. Hence the key point is a guard period must be provided between the last DL symbol in DL slot and the first UL symbol in UL subband. At least the following mechanisms can be considered in the further study:

- Option 1: The guard period is configured by gNB for SBFD aware UE.

- Option 2: UE does not expect to transmit in the uplink on the first G symbols within UL subband.

- Option 3: The first OFDM symbol of UL subband is always adjacent UL symbol or flexible symbol.

Option 3 is the simplest mechanism and can fully enjoy the bonus of semi-static flexible OFDM symbols. No or trivial standard impact is expected. One example is shown in Figure 21. UL subband is configured right behand semi-static UL symbol, i.e., UL region is contiguous from SBFD aware UE point of view. The flexible symbols can be used for the guard period between DL region and UL region. Unified TA value, i.e., determined by the current mechanism, is applied to SBFD symbol and UL symbol. As the legacy gap period can be fully reused, we don’t need to introduce or define additional gap period between DL symbol and SBFD symbol. The spectrum efficiency is higher compared with option 1 and option 2. Hence, we slightly prefer option 3.

Figure 17 UL subband is adjacent to UL symbol

Proposal 29: Guard period between DL region and UL subband is needed and at least the following mechanism can be considered.

- Option 1: The guard period is configured by gNB for SBFD aware UE.

- Option 2: UE does not expect to transmit in the uplink on the first G symbols within UL subband.

- Option 3: The first OFDM symbol of UL subband is always adjacent UL symbol or flexible symbol.

Conclusion

In this contribution, we provide our views on subband non-overlapping full duplex. We have the following observations:

Observation 1: There are several mechanisms to realize TDD UL-DL configuration which leads to different TDD periodicity:

- Single TDD periodicity provided by tdd-UL-DL-ConfigurationCommon

- Dual TDD periodicity provided by tdd-UL-DL-ConfigurationCommon

- Additional TDD UL-DL configuration provided by tdd-UL-DL-ConfigurationDedicated on top of common configuration

- No TDD periodicity is provided by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated

Observation 2: A unified solution to indicate the SBFD subband time domain location should be employed in order to simplify specification and gNB operation.

Observation 3: SLIV-like method is not sufficient for indicating SBFD time location for discrete DL region hence a unified indication is impossible across different TDD configuration scenarios.

Observation 4: Bitmap based method can be used to indicate SBFD time location within a time period.

- The time period is equal to TDD periodicity provided by tdd-UL-DL-ConfigurationCommon

- FFS: the time unit of bitmap

- FFS: how to determine time period if gNB doesn’t provide any TDD configuration

Observation 5: If bitmap-based mechanism or SLIV-based mechanism is used to indicate SBFD subband time domain location, how to define guard period between DL symbol and first symbol occupied by UL subband needs further study.

Observation 6: A subband-specific TDD UL-DL configuration can be used to indicate SBFD subband time domain location.

Observation 7: It is a typical case for a self-contained slot that it consists of both SBFD and non-SBFD symbols.

Observation 8: The maximum number of transition point between DL/UL to UL/DL is a generic issue for SBFD operation.

Observation 9: For PUCCH without repetition, current specification is sufficient to avoid PUCCH transmission across SBFD symbols and non-SBFD symbols.

Observation 10: The conflicted dynamic downlink and dynamic uplink on the same SBFD symbol may have different priorities.

Observation 11: Semi-static DL and dynamic UL may collide on SBFD symbol.

Observation 12: The impacts on SSB/CSS from UL transmission within UL subband may not always be an issue.

Observation 13: The conflict between semi-static DL and semi-static UL may occur.

Based on the aforementioned discussion and observation, we have the following proposals:

Proposal 1: For semi-static configuration of subband time location for SBFD operation, the following mechanisms can be considered:

- gNB indicates SBFD subband time domain location via a bitmap.

- gNB indicates SBFD subband time domain location via SLIV indication with assuming UL subband cannot be located on UL symbols.

- gNB indicates SBFD subband time domain location via a subband-specific TDD UL-DL configuration.

Proposal 2: UL subband cannot be applicable to UL symbols, including semi-static UL symbol and dynamic UL symbol indicated via SFI.

- If gNB indicates UL subband is present on UL symbols, UE ignores the UL subband configuration on UL symbols and transmits uplink on UL symbol as normal.

Proposal 3: Cell-specific signalling is sufficient to configure SBFD subband time location.

Proposal 4: If UL subband is configured on semi-static flexible symbol and gNB further adjust direction for the symbols, clarify behaviour of SBFD aware UE at least for the following cases:

- Semi-flexible symbol is indicated as DL symbol on which UL subband is configured

- Semi-flexible symbol is indicated as UL symbol on which UL subband is configured

- Semi-flexible symbol is indicated as SFI flexible symbol on which UL subband is configured

Proposal 5: A slot can consist of both SBFD symbols and non-SBFD symbols.

Proposal 6: Study the maximum number of transition point between DL/UL and UL/DL within a period.

Proposal 7: Only one transition point is allowed within a slot consisting of both SBFD symbols and non-SBFD symbols.

- Flexible symbols between DL symbol and UL symbol can be used as guard period for switching between SBFD symbol and non-SBFD symbol.

Proposal 8: Frequency location of guard subband(s) are explicitly configured if any. DL subband(s) are implicitly derived as the RBs which are not within UL subband or guard subband(s).

Proposal 9: For configuring frequency location of UL subband, the following mechanism can be considered:

- Reuse the same mechanism of configuring BWP to configure the frequency locations of subband for SBFD operation, i.e., the starting position and length are indicated.

- If gNB doesn’t provide any configuration for UL subband frequency location, a default BWP is used to determine the frequency location of the UL subband.

Proposal 10: Cell-specific RRC signalling is used to configure SBFD subband frequency location as baseline.

Proposal 11: Capture it in TS38.214 that only RBs contained in DL subband are available for PDSCH.

Proposal 12: The existing mechanisms to indicate frequency resources for DL transmission is sufficient for SBFD symbols, including PDSCH and CSI-RS.

Proposal 13: For CSI-RS resource configuration on SBFD slot, non-contiguous CSI-RS resource configuration via implicit mechanism is supported on SBFD slot, i.e., option 2-2.

Proposal 14: For a subband-edge frequency unit, i.e., RBG, PRG and CSI-RS resource, only the RBs contained by DL subband are used for DL transmission.

Proposal 15: For UL transmission and DL reception, it can be across SBFD symbols and non-SBFD symbols in different slots.

- It is possible that the transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only.

Proposal 16: For UL transmission and DL reception across SBFD symbols and non-SBFD symbols in different slots, further study option1 and option 3.

Proposal 17: For PUSCH repetition without frequency hopping, the following mechanisms can be considered for PUSCH across SBFD slot and non-SBFD slot:

- Option1: PUSCH repetition is mapped on non-SBFD slot only if PUSCH resource exceeds UL subband, otherwise PUSCH repetition can be mapped on non-SBFD slot and SBFD slot

- Option2: PUSCH repetition is scheduled on non-SBFD slot and SBFD slot with confining frequency resources in UL subband

- Option3: PUSCH repetition is scheduled on non-SBFD slot and SBFD slot, the frequency allocation on UL slot and SBFD slot can be different

Proposal 18: For PUSCH repetition with frequency hopping, the following mechanisms can be considered for PUSCH across SBFD slot and non-SBFD slot:

- Option1: PUSCH repetition is mapped on non-SBFD slot only if one of the hops exceeds UL subband, otherwise PUSCH repetition can be mapped on non-SBFD slot and SBFD slot

- Option2: PUSCH repetition is scheduled on non-SBFD slot and SBFD slot, same frequency hopping offset list is applied

- Option3: PUSCH repetition is scheduled on non-SBFD slot and SBFD slot, separate frequency offsets list are applied to UL slot and SBFD slot respectively

Proposal 19: For PUCCH repetition without frequency hopping, the following mechanisms can be considered for PUCCH across SBFD slot and non-SBFD slot:

- Option1: PUCCH repetition is mapped on non-SBFD slot only if PUCCH resource exceeds UL subband, otherwise PUCCH repetition can be mapped on non-SBFD slot and SBFD slot.

- Option2: PUCCH repetition is indicated on non-SBFD slot and SBFD slot with confining frequency resources in UL subband.

- Option3: PUCCH repetition is indicated on non-SBFD slot and SBFD slot, the frequency allocation on UL slot and SBFD slot can be different.

Proposal 20: For PUCCH repetition with frequency hopping, the following mechanisms can be considered for PUCCH across SBFD slot and non-SBFD slot

- Option1: PUCCH repetition is mapped on non-SBFD slot only if one of the hops exceeds UL subband, otherwise PUCCH repetition can be mapped on non-SBFD slot and SBFD slot

- Option2: PUCCH repetition is indicated on non-SBFD slot and SBFD slot, same frequency hopping offset list is applied

- Option3: PUCCH repetition is indicated on non-SBFD slot and SBFD slot, separate frequency offsets list are applied to UL slot and SBFD slot respectively

Proposal 21: For PUSCH with TBoMS, the following mechanisms can be considered for PUSCH across SBFD slot and non-SBFD slot:

- Option1: PUSCH with TBoMS is mapped on UL slot only if resource allocation of PUSCH exceeds UL subband,  otherwise PUSCH with TBoMS can be mapped on non-SBFD slot and SBFD slot

- Option2: PUSCH with TBoMS is mapped on non-SBFD slot and SBFD slot with confining frequency resources in UL subband

Proposal 22: For multi-PUSCH scheduled by a single DCI, the following mechanisms can be considered for PUSCH across SBFD slot and non-SBFD slot:

- Option 1: All PUSCHs are mapped on non-SBFD slot only if resource allocation of PUSCH exceeds UL subband, otherwise PUSCHs are mapped on both non-SBFD slot and SBFD slot.

- Option 2: Multi-PUSCH is scheduled on non-SBFD slot and SBFD slot with confining frequency resources in UL subband.

- Option 3: Multi-PUSCH is scheduled on non-SBFD slot and SBFD slot, the frequency allocation on UL slot and SBFD slot can be different.

Proposal 23: For multi-PDSCH scheduled by a single DCI, the following mechanisms can be considered for PDSCH across SBFD slot and non-SBFD slot:

- Option 1: All PDSCHs are mapped on non-SBFD slot only if resource allocation of PDSCH exceeds DL subband, otherwise PDSCHs are mapped on non-SBFD slot and SBFD slot.

- Option 2: Multi-PDSCH is scheduled on non-SBFD slot and SBFD slot with confining frequency resources in DL subband.

- Option 3: Multi-PDSCH is scheduled on non-SBFD slot and SBFD slot, the frequency allocation on DL slot and SBFD slot can be different.

Proposal 24: PDCCH monitoring occasions across SBFD slot and non-SBFD slot are allowed

- For PDCCH, it is up to gNB to avoid CORESET is assigned on unavailable resources in frequency domain.

Proposal 25:  Current mechanism is sufficient for SPS PDSCH and CG PUSCH across SBFD slot and non-SBFD slot if repetition is not configured.

- For a SPS configuration or CG configuration, it is only located on non-SBFD slots if the particular transmission exceeds DL subband or UL subband.

- Otherwise, SPS PDSCH or CG PUSCH can locates on both SBFD slot and non-SBFD slot.

Proposal 26: For PDSCH repetition, the following mechanisms can be considered for PDSCH across SBFD slot and non-SBFD slot:

- PDSCH repetition is mapped on non-SBFD slot only if PDSCH resource exceeds DL subband, otherwise PDSCH repetitions can be mapped on non-SBFD slot and SBFD slot.

- PDSCH repetition is scheduled on non-SBFD slot and SBFD slot with confining frequency resources in DL subband

- PDSCH repetition is scheduled on non-SBFD slot and SBFD slot, the frequency allocation on DL slot and SBFD slot can be different

Proposal 27: For SBFD-aware UEs, both option 1-1 and option 2-2 are supported for CSI report associated with periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both SBFD and non-SBFD symbols.

- FFS whether and how to support option 2-1

Proposal 28: The following conflicting cases of DL reception and UL transmission are identified:

- Type 1: Dynamic DL on SBFD symbol conflicts with Dynamic UL on the UL subband.

- Type 2: Semi-static DL on SBFD symbol conflicts with Dynamic UL on the UL subband.

- Type 3: Broadcast DL on SBFD symbol conflicts with UL on the UL subband.

- Type 4: Dynamic DL on SBFD symbol conflicts with semi-static UL on the UL subband, including CG PUSCH, periodic uplink reference signal and periodic PUCCH.

- Type 5: Semi-static DL on SBFD symbol conflicts with semi-static UL on the UL subband.

Proposal 29: Guard period between DL region and UL subband is needed and at least the following mechanism can be considered.

- Option 1: The guard period is configured by gNB for SBFD aware UE.

- Option 2: UE does not expect to transmit in the uplink on the first G symbols within UL subband.

- Option 3: The first OFDM symbol of UL subband is always adjacent UL symbol or flexible symbol.

Reference

  - RP-234035, New WID: Evolution of NR duplex operation: Sub-band full duplex(SBFD)

  - RAN1#110bis e-meeting chairman notes

  - R4-2214376,  Reply LS on interference modelling for duplex evolution, RAN WG4

  - TS38.214, v17.3.0

  - R4-1804119, On NR NTA offset, Intel Corporation

  - R1- 2301411, Feasibility and techniques for subband non-overlapping full duplex, Qualcomm Incorporated

  - R1-2304792, Sub-band non-overlapping full duplex, Ericsson