3GPP TSG RAN WG1 #116			R1-2400563

Athens, Greece, February 26th – March 1st, 2024

Source: 	     Xiaomi

Title:	Discussion on frame structure and timing aspects for Ambient IoT

Agenda item: 	9.4.2.2

Document for:  Decision

Introduction

In RAN#102 plenary meeting, the SID on solutions for Ambient IoT has been approved [1], including the following objectives which are led by RAN1:

General Scope The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement. - Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor - FR1 licensed spectrum in FDD. - Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s). - Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). - From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case. Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum. The following objectives are set, within the General Scope: Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1.

In this contribution, we would like to provide our initial views on the frame structure and timing aspects for Ambient IoT, which may include the aspects of synchronization and timing, random access, scheduling and timing relationships, etc.

Synchronization and timing

In the NR/LTE/NB-IoT system, the UE can maintain the synchronization with NW side by monitoring the periodically transmitted synchronization signal, such as PSS/SSS/PBCH; and for UL, slot/symbol boundary can be aligned b/w UE and NW side, by adopting a proper TA.

However, in Ambient IoT, especially for the type ⅰ device in SID which is limited by ~1 µW peak power consumption, monitoring such kind of periodic synchronization signal has a very high possibility to increase the power consumption and break the complexity limitation, therefore, it is difficult to maintain long-term synchronization at least for this type of device. Moreover, since a harmonized air interface design for the two types of devices is pursued, it seems unnecessary to introduce the periodically signal for synchronization.

Observation 1: At least for type ⅰ device in Ambient IoT, long-term synchronization with gNB (in topology 1) or intermediate UE (in topology 2) is difficult to be achieved due to monitoring periodic synchronization signal has a very high possibility to break the power consumption limitation.

Proposal 1: Support a periodic synchronization signal (e.g., PSS/SSS/PBCH) is not pursued in Ambient IoT.

- Due to Ambient IoT is target for a harmonized air interface design for the two types of devices and at least the type ⅰ device cannot support the monitoring of this kind of synchronization signal.

In RFID, which is more like an asynchronous system, the transmission/reception timing alignment b/w reader and tags can only be achieved by transmitting and monitoring the preamble/frame-sync in both UL and DL, during the transmission and reception procedure, which can be regarded as a kind of short-term synchronization.

Since long-term synchronization may not be achievable for type ⅰ device in Ambient IoT as mentioned, the approach similar to RFID can be considered for transmission/reception timing alignment b/w gNB (in topology 1) or intermediate UE (in topology 2), and device for Ambient IoT DL/UL transmission/reception. Therefore, RAN1 may need to study how to design the preamble /sync-frame for both DL and UL, which can refer to our companion contribution [2].

Proposal 2: RAN1 needs to study how to achieve timing alignment b/w gNB (in topology 1) or intermediate UE (in topology 2), and the devices, for Ambient IoT DL/UL transmission/reception procedure.

- Introducing UL and DL preamble/frame-sync like in RFID can be considered as a starting point for a harmonized air interface design.

Random access

As aforementioned, Ambient IoT is more like an asynchronous system, which means some definitions in NR related to random access procedure cannot be reuse, such as Timing Advance; and, Ambient IoT device may not have the ability to generate a sequence from a certain number of candidates (such as 64 in NR), so it seems impossible to introduce the RACH preamble in Ambient IoT.

Observation 2: Some concepts in NR related to RACH procedure is unnecessary to be introduced for Ambient IoT, e.g., timing advance, preamble, etc.

Then, a RFID-like asynchronous random access procedure can be considered as a starting point, in which one round of random access procedure is composed by a set occasions and used for a population of devices to establish connection with the gNB or intermediate UE. It is started by a DL command like Query in RFID, and can be divided into 2Q occasions. For each occasion, the starting boundary is marked by another DL command like QueryRep and until the next time reception of this command.

Proposal 3: In Ambient IoT, one round of random access is composed by a set of occasions and used for a population of devices to establish connection with the gNB or intermediate UE

- Each round of random access is started by a DL command like Query in RFID which can be divided into 2Q occasions;

- For each random access occasion, the starting boundary is marked by a DL command like QueryRep in RFID and until the next time reception of this command.

For each Ambient IoT device, it can randomly select an occasion to perform its access based on the indicated Q value from gNB or intermediate UE, which can reduce the collision possibility among the multiple accessing devices in one round of random access. Besides, there may need some enhancements in Ambient IoT to guarantee more devices can successfully in one round of random access procedure compared to RFID because there may be more devices within gNB’s coverage in topology 1,than the number of tags in which are within the reader’s coverage RFID.

Figure 1 Illustration on the collision avoidance mechanism in RFID

Proposal 4: In one round of random access, a device can randomly select an occasion to perform its access based on the indicated Q value.

- FFS: whether some enhancement is needed to improve the accessing efficiency without increasing the collision possibility.

During the occasion which has been selected by a specific device, the device should perform the transmission of a randomly generated binary stream first, which can be regarded as a temporary identification of the device (like RN16). Then, after receiving an “ACK” for contention resolution which including the same binary stream from gNB or intermediate UE, the device can be regarded as successfully accessed. Finally, in this occasion, a device ID (e.g., EPC code) will be transmitted from the device, and there may be some subsequent DL/UL data transmission to/from specific device(s), for the potential further operations, e.g., Read, Write, etc.

Figure 2 Illustration of random access and the following transmission procedure (in one occasion) in Ambient IoT

Proposal 5: In one occasion of random access, the following procedures can be considered as starting point:

- A random binary stream is generated and transmitted as temporary identification of a device (similar to RN16);

- An “ACK” signalling for contention resolution is transmitted by gNB or intermediate UE;

- After successful access, Ambient IoT device(s) can transmit the Device ID (e.g., EPC code) to gNB or intermediate UE, and optional subsequent DL/UL data transmission/reception can be performed b/w the device and gNB or intermediate UE.

Frame structure

Frequency domain structure

In NB-IoT, one or more PRBs can be used for transmission, this kind of alignment with NR/LTE system can obtain better performance and improve the resource efficiency, especially for in-band deployment scenario. Further, with the implementation of a narrow band for transmission, the device complexity and peak power consumption can be limited accordingly. Another reason is that the peak data rate in Ambient IoT is not very high (maximum not less than 5 kbps, and minimum not less than 0.1 kbps). Thus, based on above analysis, for DL, one or more PRBs can be used for Ambient IoT transmission.

Figure 3 Frequency resource for Ambient IoT DL transmission

Proposal 6: For DL frequency domain structure, one or more PRBs can be used for Ambient IoT DL transmission.

For Ambient IoT UL transmission, since it is a kind of backscattering transmission based on the receiving CW of a device, and only simple and limited number of hardware components can be arranged at least in a type ⅰ device, i.e., amplifier, oscillator, and FFT module may not be arranged at least for type ⅰ device. So, from our perspective, at least single carrier transmission can be supported for Ambient IoT UL transmission, i.e., a single channel bandwidth is occupied by a device for UL transmission. To align with the NR resource grid, especially for the in-band scenario, the single carrier bandwidth size can be an integer multiple numbers of subcarrier size in NR.

Proposal 7: For UL frequency domain structure, the resource for transmission can at least be a sing carrier bandwidth, the size of the bandwidth can be an integer multiple numbers of subcarrier size of NR, e.g., 15kHz, 30kHz, 150kHz, 300KHz, etc.

Time domain structure

Similar to LP-Wus, to achieve better co-existence performance and reuse the transmitter of NR system as much as possible, for DL Ambient IoT symbol (e.g., OOK symbol), the boundary can be aligned with NR OFDM symbol; but how many symbols can map to one OFDM symbol may need to be FFS, the progress in LP-Wus WI can be a reference.

Figure 4 Aligned Ambient IoT symbol(s) within one NR OFDM symbol

Proposal 8: For DL time domain structure, M DL Ambient IoT symbols (e.g., OOK symbols) can be mapped to one OFDM symbol, and the symbol boundary can be aligned accordingly.

- FFS the value of M, the progress in LP-Wus WI can be a reference.

As aforementioned, Ambient IoT device is difficult to maintain a long-term synchronization, then the UL transmission is more-like an asynchronization transmission and cannot align with the boundary of NR symbol/slot. Therefore, the mechanism in RFID can be regarded as a starting point, in which the timing parameters are counted by millisecond or the integer multiple of symbol length (e.g., TRcal with potential divided ratio).

Proposal 9: For UL, asynchronized transmission is starting point, i.e., the transmission starting offset and duration can only be calculated by millisecond or microsecond, or the number of UL symbol length.

Scheduling

For scheduling, at first, time gap b/w DL/UL transmission, UL/DL transmission, DL/DL transmission can be considered which may refer to RFID design, where the exact values may depend on the processing time of the gNB or intermediate UE, and the Ambient IoT device. Besides, since the clock timing may be drifted during the transmission, some margin may need to be considered when defining the link timing parameters.

Figure 5 Illustration of link timing parameters in RFID [3]

Proposal 10: Link timing parameters, i.e., time gap b/w adjacent DL/UL transmission, UL/DL transmission, DL/DL transmission can be defined for Ambient IoT, with the consideration of device’s processing time, potential margin, etc.

For DL, only DL command like signal in RFID may need to be transmitted. Further, there seems to be unnecessary to introduce PDCCH-like signal as mentioned in our companion tdoc [2], so we think whether DL scheduling is necessary in Ambient IoT can be further studied.

Proposal 11: Whether DL scheduling is needed in Ambient IoT needs to be further studied in RAN1.

For UL, how to define the scheduling granularity can refer to NB-IoT, where one or more RUs can be allocated for scheduling UL data. In Ambient IoT, the frequency domain granularity for a Resource Unit can be a single carrier bandwidth; the time domain granularity for a Resource Unit can be defined for each UL bandwidth size, separately, similar to NB-IoT.

Figure 6 Illustration of RU granularity for Ambient IoT UL

Proposal 12: Resource Unit (RU) is used as granularity for scheduling the UL data from Ambient IoT devices, one RU can be defined as a UL single carrier bandwidth in frequency domain, and has the corresponding length for each bandwidth size in time domain.

To increase the coverage performance, repetition mechanism is supported in NR/NB-IoT, where for different coverage level defined in NB-IoT, different number of repetitions can be indicated by the corresponding fields in the DCI. Similarly, since coverage performance is also a key KPI for designing Ambient IoT, Bit/TB level repetition can also be considered to improve the coverage performance, the repetition number indicator can be included in the corresponding DL command. Further, enhancements for determining the repetition number can be further considered in RAN1 to obtain better scheduling performance.

Proposal 13: Bit/TB level repetition can be considered to improve the coverage performance.

- FFS: Further enhancements on the repetition number determination mechanism.

For UL scheduling, compared with RFID, more devices would access the NW from one reader (gNB or intermediate UE) due to the coverage may be larger, so it would be benefit to consider both one-to-one and one to multiple scheduling, especially for the UL data transmission other than the device ID, which has larger payload size, so we propose:

Proposal 14: To provide more efficiency, both one-to-one and one to multiple scheduling can be considered.

Conclusion

In this contribution, initial views on frame structure and timing aspects for Ambient IoT were provided. Based on the discussion, the following observations and proposals are proposed:

Observation 1: At least for type ⅰ device in Ambient IoT, long-term synchronization with gNB (in topology 1) or intermediate UE (in topology 2) is difficult to be achieved due to monitoring periodic synchronization signal has a very high possibility to break the power consumption limitation.

Observation 2: Some definitions in NR related to RACH procedure may not be needed in Ambient IoT, e.g., timing advance, preamble, etc.

Proposal 1: Support a periodic synchronization signal (e.g., PSS/SSS/PBCH) is not pursued in Ambient IoT.

- Due to Ambient IoT is target for a harmonized air interface design for the two types of devices and at least the type ⅰ device cannot support the monitoring of this kind of synchronization signal.

Proposal 2: RAN1 needs to study how to achieve timing alignment b/w gNB (in topology 1) or intermediate UE (in topology 2), and the devices, for Ambient IoT DL/UL transmission/reception procedure.

- Introducing UL and DL preamble/frame-sync like in RFID can be considered as a starting point for a harmonized air interface design.

Proposal 3: In Ambient IoT, one round of random access is composed by a set of occasions and used for a population of devices to establish connection with the gNB or intermediate UE

- Each round of random access is started by a DL command like Query in RFID which can be divided into 2Q occasions;

- For each random access occasion, the starting boundary is marked by a DL command like QueryRep in RFID and until the next time reception of this command.

Proposal 4: In one round of random access, a device can randomly select an occasion to perform its access based on the indicated Q value.

- FFS: whether some enhancement is needed to improve the accessing efficiency without increasing the collision possibility.

Proposal 5: In one occasion of random access, the following procedures can be considered as a starting point:

- A random binary stream is generated and transmitted as temporary identification of a device (similar to RN16);

- An “ACK” signalling for contention resolution is transmitted by gNB or intermediate UE;

- After successful access, Ambient IoT device(s) can transmit the Device ID (e.g., EPC code) to gNB or intermediate UE, and optional subsequent DL/UL data transmission/reception can be performed b/w the device and gNB or intermediate UE.

Proposal 6: For DL frequency domain structure, one or more PRBs can be used for Ambient IoT DL transmission.

Proposal 7: For UL frequency domain structure, the resource for transmission can at least be a sing carrier bandwidth, the size of the bandwidth can be an integer multiple numbers of subcarrier size of NR, e.g., 15kHz, 30kHz, 150kHz, 300KHz, etc.

Proposal 8: For DL time domain structure, M DL Ambient IoT symbols (e.g., OOK symbols) can be mapped to one OFDM symbol, and the symbol boundary can be aligned accordingly.

- FFS the value of M, the progress in LP-Wus WI can be a reference.

Proposal 9: For UL, asynchronized transmission is starting point, i.e., the transmission starting offset and duration can only be calculated by millisecond or microsecond, or the number of UL symbol length.

Proposal 10: Link timing parameters, i.e., time gap b/w adjacent DL/UL transmission, UL/DL transmission, DL/DL transmission can be defined for Ambient IoT, with the consideration of device’s processing time, potential margin, etc.

Proposal 11: Whether DL scheduling is needed in Ambient IoT needs to be further studied in RAN1.

Proposal 12: Resource Unit (RU) is used as granularity for scheduling the UL data from Ambient IoT devices, one RU can be defined as a UL single carrier bandwidth in frequency domain, and has the corresponding length for each bandwidth size in time domain.

Proposal 13: Bit/TB level repetition can be considered to improve the coverage performance.

- FFS: Further enhancements on the repetition number determination.

Proposal 14: To provide more efficiency, both one-to-one and one to multiple scheduling can be considered.

# Reference

[1] RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things), RANP#102, August, 2023.

[2] R1-2400564, Discussion on Downlink and uplink channel/signal aspects for Ambient IoT, xiaomi, RAN1#116, February 26th – March 1st, 2024

[3] EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard, Specification for RFID Air Interface Protocol for Communications at 860 MHz – 960 MHz