3GPP TSG RAN WG1 #116                                          	          R1-2400566

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.5.1

Source:	Xiaomi

Title:                     Discussion on On-demand SSB SCell operation

Document for:	Discussion and Decision

Introduction

In RAN#102 meeting, the new WID of Rel-19 Network Energy Saving was approved [1]. The following objectives are included:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we discuss necessary procedures and signalling to support on-demand SSB, including the possible methods to trigger SSB. Considering SSB is essential in NR system, we also provide our analyses for the impacts on UE behaviours if on-demand SSB is applied.

Discussion

Mechanisms of SSB triggering

SSB is transmitted in a periodic and cell-specific manner, which is the props of beam operation and RRM measurement. Accordingly, always-on SSB bring mandatory power consumption and make it difficult for gNB to turn into sleep mode if not impossible. Basically, the power consumption caused by SSB transmission along with SSB transmission at least comes from the following two aspects:

- The SSB overhead in terms of occupied OFDM symbols within a SSB transmission periodicity is not small. On these symbols, gNB has to transmit SSB and has to meet certain requirement.

- gNB has to keep awake in order to transmit SSB on the predefined or configured SSB occasion. It makes pretty difficult that gNB turns into sleep mode even if the traffic load is quite low.

As shown in Figure 1, SSB overhead in time domain within 20 ms periodicity is 5.7%-11.42% depending on deployment. On-demand SSB technique can be used to shut down SSB when necessary, e.g., traffic load is pretty low, or the number of UE camping on SCell is quite few. Accordingly, the power consumption caused by SSB transmission can be significantly reduced. On the other hand, on-demand SSB technique is capable to resume SSB transmission once conditions changed.

Figure 1 SSB patterns defined in current specification

As captured in the WID, there are three potential triggering methods for on-demand SSB:

- Method 1: UE uplink wake-up-signal

- Method 2: Cell on/off indication via backhaul

- Method 3: Scell activation/deactivation signalling

For method 1, the SSB will be turned on in light of UE’s demand. For method 2 and method 3, SSB on/off totally depends on gNB implementation, i.e., gNB can determine SSB on or SSB off alone without considering UE’s requirement.

Observation 1: Different triggering method for on-demand SSB has diverse impacts on UE:

- WUS based SSB triggering fully take the requirement at UE side into consideration

- Cell on/off indication based SSB triggering is transparent to UE

- SCell activation/deactivation based SSB triggering is fully gNB implementation while non-transparent to UE

UE uplink wake-up-signal based SSB triggering

Theoretically, gNB can determine whether to transmit SSB or not totally depending on its requirement or decision. However, network-only mechanism cannot perfectly fit the requirement or conditions at UE side. For example, UE may need to request gNB to send SSB in order to acquire more accurate measurement and prepare for DL reception/UL transmission. Given that, it is reasonable to allow UE to send wake-up-signal for requesting SSB so as a better trade-off between energy saving and QoS can be achieved.

As shown in Figure 2, WUS sent by NES UE can be used to request SSB on WUS resource if necessary.

Figure 2 Procedures for UE WUS transmission

The wake-up-signal can be just a “indicator” to inform NW that SSB is required. gNB may determine whether to resume SSB or not once it receives WUS from UE side. Furthermore, the wake-up-signal can carry more information, e.g., the expected periodicity, the expected SSB index, etc. As an example, Figure 3 shows a case wherein WUS is not only used to request SSB but also indicate the expected SSB index. To be specific, gNB receives two WUS from UE1 and UE2. SSB1 and SSB2 are required by UE1 and UE2 respectively. gNB may decide to only transmit SSB1 and SSB2 within a SSB burst according to the received WUSs. Hence, additional NES gain can be harvested via avoiding to transmit redundant SSB.

Figure 3 Transmitted SSB in limited beams for DL sync.

All in all, we need first to achieve a common understanding on the functionality and content of WUS. We have the following proposal:

Proposal 1: The wake-up-signal is at least used to request the resume of SSB transmission.

- FFS: whether wake-up-signal can be used to carry other relevant information of SSB.

Regarding the physical structure of wake-up-signal, we prefer to reuse current channel or signal as much as possible. The following uplink channel or signal can be considered as starting point:

- Preamble, e.g., a set of preambles can be used to indicate gNB that UE needs to receive SSB. The set of preambles can be specific for SSB requesting. The related RO can be further studied.

- PUCCH, e.g., PUCCH format#0 can be a starting point. A ZC sequence with predefined cyclic shift could be used for SSB requesting.

- PUSCH, e.g., a predefined CG PUSCH can be used as a starting point.

Proposal 2: For UE wake-up-signal, reusing existing uplink channel or uplink signal as much as possible. At least the following channel/signal can be considered as starting point:

- Option 1: WUS is carried by PRACH

- Option 2: WUS is carried by PUCCH

- Option 3: WUS is carried by CG PUSCH

The wake-up-signal requesting SSB can be sent to SCell or PCell, as shown in Figure 4.

Figure 4 UE wake-up-signal to SCell or PCell

It is more straightforward to send WUS to target on-demand SSB SCell.  For an RRC_CONNECTED mode UE, it can obtain WUS configuration on the SCell via RRC signalling. However, if the SCell is deactivated, it may be problematic for the UE to send WUS as the all of existing uplink channel/signals are not allowed for the deactivated SCell. Therefore, if WUS transmission on SCell is pursued, at least we need to look into the restrictions on uplink transmission when the SCell is deactivated. Furthermore, a basic assumption on on-demand SSB is that it is applied to a SCell which is non-co-located with PCell. If SSB is turned off for a while, it is high possible that UE cannot sustain DL synchronization with SCell and lost the DL timing. It may make it impossible for UE to determine the UL timing of WUS.

On the other hand, SSB on PCell is always on and the UL synchronization is sustained at UE side. However, we need to further consider how to configure WUS resource for PCell and the relationship between WUS and target SCell. Furthermore, if UE sends WUS to PCell and request SSB transmission on SCell, extra information exchange between PCell and SCell is needed.

Proposal 3: Further study how to send WUS requesting SSB on a SCell, i.e., UE sends WUS to PCell/PSCell or UE sends WUS to target SCell.

Cell on/off indication via backhaul based SSB triggering

The basic idea to realize on-demand SSB according cell on/off indication via backhaul is that PCell has full power to control SCell as they belong to same DU and share same MAC entity. If ideal backhaul is available between PCell and SCell, which is a typical case, PCell can obtain the conditions on SCell in time. From this point of view, it is possible that SCell can adjust its SSB transmission behaviour according to the cell on/off indication from PCell. To be specific, if PCell send a cell off indication to SCell via backhaul, the SCell should stops to transmit SSB. There is no signalling exchange between network and UE.

However, cell on/off indication via backhaul based SSB triggering is totally transparent to UE. If UE follows legacy behaviour without knowing that SSB is shut down, there will be negative impacts on legacy procedures. For example, UE has to sustain detection on time occasion without SSB transmission which leads to unnecessary power consumption. More importantly, UE may derive incorrect measurement results with the assumption that SSB is still transmitted. It may cause serious problems on RRM procedure and BFR procedure as the accuracy of measurement may be significantly degraded.

In addition, as shown in Figure 5, UE may always need to detect several consecutive SSB bursts before it synchronizes to the SCell resuming SSB transmission. Without any UE indication, NW has no idea on the DL synchronization status at UE side. Hence, gNB has to schedule UE with a conservative method in order to avoid degradation of data transmission due to out of synch issue. Hence large transmission delay can be perceived.

Therefore, we prefer not support cell on/off indication based SSB triggering method.

Figure 5 Delay in data transmission

Proposal 4: Cell on/off indication based SSB triggering method should be deprioritized as there are many negative impacts on legacy procedures if UE follows legacy behaviours without knowing that SSB is shut down.

Scell activation/deactivation signalling based SSB triggering

gNB activates an inactive SCell via MAC CE according the timeline defined in TS38.213. [1] Basically, UE needs to perform AGC settling and T/F tracking during SCell activation procedure. Depending on different conditions, e.g., frequency range and/or SCell status, the time needed for T/F tracking can be diverse. Furthermore, CSI-RS in addition to SSB may be needed for beam operation in FR2. In order to expedite SCell activation procedure, fast SCell activation procedure has been realized by introducing temporary RS as shown in Figure 6. To be specific, fast SCell activation can only be applied to known SCell. If the deactivated SCell is unknown to UE, no much can be done and the procedure define in TS38.133 is the only way to go. [3]

All in all, no matter SCell is activated or deactivated, SSB is needed as UE need to perform measurement and hence to sustain basic functionality.

Figure 6 TRS based fast scell activation

Regarding to Scell activation/deactivation signalling based SSB triggering, we firstly need to clarify the common understanding among companies.  There may be different understandings according to aforementioned analyses:

- Understanding#1: On-demand SSB triggered by Scell activation signalling is only needed during SCell activation procedure. During SCell activation, additional SSB can be triggered automatically to speed up SCell activation. It is particularly helpful for the case wherein SCell is unknown. After SCell is activated, on-demand SSB is not needed as gNB has full power to control the SSB transmission.

Figure 7 Explanation for Understanding#1

- Understanding#2: SCell activation/deactivation procedure is reused as a mechanism to trigger SSB on/off. To be specific, gNB reuse SCell activation/deactivation mechanism as much as possible to indicate UE SSB is on or off.

Figure 8 Explanation for Understanding#2

- Understaanding#3: On-demand SSB can be used to expedite SCell activation procedure. After SCell is activated, on-demand SSB is still possible via WUS transmitted by UE.

Porposal 5: For scell activation/deactivation signaling based SSB triggering, we need to first achieve common understanding on the relationship between on-demand SSB and Scell activation/deactivation signaling

- Case1: Scell activation signalling based SSB triggering is only needed during SCell activation procedure. After SCell is activated, gNB has full power to control the SSB transmission

- Case2: SCell activation/deactivation signalling is reused as a mechanism to indicate UE SSB is on or off

- Case3: On-demand SSB can be used to expedite SCell activation procedure. After SCell is activated, UE wake-up-signal can be used to trigger SSB

Impacts on UE behaviours

Whether UE needs to recognize SSB transmission status

No matter which direction is adopted for SSB triggering, UE may not be able to recognize SSB transmission status. The possible issues for each mechanism are briefly analysed as below:

- For WUS based SSB triggering, UE may assume SSB will be transmitted after WUS transmission. However, gNB may or may not transmit SSB immediately even if it successfully receives WUS.  On the other hand, it is very difficult for UE to recognize the transition of SSB transmission in time.

- For cell on/off indication based SSB triggering, SSB on/off transition is totally transparent to UE.

- For SCell activation/deactivation based SSB triggering, UE can only know SSB on/off during SCell activation procedure or at most know when SSB is turned on. It still has no idea on when SSB is turned off.

Observation 2: No matter which mechanism is adopted for SSB triggering, UE may not be able to recognize SSB transmission status.

It is well known that SSB is one of the most essential DL channels/signals in NR system. It is widely used for many basic UE procedures, including RRM, RLM, BFR, etc. If UE cannot recognize SSB transmission status, there are at least two issues need to be further considered:

- If UE doesn’t know SSB has been turned off, it tries to receive SSB as normal. Unnecessary power consumption is inevitable.  More importantly, it makes more difficult for UE to convert into sleep mode, which leads to more power wastage. This issue is more serious when SSB periodicity is small.

- If UE doesn’t know SSB has been turned off, failure on reception of turn-off SSB is wrongly considered as degradation of channel condition. Although it is typically that the measurement results for each SSB sample are filtered before reporting, the accuracy of final measurement report is definitely deteriorated.

Power saving is very important for improving UE performance. The accuracy of measurement for reporting is essential for UE. Therefore, it is very important for UE to recognize the status of SSB transmission.

Take SCell management as an example. The measurement resource for RRM could be SSB transmitted by serving cell or neighbour cell. Based on the configuration, UE performs measurement and report the measurement results to NW. The measurement results can be used for SCell management.

As shown in Figure 9, the SCell can be added or released for a UE based on the RRM measurement results based on 38.331. [4] For example, if the RRM measurement result of SCell is worse than a configured threshold, PCell should release the SCell because it is no longer qualified for data transmission.

Figure 9 scell addition and release

On a SCell supporting on-demand SSB, as shown in Figure 10, if UE still performs RRM measurement without considering SSB on/off, the measurement results of the SCell is possibly worse than SCell release threshold. Consequently, PCell would release the SCell according to the report. However, the degradation of measurement result comes from SSB off rather than from channel condition deterioration. In the other words, a SCell supporting on-demand SSB is mistakenly removed from serving cell list. It will increase transmission delay and bring negative impacts on system performance.

Figure 10 RRM measurement based on on-demand SSB

Regarding the mechanism for UE obtain SSB transmission status, either explicit indication from gNB side or implicit determination from UE side can be further studied.

Proposal 6:  UE needs to recognize the transmission status of SSB in order to avoid wasting power and guarantee accurate measurement result.

- FFS: detail mechanisms for UE to identify the transmission status of SSB

Conclusion

In this contribution, we discussed the procedures and signalings to support on-demand SSB and the impacts of on-demand SSB on UE behaviours were analysed. The proposes and observations are summarized as following:

Observation 1: Different triggering method for on-demand SSB has different diverse impacts on UE:

- WUS based SSB triggering fully take the requirement at UE side into consideration

- Cell on/off indication based SSB triggering is transparent to UE

- SCell activation/deactivation based SSB triggering is fully gNB implementation while non-transparent to UE

Observation 2: No matter which mechanism is adopted for SSB triggering, UE is not able to recognize SSB transmission status.

Proposal 1: The wake-up-signal is at least used to request the resume of SSB transmission.

- FFS: whether wake-up-signal can be used to carry other relevant information of SSB.

Proposal 2: For UE wake-up-signal, reusing existing uplink channel or uplink signal as much as possible. At least the following channel/signal can be considered as starting point:

- Option 1: WUS is carried by PRACH

- Option 2: WUS is carried by PUCCH

- Option 3: WUS is carried by CG PUSCH

Proposal 3: Further study how to send WUS requesting SSB on a SCell, i.e. UE send WUS to PCell/PSCell or UE send WUS to target SCell.

Proposal 4: For cell on/off indication based SSB triggering method, there are many negative impacts on legacy procedures if UE follows legacy behaviours without knowing that SSB is shut down

Porposal 5: For scell activation/deactivation signaling based SSB triggering, we need to first achieve common understanding on the relationship between on-demand SSB and Scell activation/deactivation signaling

- Case1: Scell activation signalling based SSB triggering is only needed during SCell activation procedure. After SCell is activated, gNB has full power to control the SSB transmission

- Case2: SCell activation/deactivation signalling is reused as a mechanism to indicate UE SSB is on or off

- Case3: On-demand SSB can be used to expedite SCell activation procedure. After SCell is activated, UE wake-up-signal can be used to trigger SSB

Proposal 6:  UE needs to recognize the transmission status of SSB in order to avoid wasting power and guarantee accurate measurement result.

- FFS: detail mechanisms for UE to identify the transmission status of SSB

References

- RP-234065, “New WID: Enhancements of network energy savings for NR”, TSG RAN Meeting #102, Edinburgh, Scotland, December 11th-15th, 2023.

- TS 38.213, “NR; Physical layer procedures for control”, V18.1.0, January, 2024.

- TS 38.133, “NR; Requirements for support of radio resource management”, V18.4.0, January, 2024.

- TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”, V18.0.0, January, 2024.