3GPP TSG RAN WG1 #116                                                           R1-2400567

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	9.5.2

Source:	Xiaomi

Title:	Discussion on on-demand SIB1 for Idle/Inactive mode UEs

Document for:	Discussion

Introduction

In RAN#102 meeting, the new WID of Rel-19 Network Energy Saving was approved.[1] On-demand SIB1 for UEs in idle/inactive mode is captured by the following objective:

2. Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105.

In this contribution, we provide our analyses on on-demand SIB1 for Idle/Inactive UE.

Discussion

Motivation for on-demand SIB1

As for Rel-19 NES WI, a checkpoint is specified for on-demand SIB1. Hence, the motivation for on-demand SIB1 needs to be carefully studied. SIB1 occupies non-negligible time and frequency resources. Assuming 20ms periodicity, the SIB1 overhead in time domain is 2.85%~34.28% depending on deployment and configuration, as shown in Table 1. L is the number of OFDM symbols occupied by SIB1.

Table 1 Time domain resource occupancy radio of SIB1

| Case | SIB1 with L=4 | SIB1 with L=12 |
| --- | --- | --- |
| Case A (15kHz, <=3GHz) | 5.71% | 17.14% |
| Case A (15kHz, >3GHz) | 11.42% | 34.28% |
| Case B (30kHz, <3GHz) | 2.85% | 8.57% |
| Case B (30kHz, >3GHz) | 5.71% | 17.14% |
| Case C (30kHz, FR1) | 2.85% | 8.57% |
| Case D (120kHz, FR2) | 11.42% | 34.28% |
| Case E (240kHz, FR2) | 5.71% | 17.14% |

Observation 1:  The overhead of SIB1 is 2.85%~34.28% in terms of OFDM symbols within SIB1 periodicity.

During Rel-18 NES study item, it is a common understanding that NES techniques should be applicable to diverse scenarios so that operators have chance to reduce OPEX for any potential deployment, including Urban Micro scenario, Macro scenario, InH scenario, etc. [2]

At least for Urban Micro scenario and InH scenario, the number of UE camping on a cell may be changing significantly with time going by.  Furthermore, network may decide to shut down part of cells once the overall traffic load is not that high. To be specific, gNB can shut down SIB1 transmission on a cell while sustains basic operation on the neighboring cell. Currently, gNB has to periodically transmit SIB1 even if there is no UE camping on the cell. It is obvious that permanent SIB1 transmission will waste energy as there may be unnecessary to keep transmitting SIB1 in some cases.  On-demand SIB1 can be used to resolve the issues on energy waste caused by unnecessary SIB1. In Figure 1, one exemplary procedure of on-demand SIB1 is shown as below:

- Step 1: gNB stops transmitting SIB1 when it recognizes that there is no UE on current cell or the traffic load is pretty low.

- Step 2: UE1 decides to camp on the target cell and sends WUS to trigger SIB1 transmission;

- Step 3: gNB receives the WUS request and resumes the SIB1 transmission;

- Step 4: UE1 moves to another cell or traffic load becomes very low, the gNB stops SIB1 transmission again.

Figure 1  Scenario for on-demand SIB1 scheme

Observation 2:  On-demand SIB1 at least can be used to avoid energy waste for the following cases:

- There is no UE within the coverage of a serving cell.

- The overall traffic load is quite low and there is no need to provide service via the target cell.

The NES performance of on-demand SIB1 has been evaluated during Rel-18 NES SI. [3] The simulation results show that with on-demand SIB1 at empty load with baseline of 20ms SSB/SIB1 periodicity, 5.8%~8.6% BS energy savings can be achieved at SIB1 transmission rate of 20%~5% for one SSB beam, and the gains can increase to 32.1%~38.8% for 8 beams case for a same SIB1 transmission rate range.

Observation 3:  BS energy savings gain brought by on-demand SIB1 can be up to 38.8% for 8 beams case.

Based on the aforementioned observation, we can conclude that for the scenario where the number of UEs fluctuates over time, significant NES gain can be achieved by reducing the SIB1 transmission in time domain.

Proposal 1:  On-demand SIB1 for idle/inactive mode UE should be supported for Rel-19 NES.

Procedure for On-demand SIB1

In this section, we focus on the procedure of on-demand SIB1. From our understanding, the basic procedure at UE side at least includes:

- Obtain the WUS configuration.

- Send WUS to request SIB1 transmission on target cell.

- 2.2.1 WUS configuration

Basically, WUS configuration is a prerequisite for UE to send WUS. There are two potential directions to obtain WUS configuration for a NES UE, which are summarized as below:

- Case#1: UE obtains WUS configuration from a neighboring cell.

- Case#2: UE obtains WUS configuration from the on-demand SIB1 cell.

WUS configuration from a neighboring cell:

For case#1, the WUS configuration can be carried by SI, e.g., SIBn. SIB1 is a first essential SI that UE needs to obtain after MIB. The configuration of sequential SI is also carried by SIB1. Hence, SIB1 is a prerequisite for SI acquisition. From this aspect, neighboring cell with normal SIB1 transmission is a promising choice for WUS configuration.

In theory, either SIB1 or SIBn can be used to carry WUS configuration. Considering that SIB1 is a prerequisite for SIBn, UE can obtain the WUS configuration faster if WUS configuration is carried by SIB1. However, it will increase the overall payload of SIB1 which may exceed the maximum packet size. Another choice is to carry WUS configuration with SIBn. The following two options can be further studied:

- Option 1: The WUS configuration is carried by one of existing SIBn IE, e.g., SIB2, SIB3, SIB4;

- Option 2: The WUS configuration is carried by a newly defined SIBn, e.g., SIB 22;

Proposal 2:  If WUS configuration is obtained by neighboring cell, it can be carried by either SIB1 or SIBn.

If UE obtains the WUS configuration from neighboring cell, the procedure to determine a camping cell needs further discussion. Currently, the UE camps on a cell if cell selection/reselection criterion is fulfilled. From our understanding, current cell selection/reselection procedure still works for UE to choose a proper cell to camp on in on-demand SIB1 scenario. Taking the scenario as shown in  Figure 2 as an example, the procedure for UE camping on a cell is summarized as below:

- Step#1:  Obtain the SSB of neighboring cell by cell search.

- Step#2: Obtain SIB1 and camp on the neighboring cell.

- Step#3: Obtain the WUS configuration by SIBn.

- Step#4: When cell reselection criterion is fulfilled, it searches SSB on another cell. The WUS configuration takes effect only when UE camps on the target NES cell.

Figure 2 An example scenario of choosing a proper cell to camp on by cell reselection

Proposal 3:  UE can camp on the on-demand SIB1 cell following current cell selection/reselection procedure.

WUS configuration from on-demand SIB1 cell:

For case#2, several parallel methods can be considered for WUS configuration acquisition. To be specific, if SIB1 is transmitted, the UE can obtain WUS configuration by SI. For instance, if on-demand SIB1 cell receives WUS requesting SIB1 from UE1, it may resume SIB1 transmission. During this period, UE2 can receive SIB1 on this cell as usual and obtain WUS configuration.

In the contrary, if SIB1 is not available, the UE needs to obtain WUS configuration before SIB1 transmission, which may lead to non-trivial specification efforts. Hence, the necessity for the second way needs to be carefully studied.

Observation 4:  If UE needs to obtain the WUS configuration from on-demand SIB1 cell, non-trivial specification efforts may be needed.

Unlike case#1 where UE needs to obtain WUS configuration from a neighboring cell, WUS configuration is obtained from on-demand SIB1 cell for case#2.

Taking the scenario as shown in Figure 3 as an example, the on-demand SIB1 procedure is summarized as below:

- Step#1: Obtain the SSB of serving cell by cell search.

- Step#2: Obtain the WUS configuration.

- Step#3: Send WUS and attempt to obtain SIB1.

Figure 3 An example scenario of SIB1 acquisition without the assistance of neighboring cell

For UE camping on a on-demand SIB1 cell, WUS configuration is a prerequisite for SIB1 triggering. To expedite the procedure of obtaining WUS configuration during cell search, following mechanisms can be considered:

- One cell carries WUS configuration of multiple cells.

- Identical WUS configuration is broadcasted by multiple cells.

No matter which option is adopted in the end, UE can obtain WUS configuration for on-demand SIB1 cells once it successfully receives a SIB1/SIBn on any neighbouring cell.

Proposal 4:  The following mechanisms can be considered for how to carry WUS configuration:

- One cell carries WUS configuration of multiple cells.

- Identical WUS configuration is broadcasted by multiple cells.

- 2.2.2 Design on WUS signal

Per objective captured in WID, only existing signal/channel can be used as WUS signal. Accordingly, ‘the existing signal/channel’ needs to be clarified. One direction is only the existing signal/channel available for idle/inactive mode UEs can be reused as WUS, i.e., only Msg1, Msg3 or MsgA are considered. On the other hand, any kind of existing uplink channel/signals can be considered, e.g., PUCCH and PUSCH which are only available for connected mode UEs.

The intention for reusing existing signal/channel is to avoid introducing uncontrollable specification efforts. From this point of view, there is no much difference on which existing channel/signal is used as WUS. Even if Msg1 is adopted for WUS triggering, it is a kind of new ‘signaling’ for Idle/Inactive UE as the functionality is totally different.  Therefore, all the existing uplink signals/channels should be considered as the potential options for WUS.

Based aforementioned analyses, at least the following options can be considered:

- Option 1: Reuse preamble as WUS to request SIB1.

- Option 2: Reuse PUSCH as WUS to request SIB1.

- Option 3: Reuse preamble and PUSCH as WUS to request SIB1.

Option 1 follows current framework of on-demand SI. If the UE wants to request beam-based SIBn transmission, it can send the SI request by the preamble if SI-RequestConfig is configured. gNB can differentiate the usages of preamble, i.e., initial access or on-demand SI request, via separate RO/preamble. Similarly, if preamble is reused for on-demand SIB1 request, a separate RO/preamble configuration may be needed. Actually, preamble is the best choice as UE may not achieve UL synchronization when it needs to request SIB1. However, it will consume additional RO/preamble resources, which may lead to resource segmentation and degrade PRACH capacity.

Figure 4 Example for WUS triggering by preamble

For option 2, UE can send WUS carried by pre-configured PUSCH to request SIB1 if necessary. Option 2 works well if the UL synchronization has been achieved. Otherwise, there may be some issues for gNB to receive the PUSCH.

Figure 5 Example for WUS triggering by PUSCH

Option 3 is a compromise between option 1 and option 2.  To be specific, the UE first sends preamble for UL synchronization and then sends WUS carried by PUSCH. Option 3 can alleviate the issue for PRACH capacity degradation as it doesn’t need extra preamble for SIB1 requesting purpose. The preamble is sent once UL synchronization cannot be sustained or has not been obtained. One issue is that UE has to perform UL synchronization first before sending WUS. Furthermore, option 3 may increase the overhead for WUS transmission as UE needs to transmit different signal/channel. Correspondingly, the signalling overhead increases with the separate configuration of Preamble and PUSCH at gNB side.

Figure 6 Example for WUS triggering by Preamble & PUSCH

Proposal 5: For WUS signal design, the following options can be considered:

- Reuse preamble as WUS requesting SIB1.

- Reuse PUSCH as WUS requesting SIB1.

- Reuse preamble and PUSCH as WUS requesting SIB1.

Others

Currently, if UE fails to obtain SIB1 on a serving cell, the cell is regarded as barred and is taken out from the candidate cell list of cell selection/reselection for 300s. The basic principle of above mechanism is to exclude the unqualified cell for data transmission. Following legacy procedure, NES UE may unexpectedly bar a NES cell if it doesn’t receive SIB1 during SIB1 off period. Consequently, the UE may have to search for another cell. In the end, it may be hard for a NES UE to select any cell if lots of NES cell determines to turn off SIB1. It will decrease the network communication performance.

To solve that problem, the UE needs to be informed that the serving cell is an on-demand SIB1 cell. If UE doesn’t not receive SIB1 on an on-demand SIB1 cell, this cell should not be simply removed from serving cell list and barred for 300s. To help UE to identify cell type, at least the following two options can be considered:

- Option 1: UE identifies the on-demand SIB1 cell by WUS configuration implicitly.

- Option 2: UE identifies the on-demand SIB1 cell by explicit signaling.

Figure 7  Example procedure of UE camping on a cell

Option 1 works based on the perquisite that the WUS configuration carries the information of cell ID. Once the UE obtains the configuration specific to one serving cell, the UE can determine it to be a on-demand SIB1 cell.

For Option 2, signalling mechanism needs further study. Hence, specification efforts may be non-trivial.

Proposal 6: To help UE to identify the on-demand cell, the following options can be considered:

- Option 1: UE identifies the on-demand SIB1 cell by WUS configuration implicitly.

- Option 2: UE identifies the on-demand SIB1 cell by explicit signaling.

Conclusion

In this contribution, we provide our views on on-demand SIB1. We have the following observations and proposals:

Observation 1:  The overhead of SIB1 is 2.85%~34.28% in terms of OFDM symbols within SIB1 periodicity.

Observation 2:  On-demand SIB1 at least can be used to avoid energy waste for the following cases:

- There is no UE within the coverage of a serving cell.

- The overall traffic load is quite low and there is no need to provide service via the target cell.

Observation 3:  BS energy savings gain brought by on-demand SIB1 can be up to 38.8% for 8 beams case.

Observation 4:  If UE needs to obtain the WUS configuration from on-demand SIB1 cell, non-trivial specification efforts may be needed.

Proposal 1:  On-demand SIB1 for idle/inactive mode UE should be supported for Rel-19 NES.

Proposal 2:  If WUS configuration is obtained by neighboring cell, it can be carried by either SIB1 or SIBn.

Proposal 3:  UE can camp on the on-demand SIB1 cell following current cell selection/reselection procedure.

Proposal 4:  The following mechanisms can be considered for how to carry WUS configuration:

- One cell carries WUS configuration of multiple cells.

- Identical WUS configuration is broadcasted by multiple cells.

Proposal 5: For WUS signal design, the following options can be considered:

- Reuse preamble as WUS requesting SIB1.

- Reuse PUSCH as WUS requesting SIB1.

- Reuse preamble and PUSCH as WUS requesting SIB1.

Proposal 6: To help UE to identify the on-demand cell, the following options can be considered:

- Option 1: UE identifies the on-demand SIB1 cell by WUS configuration implicitly.

- Option 2: UE identifies the on-demand SIB1 cell by explicit signaling.

References

- RP-234065, “New WID: Enhancements of network energy savings for NR”, TSG RAN Meeting #102, Edinburgh, Scotland, December 11th-15th, 2023.

- RP-220297, Huawei, “Revised SI: Study on network energy savings for NR”, TSG RAN Meeting #95e, Electronic Meeting, March 17-23, 2022.

- R1-2212128, Qualcomm Incorporated, “NW energy savings performance evaluation”, TSG RAN WG1 #111	, Toulouse, France, November 14th – 18th, 2022.