3GPP TSG RAN WG1 #116			     R1-2400570

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	9.6.2

Source:	             Xiaomi

Title:	Discussion on LP-WUS operation in IDLEINACTIVE modes

Document for:	Discussion

Introduction

In RAN #102 meeting, R19 LP WUS WI is approved, and the objectives are is as follows [1],

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - This objective is to be further refined in RAN#103

In the following, we provide some discussions on LP-WUS operation in IDLEINACTIVE modes.

Configuration of LP-WUS/ LP-SS

LP-WUS/LP-SS for RRC idle/inactive modes are cell common signals, and the configuration information can be carried in SIB for broadcasting. The configuration of LP-WUS/LP-SS includes at least frequency location, bandwidth, time domain resources and power offset.

Frequency location

In R18 SI, it was agreed that at least LP-WUS and signals/channels by MR can be on the same carrier in the band, and further study whether LP-WUS and signals/channels used by MR can be different carriers or different bands. How to configure the frequency location may up to gNB deployment flexibility. But besides network flexibility, UE LP WUR capability should also be considered. LP WUR has much lower cost and lower complexity compared to MR, and its hardware may not be able to support as many bands as MR. It is possible and even necessary to support the case when LP-WUS/LP-SS and signals/channels used by MR can be on different carriers or different bands. The detailed LP WUR supporting bands requirement is up to RAN4 to decide.

Proposal 1: Restricted by LP WUR capability, LP-WUS/LP-SS and signals/channels used by MR can be on different carriers or different bands.

Proposal 2: The detailed LP WUR supporting bands requirement is up to RAN4 to decide.

Bandwidth

Bandwidth of LP WUS was discussed in R18 SI and had an initial agreement that the BW of one LP-WUS is not greater than X (FFS X is 5 or 20) MHz for FR1. Intuitively, larger bandwidth would increase the LP WUS coverage and detection performance. The gain mainly comes from increased frequency diversity, which will be saturated after the bandwidth is sufficiently large. On the other hand, a larger bandwidth may increase resource overhead and also difficulties of coexistence with legacy channels. The coverage target is to achieve similar coverage performance as PUSCH Msg 3. From our simulation in [2], when the allocated bandwidth increased to 12RB with SCS 30kHz, the coverage target will be met. Thus bandwidth not greater than 5MHz is quite enough for MC-OOK modulation. To reduce the complexity，it is generally preferred that LP-SS uses the same  bandwidth as LP-WUS.

Proposal 3: Support BW of LP-WUS/LP-SS is not greater than 5 MHz for FR1.

Bandwidth of LP WUS can also be configurable, considering different network deployment requires different LP WUS coverage. For example, for small cell deployment, BW of LP WUS can be comparatively smaller since a small bandwidth can already satisfy the coverage and detection performance requirement, while as for Macro cell deployment, the required bandwidth for LP WUS would be larger. And based on similar reason, the BW of LP WUS for RRC idle/inactive and RRC connected can also be different and configured separately, because for LP WUS for RRC idle/inactive state should aim to guarantee whole cell coverage, or achieve coverage as large as possible, so the required BW should be larger, while as for RRC connected, LP WUS is devoted to certain UE or UE group, and the LP WUS BW can be decided by the location or channel state of the target UE or UE group.

Proposal 4: Support BW of LP-WUS is configurable, and support LP WUS BW being different for RRC idle/inactive and RRC connected states.

Time domain resources

Beam sweeping pattern should be considered for LP-SS and LP WUS. The periodicity of LP-SS/LP-WUS should be an integer multiple of the SSB periodicity and the sweeping pattern should be aligned with the SSB/CORESET 0 beam sweeping pattern, allowing for the transmission of LP-SS and SSB using the same beam.

Proposal 5: Support beam sweeping pattern of LP-WUS.

Power offset

Power boosting can be considered for coverage enhancement of LP-WUS/LP-SS, and the power offset Δoffset of LP-SS/LP-WUS compared to the power of SSB can be configured for better/sufficient link performance of LP-SS/LP-WUS detection. UE can also estimate the receiving performance of LP-WUS/LP-SS based on the power offset thus determine whether to activate LR.

Proposal 6: The power offset of LP-SS/LP-WUS compared to the power of SSB should be configured.

Activation/deactivation of LP-WUS/LP-SS monitoring

After a RRC idle/inactive UE acquires the configuration of LP-WUS/LP-SS, the UE should determine whether to activate or deactivate LP WUR for LP-WUS/LP-SS monitoring. The following three alternatives are discussed in R18 SI,

- Alt 1a:

  - gNB transmits legacy paging indication and LP-WUS

  - UE activation and/or deactivation of LP-WUS monitoring is up to UE implementation.

  - This behavior may apply based on channel condition, e.g. when coverage is sufficient/insufficient.

- Alt 1b:

  - gNB transmits legacy paging indication and LP-WUS

  - UE activation and/or deactivation of LP-WUS monitoring is based on preconfigured criteria

  - This behavior may apply based on channel condition, e.g. when coverage is sufficient/insufficient.

- Alt 2:

  - activation and/or deactivation of LP-WUS monitoring in a cell is based on signalling.

For Alt 2, whether UE will apply the LP WUS monitoring mechanism or not is based on gNB signalling, and some company propose it can be based on RRC release signalling. The gNB can have clear information about the UE’s channel quality at the moment when it is released, for example, the channel quality of the UE is not good and gNB decide to deactivate LP WUS monitoring for the UE. But after the RRC release, UE may wander and have good channel quality, but to obey the gNB’s indication, the UE still can not activate LP WUS monitoring, which will increase UE power consumption. So we do not think Alt 2 is a good solution for RRC idle/inactive state.

Alt 1a/1b is better for UE power saving since UE can decide to activate/deactivate LP WUS monitoring mechanism based on actual channel condition flexibly, and Alt 1b seems a better solution that UE behaviour is specified. Alt 1a/1b may need to be further elaborated since a UE may operate in multiple modes, e.g., RRM full offloading to LR, RRM relaxation for MR assisted by LR, RRM by MR only.

LR activation

UE can predict the reception performance of LR, based on the measurement of SSB by MR. If the measurement result, e.g. RSRP/RSRQ/RSSI, of SSB is better than a threshold, UE can infer that the reception performance of LP-SS is good enough for synchronization and RRM measurement, and the detection performance of LP WUS is also good enough to achieve BLER target. The threshold is related to LR sensitivity and also other factors such as the power offset of LP-SS/LP-WUS compared to SSB, the periodicity of LP-SS. In general, two options can be considered to activate LP-WUR,

- Option 1: If the estimated RRM measurement by LP-SS/LP WUS can satisfy the configured conditions, then UE can decide to activate LP-SS based RRM and/or LP WUS monitoring.

- Option 2: If the RRM by MR can satisfy the configured conditions, then UE can decide to activate LP-SS based RRM and/or LP WUS monitoring.

The related RRM requirement by MR and LR is up to RAN4 to decide. With a common LP-SS/LP-WUS signal, the detection performance is different for an OOK-based receiver and an OFDM-based receiver since the two LR types has different receiver sensitivities. The corresponding conditions for LR activation can be different.

LR deactivation

When UE falls into ultra-deep sleep mode, UE monitors LP-SS/LP-WUS by LR with MR deactivated. The following conditions may lead to MR activation and LR deactivation:

(1) UL transmission happens;

(2) LP WUS indicating the UE to wake up is received;

(3) The reception performance of LP-SS/LP-WUS falls below LR sensitivity;

(4) The reception performance of LP-SS/LP-WUS falls below a threshold, which triggers the MR waking up for neighbouring cell RRM measurement.

UE can predict the RRM performance of MR, based on the measurement by LR. Two options can be considered to deactivate LP-WUR

- Option 1: If the estimated RRM measurement of SSB can satisfy the configured conditions, UE may wake up MR for serving cell and/or neighboring cell RRM measurement.

- Option 2: If the RRM by LR can satisfy the configured conditions, UE may wake up MR for serving cell and/or neighboring cell RRM measurement.

Similarly, with a common LP-SS/LP-WUS signal, the corresponding conditions for LR deactivation can be different for OOK-based receiver and OFDM-based receiver.

Proposal 7: Support UE activation and/or deactivation of LP-WUS monitoring based on preconfigured criteria

Proposal 8: LR sensitivity requirement for OOK-based LR and OFDM-based LR should be defined by RAN4.

Proposal 9: Conditions to activate LR for UE with OOK-based LR and UE with OFDM-based LR can be different.

Proposal 10: LR is deactivated and MR is activated when (1) UL transmission happens; (2) LP WUS indicating the UE to wake up is received; (3) The reception performance of LP-SS/LP-WUS falls below LR sensitivity; (4) The reception performance of LP-SS/LP-WUS falls below a threshold, which triggers the MR waking up for neighbouring cell RRM measurement.

Proposal 11: Conditions to activate MR for serving/neighbour cell measurement for UE with OOK-based LR and UE with OFDM-based LR can be different.

- 3 LP WUS monitoring

Since periodical LP-SS is supported, UE has frame level/symbol level timing information, and can support duty cycle LP WUS monitoring. Within a duty cycle, there can be multiple LP-WUS monitoring occasions (LOs), each LO corresponds to a group of UEs. For example, assuming there are N LOs within duty cycle TLP-WUS, a UE can determine its corresponding LO index by its UE ID, similar as the mapping method used in PF/PO. A related figure is shown in Fig.1 with 4 LOs within a duty cycle TLP-WUS.

Fig.1 Multiple LOs within one duty cycle and each corresponds to a UE group

Proposal 12: Support duty cycle monitoring of LP-WUS, with multiple LP WUS monitoring occasions (LOs) within a cycle. And UE can determine its corresponding LO index by its UE ID.

To reduce resource overhead, gNB does not need to transmit LP WUS in every LO, instead, gNB only transmits LP WUS in the LO when there is one or more UE corresponding to the LO needs to wake up. At least wake up indication at the granularity of UE subgroup should be carried in LP WUS. For example, there can be K bits payload in LP-WUS, and each bit is bitmapped to one UE subgrouping, bit “1” indicates waking up and bit “0” otherwise. UE can determine its corresponding bit index within the K bits by UE ID, or by core network configuration. The detailed UE subgrouping method is up to RAN2 discussion.

Proposal 13: At least wake up indication at the granularity of UE subgroup should be carried in LP WUS.

Procedures after wake up by LP-WUS

Depending on whether UE needs to monitor PEI or not, after woke up by LP WUS, UE can have two different procedures,

Procedure 1: after woke up by LP-WUS, UE monitors PEI, and UE will monitor PO if the PEI indicates UE to do so.

Procedure 2: after woke up by LP-WUS, UE directly monitor PO without monitoring PEI.

LP-WUS and PEI are both wake up indications to inform UE whether to monitor PO. If LP WUS applies the same UE subgrouping as PEI, the UE may not need to monitor PEI again after woke up by LP WUS, since no additional information can be provided by PEI. The only benefit for additional PEI detection is for the case of false alarmed LP-WUS. If LP WUS applies different UE subgrouping from PEI, the combination of LP WUS and PEI can provide UE a finer granularity wake up indication that can reduce unnecessary wake up, so it would be meaningful for UE to monitor PEI again after woke up by LP WUS.

For R17 PEI, UE subgrouping based on UE ID and NAS configuration are both adopted, and similar UE subgrouping methods can also be considered for LP WUS. It is possible the network configures different UE subgrouping for PEI and LP WUS, and UE may not have a full knowledge of whether the UE subgrouping(especially for the subgrouping based on NAS configuration) applied for PEI and LP WUS are the same or not, gNB can configure whether UE should go with Procedure 1 or Procedure 2.

Proposal 14: UE can be configured whether to go with Procedure 1(LP-WUS->PEI->PO) or Procedure 2(LP-WUS-> PO).

A UE only needs to monitor one LO in every duty cycle TLP-WUS. If the UE detects in its corresponding LO a LP WUS which indicates the UE to wake up, UE will wake up its MR after a wake-up delay to monitor its corresponding PEI/PO. As shown in Fig.2. If gNB does not know the wake-up delay of the UE, gNB may transmit paging too early before MR wake up, UE will miss the paging; or, gNB may transmit paging too late, UE has to keep on monitoring PEI/PO for multiple duty cycles until it receives the paging, which is not power saving. So it is necessary for gNB to know the wake-up delay of the UE and gNB transmit paging in the first PEI/PO corresponding to the UE after MR wakes up. The wake-up delay may be dependent on whether a UE will monitor PEI or PO after an indication of LP-WUS.

Proposal 15: Suggest to specify wake-up delay for activating MR from sleep state.

Fig.2 UE monitor its first PO after woke up by LP WUS

RRM measurement

RRM measurement is an essential issue that impacts UE power saving, since UE has to do serving cell/neighbor cell measurement periodically. In FR1, the periodicity can be quite dense, i.e. per DRX cycles for serving cell measurement for DRX cycle >= 1.28s, and for neighboring cell measurement, K1=3 times relaxation in Rel-16 or K3=6 times relaxation for eRedCap applies under certain conditions.

To reduce power consumption, offloading RRM measurement to LR was discussed in R18 SI. The measured reference signal can be LP-SS and PSS/SSS (only for OFDM-based LR). The commonly used RRM measurement metrics RSSI/RSRP/RSRQ/SINR can also be applied for LP-SS, but the metric definition may need some change for OOK-based LR, and for OFDM-based LR with time domain detection. For example, for SSS based measurement, RSRP is defined in the granularity of RE, whereas OOK-based LR and OFDM-based LR with time domain detection does not have the capability to handle signals on RE level, so the RSRP definition should be updated. The discussion can be left to RAN 4.

Proposal 16: RRM measurement reference signal can be LP-SS and PSS/SSS.

Proposal 17: For LP-SS, RRM measurement metrics can be RSSI/RSRP/RSRQ/SINR.

Proposal 18: For OOK-based LR and OFDM-based LR with time domain detection, how to modify the definition of the metrics is for RAN4 to decide.

Serving cell measurement

Offloading serving cell RRM measurement to LR will bring a huge power saving benefit. Totally offloading and partially offloading from MR to LR can both be considered. Totally offloading means MR does not do serving cell measurement at all and serving cell RRM measurement is fully offload to LR. And partially offloading means, besides serving cell RRM measurement by LR, MR also needs to do serving cell measurement, maybe in relaxed periodicity. Comparatively, totally offloading has more power saving gain, and partially offloading provides UE more accurate measurement results reflecting the channels for MR.

Proposal 19: Totally offloading and partially offloading of serving cell measurement from MR to LR can both be considered.

Serving measurement will be offloaded to LR naturally after LR activation, but whether to apply totally offloading or partially offloading should be considered. Alt 1 is based on measurement results by LR. For example, assuming two RSRP or RSRQ thresholds Thres1 and Thres2, and Thres1 represents better channel quality than Thres2. Totally offloading can happen when RSRP/RSRQ measurement result of LP-SS is better than Thres1, and partially offloading happens when RSRP/RSRQ measurement result of LP-SS is better than Thres2 but worse than Thres2. And for OOK-based LR and OFDM-based LR, the thresholds can be determined separately. Alt 2 is based on gNB configuration. For example, gNB can configure only totally or only partially offloading is allowed by explicit signaling. And the explicit signaling can be configured separately for OOK-based LR and OFDM-based LR.

Proposal 20: Criteria for whether to apply totally offloading or partially offloading can be based on (1)measurement results by LR, (2) gNB configuration.

Neighbouring cell measurement

Based on the evaluations in R18 SI, OOK based signal has much worse detection performance than PDCCH and can hardly provide whole cell coverage, thus neighboring cell measurement by OOK-based LR is not feasible and only can be done by MR. OFDM-based LR has similar or slightly worse detection performance compared to MR, it is possible that OFDM-based LR can receive LP-SS/PSS/SSS from neighboring cell, so neighboring cell measurement by OFDM-based LR can be considered. And to facilitate neighboring cell measurement, OFDM-based LR should be aware of the measured neighboring cell ID, which can be carried by LP-SS.

It is necessary to further relax neighbouring cell measurement by MR at least to the same level as serving cell measurement, otherwise, neighbouring cell measurement will become the bottleneck for power saving. The update to the existing conditions, e.g., low mobility condition and not at cell edge condition can be discussed when LP-SS/PSS/SSS based RRM by LR is enabled.

Proposal 21: Support neighbouring cell measurement by MR.

Proposal 22: Support neighbouring cell measurement by OFDM-based LR.

Proposal 23: Support relaxing of neighbouring cell measurement by MR.

LP WUS for eDRX

eDRX is an extreme power saving technique, but with very long paging latency. For example in RRC idle mode, when TeDRX, CN is longer than 1024 radio frames, PTW is configured and only within the PTW UE will monitor PO and outside the PTW, UE will not monitor PO. If a paging information arrives just after the end of the PTW, gNB has to wait until the PTW in the next eDRX cycle to transmit the paging. To reduce latency, LP-WUS can be combined with eDRX, and when UE receives LP WUS outside the PTW, UE can wake up its MR to monitor paging. A related figure is shown in Fig.3.

Fig.3 LP WUS combined with eDRX

Proposal 24: Support LP WUS applied for eDRX to reduce paging latency.

Conclusions

For LP WUS operation in idle/inactive mode, we have the following proposals,

Proposal 1: Restricted by LP WUR capability, LP-WUS/LP-SS and signals/channels used by MR can be on different carriers or different bands.

Proposal 2: The detailed LP WUR supporting bands requirement is up to RAN4 to decide.

Proposal 3: Support BW of LP-WUS/LP-SS is not greater than 5 MHz for FR1.

Proposal 4: Support BW of LP-WUS is configurable, and support LP WUS BW being different for RRC idle/inactive and RRC connected states.

Proposal 5: Support beam sweeping pattern of LP-WUS.

Proposal 6: The power offset of LP-SS/LP-WUS compared to the power of SSB should be configured.

Proposal 7: Support UE activation and/or deactivation of LP-WUS monitoring based on preconfigured criteria

Proposal 8: LR sensitivity requirement for OOK-based LR and OFDM-based LR should be defined by RAN4.

Proposal 9: Conditions to activate LR for UE with OOK-based LR and UE with OFDM-based LR can be different.

Proposal 10: LR is deactivated and MR is activated when (1) UL transmission happens; (2) LP WUS indicating the UE to wake up is received; (3) The reception performance of LP-SS/LP-WUS falls below LR sensitivity; (4) The reception performance of LP-SS/LP-WUS falls below a threshold, which triggers the MR waking up for neighbouring cell RRM measurement.

Proposal 11: Conditions to activate MR for serving/neighbour cell measurement for UE with OOK-based LR and UE with OFDM-based LR can be different.

Proposal 12: Support duty cycle monitoring of LP-WUS, with multiple LP WUS monitoring occasions (LOs) within a cycle. And UE can determine its corresponding LO index by its UE ID.

Proposal 13: At least wake up indication at the granularity of UE subgroup should be carried in LP WUS.

Proposal 14: UE can be configured whether to go with Procedure 1(LP-WUS->PEI->PO) or Procedure 2(LP-WUS-> PO).

Proposal 15: Suggest to specify wake-up delay for activating MR from sleep state.

Proposal 16: RRM measurement reference signal can be LP-SS and PSS/SSS.

Proposal 17: For LP-SS, RRM measurement metrics can be RSSI/RSRP/RSRQ/SINR.

Proposal 18: For OOK-based LR and OFDM-based LR with time domain detection, how to modify the definition of the metrics is for RAN4 to decide.

Proposal 19: Totally offloading and partially offloading of serving cell measurement from MR to LR can both be considered.

Proposal 20: Criteria for whether to apply totally offloading or partially offloading can be based on (1)measurement results by LR, (2) gNB configuration.

Proposal 21: Support neighbouring cell measurement by MR.

Proposal 22: Support neighbouring cell measurement by OFDM-based LR.

Proposal 23: Support relaxing of neighbouring cell measurement by MR.

Proposal 24: Support LP WUS applied for eDRX to reduce paging latency.

- References

- RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), 3GPP TSG RAN Meeting #102.

- R1-2307356, Evaluation on low power WUS, 3gpp RAN1 #114 meeting, Xiaomi.