3GPP TSG RAN WG1 #116		                                           R1-2400592

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.6.3

Source: 	OPPO

Title:	Consideration on wake-up procedure in connected mode

Document for:	Discussion & Decision

Introduction

In the WID the scope of LP-WUS wake-up procedure in connected mode are mainly in:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

In this contribution we have some initial considerations on the topic.

Wake-up signal for CONNECTED mode

In the work item, the wake-up signal is mainly designed for the IDLE/INACTIVE modes. In that case, we consider kind of medium payload such as 8 bits or 16 bits indication as example. That may be carried in the form of possible group indication by bit map for IDLE/INACTIVE modes. Indication occasion and bit field to a UE is by configuration. And one UE or more UEs can be assigned to one bit field. It is also possible for the same indication scheme applied to the CONNECTED mode case. The overlaid sequences can serve the same functionality as we expect CONNECTED mode also can support different types of wake-up receivers.

Figure 1. LP-WUS Waveform by OOK overlaid with sequences.

Considering the unified WUS signal for different RRC modes, OOK wake-up information and overlaid sequences can independently wake up main receiver operation, which will be essentially PDCCH detection.  In the figure as example without the Manchester coding, the sequence “S1” will represent same information of “1001…”.

Proposal 1: All RRC modes are sharing the same WUS signal in coding, payload and modulation.

Wake-up mechanisms

The mechanism for using the information by WUS signal for triggering PDCCH detection can consider several options. The detection means directly turning on the search space monitoring. Or, the detection is bundled with the timer-based behavior like DRX.

Waking up Search Space monitoring directly:

The Search Space can be set enabled within a duration or for a number of monitoring occasions. The WUS monitoring periodicity can be configured large than the search space periodicities. One WUS detection can control the following set of SS monitoring.

Figure 2. Search Space monitoring is directly waked up by LP-WUS

The WUS information bit for the UE in the payload can indicate that. This means the1bit can indicates a group of PDCCH monitoring occasions.

Indication DRX ON:

In the earlier power saving enhancement, a wake-up signal was used to indicate DRX ON. That indirectly enable and disable PDCCH detection through the whole mechanism mainly in higher layer. It can be a option by replace the PDCCH-based WUS with the newly introduced low-power WUS.

This scheme should mandate configuration of CDRX with LP-WUS.

Figure 3. Indication DRX ON by LP-WUS

For those options the simple solution should be the direct wake-up of search space monitoring. That does not mandate the DRX configurations. For the second option, it may introduce more higher layer impact, considering the difference between LP-WUS and PDCCH-based WUS.

Proposal 2: RAN1 study and down-select the options of LP-WUS wake-up in CONNECTED mode: LP-WUS waking up Search Space monitoring directly; LP-WUS waking up DRX ON.

Interworking with legacy power saving schemes.

The LP-WUS should be interwork with legacy power saving schemes in CONNECTED mode, which are PDCCH-based WUS (DCP, Format 2_6), Cross-slot scheduling & PDCCH Skipping. The DCP should be able to indicate wake-up with LP-WUS. Thus, rules should be discussed on how duplicated indications applied, especially if the DRX-based indication is adopted for LP-WUS.

When multi-carrier operation enabled, the LP-WUS indication applied to multiple carriers. It may indicate multiple carriers wake-up for those activated carriers. In the Scell dormancy scheme, DCI format 2_6 can indicate the Scell dormancy. For LP-WUS, the payload is expected to be much smaller that format 2_6. Those additional indication should be not introduced.

Proposal 3: The LP-WUS should be interwork with legacy power saving schemes in CONNECTED mode. Only wake-up bits are carried by LP-WUS, e.g. no Scell dormancy indication.

Activation and deactivation for the LP-WUS in connected mode

Even without multiplexing cell ID in LP-WUS signal, we consider that wake-up signal is assumed to be associate with cell. In CONNECTED mode, UE from different cells should be triggered individually. Under the same network topology, LP-WUS coverage is smaller that normal NR signals. To operate in the blank coverage, the UE should be able to activated or deactivated for LP-WUS detection.

In the study phase, it identified few candidates for activation and deactivation solution:

-	In RRC CONNECTED mode, LP-WUS monitoring can be activated/deactivated by at least one or more of

-	by gNB RRC signaling, with or without UE assistance.

-	by gNB L1/L2 LP-WUS activation/deactivation signaling, with or without UE assistance.

-	based on pre-configured condition(s), such as timer.

-	LP-WUS monitoring by UE is known to gNB, study whether it could be transparent to gNB.

-	other options are not precluded.

For those options with signaling, the gNB should have sufficient information to decide whether set the UE to monitor the LP-WUS. Even in the CONNECTED mode, UE report should be needed. Otherwise, UE may not able to monitor LP-WUS in the coverage gap area of LP-WUS. That report is one type of UE assistance and could be in form of measurement.

For the option with pre-configured condition(s), timer should be configured as the LP-WUS have monitoring duty-cycled. Additionally, further conditions can be considered. One is the decoding of LP-WUS. Consider the UE is under poor coverage, the decoding of LP-WUS will not CRC-passed. A few that failed decoding could trigger the UE to not receive LP-WUS for some time. Another possibility is based on the measurement either on NR signal or LP-SS.

In the above schemes of pre-configured condition, the gNB may not know if UE is monitoring LP-WUS or not in the CONNECTED mode. That is due our understanding that the pre-configured condition could be UE side procedure like measurement. It may not be reported to gNB in time.

Proposal 4:

For the LP-WUS Activation and deactivation options by signaling, measurement of UE is used as UE assistant information.

For the LP-WUS Activation and deactivation options by pre-configured condition(s), decoding of LP-WUS or measurement of UE are included in condition(s).

Measurement procedure

In the CONNECTED mode, MR measurements are available. That can also serve the purpose for determining the LP-WUS monitoring. The LP-SS already configure for IDLE/INACTIVE modes. It is possible to be utilized for the measurement, which is more suitable for determining LP-WUS monitoring. It can be further justified that RAN1 should introduce LP-SS measurement in CONNECTED mode.

Proposal 5: RAN1 can assume that MR measurements are available for the LP-WUS activation and deactivation. FFS if RAN1 should introduce LP-SS measurement in CONNECTED mode.

Conclusion

In this contribution, we had discussed and analyzed lower power WUS in CONNECTED mode. We basically consider unified signal for different RRC modes and the procedure should be based on that unified design.

Regarding the LP-WUS procedure and related issues for CONNECTED mode, we have following proposals:

Proposal 1: All RRC modes are sharing the same WUS signal in coding, payload and modulation.

Proposal 2: RAN1 study and down-select the options of LP-WUS wake-up in CONNECTED mode: LP-WUS waking up Search Space monitoring directly; LP-WUS waking up DRX ON.

Proposal 3: The LP-WUS should be interwork with legacy power saving schemes in CONNECTED mode. Only wake-up bits are carried by LP-WUS, e.g. no Scell dormancy indication.

Proposal 4:

For the LP-WUS Activation and deactivation options by signaling, measurement of UE is used as UE assistant information.

For the LP-WUS Activation and deactivation options by pre-configured condition(s), decoding of LP-WUS or measurement of UE are included in condition(s).

Proposal 5: RAN1 can assume that MR measurements are available for the LP-WUS activation and deactivation. FFS if RAN1 should introduce LP-SS measurement in CONNECTED mode.

Reference

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo