3GPP TSG RAN WG1 #116	                	    R1-2400598

Athens, Greece, February 26th – March 1st, 2024

Source:	OPPO

Title:	   Discussion on UE features under Topic A

Agenda Item:	8.12.5

Document for:	Discussion and Decision

# Introduction

This contribution discusses our views on UE features for Rel-18 XR.

# UE features for Rel-18 XR

For FG50-1b

Figure 1 Relationship between FG50-1b and FG 11-9a

It can be seen from Figure 1 that a UE supporting FG 50-1b (joint release in Rel-18) must already support both single-PUSCH CG (FG 5-19/5-20) and multi-PUSCH CG (FG 50-1), but not necessarily support any of FG11-9 (multiple single-PUSCH CG configurations), FG11-9a (joint release of multiple single-PUSCH CG configurations) and FG50-1a (multiple multi-PUSCH CG configurations).

There was a proposal in RAN1 #115 to ignore FG11-9a if UE reports FG50-1b. However, even if FG11-9a is ignored, a reported “no-support” in FG11-9 still plays a role in disallowing multiple single-PUSCH CG configurations in the joint release. Meanwhile, the currently agreed FG50-1a already excludes the chance to ignore FG11-9 (there is no such text for “ignoring”). Therefore, ignoring FG11-9a is not enough to finalize the logic and therefore is not an appropriate direction in our view.

With FG11-9 and FG11-9a being taken into account, Table 1 shows the applicable CG configurations in Rel-18 joint release based on various combinations of supporting on {FG11-9, FG11-9a, FG50-1a}, which also reflects a fact that the impact of FG11-9 to Rel-18 joint release can be well masked by the impact of FG11-9a.

Table 1 Applicable CG configurations for joint release in FG50-1b

| Support of FG11-9 | Support of FG11-9a | Support of 50-1a | Applicable CG configurations for FG50-1b joint release |
| --- | --- | --- | --- |
| N | N | N | Up to one single-PUSCH CG configuration and up to one multi-PUSCH CG configuration |
| N | N | Y | Up to one single-PUSCH CG configuration and multiple multi-PUSCH CG configurations |
| Y | N | N | Up to one single-PUSCH CG configurations and up to one multi-PUSCH CG configuration |
|  | Y |  | Multiple single-PUSCH CG configurations and up to one multi-PUSCH CG configuration |
| Y | N | Y | Up to one single-PUSCH CG configurations and multiple multi-PUSCH CG configurations |
|  | Y |  | Multiple single-PUSCH CG configurations and multiple multi-PUSCH CG configurations |

In case UE supports both FG11-9a and FG50-1b, the values for M in both FGs should be the same, so that there is no need to distinguish further in DCI field size determination.

Proposal-1: Adopt following for FG50-1b (Rel-18 XR joint release).

| 50. NR_XR_Enh | 50-1b | Joint release in a DCI for two or more configured grant Type 2 configurations, including multi-PUSCH CG configuration(s), for a given BWP of a serving cell | M<=4 bits indication in the Release DCI is used for indicating which CG configuration(s) is/are released, where the association between each state indicated by the indication and the CG configuration(s) is - Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple CG configurations to be released - In case of no higher layer configured state(s), separate release is used where the release corresponds to the CG configuration index indicated by the indication FFS whether/how to capture the relationship with 11-9a | One of {50-1, 50-1a} | Yes | N/A |  | Per band | N/A | N/A | N/A | Regarding the interpretation of UE capabilities in case of cross-carrier operation, support of FG50-1b is based on the support of this capability for the band of the scheduled/triggered/indicated cell only Note: For the case of joint release in a DCI for two or more configured grant Type 2 configurations, including multi-PUSCH CG configuration(s), for a given BWP of a serving cell, the reporting of this FG applies, i.e., ignore FG 11-9a If UE does not support FG50-1a, there can be up to one multi-PUSCH CG configuration in the configured states. If UE supports FG11-9a, the value of M associated with FG11-9a is equal to the value of M associated with this FG. If UE does not support FG11-9a, there can be up to one single-PUSCH CG configuration in each of the configured states. If UE supports FG11-9a but does not support this FG, the UE does not expect to be indicated for joint release including multi-PUSCH CG configuration(s) | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Conclusion

This contribution concludes with following proposals:

Proposal-1: Adopt following for FG50-1b (Rel-18 XR joint release).

| 50. NR_XR_Enh | 50-1b | Joint release in a DCI for two or more configured grant Type 2 configurations, including multi-PUSCH CG configuration(s), for a given BWP of a serving cell | M<=4 bits indication in the Release DCI is used for indicating which CG configuration(s) is/are released, where the association between each state indicated by the indication and the CG configuration(s) is - Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple CG configurations to be released - In case of no higher layer configured state(s), separate release is used where the release corresponds to the CG configuration index indicated by the indication FFS whether/how to capture the relationship with 11-9a | One of {50-1, 50-1a} | Yes | N/A |  | Per band | N/A | N/A | N/A | Regarding the interpretation of UE capabilities in case of cross-carrier operation, support of FG50-1b is based on the support of this capability for the band of the scheduled/triggered/indicated cell only Note: For the case of joint release in a DCI for two or more configured grant Type 2 configurations, including multi-PUSCH CG configuration(s), for a given BWP of a serving cell, the reporting of this FG applies, i.e., ignore FG 11-9a If UE does not support FG50-1a, there can be up to one multi-PUSCH CG configuration in the configured states. If UE supports FG11-9a, the value of M associated with FG11-9a is equal to the value of M associated with this FG. If UE does not support FG11-9a, there can be up to one single-PUSCH CG configuration in each of the configured states. If UE supports FG11-9a but does not support this FG, the UE does not expect to be indicated for joint release including multi-PUSCH CG configuration(s) | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Reference

- TS 38.822 v17.1.0,  NR User Equipment (UE) feature list (Release 17)

- R1-2312572, Updated RAN1 UE features list for Rel-18 NR after RAN1#115, Moderators (AT&T, NTT DOCOMO, INC.)