3GPP TSG RAN WG1 #116		R1-2400601

Athens, Greece, February 26th - March 1st, 2024

Source:	OPPO

Title:	Discussion on adaptation of common signal/channel transmission

Agenda Item:	9.5.3

Document for:	Discussion and Decision

Introduction

In RAN meeting #102, the new WID regarding enhancements of network energy savings for NR [1] was approved. One objective of the work item is the following.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we provide our views on adaptation of common signal/channel transmission for Rel-19 NES.

Discussion

In Rel-18 NES, to achieve network energy saving gain, the gNB can transmit and/or receive UE-specific signal/channel in centralized time period, by sacrificing the delay of UE data transmission in exchange for network energy saving. Specifically, the gNB may configure cell DTX and/or cell DRX for the UE.  During the non-active period of the cell DTX, the gNB is not expected to transmit the DL UE-specific signal/channel, and the UE is not expected to receive the DL UE-specific signal/channel accordingly. During the non-active period of the cell DRX, the UE is not expected to transmit the UL UE-specific signal/channel, and the gNB is not expected to receive the UL UE-specific signal/channel accordingly. In this way, the network energy consumption can be reduced.

In Rel-19 NES, in order to further improve network energy saving gain, the gNB may consider reducing the transmission and reception of common signal/channel at least in time domain by introducing adaptation transmission. The common signal/channel include SSB, PRACH, and Paging.

## 2.1 Adaptation of SSB in time domain

In order to perform the adaptation of SSB in time domain for NES cell, the following two candidate schemes are considered.

- Alt-1: Two SSB configurations with different period durations are configured for the NES cell. The SSB transmission opportunities corresponding to the SSB configuration with long-period are a subset of the SSB transmission opportunities corresponding to the SSB configuration with short-period. Which SSB configuration is adopted for the NES cell is indicated by gNB via dynamic indication information (e.g., GC-PDCCH).

- Alt-2: Enhancement to the cell DTX configuration for the NES cell. The SSB transmission opportunities are invalid during the non-active periods of the enhanced cell DTX. The activation or deactivation of the enhanced cell DTX of the NES cell is indicated by gNB via dynamic indication information (e.g. GC-PDCCH).

Figure 1 shows an example of two SSB configurations with different period durations for the NES cell (Alt-1). Figure 2 shows an example of the enhanced cell DTX for the NES cell (Alt-2).

Figure 1. Two SSB configurations with different period durations for the NES cell

Figure 2. The enhanced cell DTX for the NES cell

For Alt-1, the gNB may set the SSB configuration with long-period as the default SSB transmission configuration, and the SSB configuration with short-period is used to improve the performance of the Rel-19 NES UEs. The advantage of Alt-1 is that it may reduce the impact to legacy UEs for camping on the NES cell, as long as the SSB transmission opportunities which can be identified by legacy UEs are not changed. However, since legacy UEs can only has the ability to read the SSB configuration with long-period, the delay of initial access as well as the SSB-based RRM measurement accuracy of legacy UEs in the NES cell will be greatly affected when compared to accessing to a non-NES cell.

For Alt-2, the enhanced cell DTX is configured to the Rel-19 NES UEs for the NES cell. Because legacy UEs do not have the ability to read the configuration of the enhanced cell DTX, to reduce the impact as much as possible, it is better to transfer legacy UEs to a non-NES cell by gNB implementation. For example, the gNB may configure a handover command to a legacy UE in connected mode to switch its serving cell from the NES cell to a non-NES cell, or the gNB may configure a bar signaling to refuse legacy UEs in idle/inactive mode camping on the NES cell. In this way, the impact to legacy UEs when a cell is operating in Rel-19 NES mode can be avoided.

Each scheme has its advantages and disadvantages, both of which can be considered as candidate schemes to reduce the transmission and reception of SSB in time domain in the NES cell.

Proposal 1: Two schemes can be considered as candidate schemes for adaptation of SSB in time domain for the NES cell:

- Alt-1: Two SSB configurations with different period durations are configured for the NES cell.

- Alt-2: Enhancement to the cell DTX configuration for the NES cell.

## 2.2 Adaptation of PRACH in time domain

Similar to SSB, the adaptation of PRACH in time domain can also include the following two candidate schemes.

- Alt-1: Two PRACH configurations with different period durations are configured for the NES cell. The ROs corresponding to the PRACH configuration with long-period are a subset of the ROs corresponding to the PRACH configuration with short-period. Which PRACH configuration is adopted for the NES cell is indicated by gNB via dynamic indication information (e.g., GC-PDCCH).

- Alt-2: Enhancement to the cell DRX configuration for the NES cell. The ROs are invalid during the non-active periods of the enhanced cell DRX. The activation or deactivation of the enhanced cell DRX of the NES cell is indicated by gNB via dynamic indication information (e.g. GC-PDCCH).

Similarly, both Alt-1 and Alt-2 can be considered as candidate schemes to reduce the transmission and reception of PRACH in time domain in the NES cell.

Proposal 2: Two schemes can be considered as candidate schemes for adaptation of PRACH in time domain for the NES cell:

- Alt-1: Two PRACH configurations with different period durations are configured for the NES cell.

- Alt-2: Enhancement to the cell DRX configuration for the NES cell.

In addition, for simplicity, a unified scheme can be adopted to reduce the transmission and reception of SSB and PRACH in time domain in the NES cell.

Proposal 3: Support unified design for adaptation of SSB and PRACH in time domain for the NES cell.

## 2.3 Adaptation of PRACH in spatial domain

In our view, if the adaptation of PRACH in spatial domain is supported, i.e., the non-uniform PRACH resources are associated with different SSB indexes, while the adaptation of SSB in spatial domain is not supported, i.e., the SSB indexes in the SSB burst for transmission are kept unchanged, the energy saving gain would not be obvious.

Moreover, to support non-uniform PRACH resources per SSB, it is necessary to enhance the mapping of the association relationship between SSB and RO, which requires a lot of additional standards effort as well as a large UE implementation complexity.

Therefore, it is proposed to deprioritize the discussion of adaptation of PRACH in spatial domain, or to consider adaptation of PRACH in spatial domain together with adaptation of SSB in spatial domain if not deprioritized.

Proposal 4: Deprioritize the discussion of adaptation of PRACH in spatial domain.

## 2.3 Adaptation of paging occasions

For the adaptation of paging in time domain, enhancements to calculation formula of paging occasion index in a paging frame to realize the compacted POs in a PF may be considered for Rel-19 NES UE in the NES cell. Considering legacy UEs do not have the ability to enhance the PO calculation formula, to reduce the impact, it is better to transfer legacy UEs to a non-NES cell by gNB implementation, as discussed above.

Proposal 5: Enhancements to calculation formula of paging occasion index in a paging frame to realize the compacted POs in a PF may be considered.

Conclusion

In this contribution, we provide our views on adaptation of common signal/channel transmission for Rel-19 NES. The following proposals are provided.

Proposal 1: Two schemes can be considered as candidate schemes for adaptation of SSB in time domain for the NES cell:

- Alt-1: Two SSB configurations with different period durations are configured for the NES cell.

- Alt-2: Enhancement to the cell DTX configuration for the NES cell.

Proposal 2: Two schemes can be considered as candidate schemes for adaptation of PRACH in time domain for the NES cell:

- Alt-1: Two PRACH configurations with different period durations are configured for the NES cell.

- Alt-2: Enhancement to the cell DRX configuration for the NES cell.

Proposal 3: Support unified design for adaptation of SSB and PRACH in time domain for the NES cell.

Proposal 4: Deprioritize the discussion of adaptation of PRACH in spatial domain.

Proposal 5: Enhancements to calculation formula of paging occasion index in a paging frame to realize the compacted POs in a PF may be considered.

Reference

- RP-234065, New WID: Enhancements of network energy savings for NR, Ericsson (Moderator).