3GPP TSG-RAN WG1 Meeting #116			R1-2300612

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	9.4.2.2

Title:	Initial considerations on frame structure and timing aspects of A-IoT communication

Source:	OPPO

Document for:	Discussion and Decision

1. Introduction

In RAN#102, a new Rel-19 study item description for solutions for Ambient IoT in NR is approved in [1], which was based on a RAN level study outcome / technical report [2]. Related parts of the approved R19 SID are captured in the following.

General Scope The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement. - Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor - FR1 licensed spectrum in FDD. - Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s). - Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum. The following objectives are set, within the General Scope: - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1.

Since RAN1#116 is the first WG meeting discussing this topic, in this contribution we provide some background information and our views on the following aspects related to this agenda item.

- General aspects / backgrounds of A-IoT communication

  - Device types (I and II)

  - Topology 1

  - Topology 2

- Synchronization and timing aspects

  - Time and frequency synchronization in Topology 1

  - Time and frequency synchronization in Topology 2

- Random access aspects

  - Purposes of RA and its potential operations in A-IoT communication

- Scheduling and timing aspects

  - Dynamic scheduling and configured scheduling

  - Search space configuration

2. General aspects and background of A-IoT communication

Since the topic on Ambient IOT (A-IoT) is very new and there has been no discussion so far in RAN1 in the past, the main intention / motivation of this section is to provide, according to our understandings, discussion on some general aspects and background information of A-IoT communication related to this agenda item.

2.1 Device types (I and II)

According to the A-IoT SID [1] (also captured in Section 1), two new device types that somewhat different to the device categorization in TR 38.848 [2] (A, B and C) are define and included for the RAN WG study. For simplicity, we referred them in this contribution as Type I and Type II device.

Type I device: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. Type II device: ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. X is to be decided in WGs.

As can be seen, device Type I is defined to have a lower peak power consumption of ~1µW. Although neither DL nor UL amplification is to be assumed for the Type 1 device, when considering the device still need to perform various functions such as synchronization, data encoding and decoding, signal modulation/demodulation, higher layer processing and etc. for A-IoT communication in uplink and downlink, the “~1 µW” is considered in our view to be on the low end. In our view, a reasonable range for Type I device should be “~4µW”.

For device Type II, “a few” hundred peak power consumption still needs to be defined/decided in RAN1. In our understanding, the transmission and reception distance range for this device Type II is expected to be higher than the device Type I, thus requiring both DL and UL amplification in the device). Consequently, this will the main driver for the power consumption difference to the device Type I. In addition, signal waveform generation is also required for this device Type II with active transmission, as it does not need to rely on a carrier wave provided externally to perform backscattering transmission. This would also add some power consumption to the device. Overall, in our view, a peak power consumption at around 500 µW would be a reasonable value for the device Type II.

Observation 1: For Type I device (lower peak power consumption in SID), the current value in the SID of ~1 µW seems to be on the low end, and it may not be achievable. In our view, a reasonable value for device Type I should be ~4 µW.

Observation 2: For Type II device (higher peak power consumption in SID), the exact value / range is not yet defined. In our view, considering the DL and UL power amplification and signal waveform generation for UL transmissions, a reasonable value for device Type II should be ~500 µW.

2.2 Topology 1 (Direct A-IoT communication between BS and A-IoT devices)

Figure 1, connectivity topology 1 with an external carrier wave node (CWN)

According to the definition of Topology 1 in [2] for A-IoT communication, “the A-IoT device directly and bidirectionally communicates with a BS. The communication between the BS and the A-IoT device includes data and/or signaling. This topology includes the possibility that the BS transmitting to the A-IoT device is different from the BS receiving from the A-IoT device.”  In our understanding, “the possibility that the BS transmitting to the A-IoT device is different from the BS receiving from the A-IoT device” is to resolve a full duplex problem when the same BS is used for both transmitting a carrier wave and receiving backscattering to/from the A-IoT device simultaneously on the same carrier. However, to study and enable a “dual-BS” operation in Topology 1 would be more complicated, involve multiple WGs (RAN1 and RAN3) and more specification impact than using an external carrier wave node (CWN). Since in the SID [1], a carrier wave can be provided externally for both Type I and Type II devices, we assume an external CWN will be used in Topology 1 to provide the carrier wave needed for backscattering transmission from the A-IoT device.

In terms of spectrum usage in Topology 1, as it is captured in the SID that FR1 licensed spectrum in FDD shall be considered for the study, we assume that A-IoT DL communication from the BS to A-IoT devices is transmitted using a carrier in the DL frequency range of a FDD band (i.e., a DL carrier in FDD band). For A-IoT UL communication from the A-IoT device to the BS, a carrier in the UL frequency range of a FDD band is used (i.e., a UL carrier in FDD band).

Observation 3: In Topology 1, if the BS transmitting to the A-IoT device is different from the BS receiving from the A-IoT device, some coordination between the two BS is expected and should be studied and specified. Hence, the study to enable a “dual-BS” operation would be more complicated, involve multiple WGs (RAN1 and RAN3) and more specification impact than using an external carrier wave node (CWN).

Proposal 1: In Topology 1, the same BS transmitting to and receiving from the A-IoT device should be assumed during the study to minimize the work and spec impact. Instead, an external carrier wave node (CWN) should be used to provide a carrier wave for A-IoT device’s backscattering operation.

- FFS any specification impact / support is needed for transmitting a carrier wave from the CWN

Observation 4: In Topology 1, in terms of frequency spectrum usage/allocation,

- it is assumed that a DL frequency carrier of FDD band is used for A-IoT DL communication from the BS to A-IoT devices, and

- it is assumed a UL frequency carrier of FDD band is used for A-IoT UL communication from the A-IoT devices to the BS.

2.3 Topology 2 (Indirect A-IoT communication between BS and A-IoT devices via intermediate UE)

Figure 2, A-IoT connectivity topology 2

According to the definition of Topology 2 in [2] for A-IoT communication, “the Ambient IoT device communicates bidirectionally with an intermediate node between the device and BS. In this topology, the intermediate node can be a relay, IAB node, UE, repeater, etc. which is capable of Ambient IoT. The intermediate node transfers Ambient IoT data and/or signaling between BS and the Ambient IoT device.” Then according to the SID, it is captured that “UE as intermediate node under NW control” and “UE as intermediate node, under network control” under the General Scope section. Therefore, it is our understanding that the bidirectional communication between the BS and the intermediate UE is assumed to be a normal cellular NR DL and UL connection using the Uu interface. As such, a normal DL frequency carrier and a UL frequency carrier in FDD band will be used for this Uu communication link. On the other hand, the A-IoT communication between the intermediate UE and A-IoT devices are expected to carry out using a same or different UL frequency carriers, since the regulation in some countries prohibits the use of the DL frequency spectrum in a FDD band for terminals/devices with mobility (not a fixed and planned location). However, it is yet unclear whether the intermediate UE and A-IoT devices uses the same UL frequency carrier for both A-IoT DL (intermediate UE to A-IoT devices) and A-IoT UL (A-IoT devices to intermediate UE) communications to deliver DL and UL data / control signaling information (i.e., in a TDD manner), or two different/separate UL frequency carriers are used for A-IoT DL and A-IoT UL communications (i.e., in a FDD manner).

In addition, the RAN-level study TR 38.848 [2] mentioned that “In all these topologies, the Ambient IoT device may be provided with a carrier wave from other node(s) either inside or outside the topology.” Since at least Type I devices (and some Type II devices) should be also supported in Topology 2, they will require a carrier wave to perform backscattering transmissions. Then the question would be whether the carrier wave should assume to be outside the Topology 2 (e.g., to use a CWN) or inside the Topology 2 (e.g., the intermediate UE transmits a carrier wave in the UL frequency spectrum of a FDD band). In the case if the carrier wave is provided/transmitted by the intermediate node UE and the intermediate UE also need to receive backscattering transmissions from the A-IoT, to avoid this full duplex problem, in our view the carrier wave should be provided in a different UL frequency in the FDD band to the scheduled/configured A-IoT UL communication frequency carrier or channel. That is, a A-IoT device should be capable of performing a frequency shift of a received carrier wave to the scheduled A-IoT UL frequency carrier or channel.

Observation 5: In Topology 2, in terms of frequency spectrum usage/allocation,

- it is assumed a normal/existing DL frequency carrier and a UL frequency carrier of a FDD band are used for the Uu DL and UL communication between the BS and the intermediate UE, respectively, and

- it is assumed that at least one UL frequency carrier of a FDD band is used for the bidirectional A-IoT DL and UL communication between the intermediate UE and the A-IoT devices.

  - FFS whether a same UL frequency carrier or two separate/different UL frequency carriers are used for A-IoT DL and A-IoT UL communications between the intermediate UE and the A-IoT devices

Observation 6: In Topology 2, if Type I devices and some Type II devices that are only capable of backscattering transmissions for A-IoT UL communication should be supported, a carrier wave should be provided.

- FFS whether the carrier wave should be provided/transmitted from a node outside the topology (e.g., CWN) or inside the topology (e.g., by the intermediate UE)

Proposal 2: In Topology 2, if the carrier wave is provided/transmitted by the intermediate node UE and the intermediate UE also need to receive backscattering transmissions from the A-IoT, to avoid this full duplex problem, an A-IoT device should be capable of performing a frequency shift of a received carrier wave to the scheduled A-IoT UL frequency carrier or channel.

Proposal 3: For simplification of discussion in RAN1, we propose to have a unified naming for the A-IoT communication interface from the perspective of an A-IoT device. That is,

- In Topology 1,

  - “A-IoT DL” and “A-IoT UL” are used for the bidirectional A-IoT communication between the BS and the A-IoT device.

- In Topology 2,

  - The existing “Uu DL” and “Uu UL” are used for the bidirectional cellular communication between the BS and the intermediate UE.

  - “A-IoT DL” and “A-IoT UL” are used for the bidirectional A-IoT communication between the intermediate UE and the A-IoT device.

- Synchronization and timing aspects

While it is well established that frequency and timing synchronization between the transmitter and receiver nodes is vitally essential in any wireless communication, the existing synchronization mechanism/principle in NR DL and UL communication may not be directly applicable to the A-IoT communication, where it relies on UE to detect SSB bursts from gNB to acquire carrier frequencies for both DL and UL, and derive the exact radio frame number and slot boundaries for both transmission and reception timings. This assumes that UE with stable crystal oscillator is able to accurately convert the frequency from DL to UL and maintain a stable sampling timing. Of course, the periodic transmission of SSB bursts from gNB helps the UE to maintain both frequency and timing synchronization in both DL and UL directions.

However, for a low-power and low-complexity/cost receiver and frequency oscillator design that is expected in A-IoT devices with a certain sampling frequency offset (SFO) error that is expected to be much higher than in a normal UE, the frequency conversion from SS reception in a A-IoT DL carrier to a A-IoT UL carrier/channel for transmission would be quite unstable. Furthermore, the SFO also suffers from an accumulation effect over time. The longer the time period without retuning the timing synchronization, the SFO error gets larger.

Observation 7: Due to a low-power and low-complexity/cost receiver and frequency oscillator design in A-IoT devices with a certain SFO error that is expected to be much higher than in a normal UE, and the SFO also suffers from an accumulation effect over time. It seems the frequency and timing synchronization strategy / mechanisms for A-IoT DL reception and A-IoT UL transmission should be separately considered.

Furthermore, for Type I and Type II A-IoT devices that perform backscattering transmissions based on a carrier wave for A-IoT UL communication, it is unnecessary for these devices to convert a A-IoT DL carrier frequency (based on SS reception) to a A-IoT UL carrier frequency for transmission.

Proposal 4: For A-IoT DL and A-IoT UL communications, separate and different frequency and timing synchronization strategies / mechanisms should be considered and designed.

Moreover, frequency and timing synchronization mechanism also needs to be differentiated between Topology 1 and Topology 2 since the involved communication nodes are different (BS in Topology 1 vs. intermediate UE in Topology 2) and the A-IoT DL carrier frequencies are also expected to be different (FDD DL frequency in Topology 1 vs. FDD UL frequency in Topology 2).

Proposal 5: For A-IoT communication in Topology 1 and 2, the mechanism of frequency and timing synchronization should be also differentiated, since the involved communication nodes and A-IoT DL frequency spectrum are different in these two topologies.

3.1 Time and frequency synchronization in Topology 1

- A-IoT DL direction

In Topology 1, since A-IoT DL communication is directly transmitted from the BS to A-IoT devices to deliver data and/or signaling messages (without routing through an intermediate node as in Topology 2), the existing principle of synchronization signal (SS) transmission in NR DL could be used as a baseline/starting point. That is, the BS could directly transmit SS over the A-IoT DL interface to be used by A-IoT devices as the basis/reference to perform frequency and timing synchronization. However, the expected traffic A-IoT DL communication could be quite infrequent (e.g., once in a few mins, 1 hour, a few hours, one day or a few days) or even event-triggered. It would be power and resource wasteful (environmentally unfriendly) and may create unnecessary DL interference to a neighbor cell. One could consider the transmission of SS from BS in A-IoT DL is on a need basis, aperiodic and/or event-triggered.

Observation 8: Constant transmission of SS by the BS in A-IoT DL would be power and resource wasteful (environmentally unfriendly) and may create unnecessary DL interference to others, when there is no A-IoT DL transmission. Furthermore, cell search/selection/re-selection, mobility and handover are not needed in A-IoT communication in Rel-18.

Proposal 6: The transmission of SS by the BS in A-IoT DL direction could be on a need basis, aperiodic and/or event-triggered, since the A-IoT DL traffic could be quite infrequent.

- A-IoT UL direction

As mentioned earlier, for Type I and Type II devices that perform backscattering transmission reflecting from a carrier wave provided by a carrier wave node (CWN), then it is unnecessary for these devices to perform a frequency synchronization procedure / function for the A-IoT UL communication, as long as the carrier wave is transmitted in the A-IoT UL carrier/channel where a device is scheduled to transmit. However, the UL spectrum allocated for A-IoT communication could contain multiple UL channels (e.g., each having a BW of 180kHz). When a device is scheduled to transmit A-IoT UL, it is expected that only one of such channels will be used. Then the question is, how would the CWN know which A-IoT UL carrier or channel to provide a carrier wave. If the CWN transmits carrier waves in all A-IoT UL channels, then it is very power inefficient when there is no scheduled device transmission in some of the channels.

Furthermore, the same power and interference problems exist (as in A-IoT DL SS transmission) if carrier wave(s) is constantly transmitted even when there is no scheduled A-IoT UL transmission to the devices. Therefore, a mechanism should be studied for the CWN to determine the starting time and the time duration to transmit a carrier wave.

Observation 9: For A-IoT devices that perform backscattering transmissions based on a provided carrier wave in Topology 1, it is not necessary for the devices to perform frequency synchronization to a UL carrier for A-IoT UL transmission.

Proposal 7: When a carrier wave is provided by a carrier wave node (CWN) for A-IoT UL transmission, RAN1 should study the following aspects.

- FFS how to control the A-IoT UL carrier or channel(s) in which the carrier wave should be transmitted

- FFS the timing and duration in which the carrier wave should be transmitted

3.2 Time and frequency synchronization in Topology 2

In Topology 2, the DL and UL time and frequency synchronization between the BS and intermediate UE would simply following the existing NR Uu mechanism. In our view, there is nothing that needs to be optimized in this Uu link for A-IoT communication in Topology 2.

As for the A-IoT DL and A-IoT UL communications between an intermediate UE and A-IoT devices, as mentioned earlier, both are expected to operate in the UL frequency spectrum of a FDD band. So, they are both communicating on UL carrier(s). But whether the same UL carrier or different UL carriers are used for the A-IoT DL and A-IoT UL communications, this is one aspect that should be studied and determined first in RAN1, as this will have impacts to the time and frequency synchronization design.

Proposal 8: In Topology 2, when both A-IoT DL and A-IoT UL communications between an intermediate UE and A-IoT devices operate in the UL frequency spectrum of a FDD band, RAN1 should study whether the intermediate UE and A-IoT device transmit using a same UL carrier or different UL carriers.

- A-IoT DL direction

For the A-IoT DL communication from the intermediate UE to A-IoT devices, although it is expected to operate on a UL frequency carrier of a FDD band, the mechanism by which A-IoT devices to achieve time and frequency synchronization to the intermediate UE should be aligned with A-IoT DL communication in Topology 1 to minimize standards/specification efforts and to have a harmonized and non-topology dependent design for A-IoT devices. In our view, this design principle for time and frequency synchronization should be achievable, as the same SS burst from the BS could be also transmitted by the intermediate node UE on a need basis, aperiodic and/or event-triggered (e.g., by the BS).

Proposal 9: In Topology 2, for the A-IoT DL communication from the intermediate UE to A-IoT devices, the mechanism by which A-IoT devices to achieve time and frequency synchronization to the intermediate UE should align with A-IoT DL communication in Topology 1 to minimize standardization / specification efforts and to have a harmonized and non-topology dependent design for A-IoT devices.

- A-IoT UL direction

For the A-IoT DL communication from A-IoT devices to the intermediate UE, as mentioned earlier, the design / mechanism to achieve time and frequency synchronization could depend on whether they are operating on a same UL frequency carrier or different UL carriers.

For example, when a same UL carrier is assumed, the time and frequency synchronization achieved in the A-IoT DL direction could be used for the A-IoT UL transmission, at least for Type II active devices to generate a signal carrier for the transmission. For Type I and Type II backscattering passive devices, a carrier wave should be provided. If the carrier wave is provided by the intermediate UE on the same UL carrier where it should also receive the backscattering transmission from the devices, there would be a full duplex issue for the intermediate UE (to transmit and receive simultaneously). In this case, an external CWN is needed to provide the carrier wave to the devices and thus the intermediate UE would be able to receive the backscattered transmission.

Proposal 10: In Topology 2, when a same UL frequency carrier is configured and used for both A-IoT DL and A-IoT UL communications between the intermediate UE and A-IoT devices, the time and frequency synchronization achieved in the A-IoT DL can be used by Type II active devices for A-IoT UL transmission.

- FFS any solution needed for a timing drift issue caused by accumulation of SFO error over time when the time gap between A-IoT DL and UL communication is larger than a certain threshold

Proposal 11: In Topology 2, when a same UL frequency carrier is configured and used for both A-IoT DL and A-IoT UL communications between the intermediate UE and A-IoT devices and a carrier wave is needed for Type I and Type II passive devices for backscattering transmission, the carrier wave should be provided by an external CWN to avoid a full duplex issue for the intermediate UE.

When different/separate UL frequency carriers are assumed for A-IoT DL and A-IoT UL communications, the time and frequency synchronization achieved by the devices in the A-IoT DL should not be directly reused for A-IoT UL due to the frequency conversion error. As such, a different mechanism is needed for Type II active devices, e.g., the intermediate UE transmits a SS burst on the UL frequency carrier where a A-IoT UL transmission from the devices is scheduled.

Proposal 12: In Topology 2, when different/separate UL frequency carriers are configured and used for A-IoT DL and A-IoT UL communications between the intermediate UE and A-IoT devices, a new mechanism should be studied for Type II active devices to acquire time and frequency synchronization to the intermediate UE.

For Type I and Type II backscattering passive devices, once again a carrier wave should be provided for the backscattering transmission. This time, the carrier wave could be transmitted directly from the intermediate UE on the UL frequency carrier of A-IoT DL communication. If the UL frequency separation between the A-IoT DL carrier and A-IoT UL carrier is not large, it is possible for A-IoT passive devices to perform a frequency shift of the carrier wave from the A-IoT DL carrier to the A-IoT UL carrier for the backscattering transmission. As such, no external CWN is necessary.

Proposal 13: In Topology 2, when different/separate UL frequency carriers are configured and used for A-IoT DL and A-IoT UL communications between the intermediate UE and A-IoT devices, a carrier wave can be provided by the intermediate UE on the UL frequency carrier of A-IoT DL communication, without the need of an external CWN.

- A-IoT passive device performs a frequency shift of the provided carrier wave from the A-IoT DL carrier to the A-IoT UL carrier for the backscattering transmission if the frequency separation is not large

4. Random access aspects

4.1 Purposes of RA and its potential operations in A-IoT communication

In NR Uu, random access procedures are used for various purposes, such as UE initiating or triggered to initiate a RRC connection after paging, re-acquiring UL synchronization, etc. But some of these purposes may not be needed in A-IoT communication, since no RRC connection is needed anymore and synchronization may be only required at the time of communication (before A-IoT DL data reception or A-IoT UL transmission).

Different to the existing/normal NR Uu operation, in A-IoT communication there could be hundreds or more of A-IoT devices/tags triggered to transmit within the communication area/range of a BS or intermediate UE. The purpose(s) of random access in A-IoT is no longer about requesting a RRC connection and to acquire UL synchronization as such, but rather to differentiate, possibly to assign IDs, to resolve contentions and to avoid transmission collisions among the devices using a same time, frequency and/or code resource.

Observation 10: Since no RRC connection and re-acquiring UL synchronization is needed in A-IoT, random access procedures in the PHY layer should serve somewhat different purposes to the existing Uu operation, such as to differentiate, possibly to assign IDs and to resolve/avoid transmission collisions among the devices using a same time, frequency and/or code resource.

In many ways, the design and operation of random access procedures involves cross layers between L1 and MAC layer. And some may need to be decided by RAN2 before any PHY layer design can begin (e.g., whether or not one or more ID(s) needs to be assigned and if they can / should be used for scheduling and avoiding transmission collisions). As a first step for this random access study in RAN1, however, we could begin top-level discussions and design approaches to resolve contentions and avoid collisions among A-IoT devices using a same time, frequency and code domain resources.

Observation 11: As a first step for this random access study, RAN1 could begin top-level discussions and design approaches to resolve contentions and avoid collisions among A-IoT devices using a same time, frequency and code domain resources.

In terms of contention resolution and transmission collision avoidance, it can be highly related to how the scheduling works in A-IoT UL transmissions. In Topology 1, A-IoT UL transmissions should be scheduled or configured by the BS/gNB. But the BS/gNB may not always know the exact number of devices are within its communication range/coverage (since there is no RRC states), this makes the A-IoT UL scheduling harder to avoid contentions/collisions. One potential approach is the ALOHA random access that was developed in the 1970’s and used by Wi-Fi in the past for coordinating the access of large numbers of devices/transmitters in a shared communication channel, where each device accesses a communication channel at random time to transmit. The mechanism relies on repetitions to handle collisions and improve reliability. If a transmission is lost, it is left to the receiver to deal with the lost data. In some ways, this mechanism is not a perfect solution and may require enhancement/modification for the A-IoT communication.

Proposal 14: For the problem of an unknown and potential large number of devices trying to access a A-IoT UL channel/carrier for transmission, the ALOHA random access mechanism could be used as a starting point to develop a suitable solution for A-IoT UL communication.

5. Scheduling and timing aspects

5.1 Dynamic scheduling and configured scheduling

In legacy NR system, both dynamic scheduling and configured scheduling are supported. Dynamic scheduling is used for one shot scheduling, which is based on DCI. This mechanism can also be supported for A-IoT system. DCI can be used for DL or UL scheduling. For topology 1, the DCI is transmitted directly from gNB to A-IoT device. For topology 2, the scheduling information is transmitted from gNB to intermediate node, and transferred from intermediate node to A-IoT device.

Proposal 15: DCI based dynamic scheduling is supported for A-IoT system.

- For topology 1, the DCI is transmitted from gNB to A-IoT device.

- For topology 2, the DCI transmitted from gNB is transferred by intermediate node to A-IoT device.

For dynamic scheduling, both common and UE-specific scheduling can be supported. For example, for rUC1 (indoor inventory), gNB can enable all A-IoT devices in the inventory to report its identification by sending a common DCI or query. For rUC4 (indoor command), gNB can send a command to an A-IoT device by sending a UE-specific DCI.

Proposal 16: Both common and UE-specific DCI can be supported for A-IoT system.

Configured grant or semi-persistent scheduling which is used to configure periodic transmission resources are supported and more suitable for periodic transmission. This kind of scheduling is benefit for scheduling signaling overhead reduction. If periodic transmission is supported for A-IoT system, configured scheduling can be supported. according to the SID, two kinds of use cases, rUC1 (indoor inventory) and rUC4 (indoor command), are supported. For these two kinds of use cases, it is less motivated to introduce periodic transmission.

Observation 12: It is less motivated to introduce periodic transmission for A-IoT system.

Proposal 17: Configured grant or semi-persistent scheduling are down-prioritized in R19.

5.2 Search space configuration

To support dynamic scheduling, search space can be configured to A-IoT device to assist DCI monitoring. While CORESET, which is used to configure frequency resource of control channel may not be needed.

Proposal 18: For A-IoT system, search space for DCI monitoring can be supported. While CORESET may not be needed.

For both types of A-IoT devices, power can be harvest from environment and stored. While only limit power can be stored. Both reception and transmission will consume power. If the stored power is lower than a threshold, A-IoT device cannot perform reception or transmission until it has harvested enough power. Considering that, the search space configuration should take the time for power harvesting into account. If the DCI is transmitted in case the stored power of A-IoT device is lower than the threshold, A-IoT device cannot perform DCI monitoring, which will waste the DCI occasion. Furthermore, the DCI scheduling DL reception or UL transmission should be also located when the power of A-IoT device is higher than a threshold. Otherwise, A-IoT device cannot perform DL reception or UL transmission.

Proposal 19: Considering power harvesting of A-IoT device,

- The DCI should be transmitted when the power of A-IoT device is higher than a threshold.

- The DCI scheduled DL reception or UL transmission should be located when the power of A-IoT device is higher than a threshold

6. Conclusion

Device types (I and II)

Observation 1: For Type I device (lower peak power consumption in SID), the current value in the SID of ~1 µW seems to be on the low end, and it may not be achievable. In our view, a reasonable value for device Type I should be ~4 µW.

Observation 2: For Type II device (higher peak power consumption in SID), the exact value / range is not yet defined. In our view, considering the DL and UL power amplification and signal waveform generation for UL transmissions, a reasonable value for device Type II should be ~500 µW.

Topology 1

Observation 3: In Topology 1, if the BS transmitting to the A-IoT device is different from the BS receiving from the A-IoT device, some coordination between the two BS is expected and should be studied and specified. Hence, the study to enable a “dual-BS” operation would be more complicated, involve multiple WGs (RAN1 and RAN3) and more specification impact than using an external carrier wave node (CWN).

Proposal 1: In Topology 1, the same BS transmitting to and receiving from the A-IoT device should be assumed during the study to minimize the work and spec impact. Instead, an external carrier wave node (CWN) should be used to provide a carrier wave for A-IoT device’s backscattering operation.

- FFS any specification impact / support is needed for transmitting a carrier wave from the CWN

Observation 4: In Topology 1, in terms of frequency spectrum usage/allocation,

- it is assumed that a DL frequency carrier of FDD band is used for A-IoT DL communication from the BS to A-IoT devices, and

- it is assumed a UL frequency carrier of FDD band is used for A-IoT UL communication from the A-IoT devices to the BS.

Topology 2

Observation 5: In Topology 2, in terms of frequency spectrum usage/allocation,

- it is assumed a normal/existing DL frequency carrier and a UL frequency carrier of a FDD band are used for the Uu DL and UL communication between the BS and the intermediate UE, respectively, and

- it is assumed that at least one UL frequency carrier of a FDD band is used for the bidirectional A-IoT DL and UL communication between the intermediate UE and the A-IoT devices.

  - FFS whether a same UL frequency carrier or two separate/different UL frequency carriers are used for A-IoT DL and A-IoT UL communications between the intermediate UE and the A-IoT devices

Observation 6: In Topology 2, if Type I devices and some Type II devices that are only capable of backscattering transmissions for A-IoT UL communication should be supported, a carrier wave should be provided.

- FFS whether the carrier wave should be provided/transmitted from a node outside the topology (e.g., CWN) or inside the topology (e.g., by the intermediate UE)

Proposal 2: In Topology 2, if the carrier wave is provided/transmitted by the intermediate node UE and the intermediate UE also need to receive backscattering transmissions from the A-IoT, to avoid this full duplex problem, an A-IoT device should be capable of performing a frequency shift of a received carrier wave to the scheduled A-IoT UL frequency carrier or channel.

Proposal 3: For simplification of discussion in RAN1, we propose to have a unified naming for the A-IoT communication interface from the perspective of an A-IoT device. That is,

- In Topology 1,

  - “A-IoT DL” and “A-IoT UL” are used for the bidirectional A-IoT communication between the BS and the A-IoT device.

- In Topology 2,

  - The existing “Uu DL” and “Uu UL” are used for the bidirectional cellular communication between the BS and the intermediate UE.

  - “A-IoT DL” and “A-IoT UL” are used for the bidirectional A-IoT communication between the intermediate UE and the A-IoT device.

Synchronization

Observation 7: Due to a low-power and low-complexity/cost receiver and frequency oscillator design in A-IoT devices with a certain SFO error that is expected to be much higher than in a normal UE, and the SFO also suffers from an accumulation effect over time. It seems the frequency and timing synchronization strategy / mechanisms for A-IoT DL reception and A-IoT UL transmission should be separately considered.

Proposal 4: For A-IoT DL and A-IoT UL communications, separate and different frequency and timing synchronization strategies / mechanisms should be considered and designed.

Proposal 5: For A-IoT communication in Topology 1 and 2, the mechanism of frequency and timing synchronization should be also differentiated, since the involved communication nodes and A-IoT DL frequency spectrum are different in these two topologies.

Synchronization (Topology 1)

Observation 8: Constant transmission of SS by the BS in A-IoT DL would be power and resource wasteful (environmentally unfriendly) and may create unnecessary DL interference to others, when there is no A-IoT DL transmission. Furthermore, cell search/selection/re-selection, mobility and handover are not needed in A-IoT communication in Rel-18.

Proposal 6: The transmission of SS by the BS in A-IoT DL direction could be on a need basis, aperiodic and/or event-triggered, since the A-IoT DL traffic could be quite infrequent.

Observation 9: For A-IoT devices that perform backscattering transmissions based on a provided carrier wave in Topology 1, it is not necessary for the devices to perform frequency synchronization to a UL carrier for A-IoT UL transmission.

Proposal 7: When a carrier wave is provided by a carrier wave node (CWN) for A-IoT UL transmission, RAN1 should study the following aspects.

- FFS how to control the A-IoT UL carrier or channel(s) in which the carrier wave should be transmitted

- FFS the timing and duration in which the carrier wave should be transmitted

Synchronization (Topology 2)

Proposal 8: In Topology 2, when both A-IoT DL and A-IoT UL communications between an intermediate UE and A-IoT devices operate in the UL frequency spectrum of a FDD band, RAN1 should study whether the intermediate UE and A-IoT device transmit using a same UL carrier or different UL carriers.

Proposal 9: In Topology 2, for the A-IoT DL communication from the intermediate UE to A-IoT devices, the mechanism by which A-IoT devices to achieve time and frequency synchronization to the intermediate UE should align with A-IoT DL communication in Topology 1 to minimize standardization / specification efforts and to have a harmonized and non-topology dependent design for A-IoT devices.

Proposal 10: In Topology 2, when a same UL frequency carrier is configured and used for both A-IoT DL and A-IoT UL communications between the intermediate UE and A-IoT devices, the time and frequency synchronization achieved in the A-IoT DL can be used by Type II active devices for A-IoT UL transmission.

- FFS any solution needed for a timing drift issue caused by accumulation of SFO error over time when the time gap between A-IoT DL and UL communication is larger than a certain threshold

Proposal 11: In Topology 2, when a same UL frequency carrier is configured and used for both A-IoT DL and A-IoT UL communications between the intermediate UE and A-IoT devices and a carrier wave is needed for Type I and Type II passive devices for backscattering transmission, the carrier wave should be provided by an external CWN to avoid a full duplex issue for the intermediate UE.

Proposal 12: In Topology 2, when different/separate UL frequency carriers are configured and used for A-IoT DL and A-IoT UL communications between the intermediate UE and A-IoT devices, a new mechanism should be studied for Type II active devices to acquire time and frequency synchronization to the intermediate UE.

Proposal 13: In Topology 2, when different/separate UL frequency carriers are configured and used for A-IoT DL and A-IoT UL communications between the intermediate UE and A-IoT devices, a carrier wave can be provided by the intermediate UE on the UL frequency carrier of A-IoT DL communication, without the need of an external CWN.

- A-IoT passive device performs a frequency shift of the provided carrier wave from the A-IoT DL carrier to the A-IoT UL carrier for the backscattering transmission if the frequency separation is not large

Random access

Observation 10: Since no RRC connection and re-acquiring UL synchronization is needed in A-IoT, random access procedures in the PHY layer should serve somewhat different purposes to the existing Uu operation, such as to differentiate, possibly to assign IDs and to resolve/avoid transmission collisions among the devices using a same time, frequency and/or code resource.

Observation 11: As a first step for this random access study, RAN1 could begin top-level discussions and design approaches to resolve contentions and avoid collisions among A-IoT devices using a same time, frequency and code domain resources.

Proposal 14: For the problem of an unknown and potential large number of devices trying to access a A-IoT UL channel/carrier for transmission, the ALOHA random access mechanism could be used as a starting point to develop a suitable solution for A-IoT UL communication.

Scheduling and timing

Proposal 15: DCI based dynamic scheduling is supported for A-IoT system.

- For topology 1, the DCI is transmitted from gNB to A-IoT device.

- For topology 2, the DCI transmitted from gNB is transferred by intermediate node to A-IoT device.

Proposal 16: Both common and UE-specific DCI can be supported for A-IoT system.

Observation 12: It is less motivated to introduce periodic transmission for A-IoT system.

Proposal 17: Configured grant or semi-persistent scheduling are down-prioritized in R19.

Proposal 18: For A-IoT system, search space for DCI monitoring can be supported. While CORESET may not be needed.

Proposal 19: Considering power harvesting of A-IoT device,

- The DCI should be transmitted when the power of A-IoT device is higher than a threshold.

- The DCI scheduled DL reception or UL transmission should be located when the power of A-IoT device is higher than a threshold

7. References

[1] RP-234058 “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#102

[2] TR 38.848 “Study on Ambient IoT (Internet of Things) in RAN”, V1.0.0 (2023-09)