3GPP TSG RAN WG1 #116		R1-2400613

Athens, Greece, February 26th – March 1st, 2024

Source:	OPPO

Title:                     Discussion on downlink and uplink channel/signal aspects for A-IoT

Agenda Item:       9.4.2.3

Document for:	Discussion and Decision

- Introduction

In RAN#102 meeting the SID on solutions for Ambient IoT was endorsed including following general scope and RAN1-led objections [1]:

General Scope

The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope:

- The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices:

- ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally.

- X  is to be decided in WGs.

- Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”.

- For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ.

NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement.

- Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848:

- Deployment scenario 1 with Topology 1

  - Basestation and coexistence characteristics: Micro-cell, co-site

- Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control

  - Basestation and coexistence characteristics: Macro-cell, co-site

  - The location of intermediate node is indoor

- FR1 licensed spectrum in FDD.

- Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s).

- Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command).

- From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case.

Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum.

- Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848.

Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation).

Study the feasibility and required functionalities for proximity determination (coordination with SA3 is required for privacy aspects).

- RAN1-led:

For the Ambient IoT DL and UL:

  - Frame structure, synchronization and timing, random access

  - Numerologies, bandwidths, and multiple access

  - Waveforms and modulations

  - Channel coding

  - Downlink channel/signal aspects

  - Uplink channel/signal aspects

  - Scheduling and timing relationships

  - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation.

For Topology 2, no difference in physical layer design from Topology 1.

In this paper we discuss the necessity and design principle of various DL/UL channels/signals for A-IoT.

- Discussion

2.1 Synchronization signal

For A-IoT, the following functionalities of synchronization signal may be needed:

- Time synchronization

Similar as the terminal in NR/LTE, A-IoT devices needs to detect the synchronization signal to get time synchronization.

- Frequency synchronization for A-IoT devices with LO

A-IoT devices also needs to use synchronization signal to acquire frequency synchronization, especially for A-IoT device which needs to generate the RF signals itself.

To facilitate the chip alignment between transmitter and receiver of A-IoT time domain, and maybe also in frequency domain, synchronization signal is needed. Besides, how the synchronization signal can be transmitted, e.g. periodic and/or aperiodic, should also be considered. The discussion on necessity and transmission type can be found in our companion contribution in [2]. To design synchronization signal in A-IoT, there are a lot of aspects and candidates that should be studied and discussed. However, by considering the characteristics of A-IoT devices and systems, simplicity and efficiency, synchronization signal with fixed length and fixed sequence should be supported, which can dramatically minimize the complexity in synchronization procedure at both transmitter and receiver sides.

To design the synchronization signal in A-IoT, the following aspects can be further studied:

- Study the basic sequence for A-IoT synchronization signal. In legacy NR, M-sequence is used as the basic sequence to generate PSS and SSS respectively using different cyclic shift. There are also a few candidates for the basic A-IoT synchronization signal sequence, such as M-sequence, Gold sequence, ZC sequence, Pseudo-Noise sequence. For example, ZC sequence is used as the synchronization signal generation in LTE, but its synchronization performance is degraded when there is frequency shift. For A-IoT, proper sequence shall be selected considering the detection complexity, the potential waveform that A-IoT device is able to support etc.

- Study the necessity of PSS and SSS for A-IoT synchronization signal. In legacy NR, M-sequence with cyclic shifts (0, 43, 86) is used to generate 3 different PSS which can be used to acquire synchronization timing. SSS is generated with 2 M-sequences to indicate 1008 different Physical Cell Identity (PCI). The detected PSS can help to decrease the detection number of SSS candidates to 336, which minimize the blind detection complexity. For A-IoT, there is no need to support mobility at least for the target use cases in Rel-19, much smaller number of Identities can be considered in order to reduce the detection complexity.

- Study the synchronization signal is cell specific or group specific. In legacy NR, PSS/SSS is used to distinguish 1008 different PCI, which can be considered as cell specific configuration. In A-IoT, whether cell specific synchronization is reused is not determined, while A-IoT system can be very simple. Furthermore, the A-IoT devices can also be divided into different groups since there would large number of tiny devices, and group specific can help to enhance the management efficiency. In some case, e.g. when the A-IoT device number is small, A-IoT specific differentiation can also be considered. Especially in Topology 2, intermediate nodes may be needed to transmit synchronization signal to A-IoT devices to align timing and/or frequency, and the same intermediate node may transmit different synchronization signals when it is under different cell. Therefore, the number of different Identities is determined based on how to divide the A-IoT in cell specific, group specific or individually.

- Study the information/content that synchronization signal can include in A-IoT. If the sequence is used only for synchronization to acquire timing and possibly frequency alignment, it is not necessary to carry any other information in synchronization signal, and pure sequence can also help to minimize the complexity. If other information should be conveyed by synchronization signal, e.g. cell/group identity, how to design the synchronization signal, e.g. how many cyclic shifts can contain different number of identities, should be studied.

Proposal 1: Synchronization signal with fixed length and fixed sequence should be supported for A-IoT. The following relative aspects can also be further studied:

- Basic sequence that can be used to generate synchronization signal.

- Single synchronization signal or multi-signal (e.g. PSS/SSS) can be used.

- Synchronization signal is cell specific, group specific or A-IoT specific.

- The information/content that synchronization signal can contain.

2.2 Physical Random-Access Channel

In current NR system PRACH is mainly used for UL synchronization and contention resolution. However, as analysed in our companion contribution [2] timing advance estimation may not be needed for the transmission from an A-IoT device to the reader as the target coverage of the A-IoT link is only up to 50m. But in A-IoT communication it is possible that a reader triggers/schedules multiple A-IoT devices through one control signalling. For example, when a batch of new A-IoT devices entering a warehouse, the reader inside may not know the IDs and the amount of them and trigger them to report their respective IDs group by group. In this case PRACH may be needed for contention resolution as multiple devices may report its ID at the same time. The structure of the PRACH can be further studied based on the progress on other aspects, such as waveform, frame structure and timing, etc.

Proposal 2: PRACH should be supported for contention resolution in A-IoT communication, FFS the concrete structure of the channel.

2.3 Control Channel

In existing NR system both DL control channel (PDCCH) and UL control channel (PUCCH) are supported. PDCCH can be used to indicate time/frequency resource, MCS, HARQ process number, etc., of DL/UL/SL transmission (i.e., DL/UL/SL DCI), or to serve other purposes (i.e., the purposes of served by 2-x DCI formats). PUCCH is basically used to convey UCI, such as HARQ-ACK information, SR, CSI, etc. As captured in the SID, in Rel-19 ambient IoT targets DO-DTT and DT traffic types with focus on rUC1 (indoor inventory) and rUC4 (indoor command). From our perspective DL control channel (A-PDCCH) should be introduced for A-IoT communication at least for following 2 purposes:

- To indicate the transmission from gNB or intermediate node to A-IoT device (DL transmission). Although depending on the discussion in 9.4.2.1 on general aspects of physical layer design and 9.4.2.2 on scheduling, some transmission parameters of PDSCH, such as waveform, coding scheme, HARQ process number, etc. may be fixed. To support all the use cases in rUC4 different information need to be transmitted, which would lead to different PDSCH length. As the reliability requirement of the use cases may also be different, coding rate of the PDSCH may also be adapted to the use case. From an A-IoT device perspective, it needs to know whether a PDSCH should be received, how many bits are included, and what is the coding rate, etc.

- To schedule the transmission from A-IoT device to gNB or intermediate node (UL transmission). This functionality is motivated by inventory use cases, where an A-IoT device should respond to a query transmitted from gNB or intermediate node. For this purpose, at least a UL resource should be allocated to the A-IoT device by the A-PDCCH.

Proposal 3: A-PDCCH should be supported for indicating DL transmission or scheduling UL transmission in A-IoT communication.

In NR system PDCCH is transmitted with fixed modulation and channel coding scheme, but the size of DCI conveyed in a PDDCH is dependent on DCI format. To reduce detection complexity a UE is configured with CORESET and Search Space for PDCCH monitoring. A UE needs to monitor time/frequency resources indicated by CORESET and Search space to detect all possible PDCCH formats blindly. This operation leads to considerable amount of complexity and power consumption. But the complexity and energy storage of an A-IoT device is extremely low, therefore the complexity and power consumption for detection of the A-IoT control channel should be minimized. In the sense the following principles should be considered A-PDCCH design:

- Fixed modulation and channel coding scheme should be used.

- To avoid blind detection, at a given time an A-IoT device only detect one DCI size.

- The number of DCI formats that an A-IoT device needs to support should be minimized, e.g., only DL and UL DCI format are introduced.

- FAR should be minimized to avoid unnecessary data channel reception or transmission in order to reduce power consumption.

- For power saving also, at least for high capability A-IoT device, search space or something similar can be considered for A-IoT control channel detection.

- A-IoT device considered in this release may not support high-Q filer and OFDM waveform, and the timing accuracy is also very low. The configuration of CORESET, which is mainly used to reduce PDCCH detection in time/frequency domain, is not needed for A-IoT control channel.

Proposal 5: Complexity and power consumption for A-IoT control channel detection should be minimized, the following design principles should be considered: fixed MCS, no blind detection, minimized DCI formats, minimized FAR.

If UCI is also introduced in A-IoT communication, such as to convey physical layer feedback or reporting as discussed below, PUCCH should also be considered for A-IoT (A-PUCCH).

Proposal 4: If UCI is introduced in A-IoT communication A-PUCCH should also be considered.

2.4 Data Channel

As captured in the SID, in this release the focus of this SI is on rUC1 (indoor inventory) and rUC4 (indoor command) as reproduced in Table 1 below. For indoor inventory an A-IoT device needs to report its ID in response to a query from the reader through UL data channel. The length of the ID to be reported may be dependent on the use case, but for a given A-IoT device the length of its ID is expected to be fixed.

Proposal 6: UL data channel should be introduced for support of indoor inventory use cases; from system perspective, different UL channel length should be considered; from A-IoT device perspective, only one UL channel length is supported.

For indoor command the reader needs to send various command to A-IoT devices through DL data channel for different purposes, hence the number of bits for the DL channel conveying different commands may be different. As in this release HARQ retransmission is not supported, but the reliability of transmissions may be different, different channel coding rate may be needed to achieve different reliabilities. But for a given A-IoT device whether it needs to support different DL data channel length and/or different DL channel coding rate should be FFS.

Proposal 7: DL data channel should be introduced for support of indoor command use cases; from system perspective, different DL channel length and coding rate should be considered; FFS whether different channel length and/or coding rate should be supported from A-IoT device perspective.

Table 1 rUC1 and rUC4 defined in 38.848.

| rUC1: Indoor inventory | 5.1 Automated warehousing 5.2 Medical instruments inventory management and positioning 5.4 Non-Public Network for logistics 5.5 Automobile manufacturing 5.7 Airport terminal / shipping port 5.15 Smart laundry 5.16 Automated supply chain distribution 5.18 Fresh food supply chain 5.27 End-to-end logistics 6.1 Flower auction 6.3 Electronic shelf label |
| --- | --- |
| rUC4: Indoor command | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.29 Device Permanent Deactivation 6.3 Electronic shelf label |

As an A-IoT device needs to decode both DL control channel and DL data channel, another issue is whether the transmission scheme, including waveform, modulation scheme and coding scheme, etc., of them should be the same. Obviously to use same transmission scheme for the both can simplify the A-IoT device implementation. But an A-IoT device may need to perform control channel decoding all the time, only need to decode data channel when a control channel towards to it is decoded, therefore, it is expected that the power consumption for decoding a control channel is much lower than that of decoding a data channel. For this purpose, different transmission schemes may be needed for control channel and data channel.

Proposal 8: Further study whether transmission schemes for DL control channel and DL data channel are always the same or can be different.

Phase based modulation or demodulation may not be possible for an A-IoT device due to the restriction on power consumption and the lack of PLL. If the case DMRS may not be needed for DL and UL data channels.

Proposal 9: Further study whether DMRS is needed for DL and UL data channel of A-IoT communication or not.

2.5 Channel for feedback

Indoor command use case is one of the focuses of this SI. According to [3] feedback is needed for some important indoor command use cases, e.g., 5.11 Online modification of medical instruments status and 5.26 Elderly Health Care. The feedback is not used for triggering of HARQ re-transmission but to inform the reader whether the command has been executed or not. In topology 2 resources for the command transmission from intermediate node to an A-IoT device is allocated by gNB, hence the feedback should be reported to gNB as well. But whether the feedback should be a PHY layer signalling or higher layer signalling should be further studied. If PHY layer signalling is used a new A-PUCCH to convey the feedback information should be introduced. While if higher layer signalling is used UL data channel can be reused to convey the feedback information.

Proposal 10: Feedback to indicate whether an A-IoT device has successfully execute a command or not should be supported; intermediate node should transfer the feedback to gNB in topology 2; FFS whether the feedback is conveyed by PHY layer signalling or higher layer signalling.

2.6 New Channel for Topology 2

In this SI topology 2 is supported to extend the connection between gNB and A-IoT device as illustrated in Figure 1 below, as captured in the SID the intermediate node is a UE type device under the control of NW. In this topology the time/frequency resource used by the intermediate node to transmit A-IoT control/data/signal and/or carrier wave should be allocated by the BS. In addition, the transmit power of the intermediate node toward A-IoT device should also be under the control of the BS. A new DCI format may be needed for the purposes above. As discussed above, if feedback between A-IoT device and the intermediate node is supported, the feedback should be transferred to the BS. If the feedback if conveyed by PHY layer signalling on Uu link some change to existing PUCCH transmission, including PUCCH resource/format determination, etc., is needed.

Figure 1 Topology 2

Proposal 11: A new DCI format should be introduced to schedule the intermediate node to transmit control/data/signal and/or carrier wave to A-IoT devices. FFS whether some change to existing PUCCH is needed to support transferring of the feedback from A-IoT device to intermediate node.

2.7 Proximity determination

One more objective of this SI is to study the feasibility and required functionalities for proximity determination. In current NR system proximity determination is dependent on positioning or ranging mechanism(s), the positioning or ranging mechanisms are based on GNSS signal or positioning signal transmitted/received by the devices, which are all beyond the capability of an A-IoT device.

- The positioning or ranging mechanism(s) used in current NR system for proximity determination cannot be applied to A-IoT.

According to the SID RAN3 is tasked to study the positioning of an A-IoT device. Whether additional work on proximity determination in RAN1 is needed or not is dependent on the outcome of RAN3 study. If needed, RAN1’s work should also be based on or at least take into account positioning mechanism(s) identified by RAN3.

Proposal 12: Proximity determination in RAN1 should be pending until sufficient progress on positioning is achieved in RAN3.

- Conclusion

In this paper we discussed the necessities and design principles of various DL/UL channels/signals for A-IoT, we have following observation and proposals:

- The positioning or ranging mechanism(s) used in current NR system for proximity determination cannot be applied to A-IoT.

Proposal 1: Synchronization signal with fixed length and fixed sequence should be supported for A-IoT. The following relative aspects can also be further studied:

- Basic sequence that can be used to generate synchronization signal.

- Single synchronization signal or multi-signal (e.g. PSS/SSS) can be used.

- Synchronization signal is cell specific, group specific or A-IoT specific.

- The information/content that synchronization signal can contain.

Proposal 2: PRACH should be supported for contention resolution in A-IoT communication, FFS the concrete structure of the channel.

Proposal 3: A-PDCCH should be supported for indicating DL transmission or scheduling UL transmission in A-IoT communication.

Proposal 5: Complexity and power consumption for A-IoT control channel detection should be minimized, the following design principles should be considered: fixed MCS, no blind detection, minimized DCI formats, minimized FAR.

Proposal 4: If UCI is introduced in A-IoT communication A-PUCCH should also be considered.

Proposal 6: UL data channel should be introduced for support of indoor inventory use cases; from system perspective, different UL channel length should be considered; from A-IoT device perspective, only one UL channel length is supported.

Proposal 7: DL data channel should be introduced for support of indoor command use cases; from system perspective, different DL channel length and coding rate should be considered; FFS whether different channel length and/or coding rate should be supported from A-IoT device perspective.

Proposal 8: Further study whether transmission schemes for DL control channel and DL data channel are always the same or can be different.

Proposal 9: Further study whether DMRS is needed for DL and UL data channel of A-IoT communication or not.

Proposal 10: Feedback to indicate whether an A-IoT device has successfully execute a command or not should be supported; intermediate node should transfer the feedback to gNB in topology 2; FFS whether the feedback is conveyed by PHY layer signalling or higher layer signalling.

Proposal 11: A new DCI format should be introduced to schedule the intermediate node to transmit control/data/signal and/or carrier wave to A-IoT devices. FFS whether some change to existing PUCCH is needed to support transferring of the feedback from A-IoT device to intermediate node.

Proposal 12: Proximity determination in RAN1 should be pending until sufficient progress on positioning is achieved in RAN3.

- References

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR.

- R1-2400612, Initial considerations on frame structure and timing aspects of A-IoT communication, OPPO

- 3GPP TR 22.840 V2.2.0 (2023-12) Technical Report 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Ambient power-enabled Internet of Things (Release 19)