3GPP TSG RAN WG1 #116	                                                            	 R1-2400615

Athens, Greece, February 26th – March 1st, 2024

Source:	OPPO

Title:	Enhancements to enable Tx/Rx for XR during RRM measurements

Agenda Item:	9.10.1

Document for:	Discussion and Decision

# Introduction

A work item of R19 XR for NR phase 3 was approved in RAN#102 with the following objectives [1]:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

- Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

In this contribution, we discuss the enhancements to enable transmission/reception for XR during RRM measurements.

# Discussion

There can be two different ways to “enable transmission/reception in gaps/restrictions”, where from RAN1 perspective a gap refers to a measurement gap (MG) occasion:

- The MG occasion is still valid and active, while only some specific transmissions/receptions (e.g. PDSCH/PUSCH) are individually enabled in the MG occasion. UE does not perform measurement during the time of the specific transmission/reception but may do so during the rest of time within MG occasion.

- The whole MG occasion is deactivated, and all transmission/reception functions described in clauses 5.14 of TS 38.321 are enabled.

The first one requires UE to perform both data transmission/reception and measurements in a MG occasion, which can lead to big impacts to both specification and UE implementation, e.g., switching/interruption time that is required between data transmission/reception and measurements could be time-floating and depending on dynamic conditions such as scheduling.

In contrast, the second way is simple and clean for both specifications and UE implementation. Therefore, we prefer to consider enabling of transmission/reception in a MG occasion as deactivation of the whole MG occasion.

Proposal 1: The enabling of transmission/reception in a MG occasion is realized via deactivation of the whole MG occasion.

Two use cases as following should be considered for support of enabling Tx/Rx in RRM measurement:

Use case 1: High-priority and low-latency traffic (e.g. XR) transmitted in MG occasions

If the high priority XR traffic is delivered by CG-PUSCH/SPS-PDSCH, deactivation of MG occasions can be considered as semi-static. To be more specific, deactivation of MG occasions can be configured per CG/SPS configuration, where the deactivation can be configured to apply to all MG configurations or to some specific MG configurations. If one valid CG-PUSCH/SPS-PDSCH occasion (as described in clause 6.1/5.1 of TS 38.214) corresponding to a CG/SPS configuration overlaps in time with a MG occasion that is configured in the CG/SPS configuration to allow deactivation, the MG occasion is deactivated and all the Tx/Rx in the MG occasion are enabled, as shown in Figure 1. The advantages of the semi-static scheme include compatibility with XR quasi-periodic characteristic, no additional signaling overhead, fewer spec modifications and simplified implementations in gNB and UE.

Figure 1 Semi-static deactivation of a MG

Proposal 2: Semi-static deactivation of MG occasions due to overlapping with CG/SPS can be configured per CG/SPS configuration.

- If a valid CG-PUSCH/SPS-PDSCH occasion corresponding to a CG/SPS configuration overlaps in time  with a MG occasion that is configured in the CG/SPS configuration to allow deactivation, the MG occasion is deactivated;

- Each MG deactivation configured in a CG/SPS configuration can be configured to apply to either all MG configurations or some specific MG configurations.

Alternatively, if the high-priority XR traffic is transmitted by dynamically scheduled PUSCH/PDSCH, dynamic MG deactivation should be supported and the follow schemes can be considered:

- Option 1: If a UE detects a (legacy) DCI format scheduling a PDSCH/PUSCH/PUCCH in a MG occasion, the MG occasion is deactivated and all Tx/Rx are enabled in the MG occasion, as shown in Figure 2.

Figure 2 MG deactivation triggered by a legacy scheduling DCI

- Option 2: A legacy DCI format with a new specific RNTI before a MG occasion can indicate the deactivation of the MG occasion.

- Option 3: A new Rel-19 DCI format before a MG occasion can indicate the deactivation of the MG occasion, e.g., 1-bit is added into DCI format 0_1/0_2/0_3/1_1/1_2/1_3.

Option 1 has the least specification impact, but gNB needs to perform scheduling for the MG occasion in advance. Given XR service traffic is quasi-periodic and predictable, it is not challenge for gNB to send scheduling DCI to schedule a Tx/Rx before a MG occasion. For both Option 2 and 3, PDSCH/PUSCH/PUCCH can still be scheduled before or within the MG occasion. But Option 2 may lead to excessive consumption of RNTI space, and Option 3 leads to increased DCI overhead and more spec impacts. Therefore, we slightly prefer to focus on Option 1 for further study.

Proposal 3: Dynamic deactivation of MG occasion can be considered, with the following focus:

- A DCI-based scheduling of a PDSCH/PUSCH/PUCCH in a MG occasion deactivates the MG occasion.

In order for UE to have sufficient processing time to deactivate a MG occasion, timeline of dynamic MG deactivation needs to be defined, i.e. the gap between the ending of DCI indicating MG deactivation and the starting of the MG occasion should not be less than a given minimum, as shown as “X symbols” in Figure 3.

Figure 3 Timeline for dynamic deactivation of MG

Proposal 4: If dynamic deactivation of MG occasion is supported, a corresponding timeline requirement should be adopted.

Use case 2: Tx/Rx in MG occasion which satisfies RRM relaxation condition

In R17, UE may not perform the RRM measurement in a MG occasion which satisfies RRM relaxation condition for power saving purpose. However, gNB does not know the MG occasion is skipped by the UE, and therefore does not schedule any Tx/Rx in the MG. If Tx/Rx is enabled in the MG occasions in which UE does not perform the RRM measurement, the system capacity improvement can be expected. To support Tx/Rx in these MGs, UE has to send information to gNB to help the gNB to identify the availability of these MGs for Tx/Rx. Since the RRM relaxation condition is defined in higher layer, we would prefer to leave it to RAN 2 to decide whether UE can inform gNB that a MG occasion satisfies RRM relaxation condition.

Proposal 5: It is up to RAN2 to decide whether to include RRM relaxation condition in Rel-19 XR WI, including whether/how UE should inform gNB that a MG occasion satisfies RRM relaxation condition on UE side.

# Conclusions

In this contribution, we discuss the enhancements to enable transmission/reception for XR during RRM measurements with following proposals:

Proposal 1: The enabling of transmission/reception in a MG occasion is realized via deactivation of the whole MG occasion.

Proposal 2: Semi-static deactivation of MG occasions due to overlapping with CG/SPS can be configured per CG/SPS configuration.

- If a valid CG-PUSCH/SPS-PDSCH occasion corresponding to a CG/SPS configuration overlaps in time  with a MG occasion that is configured in the CG/SPS configuration to allow deactivation, the MG occasion is deactivated;

- Each MG deactivation configured in a CG/SPS configuration can be configured to apply to either all MG configurations or some specific MG configurations.

Proposal 3: Dynamic deactivation of MG occasion can be considered, with the following focus:

- A DCI-based scheduling of a PDSCH/PUSCH/PUCCH in a MG occasion deactivates the MG occasion.

Proposal 4: If dynamic deactivation of MG occasion is supported, a corresponding timeline requirement should be adopted.

Proposal 5: It is up to RAN2 to decide whether to include RRM relaxation condition in Rel-19 XR WI, including whether/how UE should inform gNB that a MG occasion satisfies RRM relaxation condition on UE side.

# References

- RP-234080, “New WID: XR (eXtended Reality) for NR Phase 3”, MediaTek (Moderator)