3GPP TSG RAN WG1 #116	                                  R1-2400616

Athens, Greece, February 26th – March 1st, 2024

Source:	OPPO

Title:          Discussion on ISAC deployment scenarios

Agenda Item:   9.7.1

Document for:  Discussion and Decision

# Description

It was agreed in RAN #102 meeting for RAN1 to explore a study on channel modelling for ISAC for NR [1]. The study is requested to aim at a common modelling framework capable of detecting and/or tracking the following example objects and meanwhile distinguishing them from unintended objects:

- UAVs

- Humans indoors and outdoors

- Automotive vehicles (at least outdoors)

- Automated guided vehicles (e.g. in indoor factories)

- Objects creating hazards on roads/railways, with a minimum size dependent on frequency

The study also needs to consider six sensing modes, including TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic and UE monostatic.

This contribution focuses on deployment scenarios and use cases in study of ISAC channel modeling.

# Discussion

There are five use case examples in the scope of ISAC. The sensing targets in these cases include human, vehicle and UAV. For human detection case, the sensing target (human) could stay or move indoors or outdoors. Scenarios such as indoor and outdoor (UMi, UMa, RMa) can be used for human detection cases. For UAV detection and trace, the sensing target (UAV) flies or hovers in the air. Outdoor scenarios, such as UMi, UMa and RMa, can be considered for UAV related cases. Vehicle could run on the open road or in a large factory building. Table-1 shows the deployment scenarios used for each use case.

Table-1 Deployment scenario for each use case

| Use case | Related deployment scenario | Related modeling study in the past |
| --- | --- | --- |
| UAVs | Outdoor: UMa-AV, UMi-AV, RMa-AV | TR36.777, TR38.901 |
| Humans indoors and outdoors | Indoor: Indoor office Outdoor: UMa, UMi, RMa | TR38.901 |
| Automotive vehicles (at least outdoors) | Outdoor: UMa,UMi,RMa | TR38.901, TR37.885 |
| Automated guided vehicles (e.g. in indoor factories) | Indoor: Indoor factory | TR38.901 |
| Objects creating hazards on roads/railways, with a minimum size dependent on frequency | Outdoor: UMa,UMi,RMa | TR38.901, TR38.802 |

Observation 1: A fairly complete study over five example use cases in SID leads to a need of considering all the indoor scenarios (indoor office, INF) and outdoor scenarios (UMi,UMa,RMa).

Although the SID in [1] mentions only mono-static mode and bi-static mode for the sensing, our understanding is that these two terminologies refer to the sensing operation taken by a pair of Tx and Rx, which still allows so-called “multi-static mode”, where the sensing geometry is constructed over one Tx-to-target link and multiple target-to-Rx links around the same target. It is important to leave RAN1 with the clear decision whether the “multi-static” sensing mode is supported by the channel modeling methodology, because:

- If Tx-target-Rx link is to be modeled in a (semi-)deterministic way, multiple such links should share the same Tx-target channel property.

- If Tx-target-Rx link is to be modeled in a statistic way, multiple such links should experience a certain level of correlation due to sharing the same pair of Tx and target.

Proposal 1: Besides mono-static sensing and bi-static sensing, multi-static sensing mode is also supported in Rel-19 ISAC channel modeling.

- Support of multi-static mode is mostly implemented in the support of bi-static mode.

## Scenario description

To keep the scenario description simple and general, we name the sensing Tx/Rx as “sensing entity” or SE, and name the sensing target as ST.

### Indoor case

Indoor scenario is considered with human to be the sensing target in indoor office, and with automated guided vehicle to be the sensing target in indoor factory. The sensing entity (i.e., Tx/Rx) can be either TRP or UE. The details assumptions for indoor scenarios are given in Table 2-1.

Table 2-1 Assumptions for Indoor use cases

| Use cases | Human detection | Automated guided vehicles |
| --- | --- | --- |
| Scenario layout | Indoor office (TR38.901): 120m*50m*3m | Indoor factory (TR38.901): 20-160000 m2 |
| Sensing entity (SE) locations | Vertical height: 1.5m (UE), 3m (TRP) Horizontal: - Uniformly random, if height=1.5m - fixed location with ISD=20m, if height=3m | Vertical height: 1.5m (UE), [5~10]m (TRP) Horizontal: - Uniformly random, if height=1.5m - fixed location with ISD=[20m], if height=[5~10]m |
| Sensing entity (SE) mobility (horizontal) | Opt-1: 0 km/h Opt-2: 3km/h (only if height=1.5m, optional) | Opt-1: 0 km/h Opt-2: 3km/h (only if height=1.5m, optional) |
| Sensing target (ST) location | Vertical height: 1.5m Horizontal: arbitrary | Vertical height: 1.5m Horizontal: arbitrary |
| Sensing target (ST) mobility | 0, 3km/h | 3km/h |
| Minimum SE-ST distance (3D) | 1m | 1m |
| SE-ST LOS/NLOS | LOS (prioritized), NLOS | LOS (prioritized), NLOS |

### Outdoor case

The human, UAV and vehicle could be the sensing targets in outdoor scenarios. The details assumptions for outdoor scenarios are given in Table 3-1.

Table 3-1 Assumptions for Outdoor scenarios

| Use cases | Human/UAV detection | Vehicle/objects on Hwy/railway |
| --- | --- | --- |
| Scenario layout | Hexagonal grid (TR38.901) ISD = 200m(UMi), 500m(UMa), 1732m(RMa) | Straight-line (TR38.802) |
| Sensing entity (SE) locations | Vertical height: 1.5m(UE, outdoor only), 10m(TRP in UMi), 25m(TRP in UMa), 35m(TRP in RMa) Horizontal: - Uniformly random per hexagonal cell, if height=1.5m - fixed location at hexagonal cell centers, if height≥10m | Vertical height: 1.5m(UE), 25m(TRP in UMa), 35m(TRP in RMa) Horizontal: - Uniformly random over straight-line, if height=1.5m - fixed location as figure shows, if height≥10m |
| Sensing entity (SE) mobility (horizontal) | Opt-1: 0 km/h Opt-2: 3km/h (only if height=1.5m, optional) | Opt-1: 0 km/h Opt-2: 3/30/120 km/h (only if height=1.5m, optional) |
| Sensing target (ST) location | Vertical height: uniformly random in [1.5m, H], where H=1.5 for human detection use case and H=300m for UAV case. Horizontal: arbitrary in hexagonal cell. | Vertical height: 1.5m Horizontal: arbitrary over straight-line |
| Sensing target (ST) mobility | 0, 3km/h, 160km/h (UAV only) | 0, 30km/h, 120km/h |
| Minimum SE-ST distance (2D) | 10m | 10m |
| SE-ST LOS/NLOS | LOS (prioritized), NLOS | LOS (prioritized), NLOS |

Proposal 2: RAN1 takes Table 2-1 and Table 3-1 as starting point for discussion of ISAC deployment scenarios.

- Use “sensing entity” and “sensing target”, instead of UE/TRP/Tx/Rx, in description of sensing deployment scenarios.

In Table 2-1 and Table 3-1, the channel condition between sensing entity (Tx/Rx) and sensing target is modeled as LOS as prioritized over NLOS. This is because the sensing modeling relies heavily on the effective RCS size of the sensing target. However, the effective RCS size in the radar theory assumes LOS condition. It could be erroneous to apply RCS size obtained in LOS condition to NLOS, and meanwhile it could be time/effort consuming to derive/measure/validate RCS size specifically under NLOS condition.

Proposal 3: RAN1 prioritizes LOS condition over NLOS for the link between sensing entity (Tx/Rx) and sensing target.

# Conclusion

Observation 1: A fairly complete study over five example use cases in SID leads to a need of considering all the indoor scenarios (indoor office, INF) and outdoor scenarios (UMi,UMa,RMa).

Proposal 1: Besides mono-static sensing and bi-static sensing, multi-static sensing mode is also supported in Rel-19 ISAC channel modeling.

- Support of multi-static mode is mostly implemented in the support of bi-static mode.

Proposal 2: RAN1 takes Table 2-1 and Table 3-1 in this contribution as starting point for discussion of ISAC deployment scenarios.

- Use “sensing entity” and “sensing target”, instead of UE/TRP/Tx/Rx, in description of sensing deployment scenarios.

Proposal 3: RAN1 prioritizes LOS condition over NLOS for the link between sensing entity (Tx/Rx) and sensing target.

# Reference

- RP-234069 New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR

- TR 38.901 Study on channel model for frequencies from 0.5 to 100 GHz

- TR 36.777 Study on Enhanced LTE Support for Aerial Vehicles

- TR 37.885 Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR