3GPP TSG RAN WG1 #116                                                                                           R1-2400617

Athens, Greece, February 26th – March 1st, 2024

Source:	OPPO

Title:	Study on ISAC channel modelling

Agenda Item:	9.7.2

Document for:	Discussion and Decision

# Introduction

At the RAN #102 meeting, the following study objectives were agreed for channel modelling in ISAC [1]:

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects:

- UAVs

- Humans indoors and outdoors

- Automotive vehicles (at least outdoors)

- Automated guided vehicles (e.g. in indoor factories)

- Objects creating hazards on roads/railways, with a minimum size dependent on frequency

The study considers six sensing modes, including TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic and UE monostatic, and primarily focuses on a frequency range of [0.5, 52.6]GHz with scalability up to 100GHz.

For the above use cases, sensing modes and frequencies, ISAC channel modelling should be defined by using TR38.901 as a starting point, and taking into account relevant measurements, including:

- modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns;

- spatial consistency.

This contribution elaborates our views on channel modelling for ISAC.

# Discussion

Statistic channel modeling vs. deterministic channel modeling

There are typically two channel model philosophies for the sensing target related channel modeling: statistic channel modeling and deterministic channel modeling.

- Statistic channel modeling expresses the channel by an abstract channel impulse response with parameters, such as power distribution, delay spread and angular spread, which are statistically obtained from a wealth of measured data in the corresponding scenario. Although statistic channel model requires large amount of measurement data to derive the modeling parameters, once those parameters are decided, the use of such channel model normally results in lower computation complexity in simulation and less relevance to deployment map details. However, due to lack of deterministic relation between channel scatter features (such as position-related ray angles, Doppler and reflection factor) and the sensing target, statistic channel model may not be able to fully support the sensing evaluations.

- Deterministic channel modeling generates the channel by applying electromagnetic wave propagation theory to the specific geographic map environment. It does not need extensive measurements in advance but requires comprehensive environmental data for accurate calculations on signal propagation. Map-based ray tracing is one typical deterministic channel modeling. However, the high computation complexity and loss of generality due to tight dependency upon a specific environmental map are traditional concerns for ray-tracing based model. The long-time low popularity in 3GPP for using ray-tracing method for communication evaluation may also make it difficult to use the method for ISAC modeling.

Figure 1 Combination of target-related cluster modelling and background cluster modelling

In our view, the desirable channel model for sensing in ISAC study can be primarily a statistic channel modeling, with some modeling parameters derived from the deterministic sensing geometry relations among sensing Tx/Rx and sensing target, where such deterministic geometry relations are normally available even in a statistic-style evaluation, such as Tx/Rx/target dropping in each simulation run. Take an example in Figure 1, once BS (as a sensing Tx), UE (as a sensing Rx) and Target are dropped to three geographical locations in a simulation,

- In the derivation of channel characteristics around the sensed target, the power, delay, arrival angel and departure angel of the propagation path can be determined based on the actual positions of sensed object and sensing Tx/Rx. For example, the path cluster delay in the fast fading modeling is determined by both Tx-target distance and target-RX distance. Departure angel and arrival angle are determined in Tx-target and target-Rx part separately.

- The propagation channel model for background environments uses statistic channel model. All Tx-to-Rx multi-paths except Tx-target-Rx path can still apply statistic channel model to reduce simulation complexity. As in TR38.901[2], the channel model generates multiple paths and involves the angle, delay, and power information of multiple sub-paths in one path cluster. The statistical angle, delay, and power for each cluster are obtained from measured data to derive empirical formulas for wireless channel. For ISAC, sensing and communication share common Tx/Rx equipment, e.g. gNB and UE. Spectrum and network deployment are also common for both. So statistic characteristics of wireless channel for communication can be reused to avoid tremendous workload for channel measurements.

Observation 1: For integrated sensing and communication, communication function and sensing function share the same Tx/Rx units, common spectrum and deployment. So the statistic characteristics of wireless channel for communication can be applicable to sensing background channel.

Proposal 1 The channel model for sensing in ISAC study is primarily a statistic channel model, where

- Some modeling parameters for Tx-Target-Rx propagation path are derived from the deterministic geometry relations among sensing Tx/Rx and sensed target, where the derivation may follow statistic formulation such as pathloss or deterministic calculation such as departure/arrival angles;

- The other propagation paths between Tx and Rx (other than path of Tx-Target-Rx) follow the existing statistics modeling methodology such as TR38.901.

It should be noted that the above-mentioned channel modeling methodology differs from the hybrid channel model in TR38.901 in that:

- The modeling method mentioned above does not need to perform ray tracing against any environmental objects, and does not require an environmental map.

- The hybrid channel model in 38.901 brings deterministic channel modeling to the strongest channel clusters between Tx and Rx, while the Tx-target-Rx path does not necessarily result in a channel cluster among those strongest.

- The SID requires the ISAC channel model to be related to RCS size of a sensed reflector, while RCS is not a part of specified modeling algorithm in TR38.901 for map-based hybrid modeling. In fact, the whole details of map-based ray-tracing methodology are out of 38.901.

- The ISAC channel model needs to be eventually calibrated within RAN1, while TR38.901 explicitly mentions the hybrid channel model is not calibrated, at least due to no agreeable and generally applicable environment map and no specified ray-tracing algorithm. It is not expected in Rel-19 ISAC channel model SI to spend efforts for what was already proved to be difficult in the past release.

Observation 2: The map-based hybrid channel model in TR38.901 does not fit for ISAC channel model SI.

Co-existence of Tx-Target-Rx channel modeling (for sensing purpose) and Tx-Rx channel modeling (for communication purpose)

As a nature of wireless channel, the Tx-Target-Rx propagation path constructs a subset of all propagation paths between the same pair of Tx and Rx. If the ISAC channel model is designed to enable a wireless communication signal (such as a NR or 6G reference signal) to be transmitted by an Tx and received by an Rx for both sensing purpose and communication purpose, the generation of Tx-Target-Rx propagation path should not change the statistic property of Tx-Rx wireless channel too much, because otherwise the communication evaluation based on new ISAC channel model may have a risk of showing different performance from a communication-only evaluation based on existing statistics channel model. In the channel modeling methodology mentioned above, Tx-Target-Rx propagation path modeling (which can be considered quasi-deterministic) is determined in a different or even independent way from the Tx-Rx propagation path modeling (which is generated statistically). Therefore, additional step needs to be taken in the modeling to make the two separately-generated models compatible to each other. This is further elaborated below with a typical bi-static sensing mode shown in Figure 2, where

Figure 2 Sensing/communication channel in bi-static sensing structure

- {d,d1,d2} denote the LOS distances for Tx-Rx, Tx-Target and Target-Rx respectively.

- Hlarge,s(dB) and Hlarge,c(dB) represent the large-scale fading loss (including both pathloss and shadowing) respectively on Tx-Target-Rx path that is experienced by sensing signal and over all combined paths on Tx-Rx that is experienced by communication signal.

It is obvious for the statistic modeling to obey Hlarge,s(dB) > Hlarge,c(dB), regardless of LOS/NLOS condition.

Another desirable compatibility check is to minimize the impact to statistic characteristics of Tx-Rx channel model due to addition of Tx-Target-Rx path. We suggest following new steps in small-scale fading modeling:

- Once a sensing cluster on the Tx-Target-Rx path is added to existing statistic channel clusters between Tx and Rx that are generated according to TR38.901, the sensing cluster replaces a statistic cluster that is “statistically closest in time” to the added sensing cluster.

- The cluster power normalization is performed over all clusters, but without changing the powers for sensing cluster(s).

Proposal 2 ISAC channel model holds compatibility between Tx-Target-Rx channel modeling (for sensing purpose) and Tx-Rx channel modeling (for communication purpose), including:

- The large-scale fading loss in Tx-Target-Rx path modeling is always larger than that in Tx-Rx channel modeling.

- A sensing cluster on the Tx-Target-Rx path replaces an existing statistic cluster in Tx-Rx channel model that is “statistically closest in time” to the replacing sensing cluster.

- The cluster power normalization is performed over all channel clusters in Tx-Rx channel model, without changing the powers on sensing cluster(s).

An example of TR38.901 TP is given in Annex A to reflect above new steps.

Pathloss for Tx-target-Rx propagation path

Following up radar theorem, the pathloss of Tx-Target-Rx path can be formulated as following in unit of dB:

PLsd1,d2=PLd1+PLd2+10log10λ24πσRCS

where λ is the wavelength, σRCS is the effective RCS size of the target, {d1,d2} are all given in Figure 2. PL(d1) and PL(d2) represent the pathloss for Tx-Target and pathloss for Target-Rx, respectively. Both Tx-Target and Target-Rx are usually considered to have free-space propagation in radar theorem, but deserve a study whether/how to be modeled in a channel condition other than free-space for ISAC.

Table 1 Typical RCS values

| Detection target | RCS size (m2) |
| --- | --- |
| Human | 1 |
| Automobile | 10~100 |
| UAV (small) | 0.01 |
| UAV (medium) | 0.1 |

RCS size is a key factor in radar detection, and should play an important role in sensing as well. The RCS size of the target usually comes from the measurements, and there are already some well-assumed RCS for the typical detection targets, as listed in Table 1. Besides, for an accurate modeling, the RCS size is also modeled as frequency-dependent.

Proposal 3: The pathloss for Tx-Target-Rx propagation is formulated as

PLsd1,d2=PLd1+PLd2+10log10λ24πσRCS

where

- PLd1 is pathloss between Tx and target, with d1 equal to Tx-to-Target LOS distance.

- PLd2 is pathloss between Rx and target, with d2 equal to Target-to-Rx LOS distance.

- λ is wave-length of sensing signal.

- σRCS is RCS size.

RCS size is traditionally obtained from the measurements that are taken under LOS condition. It is still under industrial/academic study how to obtain RCS size under NLOS environment. It remains unclear whether there is any dependable conclusion that can be somehow generally applicable. Meanwhile, in quite of some sensing application scenarios, both Tx and Rx are intentionally placed in spots with high chance of LOS to the sensing target; in some other sensing application scenarios where NLOS is likely to occur between Tx/Rx and the target, multi-static sensing mode can be assumed to apply to increase the LOS probability. So the necessity of NLOS channel modeling is not that high in our view.

Proposal 4: For Tx-Target path and Target-Rx path, prioritize LOS modeling over NLOS modeling.

In case NLOS modeling is eventually not supported on Tx-Target-Rx, if any of Tx-target path and target-Rx path is NLOS subject to LOS/NLOS probability test, the simulation based on this ISAC channel model can claim an event of “sensing target in NLOS” for the corresponding pair of Tx and Rx.

Using micro-Doppler property in Tx-target-Rx channel modeling

It is stated in SID [1] as part of objectiveness that “The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects”. In general, the traits to enable target to be distinguished from unintended objects can be RCS, shape or micro-Doppler. A conventional air surveillance radar system can rely on the RCS of an aircraft for detection. But this may not always provide reliable detection. Take an example in case of drones, the birds can have similar physical size to drones and fly at similar altitude and speed. Shape is difficult to sketch due to limited bandwidth of sensing signal. In contrast, UAV detection/identification based on micro-Doppler is proved effective and becomes more and more attractive. It is shown in Figure 3 [3] that the micro-Doppler characteristics differ significantly between drones and birds.

Figure 3 Micro-Doppler difference between drone (left) and bird (right) [3]

Table 2 Micro-Doppler modeling

| Sensing targets | Micro-Doppler function f(t) |
| --- | --- |
| UAV | The kth blade of the dth rotor: ft=Lcosβcosφd,kt, L=(L1+L2)/2, L1 is the distance between the blade roots and the center of the rotation, and L2 as the distance between the blade tip and the center of the rotation, β is the elevation angle of the rotor to the radar line-of-sight (LOS), and φd,kt=φd,k0+ωdt is the phase of the kth blade of the dth rotor, withφd,k0 denoting the initial rotation angle, and ωd is the rotation radian frequency of the dth rotor [3]. |
| Birds | ft=lsinϑcosαcos(ϕ(t))+lcosϑsin(ϕ(t)) α is azimuth angle, ϑ is pitch angle and flapping angle is ϕ(t) ϕ(t)=θmaxsin(2πfbirdt)=θmaxsin(ω0t) θmax is maximum flapping angle, ω0=2πfbird is flapping frequency[3]. |
| Respiration rate and heartbeat rate | ft=mhsinωht+mrsinωrt mh and mrare the amplitudes of human heartbeat and respiration, and ωhand ωrare the angular frequencies of heartbeat and respiration [3]. |

To support evaluation on sensing false-alarm, the ISAC channel modeling should allow the micro-Doppler taking its effect in the modeling to enable micro-Doppler based target identification. Table 2 lists some typical micro-Doppler models, which have been well-studied [3]. It is obvious that different sensing targets would have different micro-Doppler function f(t), which is exactly the reason for being able to serve its purpose. Meanwhile, the clear differentiation among micro-Doppler functions leads to a question on whether it is a good idea or rather infeasible to capture all micro-Doppler functions into TR38.901. In our view, the ISAC channel model would become more capable if it just defines in TR38.901 a place-holder in its general form (i.e., f(t)) to allow people adding desirable micro-Doppler function at the time they use the model upon a specific sensing evaluation. In other words, making the micro-Doppler function an input to the ISAC channel model is better than making it a part of ISAC channel model. Then, all Rel-19 ISAC channel modeling needs to enable micro-Doppler based target identification/differentiation becomes quite simple: just define a place-holder in small-scale fading formula.

Similar to Doppler, micro-Doppler can be modeled in a phase term, i.e., expj2π rtx-target+rtarget-rxλf(t) where rtx-target and rtarget-rx are spherical unit vectors on Tx-to-target direction and target-to-Rx direction, λ is wavelength and f(t) is a vector of micro-Doppler model. In some literatures, f'(t) is used to denote micro-Doppler function with ft=t0tf'(α)dα or f't=ddtft.

Proposal 5: Rel-19 ISAC channel model relies on micro-Doppler to enable target identification/differentiation, without determining the exact micro-Doppler model function for the specific sensing application.

- A micro-Doppler function place-holder is defined in small-scale fading modeling.

- The exact micro-Doppler function corresponding to a sensing application will be an input to the ISAC channel model.

It should be noted that the existing TR38.901 already contains an additional modeling of time-varying Doppler shift (ref. section 7.6.6 in [2]), which can be treated theoretically as a combination of Doppler modeling and micro-Doppler modeling. However, this combination form is too high-level and is not straight-forward to work with micro-movement in sensing application. In addition, the time-varying Doppler modeling in the existing 38.901 assumes the reason causing Doppler to be time-varying happens at UE, instead of a reflection object on the propagation path. That is why the time-varying Doppler is projected onto a spherical unit vector () relating to Rx only. Therefore, we prefer a new and more specific modeling for micro-Doppler.

An example of TR38.901 TP is given in Annex B for the corresponding place-holder definition.

# Conclusions

This contribution is concluded with the following observations and proposals:

Observation 1: For integrated sensing and communication, communication function and sensing function share the same Tx/Rx units, common spectrum and deployment. So the statistic characteristics of wireless channel for communication can be applicable to sensing background channel.

Observation 2: The map-based hybrid channel model in TR38.901 does not fit for ISAC channel model SI.

Proposal 1 The channel model for sensing in ISAC study is primarily a statistic channel model, where

- Some modeling parameters for Tx-Target-Rx propagation path are derived from the deterministic geometry relations among sensing Tx/Rx and sensed target, where the derivation may follow statistic formulation such as pathloss or deterministic calculation such as departure/arrival angles;

- The other propagation paths between Tx and Rx (other than path of Tx-Target-Rx) follow the existing statistics modeling methodology such as TR38.901.

Proposal 2 ISAC channel model holds compatibility between Tx-Target-Rx channel modeling (for sensing purpose) and Tx-Rx channel modeling (for communication purpose), including:

- The large-scale fading loss in Tx-Target-Rx path modeling is always larger than that in Tx-Rx channel modeling.

- A sensing cluster on the Tx-Target-Rx path replaces an existing statistic cluster in Tx-Rx channel model that is “statistically closest in time” to the replacing sensing cluster.

- The cluster power normalization is performed over all channel clusters in Tx-Rx channel model, without changing the powers on sensing cluster(s).

An example of TR38.901 TP is given in Annex A for more details of Proposal 2.

Proposal 3: The pathloss for Tx-Target-Rx propagation is formulated as

PLsd1,d2=PLd1+PLd2+10log10λ24πσRCS

where

- PLd1 is pathloss between Tx and target, with d1 equal to Tx-to-Target LOS distance.

- PLd2 is pathloss between Rx and target, with d2 equal to Target-to-Rx LOS distance.

- λ is wave-length of sensing signal.

- σRCS is RCS size.

Proposal 4: For Tx-Target path and Target-Rx path, prioritize LOS modeling over NLOS modeling.

Proposal 5: Rel-19 ISAC channel model relies on micro-Doppler to enable target identification/differentiation, without determining the exact micro-Doppler model function for the specific sensing application.

- A micro-Doppler function place-holder is defined in small-scale fading modeling.

- The exact micro-Doppler function corresponding to a sensing application will be an input to the ISAC channel model.

An example of TR38.901 TP is given in Annex B for more details of Proposal 5.

# Reference

- RP-234069,  New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR, Nokia, Nokia Shanghai Bell

- TR38.901, v17.1.0

- Micro-Doppler Effect in Radar, 2nd edition, Victor C Chen

Annex A. TR38.901 TP example for power interaction between sensing cluster and background clusters

This example assumes LOS conditions on both Tx-Target and Target-Rx, and one channel cluster over Tx-Target-Rx path.

Section 7.5 of 38.901:

Large scale parameters:

Step 2: Assign propagation condition (LOS/NLOS) according to Table 7.4.2-1. The propagation conditions for different BS-UT links are uncorrelated.

Also, assign an indoor/outdoor state for each UT. It is noted that all the links from a UT have the same indoor/outdoor state.

Step 3: Calculate pathloss (denoted as PLc dB) with formulas in Table 7.4.1-1 for each BS-UT link to be modelled.

Step 3a: Calculate pathloss for Tx-Target-Rx, denoted as PLs(dB). Details TBD.

Step 4: Generate large scale parameters, e.g. delay spread (DS), angular spreads (ASA, ASD, ZSA, ZSD), Ricean K factor (K) and shadow fading (SF, denoted as SFc dB) taking into account cross correlation according to Table 7.5-6 and using the procedure described in clause 3.3.1 of [14] with the square root matrixbeing generated using the Cholesky decomposition and the following order of the large scale parameter vector: sM = [sSF, sK, sDS, sASD, sASA, sZSD, sZSA]T.

These LSPs for different BS-UT links are uncorrelated, but the LSPs for links from co-sited sectors to a UT are the same. In addition, these LSPs for the links of UTs on different floors are uncorrelated.

Limit random RMS azimuth arrival and azimuth departure spread values to 104 degrees, i.e., ASA= min(ASA, 104), ASD = min(ASD, 104). Limit random RMS zenith arrival and zenith departure spread values to 52 degrees, i.e., ZSA = min(ZSA, 52), ZSD = min(ZSD, 52).

Step 4a: Generate shadow fading for Tx-Target-Rx path, denoted as SFs (dB). Re-generate SFs (if necessary) to meet (PLs+SFs)>(PLc+SFc). Details TBD.

Small scale parameters:

Step 5: Generate cluster delays 

Denote N'=N+1 if LOS condition is false between Tx and Rx (Note: this is to allow a “virtual” LOS cluster); and N'=N otherwise. Delays are drawn randomly from the delay distribution defined in Table 7.5-6. With exponential delay distribution calculate

,	                                (7.5-1)

Where r is the delay distribution proportionality factor, Xn ~ uniform(0,1), and cluster index n = 1,…,N’. With uniform delay distribution the delay values  are drawn from the corresponding range. Denote τmin'=minτn', and τtarget'=τmin'+d1+d2-dc where c is the speed of light, {d1,d2,d} are respectively the LOS distances for Tx-target, target-Rx and Tx-Rx. Equally divide [0,1] into N’ intervals, where n-th internal is [n-1N',nN') for n = 1,…,N’. Find the interval index ntarget satisfying ntarget-1N'≤exp-τtarget'rτ<ntargetN'.

- If there is at least one Xn falling into ntarget-th interval, and its corresponding τn' is not τmin', discard τn' that makes τn'-τtarget' the minimum among all n’s for which Xn falls into ntarget-th interval and τn' is not τmin'.  (Note: This step means that, if the deterministic cluster delay falls into an exponential-distribution time bin that has at least one statistic cluster delay and that statistic cluster is not the LOS cluster, that statistic cluster is replaced)

- Otherwise, find a set of intervals where each interval in the set contains equally maximum number of Xn per interval. Discard τn' that makes τn'-τtarget' the minimum among all n’s for which Xn falls into an interval in the set and τn' is not τmin'. (Note: This step means that, find all time bins that have the same maximum number of τn' in the bin. Replace the one among these τn' that is closest to τtarget' and meanwhile does not correspond to the LOS cluster)

- Add τtarget' to the set of {τn'}.

If LOS condition is false between Tx and Rx, discard τn'=τmin', which leaves N samples in the set of {τn'}. (Note: This is to remove the “virtual” LOS cluster that was added earlier).

Normalise the delays by subtracting the minimum delay and sort the normalised delays to ascending order:

.

where τtarget' results in τntarget.

In the case of LOS condition, additional scaling of delays is required to compensate for the effect of LOS peak addition to the delay spread. The heuristically determined Ricean K-factor dependent scaling constant is

,	(7.5-3)

where K [dB] is the Ricean K-factor as generated in Step 4. The scaled delays

,	(7.5-4)

are not to be used in cluster power generation.

Step 6: Generate cluster powers .

Cluster powers are calculated assuming a single slope exponential power delay profile. Power assignment depends on the delay distribution defined in Table 7.5-6. With exponential delay distribution the cluster powers are determined by

(7.5-5)

for all n≠ntarget, where  is the per cluster shadowing term in [dB]. Normalize the cluster powers so that the sum of all cluster powers is equal to one, i.e.,

where

In the case of LOS condition an additional specular component is added to the first cluster. Power of the single LOS ray is:

(7.5-7)

and the cluster powers are not normalized as in equation (7.5-6) , but:

(7.5-8)

where (.) is Dirac's delta function and KR is the Ricean K-factor as generated in Step 4 converted to linear scale. These power values are used only in equations (7.5-9) and (7.5-14), but not in equation (7.5-22).

Assign the power of each ray within a cluster for  as Pn / M, where M is the number of rays per cluster.

Remove clusters of  with less than -25 dB power compared to the maximum cluster power. The scaling factors need not be changed after cluster elimination.

********* Steps 7~11 are omitted ***********

Step 11: Generate channel coefficients for each cluster n and each receiver and transmitter element pair u, s.

The method described below is used at least for drop-based evaluations irrespective of UT speeds. Relevant cases for drop-based evaluations are:

-	Case 1: For low complexity evaluations

-	Case 2: To compare with earlier simulation results,

-	Case 3: When none of the additional modelling components are turned on.

-	Case 4: When spatial consistency and/or blockage is modeled for MU-MIMO simulations

-	Other cases are not precluded

For the N – 2 weakest clusters and the sensing target cluster, say n = 3, 4,…, N and n=ntarget, the channel coefficients are given by:

(7.5-22)

where Frx,u,θ and Frx,u,ϕ are the field patterns of receive antenna element u according to (7.1-11) and in the direction of the spherical basis vectors,  and  respectively, Ftx,s,θ and Ftx,s,ϕ are the field patterns of transmit antenna element s in the direction of the spherical basis vectors,  and  respectively. Note that the patterns are given in the GCS and therefore include transformations with respect to antenna orientation as described in Clause 7.1.  is the spherical unit vector with azimuth arrival angle ϕn,m,AOA and elevation arrival angle θn,m,ZOA, given by

,	(7.5-23)

where n denotes a cluster and m denotes a ray within cluster n. For n=ntarget, M=1.  is the spherical unit vector with azimuth departure angle ϕn,m,AOD and elevation departure angle θn,m,ZOD, given by

,	(7.5-24)

where n denotes a cluster and m denotes a ray within cluster n. Also, is the location vector of receive antenna element u and is the location vector of transmit antenna element s, n,m is the cross polarisation power ratio in linear scale, and 0 is the wavelength of the carrier frequency. If polarisation is not considered, the 2x2 polarisation matrix can be replaced by the scalar  and only vertically polarised field patterns are applied.

The Doppler frequency component depends on the arrival angles (AOA, ZOA), and the UT velocity vector with speed v, travel azimuth angle ϕv, elevation angle θv and is given by

(7.5-25)

For the two strongest clusters other than sensing target cluster, say n = 1 and 2 for n≠ntarget, rays are spread in delay to three sub-clusters (per cluster), with fixed delay offset. The delays of the sub-clusters are

(7.5-26)

where  is cluster delay spread specified in Table 7.5-6. When intra-cluster delay spread is unspecified (i.e., N/A) the value 3.91 ns is used; it is noted that this value results in the legacy behaviour with 5 and 10 ns sub-cluster delays.

Twenty rays of a cluster are mapped to sub-clusters as presented in Table 7.5-5 below. The corresponding offset angles are taken from Table 7.5-3 with mapping of Table 7.5-5.

Table 7.5-5: Sub-cluster information for intra cluster delay spread clusters

| sub-cluster # | mapping to rays | Power | delay offset |
| --- | --- | --- | --- |
|  |  | 10/20 | 0 |
|  |  | 6/20 | 1.28 |
|  |  | 4/20 | 2.56 |

Then, the channel impulse response is given by:

(7.5-27)

Hu,s,backgroundNLOSτ,t=n=1n≠ntarget2i=13m∈RiHu,s,n,mNLOStδτ-τn,i+n=3n≠ntargetNHu,s,n,mNLOStδτ-τn      (7.5-27)

and

Hu,s,targetNLOSτ,t=Hu,s,ntargetNLOStδτ-τntarget                                (7.5-27a)

where  is given in (7.5-22) and  defined as:

(7.5-28)

In the LOS case, determine the LOS channel coefficient by:

(7.5-29)

where (.) is the Dirac's delta function and KR is the Ricean K-factor as generated in Step 4 converted to linear scale.

Then, the channel impulse response is given by adding the LOS channel coefficient to the NLOS channel impulse response and scaling both terms according to the desired K-factor  as

.	(7.5-30)

Hu,sLOSτ,t=1KR+1Hu,s,backgroundNLOSτ,t+KRKR+1Hu,s,1LOStδτ-τ1+Hu,s,targetNLOSτ,t           (7.5-30)

Step 12: Apply pathloss (PLc) and shadowing (SFc) for the channel coefficients.

Annex B. TR38.901 TP example to define place-holder of micro-Doppler function

Section 7.5 of 38.901:

Step 11: Generate channel coefficients for each cluster n and each receiver and transmitter element pair u, s.

*********** Unchanged text omitted **********

Then, the channel impulse response is given by:

(7.5-27)

where  is given in (7.5-22) and  defined as:

(7.5-28)

To enable evaluation with target identification based on micro-Doppler characteristics of the sensing signal, the term  in (7.5-22) and (7.5-28), where cluster n corresponds to a Tx-Target-Rx cluster, is replaced by , where  is a placeholder for function characterizing micro-Doppler effect for a specific sensing application. The function that is brought into this placeholder is considered input to the modeling and therefore is not defined in this technical report.  (Note: It is assumed here that the Tx-Target-Rx cluster is not the LOS cluster between Tx and Rx. )

In the LOS case, determine the LOS channel coefficient by:

(7.5-29)

where (.) is the Dirac's delta function and KR is the Ricean K-factor as generated in Step 4 converted to linear scale.

Then, the channel impulse response is given by adding the LOS channel coefficient to the NLOS channel impulse response and scaling both terms according to the desired K-factor  as

(7.5-30)