3GPP TSG RAN WG1 #116	 	                                     R1-2400618

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Source:	OPPO

Title:	On specification for AI/ML-based beam management

Agenda Item:	9.1.1

Document for:	Discussion

# Introduction

The SI [1] of artificial intelligent (AI) and machine learning (ML) for beam management of NR air interface has been studied and evaluated during Rel-18. In RAN#102 meeting, the use cases of beam management were selected to be specified in Rel-19 without further studying, thanks to all the efforts conducted in Rel-18.

The details of the WID [2] on beam management is listed below for reference.

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]:

  - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”)

  - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”)

  - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2.

In this contribution, we will focus on the specification impacts related to model training, model inference and model LCM.

# Specification impacts of BM-Case1 and BM-Case2

In this section, we discuss the specification impacts of both sub use cases from configuration, inference, through LCM and training.

## Association between Set A and Set B

### Construction of Set A and Set B

Agreement @ RAN1#112

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of AI model inference from the following additional aspects on top of previous agreements:

- Indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable

- Beam indication from network for UE reception

- Note: The second bullet may or may not have additional specification impact (e.g., legacy mechanism may be reused).

In above agreement, RAN1 also agreed to study the indication of the association between Set A and Set B. In our understanding, both Set A and Set B can be configured via higher layer signaling with 1-on-1 association/mapping.

- From signaling aspects, it seems flexible to configure both Set A and Set B via higher layer signaling.

In RAN1#110, two alternatives of construction of Set A/Set B for BM-Case1 were captured in the agreement below.

Agreement

For the sub use case BM-Case1, support the following alternatives for further study:

- Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A)

- Alt.2: Set B is a subset of Set A

- Note1: Set A is for DL beam prediction and Set B is for DL beam measurement.

- Note2: The beam patterns of Set A and Set B can be clarified by the companies.

We understand that how to configure Set B/Set A during training and inference phase can be up to NW. But from the evaluation results of TR 38.843, one may observe that the narrow beam setting of Set B (Alt.2) can provide better prediction accuracy than that of wide beam setting of Set B (Alt.1) when given the same ratio of Set B/Set A. The second benefit of Alt.2 comes from the fact that UE can directly measure part of Set A (i.e. Set B). Once the Top-1/K beam(s) is (are) from Set B, then the UE can save some latency in measuring the indicated TCI state(s), if necessary.

- For BM-Case1, configure Set B as a subset of Set A during training and inference phase.

To show the performance benefits of temporal domain beam prediction, the same setting of Set A and Set B is typically adopted. If Set A is a subset of Set B, it may also involve beam prediction in spatial domain as well, therefore resulting in mixed BM-Case1 and BM-Case2, i.e. the spatial and temporal domain beam prediction. To have a clear design, we suggest to split those two different domains. And of course, in practical/deployment, spatial and temporal domain can be jointly applied for Tx beam prediction.

Agreement

For the sub use case BM-Case2, further study the following alternatives:

- Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A)

- Alt.2: Set B is a subset of Set A (Set A and Set B are not the same)

- Alt.3: Set A and Set B are the same

- Note1: The beam pattern of Set A and Set B can be clarified by the companies.

- For BM-Case2, configure the same Set B and Set A during training and inference phase.

### Configuration of Set A and Set B

To enable the flexibility of AI/ML based beam prediction, NW may configure more than one sets of measurement sets (more than one Set Bs) and prediction sets (more than one Set As) for UE-side model. Depending on various conditions, such as Tx beam book, antenna array layout, it is up to NW to choose/select/activate one (a pair of associated Set B and Set A) of those configurations.

- For UE-side model, support NW to configure more than one Set Bs/Set As and then select/activate one pair of associated Set B/Set A.

For BM-Case2, there could be up to N measurement instances and F prediction instances. For each measurement instance and prediction instance, following the trajectory of the UE, different measurement set, i.e. Set B and prediction set, i.e. Set A are practical setting for the AI/ML model. To facilitate such flexibility, we believe Set B and/or Set A for BM-Case2 can be configured and/or activated on a per instance basis.

- For BM-Case2, support the configuration of Set A and/or Set B on a per instance basis.

## Inference

In RAN1#110, the following agreement on specification impacts of inference was achieved.

Agreement

In order to facilitate the AI/ML model inference, study the following aspects as a starting point:

- Enhanced or new configurations/UE reporting/UE measurement, e.g., Enhanced or new beam measurement and/or beam reporting

- Enhanced or new signaling for measurement configuration/triggering

- Signaling of assistance information (if applicable)

- Other aspect(s) is not precluded

In RAN1#112, the DL Rx beam prediction is deprioritized, since the DL Rx beamforming should be considered as UE implementation or proprietary information. In RAN#102, the DL beam pair prediction was not included this WI.

Conclusion

For the sub use case BM-Case1 and BM-Case2, “Alt.2: DL Rx beam prediction” is deprioritized.

Hence, we will focus on the inference of DL Tx beam prediction (Alt.1) for UE-side model and NW-side model respectively.

### Inference at UE side

In RAN1#110bis-e, one agreement suggests to study the specification impact of UE reporting predicted beam(s) to NW when UE-side model is applied. In RAN1#112, the follow-up agreement suggests to study the necessity, feasibility and the potential specification impact when predicted L1-RSRP and confidence/probability of model output are reported from NW to UE.

Agreement @ RAN1#110bis-e

For BM-Case1 with a UE-side AI/ML model, study the potential specification impact of L1 signaling to report the following information of AI/ML model inference to NW

- The beam(s) that is based on the output of AI/ML model inference

- FFS: Predicted L1-RSRP corresponding to the beam(s)

- FFS: other information

Agreement @ RAN1#112

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the necessity, feasibility and the potential specification impact (if needed) of the following information reported from UE to network:

- Predicted L1-RSRP(s) corresponding to the DL Tx beam(s) or beam pair(s)

  - Whether/how to differentiate predicted L1-RSRP and measured L1-RSRP

- Confidence/probability information related to the output of AI/ML model inference (e.g., predicted beams)

  - FFS: Definition/content of confidence/probability information

- Note: At least the performance and spec impact should be considered

Beam reporting and indication for Tx beam prediction

For the case of predicted Tx beam (Alt.1), UE could apply legacy beam reporting mechanism to report Top-K Tx beam(s) along with predicted L1-RSRP(s) to NW. Then NW indicates the corresponding TCI state to UE as beam indication. If UE is aware of which Rx beam to apply (e.g. specific Rx beam for Tx beam prediction) for receiving the predicted Tx beam, then there is no need for the next stage of DL Rx beam sweeping. But if UE doesn’t know which corresponding Rx beam (e.g. no fixed Rx beam for Tx beam prediction) to receive the predicted Tx beam, then there could be following-up beam sweeping procedure (e.g. P3 procedure) to find it. This operation is analogous to Rel.15 NR beam management mechanism, hence yielding minimum specification impact.

- For Tx beam prediction (Alt.1), the corresponding Rx beam could either be determined by UE as specific Rx beam or by existing Rx beam sweeping procedure.

- For Tx beam prediction (Alt.1) with UE-side model, legacy beam reporting and indication mechanism could serve as starting point.

Predicted L1-RSRP(s) reporting

In our view, the predicted L1-RSRP could be quantized and reported from UE to NW as legacy measurement-based beam reporting. Specifically, Top-K beam reporting could reuse the differential reporting format from the strongest in the metric of predicted L1-RSRP.

In our evaluation, the average error between predicted L1-RSRP of predicted beam and ideal L1-RSRP of the same beam is only marginal. Hence, we think it’s worthy to trust the predicted L1-RSRP of the predicted beam.

In addition, to differentiate predicted beam reporting and measured beam reporting, it could be up to NW to configure separate CSI reporting configurations. For instance, multiple CSI-ReportConfigs can be configured to UE, one for legacy measured beam reporting and the other for predicted beam reporting. In addition, the comparison of those two different types of beam reporting can be leveraged as a metric for the purpose of LCM.

- The reported predicted L1-RSRP and measured L1-RSRP can be differentiated via separate NW configuration.

- Predicted L1-RSRP(s) by UE-side model should be reported to NW along with predicted Top-K Tx beam(s).

Confidence/probability reporting

Consider the inference phase, the AI/ML model in general outputs a regularized vector. Each element (or let’s say each Tx beam) in this vector represents the confidence/probability of the corresponding Tx beam to be the predicted best. The sum of the output vector could be regularized to a fixed value, e.g. 100%. That’s the key output value that the AI/ML model depends on to predict the Top-K Tx beams. Specifically, simply by taking the highest K confidence out of all the Tx beams of Set A, then the multi-classification problem (Tx beam prediction) can be done.

The benefits of reporting confidence/probability come from the following aspects.

- This soft metric could help NW to understand how confident the model predicts the best Tx beams. DL Tx beam selection could depend on the confidence.

- It could help NW to carry out model monitoring, e.g. when the confidence/probability is lower than a threshold. The lower confidence results in the higher probability the model fails in prediction.

In addition, to report this confidence/probability value, the quantization on it should be done. Depending on the granularity, different number of bits can be reported.

- Confidence/probability of UE-side model output could be quantized and reported to NW.

Beam reporting for N predicted time instances

In RAN1#110bis-e, the following agreement was made for BM-Case2. For BM-Case2, the Tx beam reporting should be extended from single time instance to future multiple time instances.

Agreement

For BM-Case2 with a UE-side AI/ML model, study the potential specification impact of L1 signaling to report the following information of AI/ML model inference to NW

- The beam(s) of N future time instance(s) that is based on the output of AI/ML model inference

  - FFS: value of N

- FFS: Predicted L1-RSRP corresponding to the beam(s)

- Information about the timestamp corresponding the reported beam(s)

  - FFS: explicit or implicit

- FFS: other information

For the N future time instances of beam prediction, the possible range of N could be from 1, 2, 4 or even higher number, depending on scenarios/configurations. Let’s take N = 4 as a classic value. For Tx beam prediction, each Tx beam can be represented by SSBRI or CRI with 6 bits quantization in NR. Top-K predicted Tx beam(s) may take value as K = 1/2/4. Let’s take the max value, i.e. K = 4. With above number being fixed, the maximum number of bits for N instances could be N*K*SSBRI/CRI = 4*4*6 = 96 bits which seems not overwhelmingly high. Consider the payload size of UCI for CSI reporting, it is possible to insert the N time instances of beam prediction into single beam reporting instance.

- For BM-Case2 with UE-side model, UE reports the predicted Tx beam for multiple future time instances by single reporting instance.

To make training and inference easier to be implemented, the N future time instance(s) could be equally separated in time domain, e.g. 20ms gap between any two adjacent beam prediction instances. In general, NW can be aware of (possibly via configurations) the time domain behavior of M measurement instances, N prediction instances and beam reporting instance. After UE reporting (assuming beam reporting at 0ms), NW and UE can both know which Tx and Rx beam to apply at any given time instance, e.g. at 20ms, 40ms, 80ms, 100ms. This can be done in an implicit manner, rather than reporting explicit time stamps.

- For BM-Case2 with UE-side model, the timestamp of N future time instance(s) could be implicitly reported to NW.

Beam indication for N predicted time instances

For beam indication of BM-Case2, since NW is able to collect a few of Tx beams prediction for future N time instances, it could use legacy beam indication based on legacy TCI state to indicate instance-by-instance. But the DL signaling overhead remains unreduced. To bring more overhead reduction, consider the enhanced beam indication in temporal domain, i.e. one-shot beam indication which carries TCI states for N future time instances.

- For BM-Case2, support to indicate multiple beam indications for future N time instances with one-shot indication.

### Inference at NW side

In RAN1#111, the following agreement on L1 beam reporting to facilitate inference at NW side was achieved.

Agreement

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on the following L1 reporting enhancement for AI/ML model inference

- UE to report the measurement results of more than 4 beams in one reporting instance

- Other L1 reporting enhancements can be considered

Beam reporting of Set B measurement

For BM-Case1 with NW-side model, UE has to report the measurements of Set B to NW as input of AI/ML model instead of legacy beam reporting. As suggested in above agreement, more than 4 beams (e.g. the size of Set B) can be reported within a single reporting instance.

For fixed Set B, both NW side and UE side know the configuration of Set B, hence to save overhead in UCI, it seems possible to drop the part of SSBRIs/CRIs, i.e. keeping only L1-RSRP parts. For variable Set B, the reporting format depends on whether the variable Set B is randomly selected or pre-configured in a pattern. For randomly selected Set B, SSBRIs/CRIs should be reported to NW. But for Set B with pre-configured pattern, it seems feasible to drop SSBRIs/CRIs as well.

With Set B measurement, the model at NW-side infers the Top-K Tx beams among Set A in spatial domain.

- For BM-Case1 with NW-side model, support to reduce the reporting overhead for both fixed or variable Set B, e.g. by dropping the part of SSBRIs/CRIs.

For BM-Case2 with NW-side model, the measurement of Set B contains multiple instances. As in legacy, UE may report in multiple reporting instances. But that’s anyway not aligned with the spirit of overhead reduction for beam management. If possible, UE may assemble multiple measurements (e.g. across N measurement instances) into one beam reporting instance. In addition, the timestamp of the assembled beam reporting could be either explicitly or implicitly reported.

In RAN1#113, the following agreement supports the reporting of measurements from multiple past time instances in a single reporting instance. Of course, next level details on how these measurements of K instances can be packed up and reported with as less overhead as possible should be further discussed and specified.

Agreement

For BM-Case2, study necessity, benefit(s) and potential specification impact from the following additional aspects for AI model inference:

- Reporting information about measurements of multiple past time instances in one reporting instance for BM-Case2

  - Note: only applicable to network-side AI/ML model

- Note: The potential performance gains of measurement reporting should be justified by considering UCI payload overhead

- For BM-Case2 with NW-side model, further study how UE reports multiple instances of Set B measurements into one beam reporting instance.

Beam indication

For Tx beam prediction, NW could apply the legacy beam indication via TCI state. Possibly, the indicated (or let’s say the predicted) beam is not known to UE, e.g. no previous measurement on it. Hence UE may not know how to receive it by which the corresponding Rx beam. Therefore, a DL Rx beam sweeping (P3) procedure seems necessary in an aperiodic manner. But we don’t think it’s necessary to combine or associate the beam indication and DL Rx sweeping procedure, since these two different functions can be supported within current NR framework. More specifically, one DCI, e.g. DCI format 1_1/1_2 could trigger AP CSI-RS transmission and indicate TCI state by two separate DCI fields.

- For Tx beam prediction with NW-side model, it seems not necessary to enhance the signaling, i.e. by combining or associating Tx beam indication and DL Rx beam sweeping.

- For BM-Case1 and BM-Case2 with NW-side model, it seems not necessary to associate beam indication with Rx beam sweeping.

For BM-Case2, NW-side model could predict Top-K Tx beams for multiple future instances. Then it is possible to indicate Tx beams for multiple instances within single beam indication. The target is to further reduce the signaling overhead and signaling latency.

- For BM-Case2 with NW-side model, support the beam indication for multiple future time instances in a single beam indication.

### Assistance information

One of the key aspects of the model input is what to be input for training and inference. The inputs of AI/ML model for BM-Case1 and BM-Case2 were discussed as in the following conclusions. It seems most of alternatives include L1-RSRP measurement based on Set B as input, whereas the other one (i.e. Alt.3 in the 1st conclusion) includes CIR only.

Conclusion

Regarding the sub use case BM-Case1, further study the following alternatives for AI/ML input:

- Alt.1: Only L1-RSRP measurement based on Set B

- Alt.2: L1-RSRP measurement based on Set B and assistance information

  - FFS: Assistance information. The following were mentioned by companions in the discussion:  Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc.

    - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side.

- Alt.3: CIR based on Set B

- Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID

- Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives

- Note2: All the inputs are “nominal” and only for discussion purpose.

Conclusion

Regarding the sub use case BM-Case2, further study the following alternatives of measurement results for AI/ML input (for each past measurement instance):

- Alt.1: Only L1-RSRP measurement based on Set B

- Alt 2: L1-RSRP measurement based on Set B and assistance information

  - FFS: Assistance information. The following were mentioned by companies in the discussion:, Tx and/or Rx beam angle, position information, UE direction information, positioning-related measurement (such as Multi-RTT), expected Tx and/or Rx beam/occasion for the prediction (e.g., expected Tx and/or Rx beam angle for the prediction, expected occasions of the prediction), Tx and/or Rx  beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam pointing angles beam boresight directions (azimuth and elevation), 3dB beamwidth, etc.) , increase ratio of L1-RSRP for best N beams, UE orientation information

    - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side.

- Alt.3: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID

- Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives

- Note2: All the inputs are “nominal” and only for discussion purpose.

To sort various of the assistance information, we understand some of them relates to analog beam implementation at both sides, e.g. Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beam-width, and some information may trigger proprietary concern at UE side, e.g. UE position information, UE direction information, UE orientation information, etc.

In our evaluation, it appears that the input based only on L1-RSRP can provide good enough performance in the aspects of beam prediction accuracy and avg. L1-RSRP difference. It seems that any additional assistance information would only bring marginal performance benefits. Furthermore, in RAN1#112, the following assistant information as input were concluded due to no consensus to be supported in RAN1.

Conclusion

Regarding the explicit assistance information from UE to network for NW-side AI/ML model, RAN1 has no consensus to support the following information

- UE location

- UE moving direction

- UE Rx beam shape/direction

Conclusion

Regarding the explicit assistance information from network to UE for UE-side AI/ML model, RAN1 has no consensus to support the following information

- NW-side beam shape information

  - E.g., 3dB beamwidth, beam boresight directions, beam shape, Tx beam angle, etc.

- Note: Other information (e.g., relative information) of Tx beam(s) preserving sensitive proprietary information is a separate discussion

  - e.g., some information following the same principle of Rel-17 positioning agreement

Hence, we intend to reuse the beam reporting content in NR, e.g. L1-RSRP and Tx beam ID(s), which surely has no issues of exposing beamforming implementation and proprietary information.

- For the assistance information of BM-Case1 and BM-Case2, suggest to

  - Justify the performance benefits if assistance information is used

  - Identify whether the used assistance information would expose proprietary and/or privacy information of either NW-side or UE-side.

### Beam dwelling time

For beam prediction in temporal domain, the study conducted in Rel-18 always assume the fixed time gap between prediction instances. For quite a lot of cases when UE moves in a low speed, the best beam(s) from the 1st prediction instance to the F-th instance can be the same. One may infer that such prediction in temporal domain doesn’t provide any additional useful information, but costing reporting overhead (if UE-side model applied).

From this sense, it could be helpful for the AI/ML model to predict how long a UE would stay on one or more beam(s). We could call it as the beam dwelling time prediction. Though this prediction was not evaluated during Rel-18 SI phase, it is worthy to study and evaluate it in Rel-19 WI phase.

- For temporal domain beam prediction, suggest to study and evaluate the beam dwelling time prediction.

## Life cycle management

### UE-side model and UE/NW monitoring

In RAN1#110bis-e, the agreements on model monitoring was made when AI/ML model deployed at UE side.

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection:

- Atl1. UE-side model monitoring

  - UE monitors the performance metric(s)

  - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation

- Atl2. NW-side model monitoring

  - NW monitors the performance metric(s)

  - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

- Alt3. Hybrid model monitoring

  - UE monitors the performance metric(s)

  - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

When a UE-side AI/ML model applied, the beam prediction results are easily available at UE side. As for DL beam measurement, it is natural for UE to collect the measurement results. Then straightforwardly, UE can monitor the performance of the applied AI/ML model in terms of beam prediction accuracy. That’s the common part between Alt1 and Alt3 in above agreement. The difference part between Alt1 and Alt3 is whether the LCM decision is made by UE or by NW. The benefits of Alt1 is that UE doesn’t have to report anything to NW to make such LCM decision. But as for Alt3, UE has to report at least monitoring results to NW.

For Alt2, to facilitate NW-side model monitoring, UE has to report two types of beam-related information to NW assuming the LCM decision made based on the metric of prediction accuracy. One is the beam prediction results, i.e. Top-K beams from Set A; the other one is the best K Tx beams selected from the measurements of Set A. With both reporting results, NW is able to monitor the model performance and then make LCM decision if needed.

With above being said, we suggest to specify the UE-side model monitoring as a starting point when UE-side model applied.

- For BM-Case1 and BM-Case2 with UE-side model, at least support the (Alt1) UE-side model monitoring.

In RAN1#113, regarding performance monitoring, two agreements were achieved as follows. It’s the duty of UE to calculate performance metrics. Apparently, correct or incorrect beam prediction over one instance cannot reflect whether the AI/ML model functions properly or not. Hence, we believe the event(s) related to AI/ML prediction should be defined. If this type of event happens, the physical layer of UE may notice its upper layer, e.g. MAC layer to get involved in LCM procedure. Similar to beam failure event defined in NR MIMO, one may have similar reason to define another event, let’s call it as model failure event (MFE) for simplicity for now.

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding performance monitoring, study potential spec impact(s) from the following aspects in addition to those included in previous agreements:

- Configuration/Signalling from gNB to UE for measurement and/or reporting

- UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s)

  - FFS: definition of an event and the performance metric(s) used to identify it

- Indication from NW for UE to do LCM operations

- For BM-Case1 and BM-Case2 with UE-side model, study and specify the LCM-related event(s) which would trigger LCM procedure.

### NW-side model and NW monitoring

In RAN1#110bis-e, the agreements on model monitoring was made when AI/ML model deployed at NW side. In RAN1#111, more details were added in the following agreement.

Agreement @ RAN1#110bis-e

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the NW-side model monitoring:

- NW monitors the performance metric(s) and makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

Agreement @ RAN1#110bis-e

Regarding NW-side model monitoring for a network-side AI/ML model of BM-Case1 and BM-Case2, study the potential specification impacts from the following aspects

- Beam measurement and report for model monitoring

- Note: This may or may not have specification impact.

Agreement @ RAN1#111

Regarding NW-side model monitoring for a network-side AI/ML model of BM-Case1 and BM-Case2, study the necessity and the potential specification impacts from the following aspects:

- UE reporting of beam measurement(s) based on a set of beams indicated by gNB

- Signaling, e.g., RRC-based, L1-based

- Note: Performance and UE complexity, power consumption should be considered

When NW-sided AI/ML model adopted by NW, it seems not necessary for UE to monitor the performance.

For NW model to operate, the measurement results of Set B as input to model should be reported by UE. To compare the beam prediction and actual best beams, UE has to report the measurement results of best Tx beams (legacy beam reporting). With both measurement results reported, then NW is able to monitor the performance of AI/ML model. We then see no strong standard impacts specifically introduced by the LCM operation. And the overhead for additional DL measurement and reporting (for NW to compare predicted Top-K beam and measured best K Tx beams) would be increased accordingly.

- For BM-Case1 and BM-Case2 with a network-side AI/ML model and monitoring, there seems no additional specification impact on LCM.

### Performance metric for monitoring

As we evaluated the BM-Case1 and BM-Case2, there could be up to 20% of cases that beam prediction is incorrect when compared with the genie-aided best beam. This beam prediction inaccuracy itself cannot be predicted by NW or UE. It is necessary to monitor the performance in terms of L1-RSRP error, beam prediction accuracy, etc.

One basic assumption to monitor the performance is to compare the beam prediction with legacy beam measurements. Once the model performance is worse than a pre-defined threshold or let’s say it’s not beneficial to adopt the AI/ML model compared with fallback scheme (non-AI legacy beam management schemes), then there should be mechanism(s) for NR system to either switch AI/ML model (to a better one) or fall back to legacy beam operation.

In RAN1#112, the following agreement was made to study the performance metric for model monitoring. In RAN1#112bis-e, further study was achieved as below.

Agreement @ RAN1#112

Regarding the performance metric(s) of AI/ML model monitoring for BM-Case1 and BM-Case2, study the following alternatives (including feasibility/necessity) with potential down-selection:

- Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy

- Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER

- Alt.3: Performance metric based on input/output data distribution of AI/ML

- Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP

- Other alternatives are not precluded

- Note: At least the performance and spec impact should be considered

Agreement @ RAN1#112bis-e

For AI/ML performance monitoring for BM-Case1 and BM-Case2, study potential specification impact of at least the following alternatives as the benchmark/reference (if applicable) for performance comparison:

        Alt.1: The best beam(s) obtained by measuring beams of a set indicated by gNB (e.g., Beams from Set A)

o   FFS: gNB configures one or multiple sets for one or multiple benchmarks/references

        Alt.4: Measurements of the predicted best beam(s) corresponding to model output (e.g., Comparison between actual L1-RSRP and predicted RSRP of predicted Top-1/K Beams)

        FFS:

o   Alt.3: The beam corresponding to some or all the indicated/activated TCI state(s)

        Other alternative is not precluded.

In our understanding, the link quality related KPIs (Alt.2) can be heavily impacted by many factors, such as DL interference, blockage, etc. Therefore, it cannot reflect whether beam (pair) prediction is good enough or not.

For the L1-RSRP difference, it comes from the comparison between measured RSRP and predicted RSRP. RSRP prediction could be somehow independent on Tx beam prediction. In other words, even though RSRP prediction error is large than a threshold, e.g. 3dB or more. The Tx beam prediction accuracy could be as high as expected.

For Alt.3, the LCM mechanisms, i.e. beam comparison between predicted beam and indicated TCI state(s) should be decoupled. As in legacy beam management procedure, there could be multiple instances of beam measurement and reporting, but NW could only indicate one single beam instance regarding to low mobility UE.

For beam prediction accuracy, it needs multiple time instances to collect the statistics. But considering the fact that model change/fall/switch operation should not be carried out in a dynamic way and to avoid ping-pong LCM operation. We tend to think beam prediction accuracy related KPI is applicable for model monitoring.

- On performance metric of AI/ML model monitoring, at least support the beam prediction accuracy related KPIs (Alt.1).

## Data collection for training

### Training and inference on the same side

As mentioned in following agreements, the AI/ML model(s) for inference can be deployed at either NW side or UE side. Assuming the offline trained models, it is by default reasonable to apply the inference at each side.

Agreement

For the sub use case BM-Case1, consider both Alt.1 and Alt.2 for further study:

- Alt.1: AI/ML inference at NW side

- Alt.2: AI/ML inference at UE side

Agreement

For the sub use case BM-Case2, consider both Alt.1 and Alt.2 for further study:

- Alt.1: AI/ML inference at NW side

- Alt.2: AI/ML inference at UE side

For BM-Case1, if the trained model deployed at UE side, UE measures partial DL Tx beams in Set B, predicts the best Top-K beams and reports to NW. As a consequence, the standard impact of deploying the model at UE side during inference phase would be minimum.

For BM-Case1, if the trained model implemented at NW side, the measurements on Set B should be reported from UE to NW. Then NW infers the best Top-K beams. Currently, one beam reporting instance can only support up to 4 Tx beams associated with metrics of L1-RSRP. Hence if the size of Set B is larger than 4, e.g. 8/16/32, UE reporting mechanism should be enhanced accordingly.

- For BM-Case1, deploying AI/ML model at UE side can avoid beam reporting on Set B, therefore resulting in minimum standard impact.

For BM-Case2, Set B could be as large as Set A. Take one example from our evaluation, e.g. Set B contains 32 Tx beam. If the model (e.g. LSTM) needs as many inputs as Set A, i.e. 32 Tx beam, apparently huge beam reporting overhead of Set B can be expected from UE to NW.

- For BM-Case2, deploying UE-side AI/ML model side seems more reasonable, otherwise there could be large beam reporting on Set B when Set B is the same as Set A.

In RAN1#110, the following agreement on where to carry out the training was achieved. Both sides are applicable at current stage.

Agreement

At least for the sub use case BM-Case1 and BM-Case2, support both Alt.1 and Alt.2 for the study of AI/ML model training:

- Alt.1: AI/ML model training at NW side

- Alt.2: AI/ML model training at UE side

Note: Whether it is online or offline training is a separate discussion.

RAN1 discussed the combinations of model training entity and inference entity in RAN1#110 meeting. Obviously, there are 4 combinations to be further checked. Single-side training and inference (e.g. at either NW side or UE side) could be the simplest implementation without model delivery. In RAN1#111, the following agreement was achieved to support at least training and inference at the same side.

Agreement

For the sub use case BM-Case1 and BM-Case2, at least support Alt.1 and Alt.2 for AI/ML model training and inference for further study:

- Alt.1. AI/ML model training and inference at NW side

- Alt.2. AI/ML model training and inference at UE side

- The discussion on Alt.3 for BM-Case1 and BM-Case2 is dependent on the conclusion/agreement of Agenda item 9.2.1 of RAN1 and/or RAN2 on whether to support model transfer for UE-side AI/ML model or not

  - Alt.3. AI/ML model training at NW side, AI/ML model inference at UE side

However, the cross-side training and inference (e.g. model training at NW side and delivered to UE for inference or verse vice) would necessarily involve model transfer. In our understanding, NW pre-trained model could be cell-specific and may face generalization issue when different Rx beam settings for different UEs are adopted.

On the other hand, the UE pre-trained model could be UE-specific, which in our words may seem like “overfitting” to that particular UE only, and may not be applicable to any other UE in the same cell. But it would not affect the prediction performance of the target UE.

Since the model transfer issue including AI/ML beam management will be discussed under AI 9.1.3.3, the argument on the case that AI/ML modeling trained at NW side and delivered to UE side can be suspended in AI 9.1.1 until there is clear conclusion for this issue.

- Whether to support the model transfer for BM-Case1 and BM-Case2 trained at NW side and delivered to UE side, will highly depend on the outcome of agenda item 9.1.3.3.

### UE-side model trained at UE

In RAN1#110, the agreement on data collection for model training was firstly made to study a few of aspects.

Agreement @ RAN1#110

For the data collection for AI/ML model training (if supported), study the following aspects as a starting point for potential necessary specification impact:

- Signaling/configuration/measurement/report for data collection, e.g., signaling aspects related to assistance information (if supported), Reference signals

- Content/type of the collected data

- Other aspect(s) is not precluded

Assuming the simplest model input, i.e. only L1-RSRP measurements of Set B, and the labels of the Top-K Tx beams selected among Set A, then the data set for training can be created. During the DL beam sweeping procedure, current SSB and/or CSI-RS in NR for beam management can be reused.

We next discuss two cases in which the AI/ML model is deployed and trained at the same side. As for the cross-side model training and inference, e.g. UE-side model trained at NW, it’s up to the decision on model transfer discussion.

In RAN1#111, more refined information for UE side data collection was introduced in the following agreement. In RAN1#112bis-e, further progress on UE side model training was achieved.

Agreement @ RAN1#111

Regarding the data collection for AI/ML model training at UE side, study the potential specification impact considering the following additional aspects.

- Whether and how to initiate data collection

- Configurations, e.g., configuration related to set A and/or Set B, information on association/mapping of Set A and Set B

- Assistance information from Network to UE (If supported)

- Other aspect(s) is not precluded

Agreement @ RAN1#112bis-e

Regarding the data collection at UE side for UE-side AI/ML model, study the potential specification impact of UE reporting to network from the following aspect

- Supported/preferred configurations of DL RS transmission

- Other aspect(s) is not precluded

Agreement @ RAN1#112bis-e

Regarding the data collection at UE side for UE-side AI/ML model, study the potential specification impact (if any) to initiate/trigger data collection from RAN1 point of view by considering the following options as a starting point

- Option 1: data collection initiated/triggered by configuration from NW

- Option 2: request from UE for data collection

  - FFS: details

If the AI/ML model is deployed and trained at UE side, current NR beam measurement framework can be reused as a starting point. Assuming Set B is the subset of Set A (BM-Case1), UE only needs to measure Set A. Set B can be determined by taking a subset of Set A as model input and the Top-K beam(s) can be found in Set A as labels. Of course, the relation between Set A and Set B should be configured or indicated to UE at very beginning.

For BM-Case2, Set B can be the same as Set A, but located in different time instances. UE measures Set A/Set B in sequential time instances including measurement and prediction instances. The measurements of Set B in measurement instances can be collected as model inputs and the Top-K beam(s) of Set A in prediction instances can be found as labels.

For offline training, at least tens of thousands of samples should be collected, whereas for fine-tuning of AI/ML model, we tend to believe at least thousands of samples (smaller portion of data than that of initial offline training) should be prepared.

- For BM-Case1 and BM-Case2 with UE-side model training, data set including model inputs and labels can be collected by UE via legacy approach.

As for which side to trigger or initiate data collection, there are two operations under consideration. In our view, since the UE-side model training is conducted by UE, it makes sense for UE to initiate this procedure. Meanwhile, UE also has to report to NW (e.g. via UE capability) how to configure a) Set A (Set B is a subset of Set A) or b) Set A/Set B (Set B is different from Set A).

- For UE-side model training, it is up to UE to request the data collection followed by configuration of Set A and/or Set B from NW.

### NW-side model trained at NW

If the model is deployed and trained at NW side, the data set collected by many UEs should be reported to NW. Specifically, UE has to report the measurement of Set B as model inputs and the L1-RSRPs and/or beam ID(s) of the Top-K beam(s) selected from Set A as labels. By using legacy beam reporting, the labels of Top-K beam(s) (with K up to 4) can be marked by UE and reported to NW. However, as for Set B measurements, it seems beam reporting mechanism should be enhanced accordingly. This is similar to the beam reporting enhancement for NW-side model inference.

- For BM-Case1 and BM-Case2 with NW-side AI/ML model training, the model inputs (via enhanced beam reporting) and labels (via legacy beam reporting) for training can be reported by UE.

In RAN1#112bis-e, the following agreement were made to facilitate NW-side model training.

Agreement

Regarding data collection for NW-side AI/ML model, study the following options (including the combination of options) for the contents of collected data

- Opt.1: M1 L1-RSRPs (corresponding to M1 beams) with the indication of beams (beam pairs) based on the measurement corresponding to a beam set, where M1 can be larger than 4, if applicable

  - FFS: the range of M1

- Opt.2: M2 L1-RSRPs (corresponding to M2 beams) based on the measurement corresponding to a beam set, where M2 can be larger than 4, if applicable

  - FFS: the range of M2

- Opt.3: M3 beam (beam pair) indices based on the measurement corresponding to a beam set, where M3 can be larger than 4, if applicable

  - FFS: the range of M3

- FFS: How to select the M1/M2/M3 beam(s) or beam pair(s)

- Note: Overhead, UE complexity and power consumption should be considered for the above options

Three different operations for collecting data are listed. In our understanding, the data collected at least includes two parts. One part is the measurement results of Set B. If Set B is fixed, then Opt.2 should apply by only reporting M2 L1-RSRPs corresponding to a beam set. The other part is Top-K beam(s) selected from Set A. If predicted L1-RSRP is supported, then UE should report both L1-RSRP and Top-K beam index(es). From this sense, Opt.1 applies for best beam selection with predicted L1-RSRP.

- For NW-side model training, UE reports M2 L1-RSRPs measurements of fixed Set B (via Opt.2) to NW.

- For NW-side model training, UE reports Top-K L1-RSRP(s) and its associated Tx beam index(es) to NW (via Opt.1).

For BM-Case2, the data collected is in time domain. Hence the time stamps can be either explicitly or implicitly reported to NW.

Specifically, for explicit temporal reporting, time stamps should be included in enhanced beam reporting. The temporal order of measurement results is clear, but it comes with the cost of additional reporting. On the other hand, for implicit temporal reporting, the order from the 1st measurement instance to the K-th measurement instances or the order from the 1st prediction instance to the F-th prediction instance can be somehow reflected in the reporting format which can be known by both UE and NW. Hence, the implicit time stamps based reporting can reduce more overhead.

- For BM-Case2, the temporal domain information of collected data could be reported to NW in an implicit manner (no time stamps) by enhancing reporting format.

### Consistency between Set A and Set B

In previous meetings, the consistency between Set A and Set B during training and inference had been discussed. A few of consistency issues have been brought up to the attentions of RAN1. In addition, RAN1 already had the following observation in SI phase.

Observation

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, consistency / association of Set B beams and Set A beams across training and inference is beneficial from performance perspective.

- Note: Whether specification impact is needed is a separate discussion.

The consistency issue between Set A and Set B across training and inference may come from a few of aspects, e.g. QCL assumption between Set A and Set B, ordering of Set A and Set B, Tx beam book consistency. But in our understanding, a lot of those consistency issues can be guaranteed via NW implementation, i.e. proper Set B and/or Set A configuration. For instance, let’s assume the NW-side AI/ML model, the Tx beam book implementation is totally up to NW. We cannot find any solid reason for NW to change this beam book across training and inference phase. It should be a static setting from NW’s point of view.

- To ensure consistency between Set A and Set B across training and inference, it could be up to NW to guarantee the consistency as much as possible.

In terms of Rx beam(s) at UE side, the common understanding is that the Rx beam implementation is up to UE and its proprietary information cannot be exposed to the other side. To maintain the consistency, we could specify a rule, e.g. applying the best/fixed/same Rx beam(s) across training and inference phases. To be more specific in specific language, UE may assume the same rule for QCL assumption across training and inference phases.

- To maintain consistency of QCL assumption across training and inference phases, UE may assume the same rule for QCL assumption, e.g. apply the best/fixed/same Rx beam(s).

## AI/ML related UE capability reporting

It is necessary for UE to report its capability for AI/ML beam prediction. Before inference phase, NW has to configured both Set A (candidate beams for prediction) and Set B (measurements as input of AI/ML model). In our view, how/what NW configures both as Set A and Set B highly depends on UE capability. Let’s take the following example.

For BM-Case1, there could be UE capability on the maximum size of Set B for measurement and corresponding reporting (if model at NW side), and the maximum size of Set A for prediction (if model at UE side). Similar capability also holds for BM-Case2, and additional aspects to consider the temporal domain factor, e.g. maximum measurements instances of Set B and corresponding Set B reporting (if model at NW side), and maximum prediction instances at temporal domain (assuming UE-side model).

Given the early phase of WI, the UE capability on AI/ML beam management can be treated and discussed later when the normalize work of AI/ML for beam management are stable.

- For BM-Case1 and BM-Case2, consider the UE capability on AI/ML beam management at later stage.

# Conclusion

In this section, allow us to repeat our proposals and observations.

- For BM-Case1, configure Set B as a subset of Set A during training and inference phase.

- For BM-Case2, configure the same Set B and Set A during training and inference phase.

- For UE-side model, support NW to configure more than one Set Bs/Set As and then select/activate one pair of associated Set B/Set A.

- For BM-Case2, support the configuration of Set A and/or Set B on a per instance basis.

- For Tx beam prediction (Alt.1) with UE-side model, legacy beam reporting and indication mechanism could serve as starting point.

- Predicted L1-RSRP(s) by UE-side model should be reported to NW along with predicted Top-K Tx beam(s).

- Confidence/probability of UE-side model output could be quantized and reported to NW.

- For BM-Case2 with UE-side model, UE reports the predicted Tx beam for multiple future time instances by single reporting instance.

- For BM-Case2 with UE-side model, the timestamp of N future time instance(s) could be implicitly reported to NW.

- For BM-Case2, support to indicate multiple beam indications for future N time instances with one-shot indication.

- For BM-Case1 with NW-side model, support to reduce the reporting overhead for both fixed or variable Set B, e.g. by dropping the part of SSBRIs/CRIs.

- For BM-Case2 with NW-side model, further study how UE reports multiple instances of Set B measurements into one beam reporting instance.

- For BM-Case1 and BM-Case2 with NW-side model, it seems not necessary to associate beam indication with Rx beam sweeping.

- For BM-Case2 with NW-side model, support the beam indication for multiple future time instances in a single beam indication.

- For the assistance information of BM-Case1 and BM-Case2, suggest to

  - Justify the performance benefits if assistance information is used

  - Identify whether the used assistance information would expose proprietary and/or privacy information of either NW-side or UE-side.

- For temporal domain beam prediction, suggest to study and evaluate the beam dwelling time prediction.

- For BM-Case1 and BM-Case2 with UE-side model, at least support the (Alt1) UE-side model monitoring.

- For BM-Case1 and BM-Case2 with UE-side model, study and specify the LCM-related event(s) which would trigger LCM procedure.

- On performance metric of AI/ML model monitoring, at least support the beam prediction accuracy related KPIs (Alt.1).

- For UE-side model training, it is up to UE to request the data collection followed by configuration of Set A and/or Set B from NW.

- For NW-side model training, UE reports M2 L1-RSRPs measurements of fixed Set B (via Opt.2) to NW.

- For NW-side model training, UE reports Top-K L1-RSRP(s) and its associated Tx beam index(es) to NW (via Opt.1).

- For BM-Case2, the temporal domain information of collected data could be reported to NW in an implicit manner (no time stamps) by enhancing reporting format.

- To maintain consistency of QCL assumption across training and inference phases, UE may assume the same rule for QCL assumption, e.g. apply the best/fixed/same Rx beam(s).

- For BM-Case1 and BM-Case2, consider the UE capability on AI/ML beam management at later stage.

- From signaling aspects, it seems flexible to configure both Set A and Set B via higher layer signaling.

- For Tx beam prediction (Alt.1), the corresponding Rx beam could either be determined by UE as specific Rx beam or by existing Rx beam sweeping procedure.

- The reported predicted L1-RSRP and measured L1-RSRP can be differentiated via separate NW configuration.

- For Tx beam prediction with NW-side model, it seems not necessary to enhance the signaling, i.e. by combining or associating Tx beam indication and DL Rx beam sweeping.

- For BM-Case1 and BM-Case2 with a network-side AI/ML model and monitoring, there seems no additional specification impact on LCM.

- For BM-Case1, deploying AI/ML model at UE side can avoid beam reporting on Set B, therefore resulting in minimum standard impact.

- For BM-Case2, deploying UE-side AI/ML model side seems more reasonable, otherwise there could be large beam reporting on Set B when Set B is the same as Set A.

- Whether to support the model transfer for BM-Case1 and BM-Case2 trained at NW side and delivered to UE side, will highly depend on the outcome of agenda item 9.1.3.3.

- For BM-Case1 and BM-Case2 with UE-side model training, data set including model inputs and labels can be collected by UE via legacy approach.

- For BM-Case1 and BM-Case2 with NW-side AI/ML model training, the model inputs (via enhanced beam reporting) and labels (via legacy beam reporting) for training can be reported by UE.

- To ensure consistency between Set A and Set B across training and inference, it could be up to NW to guarantee the consistency as much as possible.

# References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#94e, Dec. 2021

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#102, Dec. 2023

# Appendix

## General scenarios

The beam prediction was the most widely considered and discussed in the pre-Rel.18 phase. To have more concrete understanding, it is illustrated in Figure 1 where up to 64 Tx beams at NW and 4 Rx beams per panel at UE are deployed. The purpose is to reduce the overhead and latency associated with conventional beam sweeping procedures at FR2.

- : Only a subset of DL beams measured among all DL beams

In our understanding, spatial domain beam prediction can be referred as a super-resolution problem as depicted in Figure 2. In our evaluation, classic deep neural network (DNN), a.k.a. full-connection model could be applicable to fulfill the prediction function.

- : Illustration of beam prediction in spatial domain

For temporal domain beam prediction as illustrated in Figure 3, multiple sequential measurements are collected as inputs to AI/ML model. Afterwards, with the inputs of the past K time instances, the model predicts the best beam(s) for the forthcoming F instances with performance metric (e.g. L1-RSRP). The merit of time domain beam prediction is to avoid or reduce the frequent beam measurement, reporting and indication when UE travels across multiple DL beams coverage. Therefore, the overhead and latency in time domain can be reduced accordingly.

- : BM-Case2 with K = 4 measurement instances and F = 4 prediction instances