3GPP TSG RAN WG1 #116                                                    	 R1-2400622

Athens, Greece, February 26th – March 1st, 2024

Source:	OPPO

Title:	Additional study on other aspects of AI/ML model and data

Agenda Item:	9.1.3.3

Document for:	Discussion

Introduction

In the WID of Rel-19 AI/ML for air interface [1], the following issues are due to additional study with corresponding checkpoints in RAN#105 (Sept ’24):

- Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1]

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection

  - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

- Model transfer/delivery [RAN2/RAN1]:

  - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

In this paper, we will discuss the areas for additional study.

Discussions

Model identification and LCM

Necessity of model ID-based operation in the LCM

Answering the necessity of model identification, the question we first need to answer is whether model ID-based operation is needed in the AI/ML LCM.

In the Rel-18 study, one controversial issue is whether to support both functionality-based LCM and model ID-based LCM. Some companies may propose to only support functionality-based LCM, while deprioritize the study for model ID-based LCM. The consideration may be that all AI/ML operations requiring NW-UE aligned understanding is covered by functionality-based LCM. The finer model activation/switching can be realized with NW and UE implementation without requiring aligned understanding. Only two-sided model requires model ID-based LCM. One-side model does not require model ID-based LCM. However, this proposal assumes that each AI/ML model requiring a NW-UE aligned understanding has a 1-to-1 mapping to an AI/ML functionality.

In Rel-18 study, it was agreed that:

Agreement

Model-ID, if needed, can be used in a Functionality (defined in functionality-based LCM) for LCM operations.

We support the intention of this agreement, which is to design a unified LCM providing both Functionality and Model-ID operations.

We think the AI/ML functionality can be defined with a similar granularity to legacy UE Feature/FG, which from our perspective, is too coarse to support scenario-dependent AI/ML models. If only one model is used for each Feature/FG, the model must have a very good generalization performance. At least for some use cases, using multiple AI/ML models for different scenarios/sites is inevitable.

Even if the legacy Feature/FG can be refined to realize the 1-to-1 mapping to each AI/ML model, the scenarios/configurations/sites for different models are not be easily described in the Feature/FG description. The scenarios/configurations/sites may be implementation details which is not suitable to be specified in the Feature/FG description. For example, an operator may have a large number of gNBs, resulting in a long list of cell IDs which cannot be reported in a UE capability report. UE capability is a quite static report, which is mainly used during the initial access procedure, not for frequent updates.

Hence the small-granularity configurations, e.g., scenarios/configurations/sites should be described in the additional conditions, which are not a part of UE capability for the AI/ML-enabled feature/FG, as agreed in In RAN1#114-bis:

Agreement

For an AI/ML-enabled feature/FG, additional conditions refer to any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG.

- It doesn’t imply that additional conditions are necessarily specified

Under an AI/ML functionality, after the additional conditions are identified for training a model, the additional conditions can be expressed by the ID of the model, or an equivalent ID although it is not called Model ID. Then in the inference procedure, gNB can indicate the ID (e.g. named Model ID or some other name) to activate/select the model corresponding to the additional conditions.

Proposal 1: Support a unified LCM providing both functionality-based and ID-based operations.

- Functionality-based operation is supported by default, in which the granularity of the functionalities is aligned with the Feature/FG in a UE capability report, i.e., conditions.

- An ID can be used on top of functionality for indication of different additional conditions, to support multiple scenarios, configurations, sites, etc. The ID can be named Model ID or some other name.

The necessity of Model identification in the LCM

In Rel-18 study, for how to ensure consistency between training and inference regarding NW-side additional conditions, 4 options were listed in an agreement:

Agreement

For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options can be taken as potential approaches (when feasible and necessary):

- Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side

- Model training at NW and transfer to UE, where the model has been trained under the additional condition

- Information and/or indication on NW-side additional conditions is provided to UE

- Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality)

- Other approaches are not precluded

- Note: it does not deny the possibility that different approaches can achieve the same function.

The model identification (Option 1) is the straightforward approach to achieve the consistency between training and inference. Model ID is the most nature way to indicate an AI/ML model after the training procedure, which is widely used in other AI/ML industries. We do not see any problem on mapping the additional conditions of a model to its model ID, and using the model ID to ensure the consistency for inference procedure. At least for Type A model identification, model ID can be directly used to achieve the consistency, and no extra mechanism is needed.

In Option 3, Information and/or indication on NW-side additional conditions is provided to UE. This approach may be workable. But it is difficult to explicitly indicate all additional conditions every time when activating/selecting a model, because there may be too much additional conditions consuming much signaling overhead. The additional conditions should be combined for signaling, e.g., using an ID for an additional condition combination (e.g. a set of scenarios/configurations/sites). However, in this way, the “additional condition combination ID” actually plays the same role with “model ID”. If so, we prefer to directly use model ID, not additional condition combination ID, because model ID is more straightforward approach which is also used by other AI/ML industries.

Option 2 may be workable. But it depends on whether the standardized model transfer/delivery is supported. We can further consider the option after the related conclusion is made (We will discuss this issue in Section 2.3).

The details of Option 4 need to be further clarified. It seems to require no signaling between gNB and UE. gNB and UE do monitoring separately, and ensure same understanding about the additional conditions relying on they output completely same monitoring results. Our preliminary observation is that the “blind consistency” is hard to be reliable. Some evaluation may be necessary to justify the feasibility of Option 4.

Therefore, we suggest to focus on the study and comparison on Option 1 and 3. Actually we see no major technical difference between the options. As we suggested in Proposal 1, an ID is needed to ensure the consistency. But the name of the ID can be discussed, e.g., Model ID or some other name. If it is named Model ID, Model identification (Option 1) can be the corresponding terminology to describe the procedure ensuring consistency between training and inference regarding NW-side additional conditions. If the ID is not named Model ID, Option 3 can be the better solution. At this moment, we see no problem to use Model ID and Model identification for definition.

Proposal 2: To study the necessity of Model identification, focus on the following two approaches to ensure consistency between training and inference regarding NW-side additional conditions.

- Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side.

- Information and/or indication on NW-side additional conditions is provided to UE.

  - Details needs to be clarified, e.g., types of information/indication and its difference from model ID.

Functionality ID and Model ID

If a unified LCM with functionality and model operation is defined, a detail of the LCM is the ID design for functionality and model.

In Rel-18 study, it was agreed that:

Agreement

- Model ID in RAN1 discussion may or may not be globally unique, and different types of model IDs may be created for a single model for various LCM purposes.

- Note: Details can be studied in the WI phase.

For functionality configuration and indication, functionality ID is a convenient tool for NW and UE. Hence functionality ID can be used for indication functionality between NW and UE.

For model indication, we believe there would be two types of AI/ML model ID to be used for different AI/ML LCM procedures:

- Local ID: AI/ML model ID used in inference procedure, e.g., for indication of model selection/activation/deactivation/switching/fallback

- Global ID: AI/ML model ID used in model transfer and test certification

For AI/ML inference procedure, AI/ML model activation, configuration and switching based on Local ID would be similar to the traditional configuration/indication procedure widely used in 5G NR air interface specifications. In the NR physical procedures, the resource and parameter set which need same understanding between NW and UE can be defined with an ID. Then they can be configured by higher-layer signaling or indicated by DCI. The AI/ML model can be configurated/indicated in a similar way. This is a simple solution and can reuse the legacy procedures as much as possible. Hence we propose to support Local model ID for enabling the same understanding between NW and UE about which model is used for AI/ML inference.

If the Model identification procedure is supported, after the consistency between training and inference regarding additional conditions is achieved via Model identification procedure, a Local ID can be allocated mapped to the additional conditions. And the Local ID can be used to activate and configure the model. The Local ID can be simple number, which may not include the explicit information of the model, e.g., use case, scenario, configuration. The Global ID for model transfer may be also needed. Whether the Global ID need to be defined in 3GPP specification, and how to map the Global ID to Local ID can be further studied.

Proposal 3: Functionality ID can be used for indication functionality between NW and UE.

Proposal 4: At least after Model identification, Local model ID can be a simple number, which is similar to the resource/configuration ID in the legacy NR specification and does not include explicit information about the model, e.g., scenarios/configurations/sites.

Procedure from functionality identification to functionality activation

In previous meetings, the basic framework of AI/ML functionality-based LCM has been agreed. But the accurate procedure from functionality identification to functionality activation is still under discussion. In RAN1#113, the moderator listed two alternatives as below:

Alt 1

- Configurable functionality is synonymous to identified functionality.

- Configured functionalities are determined by NW as a subset of identified functionalities.

- Applicable functionalities are reported from UE as a subset of configured functionalities.

- NW activates one functionality out of applicable functionalities.

Alt 2

- Applicable functionalities are reported from UE as a subset of identified functionalities.

- Configurable functionality is synonymous to applicable functionality.

- Configured functionalities are determined by NW as a subset of applicable functionalities.

- NW activates one functionality out of configured functionalities.

Figure 1: Two alternatives listed in RAN1#113 FL summary [1] for functionality-based LCM

The difference between the two alternatives is about the relation between “configured functionalities” and “applicable functionalities”.

The reason why UE needs to update the UE capability is that the actual UE capability to support an AI/ML functionality may change along with the change of resource, configurations, environment and conditions. For example, the UE’s computation power and storage available for the AI/ML inference may change. Then an AI/ML functionality supported by a UE in last moment may not be supportable in the next moment. Hence UE should be able to update the UE capability on applicable AI/ML functionalities.

On the other side the NW capability (or NW’s interest) may also change. For example, different cells may have different hardware/software configurations. An AI/ML functionality supported by this cell may not be supportable in that cell. And with a large number of AI/ML UEs in the cell, a gNB may not have enough computation power to support all identified AI/ML functionalities to be triggered for all AI/ML UEs. Hence gNB should be able to update the NW capability on applicable AI/ML functionalities by functionality configuration.

The question is: The UE capability update happens before or after the NW functionality configuration?

Alt.1 is the result of updating the UE capability after the functionality configuration. Then the procedure is as below:

(1) Potential AI/ML functionalities supported by NW and UE are identified based on UE’s and NW’s static capabilities;

(2) NW configures a functionality list, which is a sub-set of identified functionalities, according to the NW’s instantaneous interest or capability;

(3) UE updates the UE capability, and forms the applicable functionality list (which is the sub-set of configured functionality list);

(4) NW activates a functionality from the applicable functionality list.

Alt.2 is the result of updating the UE capability before the functionality configuration. Then the procedure is as below:

(1) Potential AI/ML functionalities supported by NW and UE are identified based on UE’s and NW’s static capabilities;

(2) UE updates the UE capability, and forms the applicable functionality list (which is the sub-set of identified functionality list);

(3) NW configures a functionality list, which is a sub-set of applicable functionalities, according to the NW’s instantaneous interest or capability;

(4) NW activates a functionality from the configured functionality list.

In principle, both the two alternatives work. We slightly prefer Alt.2, i.e., the configured functionalities will be the sub-set of applicable functionalities. In 5G system, NW RRC-configuration will be the last step before the dynamic indication (e.g. DCI). So it is more reasonable that after UE reports the applicable functionalities, NW will configure the list of functionalities based on the applicable functionality list.

It can be further studied how dynamic the UE reports the applicable functionalities. If the UE update is too frequent, it may result in an excessive scheduling restriction to gNB. We can consider the UE’s semi-static applicable functionality report.

Proposal 5: The AI/ML functionality identification, configuration and activation procedure can be as below:

- (1) Potential AI/ML functionalities supported by NW and UE are identified based on UE’s and NW’s static capabilities;

- (2) UE updates the UE capability, and forms the applicable functionality list (which is the sub-set of identified functionality list);

- (3) NW configures a functionality list, which is a sub-set of applicable functionalities, according to the NW’s instantaneous interest or capability;

- (4) NW activates a functionality from the configured functionality list.

Procedure from model identification to model activation

Similarly, the procedure from model identification to model activation can be as below:

(1) Potential AI/ML models supported by NW and UE are identified based on UE’s and NW’s static capabilities;

(2) UE updates the UE capability, and forms the applicable model list (which is the sub-set of identified model list);

(3) NW configures a model list, which is a sub-set of applicable models, according to the NW’s instantaneous interest or capability;

(4) NW activates a model from the configured model list.

Proposal 6: The AI/ML model identification, configuration and activation procedure can be as below:

- (1) Potential AI/ML models supported by NW and UE are identified based on UE’s and NW’s static capabilities;

- (2) UE updates the UE capability, and forms the applicable model list (which is the sub-set of identified model list);

- (3) NW configures a model list, which is a sub-set of applicable models, according to the NW’s instantaneous interest or capability;

- (4) NW activates a model from the configured model list.

On UE data collection

Among the two objectives on UE data collection in Rel-19 study, RAN2 will lead the analysis on the UE data collection mechanisms. RAN1 can help to further identify the corresponding contents of UE data collection. However, in Rel-18 study, RAN1 has replied the LS from RAN2 on Data Collection Requirements and Assumptions [2]. We assume RAN2 will carry out their work based on the LS reply, and envision RAN1 will not agree on an updated observation on the UE data collection contents which is obviously different from [2]. Hence we suggest RAN1 to wait for RAN2 progress, and first focus on other study areas. If RAN2 needs further assistance from RAN1, RAN1 can carry out additional study accordingly.

Proposal 7: On UE data collection, RAN1 waits for RAN2 progress on UE data collection mechanisms based on RAN1’s LS reply in Rel-18 study, and can carry out additional study on if RAN2 needs further assistance.

On model transfer

In Rel-18 study, the observation on model delivery/transfer was agreed:

Agreement

For model delivery/transfer to UE (for UE-side models and UE-part of two-sided models):

- Model delivery/transfer to UE, if feasible, may be beneficial to handle scenario/configuration specific (including site-specific configuration/channel conditions) models (i.e., when a single model cannot generalize well to multiple scenarios/configurations/sites), to reduce the device storage requirement.

- Model delivery/transfer to UE after offline compiling and/or testing may be friendlier from UE’s implementation point of view compared to the case without offline compiling and/or testing. On the other hand, the case without offline compiling and/or testing (that can update parameter with known model structure), may have benefit at least in terms of shorter model parameter update timescale.

- For model trained at network side, Case y (w/ network-side training) and Case z2 may incur the burden of offline cross-vendor collaboration such as sending a model to the UE-side and/or compiling a model.

- For model trained at UE side/neutral site, Case z1 and Case z3 may incur the burden of offline cross-vendor collaboration to send the trained model from the UE-side to the network, compared to Case y (w/ UE-side training) which does not have such burden.

- Model storage at the 3gpp network, compared to storing the model outside the 3gpp network, may come with 3gpp network side burden on model maintenance/storage.

- Proprietary design disclosure concern may arise from model training and/or model storage at the network side compared to other cases (such as case y with UE side training) which does not have such issue.

From the observation, the merits of model delivery/transfer in some use cases are identified, e.g., handling scenario/configuration specific models to reduce the device storage requirement. Meanwhile, the agreement also observed some requirements on model delivery/transfer (e.g. requiring offline compiling and/or testing) and problems on model delivery/transfer (e.g. burden of offline cross-vendor collaboration, burden on model maintenance/storage, proprietary design disclosure concern).

The question is that: Comparing between 3GPP standardized and non-standardized model delivery/transfer, which solution can help the industry to accept the burdens and encourage the industry to cop with the problems. From our perspective, non-standardized model delivery/transfer is still a more flexible and feasible solution.

Nowadays the non-AI 5G algorithms are deployed in non-3GPP manner, i.e., the algorithm is developed, tested by each vendor proprietarily. The AI 5G algorithms will evolve/update more frequently/actively than non-AI 5G algorithms. And some AI/ML algorithm (e.g. two-side model) needs close alignment/cooperation between UE and NW. Hence the 3GPP-based model deployment was proposed by some companies, i.e., some model deployment format may be defined in 3GPP standards to enable a more open and interoperable model deployment. The target is to enable Vendor B to compile, test and deploy Vendor A’s AI/ML model in a “plug-and-play” manner. We are open to study the possibility. But so far, we have not seen much details to justify its feasibility.

Although defining a 3GPP-standardized format for model delivery/transfer will make the model delivery/transfer slightly more convenient. But 90% of the burden for offline cross-vendor collaboration cannot be saved. It should be noticed that even the 3GPP-based model transfer is supported, it does not mean the model deployment is naturally 3GPP-based. The deployment phase refers to that after a node receives a model, some engineering operations are required to make the model available to use at that node, e.g. specific optimization, compiling and testing. The model format during transfer procedure (as a 5G control plain package) cannot be directly used for model deployment.

Whether two vendors like to collaborate with each other on a model delivery/transfer still depends on their evaluation on the performance gain the model may bring, evaluation of the acceptable burden they can accept on collaborative compiling, testing and verification. Will two vendors become interested in a model delivery/transfer simply because the delivery/transfer is based on 3GPP signaling? We do not think it is in the case.

It should be noted that the challenges are present at this moment, which will be the problem we strive to solve in future. We are open that the 3GPP standards for AI/ML-based 5G air interface provides multiple options to the industry to realize the model delivery/transfer, including non-3GPP-based and 3GPP-based ones. But a detailed analysis should be developed in Rel-19 study that to what extent the burden of offline cross-vendor collaboration, model maintenance/storage and proprietary design disclosure concern can be resolved by defining a 3GPP-standardized model delivery/transfer. If the burdens/concerns can be substantially reduced, the standardized model delivery/transfer can be supported in Rel-19. From our current perspective, compared with the non-3GPP solution, we only see the 3GPP-standardized model delivery/transfer can only bring a more clear and understandable model delivery/transfer procedure, but is not helpful for the reducing the cross-vendor collaboration.

Proposal 8: To consider the necessity of the standardized model transfer/delivery solutions, a comparison between 3GPP-standardized solution and non-3GPP solution is needed,  for resolving the burden of offline cross-vendor collaboration, burden on model maintenance/storage, proprietary design disclosure concern.

Conclusion

Based on the above discussions, our proposals for general aspect of AI/ML framework are listed below:

Proposal 1: Support a unified LCM providing both functionality-based and ID-based operations.

- Functionality-based operation is supported by default, in which the granularity of the functionalities is aligned with the Feature/FG in a UE capability report, i.e., conditions.

- An ID can be used on top of functionality for indication of different additional conditions, to support multiple scenarios, configurations, sites, etc. The ID can be named Model ID or some other name.

Proposal 2: To study the necessity of Model identification, focus on the following two approaches to ensure consistency between training and inference regarding NW-side additional conditions.

- Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side.

- Information and/or indication on NW-side additional conditions is provided to UE.

  - Details needs to be clarified, e.g., types of information/indication and its difference from model ID.

Proposal 3: Functionality ID can be used for indication functionality between NW and UE.

Proposal 4: At least for LCM with non-3GPP-based model transfer, Local model ID can be a simple number, which is similar to the resource/configuration ID in the legacy NR specification and does not include explicit information about the model, e.g., scenarios/configurations/sites.

Proposal 5: The AI/ML functionality identification, configuration and activation procedure can be as below:

- (1) Potential AI/ML functionalities supported by NW and UE are identified based on UE’s and NW’s static capabilities;

- (2) UE updates the UE capability, and forms the applicable functionality list (which is the sub-set of identified functionality list);

- (3) NW configures a functionality list, which is a sub-set of applicable functionalities, according to the NW’s instantaneous interest or capability;

- (4) NW activates a functionality from the configured functionality list.

Proposal 6: The AI/ML model identification, configuration and activation procedure can be as below:

- (1) Potential AI/ML models supported by NW and UE are identified based on UE’s and NW’s static capabilities;

- (2) UE updates the UE capability, and forms the applicable model list (which is the sub-set of identified model list);

- (3) NW configures a model list, which is a sub-set of applicable models, according to the NW’s instantaneous interest or capability;

- (4) NW activates a model from the configured model list.

Proposal 7: On UE data collection, RAN1 waits for RAN2 progress on UE data collection mechanisms based on RAN1’s LS reply in Rel-18 study, and can carry out additional study on if RAN2 needs further assistance.

Proposal 8: To consider the necessity of the standardized model transfer/delivery solutions, a comparison between 3GPP-standardized solution and non-3GPP solution is needed,  for resolving the burden of offline cross-vendor collaboration, burden on model maintenance/storage, proprietary design disclosure concern.

References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm (Moderator), 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023

- R1-2310681, Reply LS on Data Collection Requirements and Assumptions, TSG RAN WG1, 3GPP TSG-RAN WG1 Meeting #114bis, Xiamen, China, October 9th – 13th, 2023