3GPP TSG RAN WG1 #116		R1-2400624

Athens, Greece, February 26th – March 1st, 2024

Source:	OPPO

Title:	CSI enhancements for Rel-19 MIMO

Agenda Item:	9.2.2

Document for:	Discussion and Decision

Introduction

In RAN#102 meeting, the following scope was agreed for CSI enhancement in Rel-19 MIMO WID[1]:

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1

  - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

In this contribution, we discuss these CSI enhancements for Rel-19 MIMO.

Discussion

  - CSI enhancement for up to 128 CSI-RS ports

- CSI-RS configuration

To support 128 CSI-RS ports for Type I and Type II codebook, up to 4 CSI-RS resources are needed according to the WID and up to 32 ports per resource is supported. A straightforward CSI-RS configuration is reusing the mechanism of 4 CSI-RS resources in one set, e.g., CSI-RS configuration in Rel-18 CJT codebook. Besides, the mapping of CSI-RS port index to codebook can also follow Rel-18 CJT CSI enhancement.

Proposal 1: Reuse legacy mechanism (e.g., R18 CJT CSI) to configure 4 CSI-RS resources in one set for up to 128 ports transmission.

- Type I and eType II Codebook enhancement

One essential issue for Type I codebook is (N1, N2) configuration.  For larger number of antenna ports, 64, 96 and 128 ports can be considered. Accordingly, the following (N1, N2) combinations can be supported: (16, 4), (8, 8), (12, 4), (8,6), (8, 4), (16, 2). The other combinations are not precluded. For eType II codebook, the same (N1, N2) as Type I is preferred.

With increasing antenna ports, the number of orthogonal beams is increasing with narrow beamwidth, and there may not need to consider large oversampled SD basis. Furthermore, the UE complexity and feedback overhead will be increased accordingly. In this case, smaller oversampling factor can be considered.

Proposal 2: At least consider (N1, N2) from {(16, 4), (8, 8), (12, 4), (8,6), (8, 4), (16, 2)} for Type I and Type II codebook with 64/96/128 CSI-RS ports.

Proposal 3: For Type I codebook, consider to reduce over sampling factor for up to 128 ports codebook.

Regarding codebook structure, fully reusing Type I codebook design (either based on simple panel or multi-panel codebook) is straightforward and can be considered as baseline. Moreover, refining Type I codebook based on eType II scheme can be considered, such as SD basis selection and/or precise quantization, without FD basis reporting. For eType II codebook for up to 128 ports, it can be further studied whether additional values of parameter combination {L, beta, pv} is needed, with newly added {N1, N2}.

Proposal 4: The current Type I CB and eType II CB can be reused as much. Further study enhancement of SD basis selection and quantization for Type I codebook and additional parameter combinations for eType II codebook.

study

- Multi-CRI based CSI feedback

For multi-CRI based CSI feedback, multiple CRIs can be reported with corresponding RI/PMI/CQI. The feedback is similar to Rel-17 NC-JT CSI. There are two essential issues to be considered:

- The supported codebook type. For NC-JT CSI, only type I codebook is supported considering the feedback overhead for multiple TRPs. Also, CRI is not reported for legacy eType II codebook. For multi-CRI based feedback, more CSIs need to be reported than NC-JT, and the feedback overhead would be a larger issue if eType II codebook is supported. We propose to support Type I codebook as baseline for multi-CRI based feedback, and further consider eType II codebook if the time and feedback overhead is available.

- How to exploit the common part of the multiple RI/PMI/CQI to reduce the feedback overhead. Similar to NC-JT CSI, considering the number of CWs, joint CQI needs to be introduced for layers mapped to one CW. For multi-CRI based CSI feedback, if the total rank is not larger than 4, only one CQI is needed, and two CQIs for more than 4 layers. Furthermore, to simplify the design, common RI across CRIs can also be considered. To limit the complexity and overhead, the number of layers for each CRI should be restricted, e.g., not larger than 2.

Proposal 5: Type I codebook can be baseline to support multi-CRI based CSI feedback. Further consider eType II codebook if time and overhead allows.

Proposal 6: Consider joint design of RI and CQI for multi-CRI based feedback.

  - Time/frequency misalignment reporting for CJT

In Rel-18, the enhancement of CJT transmission was based on the assumption of ideal synchronization. However, depended on actual deployment, the ideal time/frequency synchronization is difficult to be achieved in some scenarios. In Rel-19, time/frequency misalignment reporting is introduced to ensure the CJT performance.

For frequency error, though multiple TRPs can be assumed to be at the same carrier frequency, oscillator stability would still lead to frequency difference between sites. In 38.104, the maximum frequency error for a base station is specified as below, which means the relative frequency of the signal received from different TRPs can be up to 0.2ppm changing over 1ms. For example, for carrier frequency of 4GHz, the maximal frequency difference observed at UE could be up to 0.8kHz. Considering CJT UEs are not expected to be at high mobility, doppler shift is relatively small compared to the frequency error and is not considered here.

#### 6.5.1.2 Minimum requirement for BS type 1-C and BS type 1-H For BS type 1-C and BS type 1-H, the modulated carrier frequency of each NR carrier configured by the BS shall be accurate to within the accuracy range given in table 6.5.1.2-1 observed over 1 ms. The frequency error requirement for NB-IoT are specified in TS 36.104 [13] clause 6.5.1. Table 6.5.1.2-1: Frequency error minimum requirement BS class; Accuracy. Wide Area BS; ±0.05 ppm. Medium Range BS; ±0.1 ppm. Local Area BS; ±0.1 ppm.

For CJT, the time synchronization error may come from two aspects: the asynchronization between TRPs and the propagation delay (spread) between different TRPs and a UE. For TRP asynchronization, there is timing requirement for TDD in 38.133 that the time phase synchronization accuracy at gNB should be better than 3 µs. It can be further discussed whether this synchronization inaccuracy needs to be fully considered or not. For FDD, there is no such requirement, and the TRPs can be assumed to be synchronous. Furthermore, due to propagation delay and different delay spreads of TRPs (e.g. LOS for one TRP and NLOS for another TRP), there can be delay offset between signal received from different TRPs even with synchronous TRPs. However, considering CJT UEs is usually at the cell edge of multiple TRPs, the propagation delay including delay spread is not expected to very large. Whether the delay offset can be larger than CP needs further discussion/evaluation.

Proposal 7: The maximal frequency misalignment among TRPs observed at UE can be assumed to be ~0.2ppm.

Proposal 8: Discuss and make decision on the following issues for time misalignment:

-  How to assume time asynchronization among TRPs especially for TDD (e.g. the 3us requirement)

- Whether the maximal delay offset among synchronous TRPs can be larger than CP or not

For measurement of time/frequency misalignment at UE, DL RS should be configured for each TRP. TRP-specific CSI-RS can be used similar to TDCP measurement. It can be further studied whether current RS (e.g. TRS) can be reused for the measurement. For report of the synchronization error, new report quantity should be defined. UE can be configured to report only time misalignment or only frequency misalignment or both. The measurement can be based on the same CSI-RS resources.

For time misalignment reporting, UE can directly report the time/delay offset between TRPs based on the measured DL RS. For example, UE can measure the timing of each TRP based on the phase change among different frequency resources of the corresponding RS, and derive the time misalignment according to the TDOA between TRPs. Alternatively, UE can report the relative phase offset (relative to reference TRP) in some frequency resource unit (e.g. PRB), and gNB can directly compensate the phase for frequency alignment. Similarly, for frequency misalignment reporting, UE can directly report the frequency offset (e.g. in Hz) between TRPs, or the relative phase offset (relative to reference TRP) in some time resource unit (e.g. symbol).

Proposal 9: For time misalignment reporting, either time offset or phase offset (in frequency) based report can be considered.

Proposal 10: For frequency misalignment reporting, either frequency offset or phase offset (in time) based report can be considered.

In Rel-18, the CSI enhancement for CJT focused on eType II codebook. For TDD, reciprocity based CJT transmission could also provide gain over S-TRP without additional specification impact, e.g. based on current CSI framework. Current antenna calibration is performed per TRP, and there could be amplitude/phase misalignment among antennae in different TRPs. However, the performance of reciprocity based CJT transmission (without or without calibration error) has not been widely evaluated in RAN1. The performance gain of inter-TRP antenna calibration is still unclear and needs further justified.

Proposal 11: Reporting for inter-TRP antenna calibration for TDD needs further justified.

Conclusion

In this contribution, we discuss potential enhancements for Rel-19 MIMO enhancement.

Proposal 1: Reuse legacy mechanism (e.g., R18 CJT CSI) to configure 4 CSI-RS resources in one set for up to 128 ports transmission.

Proposal 2: Consider (N1, N2) from {(16, 4), (8, 8), (12, 4), (8,6), (8, 4), (16, 2)} for Type I and Type II codebook with 64/96/128 CSI-RS ports.

Proposal 3: For Type I codebook, consider to reduce over sampling factor for up to 128 ports codebook.

Proposal 4: The current Type I CB and eType II CB can be reused as much. Further study enhancement of SD basis selection and quantization for Type I codebook and additional parameter combinations for eType II codebook.

Proposal 5: Type I codebook can be baseline to support multi-CRI based CSI feedback. Further consider eType II codebook if time and overhead allows.

Proposal 6: Consider joint design of RI and CQI for multi-CRI based feedback.

Proposal 7: The maximal frequency misalignment among TRPs observed at UE can be assumed to be ~0.2ppm.

Proposal 8: Discuss and make decision on the following issues for time misalignment:

-  How to assume time asynchronization among TRPs especially for TDD (e.g. the 3us requirement)

- Whether the maximal delay offset among synchronous TRPs can be larger than CP or not.

Proposal 9: For time misalignment reporting, either time offset or phase offset (in frequency) based report can be considered.

Proposal 10: For frequency misalignment reporting, either frequency offset or phase offset (in time) based report can be considered.

References

- RP-234007, 3GPP RAN#102 meeting, Samsung (Moderator), Edinburgh, Scotland, December 11-15, 2023.

- 38.104 v18.4.0

- 38.133 v18.4.0