3GPP TSG RAN WG1 #116	 	                                     R1-2400626

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Source:	OPPO

Title:	Enhancements on asymmetric DL STRP/UL MTRP scenarios

Agenda Item:	9.2.4

Document for:	Discussion

# Introduction

The WID [1] of NR MIMO evolution Phase 5 was agreed in RAN#102 meeting. The objectives related to this agenda item is listed below

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

In this contribution, we present our view on the enhancement of asymmetric DL sTRP and UL mTRP scenario.

# Discussions on asymmetric deployment

The deployment of heterogeneous networks (HetNets) is a strategy to improve user experience by combining macro cells with smaller cells (e.g., micro, pico, and femtocells). This setup caters to the differing needs of users and helps improve overall network coverage and capacity, particularly for uplink (UL) throughput.

This scenario provides a strategic advantage in network topology, catering to dense urban environments where uplink data traffic is significantly high. By offloading the uplink communication to dedicated UL-only TRPs, NW can efficiently manage the increased uplink traffic without overburdening the regular TRP, which manages both uplink and downlink. Such a deployment scenario is instrumental in improving uplink throughput and enhances network capacity and coverage.

## Scenarios and assumptions

The basic scenario of asymmetric deployment is illustrated in Figure 1. UE is engaged in downlink communication with a regular TRP capable of both downlink and uplink transmission. However, for UL communication, the UE targets a separate TRP, designated as an UL-only TRP.

- : the asymmetric deployment scenario in which a UE communicates with one single TRP for DL and with one or more TRPs for UL

In the WID [1], the assumptions for the enhancement in this agenda item was explicitly listed in what follows. From our point of view, those assumptions underlying this deployment are constrained to ensure focus and manageability in standardization efforts.

  - DL sTRP/UL mTRP deployment

  - Intra-band & intra DU & non-co-located mTRP

  - No definition change on cell

  - Reuse Rel-17/18 unified TCI framework

  - Applicable to both FR1 and FR2

## Benefits and objectives

The deployment of a heterogeneous network with asymmetric DL sTRP and UL mTRP brings several benefits to both NW and UE.

- Increased UL Throughput: By enabling UEs to select a UL-only TRP with potentially higher signal quality or lower pathloss for UL transmissions, NW can provide better UL throughput. This can be particularly beneficial in dense urban environments or indoor scenarios, where UEs might be closer to UL-only TRP that can handle UL traffic more effectively than a distant regular TRP, resulting in improved data rates.

- Enhanced Coverage: The ability for UEs to transmit to different types of nodes (regular TRP or UL-only TRP) can lead to improved coverage, especially at the cell edges or in areas where macro cell signals are weak. By leveraging multiple points of reception, the UL coverage can be significantly enhanced.

- Energy Efficiency: The selective use of UL-only TRP for UL transmissions allows for reduced power usage by UE since a closer transmitter would require less power to achieve the necessary signal quality at the TRP. Additionally, UL-only TRP can minimize or shut down DL transmissions when not needed, which saves energy at NW infrastructure.

- Offloading Traffic: Offloading UL traffic from regular TRP to UL-only TRP can alleviate congestion on regular TRP, especially during peak times. This can lead to a better balance of network load and ensure that the resources of regular TRP are used more efficiently.

- Cost-Effective Network Densification: Deploying UL-only TRPs as part of an asymmetric UL/DL scenario can be a cost-effective way to increase network density and capacity, compared to deploying additional regular TRPs.

The objectives are clearly listed in the sub-bullet of this WID. Those objectives with specification impact are

- Two closed-loop PC adjustment states for SRS, both separate from PUSCH

- Pathloss offset configurations for pathloss calculation to UL-only TRP

# Enhancements on dynamic power control for SRS

In the asymmetric deployment scenario, UE may transmit SRS towards multi-TRP including regular TRP and UL-only TRP.

However, in legacy releases, dynamic power control for SRS was inherited from the design of single TRP, i.e. single close-loop power control state. Specifically, the close-loop index (CLI) was not independently introduced along with the TPC command for SRS, e.g. in DCI format 2_3. But for PUSCH and PUCCH, two separated close-loop power control states have been already supported and SRS can be configured to follow that of PUSCH.

According to the objective, it is time to specify two close-loop power control states for SRS has come. Particularly, it is for the case when SRS is configured not to follow the close-loop power control states of PUSCH.

## Two closed-loop PC adjustment states for SRS

To enable two closed-loop power control states, the DCI carrying dynamic TPC should also indicate which closed-loop PC state the TPC associates with. From this sense, we have two signaling designs.

The first option is that directly inserting CLI into DCI, i.e. DCI format 2_3, along with TPC and SRS request. Then UE can understand which closed-loop PC state the TPC targets for. Of course, the formula of calculating transmission power of SRS in TS 38.213 should be changed accordingly. At least the CLI, i.e. (i0 or i1) should be reflected in the formula below when SRS is configured not to follow power adjustment of PUSCH.

hb,f,ci,l=hb,f,ci-i0,l+m=0CSi-1δSRS,b,f,c(m,l)

The second option is to associate CLI with indicated UL/joint TCI state. Considering the R17/R18 TCI framework assumed for this working item, CLI (same as P0, alpha and PL RS) can be configured within each UL/joint TCI state. For both regular TRP and UL-only TRP, each TRP would be associated with one indicated UL/joint TCI state. Then DCI (e.g. format 2_3) for power adjustment can indicate the index of indicated UL/joint TCI state, i.e. the 1st (k = 0) or 2nd (k = 1) indicated UL/joint TCI state to be associated with the corresponding TPC. Then UE takes the TPC and applies it to the PC adjustment state associated with the referred TCI state.

Moreover, similar changes should be conveyed in TS 38.213 as below where k = {0, 1} stands for the 1st or 2nd indicated UL/joint TCI state respectively.

hb,f,ci,k=hb,f,ci-i0,l+m=0CSi-1δSRS,b,f,c(m,k)

Note that for the second option, it applies to the SRS configured to follow the indicated UL/joint TCI state.

- To support two closed-loop power control adjustment states for SRS, discuss and decide at least the following two options

- Opt.1: directly insert CLI along with TPC command for SRS

- Opt.2: associate CLI with indicated UL/joint TCI state for SRS

Let’s next look at SRS for the case of CA. From our understanding, a UE may transmit SRS to either single CC or multiple CCs to pilot UL channel. Each CC can be indicated with an independent TPC. We believe the two separate close-loop power control adjustment states should be applied on a per CC basis. Hence, for both cases of srs-TPC-PDCCH-Group = typeA (multiple CC) and srs-TPC-PDCCH-Group = typeB (single CC), the separate closed-loop PC for single CC RAN1 will agree should be reused for multiple CCs.

- Support two closed-loop power control adjustment states for SRS when srs-TPC-PDCCH-Group = typeA or typeB.

## Extended delta value for TPC command

The existing value in Table 7.1.1-1 of TS 38.213 can provide up to 3 (accumulated) or 4 dB (absolute) Tx power change per TPC command. To achieve fast power control adjustment for SRS, bigger steps can be considered by adding more step values into Table 1.

Table 7.1.1-1: Mapping of TPC Command Field in a DCI format scheduling a PUSCH transmission, or in DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, or in DCI format 2_3, to absolute and accumulated δPUSCH,b,f,c values or δSRS,b,f,c values

| TPC Command Field | Accumulated δPUSCH,b,f,c or δSRS,b,f,c [dB] | Absolute δPUSCH,b,f,c or δSRS,b,f,c [dB] |
| --- | --- | --- |
| 0 | -1 | -4 |
| 1 | 0 | -1 |
| 2 | 1 | 1 |
| 3 | 3 | 4 |

The new values as examples in Table 1 can be found in red.

- Mapping of TPC Command Field in a DCI format 2_3 to absolute and accumulated δPUSCH,b,f,c values or δSRS,b,f,c values (with newly added values)

| TPC Command Field | Accumulated δPUSCH,b,f,c or δSRS,b,f,c [dB] | Absolute δPUSCH,b,f,c or δSRS,b,f,c [dB] |
| --- | --- | --- |
| 0 | -6 | -10 |
| 1 | -3 | -7 |
| 2 | -1 | -4 |
| 3 | 0 | -1 |
| 4 | 1 | 1 |
| 5 | 3 | 4 |
| 6 | 6 | 7 |
| 7 | 9 | 10 |

- Consider to add new delta values for closed-loop power control for SRS to achieve faster power control adjustment.

# Introducing PL offset(s)

For UL-only TRP, since no DL RS transmitted, UE cannot measure its path loss (PL) from the UL-only TRP. In legacy, the UE can measure the PL from the regular TRP. Hence, a PL offset (based on the PL RS from regular TRP) should be introduced according to the WID.

## Measurement on the PL offset between regular TRP and UL-only TRP

The first question that comes to our mind is that how could NW configure the PL offset with respect to one PL RS, in the asymmetric deployment scenario. The intuitive way is to take advantages of the channel reciprocity. Specifically, the UL PL can be used as DL PL. A UE can transmit SRS toward both UL-only TRP and regular TRP. Then it is up to NW side to estimate two PLs and then to calculate the PL gap which can be configured/used as PL offset.

For instance, UE sends one SRS resource to regular TRP which measures the received power. In addition, the UE can also report Type 3 PHR (for SRS) including Pc,max and power headroom, which can be helpful in determining the exact Tx power of SRS at regular TRP. With above information, regular TRP can derive the UL PL from UE to regular TRP.

On the other hand, UE sends another SRS resource to UL-only TRP. Similarly, the UL-only TRP can measure the received power of SRS. But at this stage of determining PL offset(s), the Tx power of the SRS cannot be calculated given the formula in TS 38.213. Hence, it is necessary to specify a pre-defined Tx power for SRS, such that both UE and UL-only TRP can know the Tx power for calculating the UL PL between UE and UL-only TRP.

With two path losses (one between UE and UL-only TRP and the other between UE and regular TRP), the PL offset(s) can be derived. It facilitates NW side to configure single or multiple PL offset(s) for UL transmission toward to UL-only TRP.

- SRS can be reused for NW to measure the UL path loss gap from UE to regular TRP and from UE to UL-only TRP.

- Study and define a mechanism to determine the Tx power of SRS toward UL-only TRP.

## PL offset(s) configuration

Once NW obtains the PL gap from UE to regular TRP and to UL-only TRP, it is now facilitated to configure the PL offset(s). Higher layer signaling-wise, the PL offset(s) can be configured in UL/joint TCI state along with the referred PL RS. The UL/joint TCI state applies to the UL-only TRP.

- Support to configure the PL offset(s) along with the associated PL RS in the UL/joint TCI state which applies to the UL-only TRP.

By checking the current IE of UL/joint TCI state in TS 38.331, the power control parameters P0, alpha and CLI can be configured within TCI state on a per channel/signal basis. But there is only one PL RS configured with each UL/joint TCI state. From this sense, we tend to think one common PL offset for PUSCH/PUCCH/SRS should be sufficient.

- PL offset(s) can be commonly configured in UL/joint TCI state for PUSCH/PUCCH/SRS toward UL-only TRP.

## PL offset activation/indication

Since UE may roam unpredictably within a cell or across multiple cells, the PL offset configuration per UL/joint TCI state seems static and thus insufficient to facilitate UE mobility.

Let’s take the scenario of Figure 2 as an example, where a UE moves away from the regular TRP under the same DL beam coverage (implying the same PL RS). Due to the change from UE position 1 to position 2, the proper PL offset should also be changed/switched accordingly, i.e. from PL offset 1 to PL offset 2.

To facilitate intra-cell beam-level mobility, it is necessary for NW to switch PL RS and/or PL offset. For this case, it is surely undesirable to involve the RRC reconfiguration to adjust the Tx power toward UL-only TRP (configuring another PL RS and/or PL offset which works more suitably).

- : the intra-cell mobility scenario in which the suitable PL offset may change given the same PL RS

To address this issue, more dynamic signaling aspects should be considered. Specifically, RRC signal could configure multiple PL offsets along with PL RS on per UL/joint TCI state basis. Then NW could adopt one MAC CE to activate one PL offset from the configured ones for the associated PL RS.

To change the PL offset in an even faster way, based on RRC configuration of multiple PL offsets per UL/joint TCI state, MAC CE could activate more than one PL offsets for more dynamic selection. Based on the activated PL offsets by MAC CE, NW could send a DCI to pick up one PL offset according to the latest measurement.

- Study and specify (if necessary) the mechanism to dynamically change the PL offset for UL-only TRP.

- MAC CE based approach

- MAC CE + DCI based approach

# Conclusion

In this section, allow us to repeat our proposals

- To support two closed-loop power control adjustment states for SRS, discuss and decide at least the following two options

- Opt.1: directly insert CLI along with TPC command for SRS

- Opt.2: associate CLI with indicated UL/joint TCI state for SRS

- Support two closed-loop power control adjustment states for SRS when srs-TPC-PDCCH-Group = typeA or typeB.

- Consider to add new delta values for closed-loop power control for SRS to achieve faster power control adjustment.

- SRS can be reused for NW to measure the UL path loss gap from UE to regular TRP and from UE to UL-only TRP.

- Study and define a mechanism to determine the Tx power of SRS toward UL-only TRP.

- Support to configure the PL offset(s) along with the associated PL RS in the UL/joint TCI state which applies to the UL-only TRP.

- PL offset(s) can be commonly configured in UL/joint TCI state for PUSCH/PUCCH/SRS toward UL-only TRP.

- Study and specify (if necessary) the mechanism to dynamically change the PL offset for UL-only TRP.

- MAC CE based approach

- MAC CE + DCI based approach

# References

- RP-234007, New WID: NR MIMO Phase 5, Samsung, RAN#102, Dec. 2023