3GPP TSG RAN WG1 #116                                  			               R1-2400629

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.11.4

Source:	InterDigital, Inc.

Title:	IoT-NTN uplink capacity/throughput enhancement

Document for:	Discussion

# Introduction

In RAN #102 meeting, the WI for R19 IoT NTN [1] is approved.

For UL capacity enhancement, IoT NTN WID has an objective to improve the UL capacity. The justification for this objective is in the following:

Need for Uplink capacity enhancement: NB-IoT NTN is already being deployed live at this very moment. In these early and upcoming deployments, it is clearly emerging that IoT-NTN, in particular NB-IoT, will have to support massive capacity in terms of number and types of UE, some of which with worse characteristics than others (e.g. low cost devices, wearables, etc). Multiplexing of UEs by usage of orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH should therefore be studied and if beneficial being specified. Therefore, in order to unlock the additional UL capacity potential, there is a need to identify methods to de-couple the UL from the DL as much as possible.

This justification is linked to the following objective:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2] - Multi-tone support for 15 kHz SCS should also be considered Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction [RAN2]: - Msg3 transmission without msg1/RAR - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In this contribution, we discuss some aspects of UL capacity enhancement for IoT NTN devices.

# Need for Uplink Capacity Enhancement for IoT NTN

Wide Beams in Satellites:

For satellite based communications, due to large distance between the satellites and the devices on the earth, the beams become very wide. Thus, cell coverage spans many dozens of kms for LEO and may even span hundred of kms for GSO.

Large Number of UEs per Beam:

Due to large coverage per satellite beam, and the exploding number of devices with varying capabilities and categories, the number of devices per beam may be very large. This amounts to very limited resource availability per device in NTN networks.

Support of Low-cost IoT Type of Devices:

NTN networks already support IoT operation, based upon NB-IoT and eMTC devices. With proliferation of satellite constellations and more devices in the field equipped with IoT NTN capabilities, this leads to a large number of devices, many thousands per sq km present in the coverage of a satellite beam.

Spectrum Scarcity:

Although NTN is supporting new device categories and new device types, the spectrum available for these devices is not scaling up. Thus, to avoid spectrum being bottleneck for communication in NTN, the devices need to be simultaneously supported for UL transmissions which is one of the major use cases in NTN.

Observation 1: Wide satellite beams, large number of UEs, IoT support and spectrum scarcity all lead to very limited uplink resource availability per UE for uplink transmission.

Uplink Coverage Enhancements with large number of repetitions:

Due to limited Tx power of the IoT devices and very large propagation distance, the uplink channels have always been considered a bottleneck for NTN networks.

For devices based on NB-IoT and eMTC, a lot of PUSCH repetitions are done typically to achieve successful detection at the network nodes even for terrestrial networks.

Observation 2: Large number of PUSCH repetitions inherent to NB-IoT and eMTC UL transmissions lead to very poor uplink throughput and capacity.

# Uplink Capacity Enhancement for IoT NTN

Rel-19 WID for IoT NTN [1] has specified the objective of uplink capacity enhancement through the use of orthogonal cover codes if found beneficial. We discuss the capacity enhancement for NPUSCH and NPRACH in the following sub-sections.

## OCC for NPUSCH Format 1

The NPUSCH is dedicated to the transmission of data and control information from the UE side. In this respect, two formats are supported: Format 1 for data and Format 2 for control information (i.e., ACK/NACK). Rel-19 scope for IoT NTN UL capacity enhancement is limited to NPUSCH Format 1. NPUSCH Format 1 supports both single tone and multi-tone based transmissions. Single tone transmissions can use 3.75 kHz and 15 kHz SCS whereas multi-tone transmissions can only use 15 kHz SCS.

3GPP introduces a concept of “Resource Unit (RU)” for both single tone and multi-tone transmissions in NB-IoT. It defines the ”resource unit (RU)”, which represents the smallest unit that can be transmitted in uplink regardless of the subcarrrier spacing being used and depends only on the transmission option (i.e., single-tone or multi-tone). For the single-tone transmissions using 15 kHz or 3.75 kHz spacing, one RU will have exactly the same number of REs (112 REs). For multi-tone transmissions, one RU will have 168 REs.

In the case of single-tone transmission, using a subcarrier spacing of 3.75 kHz or 15 kHz, the repetition process is the same for both SCSs. Thus, if NRU x NUL slots of NPUSCH data are to be transmitted where NRU is the number of resource units and NUL is the number of slots in the resource unit, and NPUSCH is scheduled to be repeated NRep times, the process is carried out as follows: One repetition comprising of NRU x NUL slots of data are first transmitted, then another NRU x NUL slots are transmitted and so on up to the NRep repetitions. For single tone transmission, the most straight-forward approach would be to apply the orthogonal cover code at the repetition level where each repetition will span NRU x NUL slots.

Proposal 1: For NPUSCH Format 1 single tone transmissions, OCC is applied at transport block repetition level.

For multi-tone transmissions, the repetitions are done different to single tone transmissions. For the case of multi-tone transmissions, the first two slots among the scheduled/assigned resource units are repeated fully or partially and then next two slots, and so on. One method to increase capacity for multi-tone transmission could be to apply the orthogonal cover code to each repetition unit of two slots. This would though require that different UEs who share the same time frequency resource using different cover codes are scheduled with the same repetition parameters.

Proposal 2: For NPUSCH Format 1 multi tone transmissions, OCC is applied at repetition unit level (e.g., 2 slots).

## OCC for NPRACH

The NPRACH is dedicated to the transmission of the NB-IoT preamble which is the first transmitted signal by the UE towards the eNB in order to request access to the network. Each preamble is composed of four groups of symbols. The subcarrier spacing used for the transmission of NPRACH is equal to 3.75 kHz. In order to reduce the probability of collision between the preambles transmitted from different UEs in a cell, the mechanism of frequency hopping is used. In NB-IoT, different coverage enhancement levels may be defined. For each CE level, broadcast system information provides the RACH related configuration parameters. For each NPRACH configuration Nrep is provided which denotes the number of repetitions per preamble attempt. The OCC integration to the NPRACH transmission can be facilitated if the cover codes are applied to the preamble repetitions which are already supported per preamble attempt.

Proposal 3: For NPRACH capacity enhancement, OCC is applied to the repetitions per preamble attempt.

In this manner, even if more than one UEs select a given preamble, they can be demultiplexed within the code domain thanks to the application of orthogonal cover codes to each preamble repetition.

# Conclusions

In this contribution, the following observations are made for IoT NTN UL capacity enhancements:

Observation 1: Wide satellite beams, large number of UEs, IoT support and spectrum scarcity all lead to very limited uplink resource availability per UE for uplink transmission.

Observation 2: Large number of PUSCH repetitions inherent to NB-IoT and eMTC UL transmissions lead to very poor uplink throughput and capacity.

The observations and the discussion in this document lead to the following proposals:

Proposal 1: For NPUSCH Format 1 single tone transmissions, OCC is applied at transport block repetition level.

Proposal 2: For NPUSCH Format 1 multi tone transmissions, OCC is applied at repetition unit level (e.g., 2 slots).

Proposal 3: For NPRACH capacity enhancement, OCC is applied to the repetitions per preamble attempt.

# Reference

- RP-234077, “New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, RAN#102, December 2023.