3GPP TSG RAN WG1 Meeting #116                             R1-2400630

Athens, Greece, February 26th – March 1st, 2024

Source:	Sharp

Title:	Discussion on general aspects of physical layer design

Agenda Item:	9.4.2.1

Document for:	Discussion and Decision

- Introduction

A new study item on solutions for Ambient IoT [1] for Rel-19 was approved in RAN#102, with objectives including the following:

- RAN1-led: For the Ambient IoT DL and UL: - [...] - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - [...]

In this document, we share our views on some general aspects of physical layer design.

- Discussion

  - Functional split

From system perspective it is obvious that UEs with ~1 µW peak power consumption (denoted by “Type-1 UEs” in this document) can only support a basic set of functionalities, and UEs with ≤ a few hundred µW peak power consumption (denoted by “Type-2 UEs” in this document) should be capable of supporting both the basic set of functionalities and an advanced set of functionalities.

We envision the basic set of functionalities to require a UE complexity level down to that of an RFID tag, and a BS (or an intermediate node) complexity level down to that of an RFID “reader”, and a total deployment cost comparable with a corresponding RFID system (e.g. ISO18000-6C). This at least implies that 1), design of both receiver and transmitter of a Type-1 UE should target the complexity of an RFID tag; 2), a network node (i.e. a BS, or an intermediate node), as well as a UE, should be allowed to only support the basic set of functionalities.

Proposal 1: A basic set of functionalities is defined to be supported by both Type-1 UEs (i.e. UEs with ~1 µW peak power consumption) and Type-2 UEs (UEs with ≤ a few hundred µW peak power consumption), and an advanced set of functionalities is defined to be supported only by Type-2 UEs.

Proposal 2: Both the following network deployment options are supported:

- Option 1: only the basic set of functionalities is enabled.

- Option 2: both the basic set of functionalities and the advanced set of functionalities are enabled.

  - Mode of deployment

In order not to increase UE complexity, it is crucial that a same physical layer design should be strived for the three modes of deployment, i.e. “in-band”, “guard-band” and “standalone”. Moreover, it is desirable to study whether the actual mode of deployment in use can be transparent to the UE at least in some deployment scenarios (e.g. for some application layer data traffic types, and/or for the basic set of functionalities).

Proposal 3: A same physical layer design is strived for the three modes of deployments, i.e. “in-band”, “guard-band” and “standalone”.

Proposal 4: RAN1 to study whether the actual mode of deployment in use can be transparent to the UE in some deployment scenarios.

  - System information

In our view, in order to minimize the per-message over-the-air signaling overhead, SI (system information) should be supported. The SI should be valid over the duration of an application layer data session.

Since this is an aspect that may have a RAN2 impact, we think RAN1 should carry out a study and conclude the feasibility (e.g. including the maximum payload size, the persistence time, etc.) as early as possible.

Proposal 5: RAN1 to study and conclude the feasibility of supporting “system information” (including the level of support by Type-1 UEs and Type-2 UEs) transmission at physical layer.

Regardless of the decision on support for SI (and the level of support for Type-1 UEs and Type-2 UEs), we think the basic set of functionalities should come with minimum over-the-air configurability. Instead, “pre-configuration” should be supported. RAN1 should evaluate the maximum payload size of the pre-configuration, for both Type-1 UEs and Type-2 UEs, taking the target costs of these UEs into account.

Proposal 6: RAN1 to study and conclude the feasibility of supporting “pre-configuration” (including the level of support by Type-1 UEs and Type-2 UEs) in A-IoT.

  - RF channel bandwidth

Regarding RF channel(s) used for A-IoT, it should be possible that for a BS or an intermediate node, multiple RF channels can be used simultaneously, e.g. each serving a different group of UEs.

For Type-1 UEs, it is sufficient to support one RF channel bandwidth in each frequency band. Support for more RF channel bandwidth values can be part of the advanced set of functionalities and can be studied with lower priority.

Proposal 7: For the basic set of functionalities, one and only one RF channel bandwidth is supported in each frequency band.

In order to facilitate support for in-band deployment, it should be possible that RF channel configuration is compatible with legacy NR Uu.

Proposal 8: The bandwidth of an RF channel in A-IoT is compatible with legacy NR Uu.

  - Waveforms, modulations and coding

Regarding waveforms and modulations, there had been proposals from some companies during the RAN level study that in order to achieve ultra-low power consumption in a Type-1 UE receiver (e.g. with envelope detection), modulation schemes like OOK (On-Off Keying) should be used. We think this is a reasonable direction at least for the downlink and for the basic set of functionalities. For any discussion on OOK-based waveforms, RAN1 should exploit the outcome of the study on LP-WUS as much as possible.

Proposal 9: For the basic set of functionalities, OOK is evaluated for downlink data and signaling transmissions.

Regarding coding, in our view, at least for the basic set of functionalities, any coding technique adopted for the downlink should come with 1), ultra-low complexity; and 2), robustness against errors; and 3), synchronous clock; and 4), facilitation of energy harvesting (regardless of the contents of the transmitted data or signaling). We propose to at least consider Manchester coding.

Proposal 10: For the basic set of functionalities, Manchester coding is evaluated for downlink data and signaling transmissions.

- Conclusion

In this contribution, we discuss some general aspects of physical layer design, and make the following proposals.

Proposal 1: A basic set of functionalities is defined to be supported by both Type-1 UEs (i.e. UEs with ~1 µW peak power consumption) and Type-2 UEs (UEs with ≤ a few hundred µW peak power consumption), and an advanced set of functionalities is defined to be supported only by Type-2 UEs.

Proposal 2: Both the following network deployment options are supported:

- Option 1: only the basic set of functionalities is enabled.

- Option 2: both the basic set of functionalities and the advanced set of functionalities are enabled.

Proposal 3: A same physical layer design is strived for the three modes of deployments, i.e. “in-band”, “guard-band” and “standalone”.

Proposal 4: RAN1 to study whether the actual mode of deployment in use can be transparent to the UE in some deployment scenarios.

Proposal 5: RAN1 to study and conclude the feasibility of supporting “system information” (including the level of support by Type-1 UEs and Type-2 UEs) transmission at physical layer.

Proposal 6: RAN1 to study and conclude the feasibility of supporting “pre-configuration” (including the level of support by Type-1 UEs and Type-2 UEs) in A-IoT.

Proposal 7: For the basic set of functionalities, one and only one RF channel bandwidth is supported in each frequency band.

Proposal 8: The bandwidth of an RF channel in A-IoT is compatible with legacy NR Uu.

Proposal 9: For the basic set of functionalities, OOK is evaluated for downlink data and signaling transmissions.

Proposal 10: For the basic set of functionalities, Manchester coding is evaluated for downlink data and signaling transmissions.

- References

- RP-234058, “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, Huawei (moderator, RAN1 Vice-Chair), RAN#102.