3GPP TSG RAN WG1 Meeting #116                             R1-2400633

Athens, Greece, February 26th – March 1st, 2024

Source:	Sharp

Title:	Discussion on waveform characteristics of externally provided carrier-wave

Agenda Item:	9.4.2.4

Document for:	Discussion and Decision

- Introduction

A new study item on solutions for Ambient IoT [1] for Rel-19 was approved in RAN#102, with objectives including the following:

- RAN1-led: For the Ambient IoT DL and UL: - [...] - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation.

In this document, we share our views on some aspects of waveform characteristics of externally provided carrier-wave (CW).

- Discussion

The transmitter of the CW can be same as the BS (in Topology 1) for downlink transmission, or same as the intermediate node (in Topology 2) for downlink transmission, or a third network node. The actual transmitter should be transparent to the UE. On the other hand, the transmit power of the CW should be carefully managed, taking into account whether a same transmitter is used for the downlink transmissions and the CW.

Proposal 1: The actual transmitter of the CW is transparent to an A-IoT UE.

In order to facilitate in-band deployment, impacts of the waveform of the CW to legacy NR Uu need to be minimized. We propose to study both unmodulated sine wave and OFDM waveform as specified for legacy NR Uu. Modulations applied by an A-IoT UE for a backscattered uplink transmission can still be simple ones efficient for extreme low power consumption, e.g. OOK.

Proposal 2: Regarding waveform of the CW, RAN1 to study both unmodulated sine wave and the OFDM waveform specified for legacy NR Uu.

- As proposed in our companion paper [2], we think RAN1 should study backscattering-based transmissions from an A-IoT UE occurring in DL spectrum, as well as in UL spectrum. From CW transmission perspective, the implication is that for a backscattering-based transmission triggered by a downlink transmission, it should be supported that the corresponding CW transmission, as well as the uplink transmission and the downlink transmission, can occur in the same DL spectrum, or in the same UL spectrum.

Proposal 3: RAN1 to study support for the case of a backscattering-based uplink transmission, a downlink transmission triggering that uplink transmission, and the CW transmission on which the uplink transmission is scattered, occur in the same DL spectrum, or in the same UL spectrum.

In order to support a large number of UEs performing backscattering-based uplink transmissions (e.g. random access triggered by a downlink transmission), in addition to use of a same RF channel for backscattering, use of a different RF channel (i.e. backscattering with frequency shifting) can also be studied. The supported levels of frequency shifting should be respectively identified for UEs with ~1 µW peak power consumption (denoted by “Type-1 UEs” in this document) and UEs with ≤ a few hundred µW peak power consumption (denoted by “Type-2 UEs” in this document).

Proposal 4: RAN1 to study backscattering based transmissions with frequency shifting as well as backscattering based transmissions without frequency shifting.

- The level of support for frequency shifting can be different between Type-1 UEs and Type-2 UEs.

In order to support adaptive data rate (e.g. down to 0.1 kbps and up to 5 kbps as captured in TR 38.848 [2]) for the uplink, it would be desirable that the CW bandwidth is also adaptive, e.g. with the actual CW bandwidth indicated in the downlink control signaling triggering the corresponding backscattering-based uplink transmission.

Proposal 5: RAN1 to study support for adaptive CW bandwidths.

- Conclusion

In this contribution, we discuss some aspects of waveform characteristics of externally provided carrier-wave, and make the following proposals.

Proposal 1: The actual transmitter of the CW is transparent to an A-IoT UE.

Proposal 2: Regarding waveform of the CW, RAN1 to study both unmodulated sine wave and the OFDM waveform specified for legacy NR Uu.

Proposal 3: RAN1 to study support for the case of a backscattering-based uplink transmission, a downlink transmission triggering that uplink transmission, and the CW transmission on which the uplink transmission is scattered, occur in the same DL spectrum, or in the same UL spectrum.

Proposal 4: RAN1 to study backscattering based transmissions with frequency shifting as well as backscattering based transmissions without frequency shifting. The level of support for frequency shifting can be different between Type-1 UEs and Type-2 UEs.

Proposal 5: RAN1 to study support for adaptive CW bandwidths.

- References

- RP- 234058, “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, Huawei (moderator, RAN1 Vice-Chair), RAN#102.

- R1-2400632, “Discussion on downlink and uplink channel/signal aspects”, Sharp, RAN1#116.