3GPP TSG RAN WG1 #116			                       R1-2400634

Athens, Greece, February 26th – March 1st, 2024

Source:	Sharp

Title:	Discussion on LP-WUS and LP-SS design

Agenda Item:	9.6.1

Document for:	Discussion and decision

Introduction

In RAN#102, a WI on low-power wake-up signal (LP-WUS) and receivers for NR was approved[1]. In this WI, LP-WUS is introduced for UE power saving, and the objectives of the WI include specifying LP-WUS and LP-SS design for both IDLE/INACTIVE and CONNECTED modes.

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - This objective is to be further refined in RAN#103

In this contribution, some considerations on the design of LP-WUS and LP-SS are discussed.

Discussion

Waveform

Waveform for LP-WUS

According to the WID, for IDLE/INACTIVE UE, OOK with overlaid OFDM sequence(s) over OOK symbol can be used for LP-WUS waveform, and the OFDM sequence(s) can carry information. To ensure that the same information will be delivered irrespective of LP-WUR type, two options can be considered:

- Option1: OOK symbols carry complete wake-up message, and the OFDM sequence can repeat part or all of the same message

- Option2: OOK symbols carry part of wake-up message, and OFDM sequences carry another part of wake-up message

If option 1 is selected, the lower category UEs only only need to detect OOK symbols to minimize device complexity. For UEs with advanced categories, they can also detect the OFDM sequences to reduce the FAR. If option 2 is considered, then all categories of UEs require the ability to detecte both OOK and OFDM sequences, careful consideration needs to be given to the selection of OFDM sequences to minimize the cost to the UEs.

Observation 1: Two options can be considered for the design of OOK and OFDM sequence for LP-WUS; the one is that OOK symbols carry complete wake-up message and OFDM sequence only repeat part or all of the same message; another is that both OOK symbols and OFDM sequence carry part of wake-up message, and UE need to combine them to obtain the complete message.

According to the WI objectives, OOK-1 and OOK-4 are both in the candidate list for the LP-WUS waveform. Following the definition in SI, OOK-1 use multiple SCs to carry single-bit in 1 OFDM symbol. OOK=1 means all SCs are modulated, and OOK=0 means all SCs are zero power. an overlaid OFDM sequence may be filled in the modulated SCs when OOK=1 symbols are transmitted. This means that UE can detect OFDM sequence only one of those bits with OOK = 1. If Manchester coding is enabled, one wake-up bit will be extended to two or more toggled OOK symbols, and then UE can always receive an OFDM sequence in the transmission duration of each wake-up bit. For OOK-4, a similar condition may also happen .

Proposal 1: Manchester coding can be a mandatory feature for LP-WUS design for OOK with overlaid OFDM sequence.

The LP-WUS shall use the same design for connected UE and idle UE, the overlaid OFDM sequence can also carry information for connected UEs, and whether the same information is delivered for all categories of WUR is also applied for connected mode can be further discussed.

Proposal 2: OFDM sequence on overlaid OOK for connected UE shall be supported and whether the same information for all UE categories needs further discussion.

Waveform for LP-SS

For UEs in idle/inactive mode, periodic LP-SS signal is beneficial for coarse time/frequency synchronization of LP-WUR and RRM measurement. To make it easy to manage UE’s profile on LP-WUS reception quality by measurement of LP-SS, the waveform type of LP-SS should be configured the same as LP-SS.

Proposal 3: LP-SS and LP-WUS should have the same waveform type in one cell.

LP-WUS structure

Regarding the structure of the LP-WUS carried by OOK symbols, two kinds of WUS, i.e.sequence-based and encoded-bits-based WUS, have been discussed.

With sequence-based options, the wake-up signal can be one of a set of sequences with good autocorrelation and cross-correlation characteristics. The UE detects it using a known sequence which has been allocated for the UE in advance; if they match over the threshold, the UE's MR is woken for further processing. The sequence type should be the first consideration. Good correlation characteristics of the sequence should be ensured even with a small ADC width to minimise the complexity of the receiver. In addition, the sequence detection must be robust enough against time/frequency errors to work with low-precision crystals. Since OOK modulation is the selected waveform for LP WUS, binary sequences such as m-sequence, PN-sequence, and gold code would be more suitable options. Furthermore, sequence CDMs may not be supported to avoid dealing with multi-bit levels in the receiver. Another issue with the sequence-based WUS is the mapping rules between sequences and UEs. For example, in a WUS monitor occasion/WUS resource, a gNB might transmit a specific sequence to wake up a UE group associated with a paging occasion. If there are N UE subgroups associated with a WUS occasion, there will be at least N different sequences associated with one resource.

Another option for WUS is to use encoded bits which may have more flexibility in transmitting wake-up indications for different UE groups. In the payload of WUS, it can arrange wake information for different UE groups sequentially to indicate whether a group is awake with the use of 0/1. An addition of the CRC bit should be made within the payload and the CRC size should meet the requirements of the FAR.

In the TR[2], the evaluation on the overhead shows that, for OOK-based LP-WUS, when the MIL target aligns to PUSCH MSG3, the carried information can be up to 24bit, and the overhead remains marginal for cells with 20MHz bandwidth.

- For 5MHz LP-WUS with single PUSCH MSG 3 as MIL target - For OFDM based LP-WUS carrying information of up to 24bits, the overhead is marginal (up to 0.16%) for 20MHz or 100MHz system BW - For OOK/FSK-2 based LP-WUS carrying information up to 24bit, the overhead is marginal (up to 1.98%) for 20MHz. Note that in this case, the overhead evaluation in some sources includes LP-WUS and LP-SS. The reported maximum payload and maximum overhead are from different sources and are not corresponds to each other.

With a larger payload size, it is possible to combine wake indications for different groups and support more refined subgroup indications to get optimum power saving.

Observation 2: Encoded bit-based WUS offers more flexibility for WUS mapping, and up to 24 bits can be carried in one OOK based LP-WUS.

Proposal 4: Support encoded bit-based message for OOK based LP-WUS.

Coverage enhancement

The sensitivity of the low-power receiver for WUS signals may be lower than that of other NR channels due to waveform character, low-power receive structure, higher noise figure and other factors. This can affect the coverage of WUS signals. To ensure consistent coverage of WUS signals with other channels, some techniques have been discussed.

For encoded bit-based WUS, we can consider using simple FEC schemes like RM coding or convolutional coding for performance enhancement. These schemes can be easily implemented on devices with lower complexity. Other low-complexity schemes, including time domain repetition, can also be considered due to their easy implementation.

RAN4 study also shows the benefits of enabling power boosting for LP-WUS. The support of LP-WUS power boosting and boosting level can be declared by the manufacturer. The gNB may provide the power offsets for LP-WUS and LP-SS in SIB to get accurate measurements.

Proposal 5: Some coverage enhancement with low complexity can be considered for LP-WUS.

Resource configuration

For a UE in idle/inactive mode, we have agreed that gNB does not need to be aware of whether the UE is monitoring LP-WUS or not, the configurations of LP-WUS resources can be provided in the system information block for all UEs. For connected UE, the configurations of LP-WUS resources can be provided by UE-specific signals.

According to studies of RAN1 and RAN4, the frequency location of LP-WUS resource can be in the same or different frequency band with signals/channel for MR. consider the common design for idle and connected UE, the elements for CSI-RS-Resource can be a reference for LP-WUS configuration, including QCL-info, powerControlOffset, periodicityAndOffset, and resourceMapping for frequency domain location based on CRB grid and without reference to special BWP.

Proposal 6: CSI-RS-Resource can be the reference for LP-WUS configuration.

Conclusion

In this contribution, we have the following observations and proposals:

Observation 1: Two options can be considered for the design of OOK and OFDM sequence for LP-WUS; the one is that OOK symbols carry complete wake-up message and OFDM sequence only repeat part or all of the same message; another is that both OOK symbols and OFDM sequence carry part of wake-up message, and UE need to combine them to obtain the complete message.

Observation 2: Encoded bit-based WUS offers more flexibility for WUS mapping, and up to 24 bits can be carried in one OOK based LP-WUS.

Proposal 1: Manchester coding can be a mandatory feature for LP-WUS design for OOK with overlaid OFDM sequence.

Proposal 2: OFDM sequence on overlaid OOK for connected UE shall be supported and whether the same information for all UE categories needs further discussion.

Proposal 3: LP-SS and LP-WUS should have the same waveform type in one cell.

Proposal 4: Support encoded bit-based message for OOK based LP-WUS.

Proposal 5: Some coverage enhancement with low complexity can be considered for LP-WUS.

Proposal 6: CSI-RS-Resource can be the reference for LP-WUS configuration.

References

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] 3GPP TR 38.869 V2.0.0, Study on low-power wake up signal and receiver for NR