3GPP TSG RAN WG1 #116			                       R1-2400635

Athens, Greece, February 26th – March 1st, 2024

Source:	Sharp

Title:	Discussion on LP-WUS operation in IDLE/INACTIVE modes

Agenda Item:	9.6.2

Document for:	Discussion and decision

Introduction

In RAN-102#, a WI on Low-power wake-up signal and receiver for NR was approved. The objectives of the WI include specifying LP-WUS and LP-SS configurations and procedures for both IDLE/INACTIVE and CONNECTED modes.

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

In this contribution, the procedures of LP-WUS/LP-SS monitoring for UE in idle/inactive mode were discussed.

Discussions

Monitoring procedures

In the previous RAN1 and RAN2 studies, for idle/inactive UE waking up by WUS, three alternatives have been provided for further discussion.

- Alt 1.1: UE could monitor paging DCI/paging;

- Alt 1.2: UE could monitor PEI, if configured and supported; FFS details on using LP-WUS and PEI together, e.g. subgrouping

- Alt 2: UE could perform random access directly, FFS on whether and what condition/requirement is needed. R2 assumes that this require that LP-WUS includes UE_ID or equivalent. (Depends on LP-WUS capacity to carry information)

Alt 1 has been agreed by RAN2 as the baseline. With this option, when UE move out of LP-WUS coverage and fall back to legacy paging DCI monitoring, gNB need not allocate additional resources for paging information.

For Alt 1.2, it works when the gNB also provides PEI and the UE also supports PEI. Since we assume that a UE needs a longer ramp-up time than a PEI after being woken up by an LP-WUS, the LP-WUS associated with a PO should always precede its associated PEI. In addition, the LP-WUS may not be specific to a single UE, which may be mistakenly awakened by the wake-up signal of another UE in the same group. To minimise FAR, the UE may also monitor the PEIs for double checking based on the UE's implementation once upon it is waked by a LP-WUS associated with the same PO for PEI.

From the perspective of RAN1, for the Alt2 scheme that performs RA directly after wake-up, an LP-WUS would need to contain a large number of UE_ID bits, which may consume too much LP-WUS resources, and therefore should not be supported.

Proposal 1: Random access directly after waking up by LP-WUS shall not be supported.

Proposal 2: It can be up to UE implementation on whether UE also monitor PEI when LP-WUS is configured.

Dynamic PO

Compared to legacy I-DRX operation, if the MR enters a ultra-deep sleep while monitoring LP-WUS in the idle/active state, the average paging latency increases significantly if legacy paging occasions are reused due to the long ramp-up time and resynchronisation process of the MR from the super-deep sleep state. Furthermore, the delay can be reduced if additional POs can be introduced in response to the wake-up signal. The evaluation results for some cases are presented in TR[2].

- Legacy PO - Results in [8A-1], [8A-4], [8A-5], [8A-7], [8A-8], [8A-11], [8A-15], [8A-17] show that compared with i-DRX with or without PEI mean paging latency (0.638~0.768) second, LP-WUS will result in mean paging latency (1.022~1.264) second, which increases mean paging latency (52%~98%) - Results in [8A-3], [8A-10], [8A-13], [8A-16] show that compared with i-DRX with or without PEI paging latency, LP-WUS will result in mean paging latency (1.053~1.909) second without baseline i-DRX latency. - Dynamic PO - Results in [8A-8], [8A-11] show that compared with i-DRX with or without PEI mean paging latency (0.638~0.687) second, LP-WUS will result in mean paging latency (0.568~0.599) second, which decreases mean paging latency (6%~17%) - Results in [8A-13] show that compared with i-DRX with or without PEI paging latency, LP-WUS will result in mean paging latency (1.08) second without baseline i-DRX latency.

According to the background of this WI, both latency reduction and power saving should be considered, so potential techniques to decrease the latency e.g. using dynamic paging occasions shall also be considered.

Proposal 3: Support dynamic PO associated with LP-WUS monitoring for UE in IDLE/ INACTIVE mode.

Duty cycle and continuous monitoring

On WUS monitoring, two options for monitoring behaviours of LP-WUR have been evaluated in SI:

-	Option1: Duty cycle, corresponds to LP-WUR switches between ON/OFF states

-	Option2: Continuous monitoring, corresponds to LP-WUR is ON all the time

For duty cycle monitoring, the UE monitors the LP-WUS only periodically within a special window and stays asleep outside the window so that the power consumption of the LR will be lower since the average power consumption will be computed over the whole cycle. gNBs can configure a suitable period to control the average delay of the paging response. Continuous monitoring requires the UE to always monitor the LP-WUS and imposes tighter limits on the power consumption of the WUR to get obvious power-saving gain.

- Significant UE power saving gain (up to more than 90%) is observed by using LP-WUS/WUR to trigger UE MR paging monitoring compared with existing I-DRX operation (with and without PEI), if sufficient relaxation to MR RRM measurement is applied. - For duty-cycled LP-WUS monitoring and same duty ratio, higher power saving gain is observed if the relative power consumption of LP-WUR ON for LP-WUS monitoring is lower. - For continuous LP-WUS monitoring, UE power saving gain is only observed if the relative power consumption of LP-WUR ON for LP-WUS monitoring is lower, e.g. no larger than 1 unit.

The evaluation results show that the power saving gain of continuous monitoring can only be observed if the power consumption of the LP-WUR is low enough compared to duty cycle monitoring. To avoid excessive limitation on the WUR, it is reasonable to support only duty cycle monitoring for UE in idle/inactive mode.

Proposal 4: Only duty cycle monitoring shall be supported for UE in IDLE/ INACTIVE mode.

LP-WUS monitoring for eDRX

In NR, an idle UE shall monitor the POs during each DRX paging cycle. if eDRX is provided, the UE shall only monitor the POs in the PTW during long eDRX cycles, and the average latency of the paging response will depend on the duration of the eDRX. According to the evaluation in the SI, if LP-WUS monitoring is also limited to POs in the PTW, the latency will increase, which is not in the context of the WI, i.e. both power saving and latency reduction should be considered.

Proposal 5: Support WUS monitoring outside of PTW when eDRX is configured for UE.

Conclusion

In this contribution, we have the following observations and proposals:

Proposal 1: Random access directly after waking up by LP-WUS shall not be supported.

Proposal 2: It can be up to UE implementation on whether UE also monitor PEI when LP-WUS is configured.

Proposal 3: Support dynamic PO associated with LP-WUS monitoring for UE in IDLE/ INACTIVE mode.

Proposal 4: Only duty cycle monitoring shall be supported for UE in IDLE/INACTIVE mode.

Proposal 5: Support WUS monitoring outside of PTW when eDRX is configured for UE.

References

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] 3GPP TR 38.869 V2.0.0, Study on low-power wake up signal and receiver for NR