3GPP TSG RAN WG1 #116			R1-2400645

Athens, Greece, February 26th – March 1st, 2024

Source:	Sharp

Title:	ISAC Channel modelling

Agenda Item:	9.7.2

Document for:	Discussion and Decision

# Background

According to RP-234069 [1], RAN1 is supposed to discuss ISAC channel modelling. The conclusion from RP-234069 [1] for ISAC channel modelling is summarized below.

- Define channel modelling details for sensing using 38.901 as a starting point, and considering relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns. - spatial consistency.

# Choice of Channel Model for ISAC

The choice of ISAC channel model can be broadly classified into three categories: Stochastic channel model, deterministic channel model and hybrid channel model.

Deterministic channel model for ISAC can provide very high accuracy however it cannot be applied in large-range coverage scenarios because of computational cost and complex implementation. In contrast, statistical model provides much lesser accuracy but can be deployed over large range scenarios due to simplicity.

Observation 1: 3GPP TR 38.901 [1] currently supports stochastic channel model (section 7 in 3GPP TR 38.901) and map-based hybrid channel model (section 8 in 3GPP TR 38.901).

Observation 2: 3GPP TR 38.901 [1] currently supports Spatial consistency only for stochastic channel model. Spatial consistency mode is not supported for Map-based hybrid channel model in 3GPP TR 38.901.

Proposal 1: 3GPP TR 38.901 [1] stochastic channel model can be extended for bi-static sensing mode for ISAC channel modeling if it can meet the requirements for a particular ISAC use case because the TX-RX are not co-located in the current TR 38.901 stochastic channel model and are separated by a distance “d”.

Proposal 2: Spatial consistency is already supported for 3GPP TR 38.901 [1] stochastic channel model; thus, we can extend the same spatial consistency procedures in 3GPP TR 38.901 for ISAC based stochastic channel model.

Note: For ISAC use cases where bi-static sensing mode cannot achieve the desired sensing KPIs/requirements using TR 38.901 stochastic channel model we may have to use a map-based hybrid channel model or a deterministic channel model. For example, it might be challenging for the bi-static sensing mode based stochastic channel model to support complex object recognition, such as human gesture recognition, breath detection, etc. These use cases might require a map-based hybrid channel model or a deterministic channel model.

Proposal 3: For mono-static sensing mode the current 3GPP TR 38.901 channel model (stochastic and map-based hybrid channel model) cannot be used because the TX and RX are not co-located in 3GPP TR 38.901 channel model and thus the channel parameters presented in 3GPP TR 38.901 for both types of channel models are invalid. We need a new channel model for ISAC for mono-static sensing mode. Mono-static configuration can be deprioritized for now due to the reason mentioned in [2]. If Mono-static mode needs to be considered it should be discussed separately from bi-static mode.

Proposal 4: Channel modeling framework for ISAC irrespective of the sensing mode (bi-static or mono-static sensing mode) and type of channel modeling methodology (stochastic, deterministic and hybrid) should contain:

- Independent Environment/Background Modeling (mandatory)

- Independent sensing clusters (mandatory)

- Shared clusters (good to have)

Proposal 5: Independent Environment/Background Modeling for bi-static sensing mode is the same as generating independent communication clusters. Independent communication clusters for bi-static sensing mode can be generated using 3GPP TR 38.901 [1] channel parameters for different scenarios (UMi, UMa, RMa. InH and InF) and channel conditions (LOS or NLOS).

Proposal 6: Independent Environment/Background Modeling for mono-static sensing mode needs to be done for each scenario and channel condition. TR 38.901 assumes separate TX and RX locations, with a distance "d" between them. This separation leads to differences in Angle of Arrival (AOA) and Angle of Departure (AOD). On the other hand, Monostatic sensing, however, uses a co-located TX and RX. This co-location creates very similar AOA and AOD, making 3GPP TR 38.901's model unsuitable to generate the environment/background.

Proposal 7: For generating independent sensing clusters, please refer to Section 3 “Modeling of Sensing Target”. In general sensing targets can be modeled using a single point or multi point model.

Proposal 8: The Shared Clusters must be jointly generated for communication and sensing channels. It can be modeled using Delay Spread (DS), Angular Spread (AS), and Sharing Degree (SD) [3]. Further discussion is required to discuss the modeling approach for the shared cluster.

# Modeling of Sensing Target

Firstly, we must prioritize the use cases.

Secondly, modeling of sensing target is highly dependent on sensing mode and frequency of interest. Thus, we need to narrow down the desired sensing mode and frequency for each use case.

Thirdly, there are many different objects under each use case that can be characterized. Thus, it is crucial to decide how granular we want the characterization to be. For instance, if we select the UAV use case one can model different types of UAVs based on size, shape, material, type of UAV, etc. Similarly, if we chose Automotive Vehicles use case one can model different types of vehicles like SUV, Sedan, Truck, etc. Again, within each type of SUV, Sedan, etc. we can have different shape, size, material, etc. For higher accuracy it’s better to model each object of interest. However, that is very time consuming and infeasible. Thus, a general yet accurate modeling of sensing target is required.

Proposal 9: After prioritizing the use cases, desired sensing mode and frequency of interest for a particular use case, select a small subset of most commonly found/used targets for each use case and create a model of the sensing target.

Proposal 10: Depending on the use case, a target can be modelled using a single point model or a multi-point model. For instance, modeling humans as targets using a single point model will be much less accurate for human gesture recognition use case but a single point model might be sufficient to for the use case to detect the presence of humans.

Proposal 11: In a single point model, the single point represents the entire sensing cluster. Whereas in the multi-point model we may extract “Key” points from the target. The identification/generation of “Key” points will depend on the target. Each “Key” point must be related to the other “Key” points for a given target and all the “Key” points put together may represent the shape of the target.

# Conclusion

Proposal 1: 3GPP TR 38.901 [1] stochastic channel model can be extended for bi-static sensing mode for ISAC channel modeling if it can meet the requirements for a particular ISAC use case because the TX-RX are not co-located in the current TR 38.901 stochastic channel model and are separated by a distance “d”.

Proposal 2: Spatial consistency is already supported for 3GPP TR 38.901 [1] stochastic channel model; thus, we can extend the same spatial consistency procedures in 3GPP TR 38.901 for ISAC based stochastic channel model.

Proposal 3: For mono-static sensing mode the current 3GPP TR 38.901 channel model (stochastic and map-based hybrid channel model) cannot be used because the TX and RX are not co-located in 3GPP TR 38.901 channel model and thus the channel parameters presented in 3GPP TR 38.901 for both types of channel models are invalid. We need a new channel model for ISAC for mono-static sensing mode. Mono-static configuration can be deprioritized for now due to the reason mentioned in [2]. If Mono-static mode needs to be considered it should be discussed separately from bi-static mode.

Proposal 4: Channel modeling framework for ISAC irrespective of the sensing mode (bi-static or mono-static sensing mode) and type of channel modeling methodology (stochastic, deterministic and hybrid) should contain:

- Independent Environment/Background Modeling (mandatory)

- Independent sensing clusters (mandatory)

- Shared clusters (good to have)

Proposal 5: Independent Environment/Background Modeling for bi-static sensing mode is the same as generating independent communication clusters. Independent communication clusters for bi-static sensing mode can be generated using 3GPP TR 38.901 [1] channel parameters for different scenarios (UMi, UMa, RMa. InH and InF) and channel conditions (LOS or NLOS).

Proposal 6: Independent Environment/Background Modeling for mono-static sensing mode needs to be done for each scenario and channel condition. TR 38.901 assumes separate TX and RX locations, with a distance "d" between them. This separation leads to differences in Angle of Arrival (AOA) and Angle of Departure (AOD). On the other hand, Monostatic sensing, however, uses a co-located TX and RX. This co-location creates very similar AOA and AOD, making 3GPP TR 38.901's model unsuitable to generate the environment/background.

Proposal 7: Sensing targets can be modeled using a single point or multi point model.

Proposal 8: The Shared Clusters must be jointly generated for communication and sensing channels. It can be modeled using Delay Spread (DS), Angular Spread (AS), and Sharing Degree (SD) [3].

Proposal 9: After prioritizing the use cases, desired sensing mode and frequency of interest for a particular use case, select a small subset of most commonly found/used targets for each use case and create a model of the sensing target.

Proposal 10: Depending on the use case, a target can be modelled using a single point model or a multi-point model. For instance, modeling humans as targets using a single point model will be much less accurate for human gesture recognition use case but a single point model might be sufficient to for the use case to detect the presence of humans.

Proposal 11: In a single point model, the single point represents the entire sensing cluster. Whereas in the multi-point model we may extract “Key” points from the target. The identification/generation of “Key” points will depend on the target. Each “Key” point must be related to the other “Key” points for a given target and all the “Key” points put together may represent the shape of the target.

# References

- [1] 3GPP, “Study on channel model for frequency spectrum from 0.5 to 100 GHz,” TR 38.901.

[2] RP-232751 RAN 102, ‘On ISAC’, Ericsson.

[3] Liu, Y., Zhang, J., Zhang, Y., Yuan, Z., & Liu, G. (2022). A Shared Cluster-based Stochastic Channel Model for Joint Communication and Sensing Systems. arXiv preprint arXiv:2211.06615. https://arxiv.org/abs/2211.06615