3GPP TSG RAN WG1 #116                                       R1-2400664

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.4.2.2

Source:	China Telecom

Title:	Discussion on frame structure and timing aspects for Ambient IoT

Document for:	Discussion

# 1		Introduction

In the RAN plenary #102 meeting, a new SID for Rel-19 Ambient IoT was agreed [1]. Based on the conclusions reached in the RAN-level study item, the objectives are formed as follows:

The following objectives are set, within the General Scope: - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1.

In this contribution, discussions based on frame structure and timing will be introduced and discussed. The possible synchronization, access and schedule methods will be mentioned and detailed procedures for those functionalities will be discussed.

# 2		Potential frame structure and timing design

Ambient IoT is targeted for ultra-low power consumption and complexity devices. As mentioned in [1], device type I is a passive device with no proactive transmitting capability and only 1µW peak power consumption allowed. Such strict requirements cause almost all current NR mechanisms to be inapplicable. For device type II, internal transmission is supported and the power consumption is relaxed to a few hundred µW, which seems possible to reuse the current NR procedure. But considering a harmonized design with minimized differences for those devices is recommended in the scope, design for device type II may be just some enhancements based on type I.

  - Synchronization

In the current NR mechanism, a whole network-wide synchronization is adopted to assist the coherent OFDM demodulation and schedule the UL/DL transmissions to avoid interference. Considering the initial SFO of clock component is up to 10X ppm for both device type I and device type II, the timing effect will be extremely poor compared with the current 0.1 ppm NR system. Hence, the whole network-wide synchronization scheme does not apply to Ambient IoT devices.

RFID provides a possible synchronization transmission method [2], a predefined preamble or frame-sync is added ahead of the RFID command or UE reply for R=>T or T=>R communication, and a dummy 1 symbol is also carried at the end of the signaling in the T=>R communication. Considering the device complexity should be comparable with RFID, such designs can be regarded as a reference.

Figure 1 frame-sync and preamble for R=>T communication

Figure 2 frame-sync and preamble for T=>R communication

UL Synchronization structure design

Based on the RFID design, the synchronization structure is different between DL and UL with the consideration of Tag complexity, and the similar consideration can be reused in Ambient IoT devices. In DL channel, calibration information is necessary due to an inaccurate clock, thus incethe structure should at least comprise a delimiter to mark the transmission starting point and a clock duration to assist in clock calibration. Meanwhile, since the SFO value X is still not confirmed, further calibration like RTcal or TRcal in RFID can also be considered in Ambient IoT DL synchronization structure. A DL command is often regarded as a short message and doesn’t need to be enhanced, but if longer DL information is necessary for some scenarios or if the SFO value X is too large, potential further calibration symbols are still needed to deal with the time offset caused by the poor clock accuracy. Such calibration symbols can be combined with delimiter and clock calibration, or inserted in the middle of such commands for distributed calibration.

Figure 3 potential synchronization structure for device type I/II

Observation 1: The DL synchronization structure may comprise a delimiter, a symbol level duration and possible further calibration.

Meanwhile, a delimiter at the end of DL command like “dummy 1” seems not necessary since continuous and large number of commands will not be received in the device in a short period, and a precise indication is not needed. But considering the timeline requirements between a DL command and a UL response are still not discussed, the end delimiter can also be added to the synchronization structure if necessary.

Observation 2: Whether the end delimiter is needed can be further discussed.

Proposal 1: In DL, the potential synchronization structure design may comprise the following components:

- A starting delimiter to mark the transmission starting point

- A clock duration to assist in clock calibration

- Some calibration symbols to assist further clock calibration if necessary

- An end delimiter to mark the transmission endpoint if necessary

UL Synchronization structure design

In the UL, since device symbol duration information can be tracked by gNB, clock calibration information is not needed in the UL synchronization, and just some predefined preamble sequences can be utilized as the starting position. Two options can be considered for the sequence generation method:

- Option 1: The sequence is generated by some predefined generation method, i.e., ZC, m, PN

- Option 2: The sequence is fixed, and is selected based on some characteristics or principle

For option 1, the legacy sequence generation method can be reused and less spec. impact may be caused, but considering some generation parameters (e.g., the u, v value for low PAPR sequence) need to be configured by NW, a DL pre-configured transmission signaling may be needed. Besides, since the complexity of devices is ultra-low, whether the generation process like ZC sequence can be supported based on the current device structure needs further discussion.

For option 2, several characteristics can be considered. In 802.11ba, potential sequence design is listed based on anti-multipath fading, nearly constant envelope and power distribution, and the trade-off between anti-fading and low PAPR [3]. Since coverage performance is a key point for Ambient IoT performance, at least the anti-multipath fading design can be referred to. Besides, considering the channel difference between WIFI and mobile communication, corresponding simulation work may be needed. It is worth noting that corresponding generation parameter indication is not necessary if a fixed sequence has been written in the spec.

Proposal 2: The potential UL preamble sequence can reuse the legacy NR sequence or select some fixed sequence.

- Pre-configuration signaling and device complexity should be considered if reusing the legacy NR sequence.

- Evaluation is necessary if a fixed sequence has been supported in the spec.

Moreover, considering the time duration of the accurate clock is not long due to the high SFO value, an end preamble sequence (e.g., dummy 1 in RFID) is necessary to help gNB confirm the current timing offset and demodulate the UL transmission data. Similar to DL transmission, if a large UL transmit block is necessary for some scenarios or the SFO value X is too large, potential further calibration symbols are also needed, which can be inserted in the middle of such commands for distributed calibration. Different from DL transmission, such calibration symbols may also be generated based on sequence, which is the same or different from the preamble sequence.

Figure 4: potential synchronization structure for device type I/II

Proposal 3: In DL, the potential synchronization structure design may comprise the following components:

- A preamble sequence to mark the transmission starting point

- Some calibration sequences to assist further clock calibration if necessary

- An end preamble to mark the transmission endpoint

  - Random access

In the current NR mechanism, UE will access the NW by a 4-step RACH procedure. However, the potential PRACH occasion depends on the CW period due to no proactive transmitting capability for device type I, and massive device access collision will occur since no SSB index indication exists to provide a different PRACH occasion. Moreover, even if the above-mentioned problem can be solved, since the whole network-wide synchronization scheme does not apply to Ambient IoT devices, TA information doesn’t have its effect anymore. Therefore, legacy 4-step RACH procedure is not suitable for the Ambient IoT system.

RFID describes a possible access and collision avoidance mechanism [3]. In the Inventory procedure, Tag will respond a random RN16 after receiving the Query command, and reply its EPC code after an ACK command. Then, the target object is identified by the Reader. To avoid violent device collisions, the RN16 value is randomly generated by a Q value carried in the Query command and the generation randomness criteria are limited to a certain range.

Based on the above information, two options are possible for the potential access mechanism:

- Option 1: Reuse the RFID mechanism. Accessing to the network is completed by an Inventory procedure.

- Option 2: Define a specific access procedure. A specific command/indication is needed instead of reusing the Inventory procedure.

For option 1, the potential access method can be designed similarly to RFID. RN16 information is utilized to decentralize the device access timing, be recognized by the core network and act as a key to help subsequent transmission. But that design means devices cannot access the network unless an Inventory procedure is executed, and the ID information is transmitted without encrypting, which is highly susceptible to pseudo-based stations.

Figure 5 potential access procedure for device type I/II

For option 2, A specific procedure similar to Inventory can be defined to collect the ID information of each device. A certain trigger period can be configured and some encryption mechanism should be considered for the ID information transmission, like the TSMI utilization in legacy NR network if option 2 is adopted. Meanwhile, since device type II is able to transmit some information based on itself, whether an extra access procedure design for device type II is needed can be further discussed.

Proposal 4: The potential access mechanism for Ambient IoT can reuse the inventory procedure in the RFID system with some enhancements.

- A specific procedure may be defined separately for device ID collection.

- Some encryption mechanisms should be considered like the TSMI utilization in NR network.

- A separate design for device type II may be needed.

  - Scheduling

In the NR system, scheduling is mainly driven by PDCCH. UE will send SR information to require a specific UL transmission resource and transmit data flow after detecting its scheduling DCI information. However, for an Ambient-IoT system, PDCCH blind detection is hard to be utilized considering the complexity of both device type I and type II. Meanwhile, the transmission in the RFID system is like a “configured grant” method. A DL command information is sent by Reader and a UL transmission is followed after this command, which seems that PDCCH is not necessary in such a system. Hence, the dynamic scheduling mechanism does not need to be applied.

Figure 6 potential scheduling mechanism for device type I/II

Scheduling content

Though the RFID scheduling mechanism can be as a reference, a corresponding detailed design needs to be discussed. Based on the RAN-level study conclusion [4], the max TB size is less than 1000bis, but no further TB size level is defined, so the specific TB size or the corresponding MCS information needs to be further discussed. Besides, considering repetition or retransmission is supported in the current NR system, whether such a mechanism can be supported in the Ambient IoT system should be further studied with the consideration of device complexity.

Observation 3: TB size, MCS level, repetition or retransmission mechanism of the scheduling needs to be studied.

Meanwhile, notice that an RFID command has its specific indication field, the corresponding ambient IoT command and its indication field should also be defined and modified. Take the ‘Query’ command as an example, it has nine fields with 22bits, and the structure is shown as follows:

Figure 7 The ‘Query’ command structure

The ‘Command’ field is utilized to distinguish the command function, ‘DR’ field is used to select a suitable transmission data rate or frequency, ‘M’ represents the coding rate and other fields also have their specific functionalities. Considering the transmission data rate and the MCS indication may be performed by a new command, whether such fields are also necessary in the ‘Query’ command for Ambient IoT command design may need further discussion. From this perspective, the command field design may be quite different between the RFID command and Ambient IoT command even if a similar procedure has been carried out. Hence, the detailed command field design needs thorough consideration.

Observation 4: Command field design may be quite different between the RFID command and Ambient IoT command even if a similar procedure has been carried out.

Scheduling timeline

In the RFID system, the timeline requirement is defined based on the gap between a DL command and UL transmission. As shown in Figure 8, the time gap between RN16 and the Query command does not exceed T1 duration, and the gap between RN16 and the ACK command does not exceed T2 duration. Based on such a design, a corresponding timeline also needs to be defined for Ambient IoT transmission. Meanwhile, the timeline requirement can also be enclosed in a DL command to indicate the next UL transmission occasion for the consideration of extra scheduling flexibility.

Figure 8 A typical RFID transmission timeline illustration

Observation 5: Timeline requirement needs to be studied, and can be predefined or enclosed in a DL command.

Proposal 5: The potential scheduling design may contain at least the following:

- The potential scheduling content such as MCS/TB size indication

- The detailed scheduling command design such as the command field design

- The possible scheduling timeline requirement

# 3		Conclusion

In this contribution, we discuss the potential design of synchronization, random access and scheduling, and have the following observations and proposals:

Observation 1: The DL synchronization structure may comprise a delimiter, a symbol level duration and possible further calibration.

Observation 2: Whether the end delimiter is needed can be further discussed.

Observation 3: TB size, MCS level, repetition or retransmission mechanism of the scheduling needs to be studied.

Observation 4: Command field design may be quite different between the RFID command and Ambient IoT command even if a similar procedure has been carried out.

Observation 5: Timeline requirement needs to be studied, and can be predefined or enclosed in a DL command.

Proposal 1: In DL, the potential synchronization structure design may comprise the following components:

- A starting delimiter to mark the transmission starting point

- A clock duration to assist in clock calibration

- Some calibration symbols to assist further clock calibration if necessary

- An end delimiter to mark the transmission endpoint if necessary

Proposal 2: The potential UL preamble sequence can reuse the legacy NR sequence or select some fixed sequence.

- Pre-configuration signaling and device complexity should be considered if reusing the legacy NR sequence.

- Evaluation is necessary if a fixed sequence has been supported in the spec.

Proposal 3: In DL, the potential synchronization structure design may comprise the following components:

- A preamble sequence to mark the transmission starting point

- Some calibration sequences to assist further clock calibration if necessary

- An end preamble to mark the transmission endpoint

Proposal 4: The potential access mechanism for Ambient IoT can reuse the inventory procedure in the RFID system with some enhancements.

- A specific procedure may be defined separately for device ID collection.

- Some encryption mechanisms should be considered like the TSMI utilization in NR network

- A separate design for device type II may be needed.

Proposal 5: The potential scheduling design may contain at least the following:

- The potential scheduling content such as MCS/TB size indication

- The detailed scheduling command design such as the command field design

- The possible scheduling timeline requirement

# 4		Reference

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG-RAN Plenary Meeting #102, Edinburgh, UK, December 11th – December 15th, 2023.

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID, Specification for RFID Air Interface Protocol for Communications at 860 MHz – 960 MHz v2.0.1, April, 2015.

- IEEE 802.11ba, Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications, Amendment 3: Wake-Up Radio Operation, 2021.

- TR 38.848 v18.0.0, Study on Ambient IoT (Internet of Things) in RAN, September, 2023.