3GPP TSG RAN WG1 #116			R1-2400665

Athens, Greece, February 26th – March 1st, 2024

Agenda item:		9.4.2.3

Source:	China Telecom

Title:	Discussion on downlink and uplink channel/signal aspects for Ambient IoT

Document for:		Discussion

Introduction

The study item on solutions for Ambient IoT (Internet of Things) was approved in RAN#102 meeting [1]. The main objectives are described as follows:

General Scope The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor - FR1 licensed spectrum in FDD. - Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s). - Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1.

This contribution provides views on the downlink and uplink channel/signal aspects for A-IoT.

Downlink and uplink channel/signal aspects

  - General aspects

Considering A-IoT devices with extremely restricted complexity and cost, it is unlikely to reuse control channel, broadcast channel and random-access channel in legacy LTE/NR system, like PDCCH, PUCCH, PBCH and PRACH. The high overhead and large complexity would be unacceptable. A new IoT data channel for DL and UL would be defined instead of utilizing legacy LTE/NR design. The simple RFID-like data channel may be a good choice. Furthermore, with the constraints of initial sampling frequency offset (SFO) up to 10X ppm for both DL and/or UL amplification in the A-IoT device, the reference signal in legacy LTE/NR system can’t be supported, like SSB, DMRS, CSI-RS, SRS, PTRS and so on.

Observation 1: For A-IoT, no need to support PUCCH, PDCCH, PBCH and PRACH channel.

Observation 2: For A-IoT, no need to support the reference signal utilizing legacy LTE/NR design, like SSB, DMRS, CSI-RS, SRS, PTRS.

Proposal 1: A new IoT data channel for DL and UL would be defined instead of utilizing legacy LTE/NR design.

  - Downlink channel/signal

For R=>T communication of RFID, an interrogator shall begin all signal transmission with either a preamble or frame-sync. A preamble would be signaled before Query command to indicate the start of an inventory round. The other data can be signaled after the frame-sync. The detailed values on delimiter, data-0, R->T calibration (RTcal) and T->R calibration (TRcal) can be found below in [2], as illustrated bellow.

Fig 1. Illustration of R=>T preamble and frame-sync

For A-IoT downlink, it is not necessary to support control information due to high overhead. It can reuse the simple RFID-like way of “Preamble/Sync + Data/Command” for downlink transmission of A-IoT devices. Preamble or fame-sync is used for the purpose of timing synchronization and can be used to indicate the position of data. The additional key point is whether to have postamble to indicate the end of data transmission. We think the necessity of postamble would be affected by deployment scenario, data size and command type. There may contain command like Select/QueryACK/NCK, and data like RN16/EPC/device ID as reused in RFID. Thus, we support two types of DL transmission for A-IoT as illustrated in Fig 1 below. If the unexpected asynchronization occurs, the postamble would be needed to tell the end of transmission and avoid worse receiving performance. The detailed consideration and design on preamble/sync and postamble can be found in [3]. Certainly, this mechanism is easy to implement with low performance, which may need to consider the additional optimized schemes to ensure the accuracy of timing synchronization.

Fig 2. Illustration of two types of DL transmission for A-IoT

Proposal 2: Support two types of RFID-like channel for A-IoT downlink transmission.

- Type 1: Preamble/Sync + Data/Command.

- Type 2: Preamble/Sync + Data/Command + Postamble.

- FFS: Details on preamble/frame-sync and postamble, if needed.

- Further study whether to consider the additional optimized schemes to ensure the accuracy of timing synchronization.

  - Uplink channel/signal

For T=>R communication of RFID, there is only the preamble mechanism to achieve timing synchronization. The UL transmission data is immediately signaled after the preamble. Two types of the preamble formats are supported by RFID devices, i.e., FM0 preamble and Miller preamble. According to the TRext value specified in Query command, e.g., TRext = 0 or TRext = 1, it can choose the FM0/Miller preamble or FM0/Miller extended preamble with pilot tone. The details can be found below in [2].

Due to the constraints of initial sampling frequency offset (SFO) up to 10X ppm for both DL and/or UL amplification in the A-IoT device, it is hard to reuse the reference signal in legacy LTE/NR system for uplink transmission of A-IoT. It can reuse the simple RFID-like way of “Preamble + Data + Postamble” for uplink transmission of A-IoT devices. The postamble is used to indicate the end of data transmission with necessary. Furthermore, for long data transmission, it may need to use midamble to divide UL transmission data into small segments. The detailed consideration and design on preamble, midamble and postamble can be found in [3]. The following figures are illustrated on short and long UL transmission for A-IoT devices.

Fig 3. Illustration of short UL transmission for A-IoT

Fig 4. Illustration of long UL transmission for A-IoT

Proposal 3: For short UL transmission of A-IoT devices, support “Preamble + Data + Postamble”.

- FFS: Details on preamble and postamble, e.g., FM0 or Miller.

Proposal 4: For long UL transmission of A-IoT devices, support “Preamble + Data + Midamble + Data + Postamble”.

- Reuse the details on preamble and postamble implied for short UL transmission of A-IoT devices.

- FFS: Details on midamble.

Conclusions

In this contribution, we have the following observations and proposals:

Observation 1: For A-IoT, no need to support PUCCH, PDCCH, PBCH and PRACH channel.

Observation 2: For A-IoT, no need to support the reference signal utilizing legacy LTE/NR design, like SSB, DMRS, CSI-RS, SRS, PTRS.

Proposal 1: A new IoT data channel for DL and UL would be defined instead of utilizing legacy LTE/NR design.

Proposal 2: Support two types of RFID-like channel for A-IoT downlink transmission.

- Type 1: Preamble/Sync + Data/Command.

- Type 2: Preamble/Sync + Data/Command + Postamble.

- FFS: Details on preamble/frame-sync and postamble, if needed.

- Further study whether to consider the additional optimized schemes to ensure the accuracy of timing synchronization.

Proposal 3: For short UL transmission of A-IoT devices, support “Preamble + Data + Postamble”.

- FFS: Details on preamble and postamble, e.g., FM0 or Miller.

Proposal 4: For long UL transmission of A-IoT devices, support “Preamble + Data + Midamble + Data + Postamble”.

- Reuse the details on preamble and postamble implied for short UL transmission of A-IoT devices.

- FFS: Details on midamble.

References

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, Huawei, 3GPP TSG RAN Meeting #102, Edinburgh, UK, December 11th -15th, 2023.

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID, Specification for RFID Air Interface, Protocol for Communications at 860 MHz – 960 MHz, Version 2.0.1.

- R1-2400664, Discussion on frame structure and timing aspects for Ambient IoT, China Telecom, 3GPP TSG RAN1 Meeting #116, Athens, Greece, February 26th – March 1st, 2024.