3GPP TSG RAN WG1 #116                                       R1-2400670

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.6.1

Source:	China Telecom

Title:	Discussion on LP-WUS/LPSS design

Document for:	Discussion

- Introduction

In the RAN plenary #102 meeting, a new WID for Rel-19 LP-WUS was agreed [1]. Based on the conclusions reached in the RAN-level study item, the objectives are formed as follows:

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported

In this contribution, discussions based on LP-WUS/LP-SS will be discussed. The potential LP-WUS/LP-SS generation method is described, and the detailed generation process is also introduced.

- LP-WUS generation process

LP-WUS is targeted for ultra-low power consumption waveform. As mentioned in [1], the target waveform is OOK-based LP-WUS with overlaid OFDM sequences. OOK-1 is derived from IEEE 802.11ba [2], and the information sequence is mapped in the corresponding OFDM symbols to represent the waveform ‘ON’, and not be mapped if the waveform ‘OFF’ needs to be conveyed. In the receiver, envelope detection will distinguish the waveform ‘ON’ from the waveform ‘OFF’. A bandpass filter is also configured on the target WUS signal bandwidth to reduce the noise impact and avoid potential interference from co-existing legacy NR signals or other WUSs. A typical option OOK-1 method can be described as follows:

Figure 1. Illustration of option OOK-1 generation process                Figure 2. Waveform of option OOK-1

In OOK-4, M-bit OOK can be transformed by DFT precoding and/or LS square. The time domain waveform can be shaped by expanding the target information bit, e.g., [0 1] -> [ 0 0 0 0 … 0 0 1 1 1 1 …. 1 1], and a DFT precoder is introduced to approach the ideal OOK waveform. Then truncation is performed under a given signal bandwidth. After OFDM modulation, the shape of the time-domain LP-WUS signal will approach the ideal time-domain OOK waveform. A bandpass filter is also configured on the target WUS signal bandwidth to reduce the noise impact and avoid potential interference from co-existing legacy NR signals or other WUSs. A typical option OOK-4 method can be described as follows:

Figure 3 Illustration of option OOK-4 generation process               Figure 4 Waveform of option OOK-4

Though candidate solutions have been adopted in the current TR [3], no more detailed waveform generation procedures are provided. Meanwhile, considering OFDM sequence is overlaid with information bits, the sequence design also needs to be discussed. Hence, the corresponding signal generation procedure and OFDM sequence design are discussed in this contribution.

  - OFDM sequence design

To obtain a constant envelope waveform and avoid the frequency spike effect, the information sequence can be the sequence with a constant envelope but different phases instead of the ‘all-one sequence’ when mapping to the frequency resource, such as the ZC sequence. The frequency spectrum will be spread out over the bandwidth and reduce the possibility of threshold effect as well based on such sequence characteristics. Since the OFDM sequence shall be overlaid on the OOK-1 design and information will be carried in such OFDM sequences, a suitable OFDM sequence shall be selected to guarantee the detection performance, such as better cross-correlation or anti-fading capability. At least three aspects shall be considered when seeking the target sequence:

- Sequence type

- Sequence generation parameter

- Payload size carried on the sequence

The different sequence has their specific properties and the detection performance will be accordingly affected, such as the cross-correlation of the ZC sequence is better than m sequence and consequently selected as the PSS generation sequence. Besides, different generation parameters may also have impacts on the detection performance even if the same sequence type is utilized. For example, the different u/v values will affect the cross-correlation performance when generating a ZC sequence, and detection performance is also different due to such a property. Finally, as mentioned above, information bits shall be carried in the OFDM sequence. Considering a typical method to convey the corresponding information bits in the sequence is to utilize their sequence properties (e.g., sequence phase, sequence generation parameter) to distinguish the carried information, a suitable information payload should be confirmed to make a trade-off between payload size and detection performance.

Proposal 1: A suitable OFDM sequence design shall be considered based on the following aspects:

- Sequence types, such as ZC sequence, Gold sequence or some predefined sequence

- Sequence generation parameters, such as u/v values in the ZC sequence generation procedure

- Payload size carried on the corresponding sequence

  - OOK waveform generation procedure

Based on the WID description, the OOK waveform generation procedure needs to be specified. In the Physical uplink control channel chapter, 3 sections are used to illustrate the generation procedure, including general format description, sequence generation method and their detailed format generation procedure [4]. Considering LP-WUS has two formats and both of them need to be generated by sequence, the PUCCH generation procedure can be as a reference for the LP-WUS description in the spec. A similar description may be recorded in the potential new spec., including OOK generation format description (e.g., OOK format 0 for OOK-1, OOK format 1 for OOK-2), sequence generation method and their generation procedure for each format.

Proposal 2: LP-WUS generation description in the potential spec. may include the following section:

- General

- Sequence generation

- LP-WUS format 0/1

OOK-1 generation procedure

Two steps are written in the PUCCH format 1 generation procedure, including sequence modulation and mapping to physical resources. In sequence modulation, a block of bits is modulated in the sequence by BPSK or QPSK. Considering OOK-1 is modulated by one OFDM sequence per OFDM symbol to represent the waveform ‘ON’, the corresponding modulation description shall be written in the sequence modulation section. Meanwhile, since such OFDM sequence also carries information bits for OFDM WUR, detailed sequence generation description needs also be adopted in the corresponding section, such as the different sequence phases corresponding to the information ‘1’ or ‘0’. After modulation, the corresponding OFDM sequence will be mapped to the frequency RE resource. Since virtual resource blocks may be used if a DL signal is mapped in the frequency resource, such a procedure may also need to be considered in the generation procedure description.

Proposal 3: OOK-1 LP-WUS generation description in the potential spec. may include the following section:

- Sequence modulation

- Mapping to virtual resource blocks if necessary

- Mapping to physical resource blocks

OOK-4 generation procedure

Similar to OOK-1 description, sequence modulation and mapping to physical resource blocks are necessary for OOK-4 generation procedure, while additional steps are needed such as DFT-precoder since OOK-4 is an approximation-based waveform generation method. Hence, such steps should be discussed.

As mentioned in the WID, OOK-4 is an approximation-based M-bit modulation LP-WUS format. Assuming a total N bit will be transmitted in the LP-WUS signal, rate matching is a necessary step to calculate the required number of frequency and time domain resources, especially when N>M that more than one OFDM symbols are needed. In that case, LP-WUS bits shall be divided into several signal blocks based on the rate matching requirement, then the approximation-based OOK-4 time domain waveform can be generated based on the value M.

Figure 5 Rate matching procedure illustration

After rate matching, the corresponding bits in each signal block will be generated into OOK waveforms. Based on the WID description, information bits will be carried on the OFDM sequence. Considering longer sequences will have better detection performance, the OFDM sequence shall be modulated or spread as the target signal bandwidth size to perform OOK modulation. If an ‘all-one’ sequence is selected to represent the waveform ‘ON’, then the input M bits can be just spread in the time domain for simplicity. For example, the input bits [0 1] can be spread as [ 0 0 0 0 … 0 0 1 1 1 1 …. 1 1], and the all-one sequence ‘1 1 1 … 1 1’ is used to perform OOK modulation. If another OFDM sequence is utilized, the all-one sequence should be replaced as the selected sequence. Then DFT precoder will be performed to generate frequency physical resource blocks.

Moreover, considering the generated spectrum energy may also be concentrated even if the OFDM sequence has been utilized to flat the spectrum, a main-lobe part repetition method can be utilized to acquire additional frequency diversity gain for anti-fading. Assuming x is the frequency domain sampling point input and m is an OFDM symbol, b is the input time domain waveform sampling, N is the total time domain waveform sampling point size and L can be regarded as the number of REs. After DFT precoding, the signal in the m-th OFDM symbol can be further expressed as:

To better illustrate the anti-fading effect based on the mentioned method, all-one sequence is utilized as an example to show the frequency diversity gain. The formula after passing the channel and the amplitude-frequency response both illustrate that frequency energy will be concentrated on the central part when OOK-4 generation is utilized:

Figure 6 The formula and amplitude frequency response for basic OOK-option 4 scheme

Based on figure 7, figure 8, figure 9 and figure 10, the main lobe of the signal in the frequency response has the most significant contribution, while the side lobe has little contribution to the signal and can be cut off to reduce the occupied frequency resource. If such side lobe can be replaced by the main lobe, the frequency diversity gain will be acquired and the threshold effect can be mitigated. We name this idea a ‘central part repetition’ scheme.

Figure 7. Frequency response and time domain waveform in all main-lobe and side-lobe

Figure 8. Frequency response and time domain waveform after cutting off some side-lobe

Figure 9. The frequency effect after some side-lobe is cut off and replaced by main-lobe

Figure 10. The time domain waveform effect after some side-lobe is cut off and replaced by main-lobe

The scheme can be described from two aspects. First, we can disperse effective energy distribution. That is to say, we can generate more DFT precoding, reserve the main lobe and reduce the power of each main lobe to guarantee constant total energy and acquire frequency diversity. Meanwhile, we can also cut off the side lobe in the allocated bandwidth and replace it with the repeated main lobe generated in the first step. The process can be further described in the following steps:

Step 1: Generate a DFT precoding input with bandwidth R

Step 2: Generate multiple DFT precoding inputs based on frequency position

Step 3: Cut off the side lobe. For simply, we can assume the size of reserved main lobe is R/X.

Step 4: Sequential splicing. Since X segment is generated, the total bandwidth of splicing size is still R.

Figure 11. Generation process of the scheme ‘central part repetition’

Based on this scheme, there will be X frequency peak spread evenly over the allocated bandwidth, and the tradeoff between a single spike and the number of side-lobe can be also found. An example with a repeat number of 8 is shown in figure 12 and figure 13. As can be seen, at least one main peak energy can be utilized to generate the time domain waveform, a more specific mathematical process of such a scheme can be referred to in Appendix B.

Figure 12 AFR for the proposed scheme                     Figure 13 PSD for the proposed scheme

Moreover, the simulation results for the proposed ‘central part repetition’ scheme are also provided. Assuming the repeat number is 8 and the cut off frequency location is 1/8 of the whole LP-WUS bandwidth, 3 chip rates are simulated with 4-bit ADC and an order 5 filter. The detailed simulation assumption can be found in Appendix A.

Figure 14 Different rates for the proposed OOK-4 scheme       Figure 15 Performance of other proposed scheme in 28kbps

Considering other solutions are also provided in the TR, a comparison of these schemes is also displayed in the following figure. For a fair comparison, the same bandwidth, total power, signal structure and receiver process are assumed, and 4bit ADC is assumed to guarantee that the detection performance is not affected. The detailed assumptions can be found in Appendix A.

As can be seen in figure 15, each solution improves the original scheme. At least 3dB performance gain can be observed from the alternative ‘1’ and ‘-1’ due to the 2-length cyclic shift lower down the high spike in the frequency domain. However, since only 2 phases are used in the time domain ‘ON’ waveform, the frequency spike still exists and the improvement effect is not idle. The whole-length ZC sequence derived from ‘flatten the spectrum’ can have the max spectrum spreading effect as well as detection performance, and the proposed scheme ‘central part repetition’ has a similar performance to the ZC sequence, hence ‘central part repetition’ scheme should be considered in the potential OOK-4 normative work.

Proposal 4: OOK-4 LP-WUS generation description in the potential spec. may include the following section:

- Rate match to generate signal blocks

- Sequence spread or sequence modulation based on the target signal bandwidth

- DFT precoder

- Central part repetition

- Mapping to virtual resource blocks if necessary

- Mapping to physical resource blocks

Proposal 5: The scheme ‘central part repetition’ should be supported based on following steps:

- Generate a DFT precoding input with bandwidth R

- Generate multiple DFT precoding inputs based on different frequency start position

- Cut off the side lobe

- Sequential splicing

- LP-SS generation process

Similar with LP-WUS signal, LP-SS is also designed based on the above-mentioned procedure, but the difference between the LP-WUS and LP-SS design is whether OFDM sequence should be overlaid. Considering the overlaid OFDM sequence is mainly designed for OFDM WUR based on cross-correlation detection, and the legacy SSS can also have the similar effect, the overlaid OFDM sequence design is not essential. SSS PRBs are able to be reserved from PBCH blocks to avoid interference from non-SSS PRBs by a baseband filter, and the corresponding detection performance of the OFDM WUR is guaranteed. Moreover, the workload may be large if an overlaid OFDM sequence design is supported. A longer LP-SS sequence design may cause too much power consumption when received by LP-WUR, and a short design will result in worse detection performance. Therefore, a trade-off of the OFDM sequence length design should be studied, large simulation work is needed, and it seems not meaningful if the final design is similar to the SSS sequence design. Considering the LP-SS design also affects the RRM measurement procedure, complex LP-SS generation will cause a large normative workload, thus the OFDM sequence overlaid design is better not to be considered in the LP-SS generation procedure.

Proposal 6: Not considering overlaid OFDM sequence in the LP-SS design.

- Monitoring

Based on the WID description, at least the duty-cycled monitoring mechanism is prioritized, and whether continuous monitoring can be supported needs further discussion. Apparently, continuous monitoring is able to guarantee that the WUS signal can be always detected, while extra power consumption is caused in UE due to its consistent monitoring. For consideration of reducing extra power consumption, only duty-cycled monitoring is better to be utilized. However, DRX cycle should be reasonable enough so that all WUS signals can be detected, otherwise at least detection performance will be affected. To realize that target, the DRX cycle needs to be small and configurable, and can be utilized when the main radio is in the sleep state.

Proposal 7: Only duty-cycled monitoring is better to be utilized in the monitor mechanism.

- Conclusion

In this contribution, we discuss the aspects on the LP-WUS generation procedure, and have the following observations and proposals:

Proposal 1: A suitable OFDM sequence design shall be considered based on the following aspects:

- Sequence types, such as ZC sequence, Gold sequence or some predefined sequence

- Sequence generation parameters, such as u/v values in the ZC sequence generation procedure

- Payload size carried on the corresponding sequence.

Proposal 2: LP-WUS generation description in the potential spec. may including the following section:

- General

- Sequence generation

- LP-WUS format 0/1

Proposal 3: OOK-1 LP-WUS generation description in the potential spec. may include the following section:

- Sequence modulation

- Mapping to virtual resource blocks if necessary

- Mapping to physical resource blocks

Proposal 4: OOK-4 LP-WUS generation description in the potential spec. may include the following section:

- Rate match to generate signal blocks

- Sequence spread or sequence modulation based on the target signal bandwidth

- DFT precoder

- Central part repetition

- Mapping to virtual resource blocks if necessary

- Mapping to physical resource blocks

Proposal 5: The scheme ‘central part repetition’ should be supported based on following steps:

- Generate a DFT precoding input with bandwidth R

- Generate multiple DFT precoding inputs based on different frequency start position

- Cut off the side lobe

- Sequential splicing

Proposal 6: Not considering overlaid OFDM sequence in the LP-SS design.

Proposal 7: Only duty-cycled monitoring is better to be utilized in the monitor mechanism.

- Reference

# RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), 3GPP TSG-RAN Plenary Meeting #102, Edinburgh, UK, December 11th – December 15th, 2023.

- IEEE P802.11ba, Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications, Amendment 3: Wake-Up Radio Operation.

- TR 38.869 v1.1.1, Study on low-power wake up signal and receiver for NR, October, 2023.

- TS 38.211 v17.4.0, Physical channels and modulation, December, 2022.

Appendix A：Simulation assumptions

Table 1. Simulation assumptions for option OOK-4.

| Attributes | Assumptions |
| --- | --- |
| MC-OOK method | Option OOK-4 |
| Carrier Frequency | 4GHz |
| Channel structure | Preamble +data +CRC: 16chips/32 chips+ 32 bits +8 CRC bits |
| Coding | 1/2 rate Manchester coding (For information bits and CRC bits) |
| Chip rate | 28kbps, 56kbps |
| SCS | 30kHz |
| WUS BW | 3RB~1.08MHz, 6RB~2.16MHz, 9RB~3.24MHz, 12RB~4.32MHz |
| Filter | 5th Order Butterworth |
| Sampling Rate | 3.84MHz |
| Channel Model | TDL-C 300ns |
| Guardband | 2RB |
| ADC bits | 1bit, 2bit, 4bit, 8bit, idle |

Appendix B：Generation process of ‘central part repetition’

As we known, the transmission process can be divided into three parts, i.e., block generation, WUS sharing, and multi-carrier modulation. The reception process can also be divided into three parts, i.e., analog to digital transform, time synchronization, and envelope detection.

After LS square pre-distortion, the signal can be expressed as

x=FinvHFinv-1FinvHb

where Finv is the N×N dimension Fourier inverse matrix. The original equation can also be simplified as

x=Finv-1FinvH-1FinvHb=Finv-1b≈Fb

where F is a N×N dimension Fourier matrix.

More specifically, the signal in an OFDM symbol can be expressed as

where w=exp2πjN, and L is the number of frequency input REs.

As mentioned in section 2.1.2, if H0 is exactly at the deep fading part of the channel, most of the signal energy will be lost, making it very difficult to distinguish between the input signal and Gaussian noise by envelope detection. In this way, we want to solve the problem from two aspects.

- We can disperse effective energy distribution. That is to say, we can increase more inputs and reduce the power of each input to guarantee constant total energy and acquire frequency diversity.

- We can also cut off the non-centralized part in the allocated bandwidth. In other words, the non-centralized part has little contribution to the waveform generation but occupies most of the bandwidth, such part can be replaced by more LS square pre-distortion inputs to get more frequency diversity gain and improve the detection performance.

Assuming R is the total allocated bandwidth of LP-WUS, Q inputs are generated with different phase rotations in the Q segments and kc is the starting position of the allocated bandwidth, three steps can be designed to achieve the scheme.

Step 1: Generate LS pre-distortion input based on bandwidth R. Since bandwidth is divided equally into Q segments, the number of LS pre-distortion inputs should be Q.

Step 2: Cut off the non-centralized part. Only the first 1Q of each input will be reserved, and a power factor is also needed to guarantee constant total energy.

Step 3: Combining all the mapping segments to produce the final frequency domain mapping result. Since the size of each segment is RQ, the total bandwidth size after combination mapping is still R.

A power factor is also needed to guarantee average EPRE of LP-WUS is the same as legacy NR signals, and the SNR is calculated based on per RE level. Under this approach, frequency diversity is acquired by frequency repetition, guaranteeing that the time domain waveforms are still distinguishable enough to be judged by the comparator after the multipath fading.