3GPP TSG RAN WG1 #116        		                           R1-2400697

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	5

Source: 	Samsung

Title: 	Discussion on RAN2 LS on RACH-less handover

Document for:	Discussion and Decision

Introduction

In RAN1#115 meeting, it discussed on RAN1 specification impact for RACH-less handover, and following agreement and conclusion were made.

Agreement The text proposal for TS 38.213 in section 3.3 in R1-2312377 is endorsed in principle. Draft CR to be reviewed and endorsed in RAN1#116. - Note: further discuss the TP for search space, repetition, power control, collision with valid PRACH occasion, TDD aspect, etc. - Note to the TS 38.213 editor: it is not expected to capture this TP for RAN#102 Conclusion Which CORESET to use for RACH-less handover can be further discussed at a future meeting.

This contribution discusses remaining issues related to the RAN2 LS on RACH-less handover, and corresponding potential RAN1 specification impact.

Discussion

Remaining issues for TP

In accordance with the agreement reached at the RAN1 #115 meeting, there is a need to discuss the remaining issues related to TP, taking into account search space, repetition, power control, collision with valid PRACH occasions, and TDD aspects. For the search space, it was proposed to consider introducing text proposal for a search space for monitoring PDCCH for initial PUSCH transmission in the case of RACH-less handover, which is similar to SDT. However, in the latest version of TS 38.331 (Appendix 5.3), it is noted that the RACH-less handover command (i.e., RACH-LessHO-r18) is transmitted in ReconfigurationWithSync, which includes ServingCellConfigCommon. As a result, the search space configuration is already specified in TS38.331 in the following procedures: ServingCellConfigCommon → DownlinkConfigCommon → BWP-DownlinkCommon → PDCCH-ConfigCommon → searchSpaceZero or commonSearchSpaceList. Therefore, an additional text proposal for the search space may be unnecessary.

Regarding repetition, there was discussion on whether to introduce repetition for initial PUSCH transmission since it is NTN. However, in our understanding, it is a new feature that has not been previously discussed in both RAN1 and RAN2, and thus it is too late to consider this new feature during the maintenance phase. If repetition is required, other alternatives can be considered, such as PRACH repetition, Msg.3 repetition, and retransmission of initial PUSCH transmission. Consequently, an additional textual proposal for repetition is unnecessary.

Regarding power control, in TS 38.213, the following text is specified for SDT. Since power control for DG PUSCH is already specified in clause 7.1.1, the following text is necessary for CG PUSCH in the case of RACH-less handover.

A UE determines a power of a PUSCH transmission as described in clause 7.1.1, where the UE obtains 𝑃𝐿𝑏,𝑓,𝑐(𝑞𝑑) using a RS resource from an SS/PBCH block with index associated with the PUSCH transmission.

Regarding collisions with valid PRACH occasions, some companies argue that it is necessary because SDT PUSCH already specifies this behavior, and a similar feature may occur in RACH-less handover. On the other hand, other companies believe that it is unnecessary because the network can avoid overlapping events by configuring PUSCH resources and PRACH resources in a TDM manner. If the network can handle these configurations easily without any overlaps, then an additional textual proposal for that is unnecessary. Otherwise, it may be necessary to consider relaxing the operational burden on the network. Thus, it is slightly preferable to consider adding relevant text for collisions with valid PRACH occasions.

Finally, regarding the TDD aspect, it is premature to consider this issue because NTN bands are currently only FDD. Based on the above reasons, we propose the following text proposals.

Proposal 1: Support the following the updated text proposals for RACH-less handover.

- Reason for change: Support RACH-less handover for NR NTN.

- Summary of change: Specify 1) the pre-allocated grant is provided with association to SSBs, and 2) the mapping between type-1 CG and SSBs in CG-SDT is baseline of how to configure pre-alocated grant mapped to SSBs

- Consequences if not approved: Incomplete support for RACH-less handover for NR NTN.

22 PUSCH transmission in NTN RACH-less handover A UE indicated to perform PUSCH transmission in RACH-less handover can be provided one or more configurations by respective one or more ConfiguredGrantConfig, for configured grant Type 1 PUSCH transmissions on the initial UL BWP [12, TS 38.331]. A UE can be provided by ntn-SSB-Subset a number of SS/PBCH block indexes NPUSCHSS/PBCH to map to a number of [valid] PUSCH occasions for PUSCH transmissions over an association period. If the UE is not provided ntn-SSB-Subset, the UE determines NPUSCHSS/PBCH from the value of ssb-PositionsInBurst in ServingCellConfigCommon. A PUSCH occasion for a PUSCH transmission is defined by a time resource and a frequency resource and is associated with a DM-RS provided by cg-DMRS-Configuration for the configuration of PUSCH transmissions. An association period, starting from frame with SFN 0, for mapping NPUSCHSS/PBCH SS/PBCH block indexes, from the number of SS/PBCH block indexes, to PUSCH occasions and associated DM-RS resources is the smallest value in the set determined by the PUSCH configuration period provided by periodicity in ConfiguredGrantConfig according to Table 19.1-1 such that NPUSCHSS/PBCH SS/PBCH block indexes are mapped at least once to [valid] PUSCH occasions and associated DM-RS resources within the association period. A UE is provided a number of SS/PBCH block indexes associated with a PUSCH occasion and a DM-RS resource by ntn-SSB-PerCG-PUSCH. If after an integer number of SS/PBCH block indexes to PUSCH occasions and associated DMRS resources mapping cycles within the association period there is a set of PUSCH occasions and associated DMRS resources that are not mapped to NPUSCHSS/PBCH SS/PBCH block indexes, no SS/PBCH block indexes are mapped to the set of PUSCH occasions and associated DMRS resources. An association pattern period includes one or more association periods and is determined so that a pattern between PUSCH occasions with associated DMRS resources and SS/PBCH block indexes repeats at most every 640 msec. PUSCH occasions and associated DMRS resources not associated with SS/PBCH block indexes after an integer number of association periods, if any, are not used for PUSCH transmissions. NPUSCHSS/PBCH SS/PBCH block indexes are mapped to [valid] PUSCH occasions and associated DMRS resources in the following order - first, in increasing order of DMRS resource indexes within a PUSCH occasion, where a DMRS resource index DMRSid is determined first in an ascending order of a DMRS port index and second in an ascending order of a DMRS sequence index [4, TS 38.211] - second, in increasing order of PUSCH configuration period indexes [A PUSCH occasion is valid if it does not overlap with a valid PRACH occasion as described in clause 8.1.] A UE determines a power of a configured grant Type 1 PUSCH transmission as described in clause 7.1.1, where the UE obtains 𝑃𝐿𝑏,𝑓,𝑐(𝑞𝑑) using a RS resource from an SS/PBCH block with index associated with the PUSCH transmission.

Which CORESET to use RACH-less handover

For path-loss determination for dynamic grant PUSCH, the following sentence was specified.

- PLb,f,c(qd) is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index qd for the active DL BWP, as described in clause 12, of carrier f of serving cell c - If the UE is not provided PUSCH-PathlossReferenceRS and enableDefaultBeamPL-ForSRS, or before the UE is provided dedicated higher layer parameters, the UE calculates PLb,f,c(qd) - using a RS resource from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB - if the UE is provided ntn-RACH-LessHO in ReconfigurationWithSync [12. TS 38.331], using a RS resource from an SS/PBCH block with same SS/PBCH block index as the one with same quasi co-location properties as for PDCCH receptions for scheduling an initial PUSCH transmission, as described in Clause 10.1, in controlResourceSetZero provided in ServingCellConfigCommon of ReconfigurationWithSync

One company raised the issue that controlResourceSetZero is used for a UE to perform a beam-sweeping procedure in order to find a beam index, whereas RAN2 agreed that only one beam information is delivered to the UE. Our understanding is that there is no specification text mandating the UE to perform beam sweeping when monitoring PDCCH in controlResourceSetZero. It is also noted that either tci-StateID-r18 or dg-beam-r18 would be indicated to the UE to monitor PDCCH. Therefore, in our understanding, a UE configured with rach-LessHO is able to monitor PDCCH candidates in controlResourceSetZero based on the beam index which is configured by either tci-StateID-r18 or dg-beam-r18. Hence, there is no further issue to address.

Observation 1: In current specification, for RACH-less handover, a UE is able to monitor PDCCH according to controlResourceSetZero with a beam index which is configured by either tci-StateID-r18 or dg-beam-r18.

Whether to consider other PUSCHs than initial PUSCH for PL determination

An additional issue was identified when discussing the draft CR on TS 38.213 based on the agreement. The aforementioned agreement was made based on a RAN2 question, which is why the agreement focused solely on the initial PUSCH transmission. However, the following two cases should be clarified:

Case 1) Retransmissions of the initial PUSCH transmission (i.e., same TB), and

Case 2) Subsequent PUSCH transmissions after the initial PUSCH transmission (i.e., different TB).

These were discussed in the previous meeting. However, some companies argued that "an RS resource from an SS/PBCH block with the same SS/PBCH block index as the one the UE uses to obtain the MIB" can be used for both Case 1 and Case 2. However, it is unclear when the UE would be scheduled to transmit PUSCHs other than the initial PUSCH before the time when the UE obtains the MIB. Additionally, although the gNB transmits SS/PBCH periodically, it does not know when the UE will obtain the MIB accurately. Therefore, it is preferable that the same path-loss determination should be applied to Cases 1 and 2, as well as the initial PUSCH transmission. With this understanding, we would like to propose the following text proposal.

Proposal 2: Support the following text propsosal for pathloss determination in case of DG PUSCH in RACH-less handover.

- Reason for change: Provide the same RS resource for dynamic grant PUSCH in RACH-less handover.

- Summary of change: Including other PUSCHs as well as initial PUSCH regarding path-loss determination for RACH-less handover.

- Consequences if not approved: Different RS resources would be applied into initial PUSCH and subsequent PUSCH (or re-transmitted PUSCH).

7 Uplink Power control ------------------------------------------------------unchanged parts are omitted--------------------------------------------------- - PLb,f,c(qd) is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index qd for the active DL BWP, as described in clause 12, of carrier f of serving cell c - If the UE is not provided PUSCH-PathlossReferenceRS and enableDefaultBeamPL-ForSRS, or before the UE is provided dedicated higher layer parameters, the UE calculates PLb,f,c(qd) - using a RS resource from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB - if the UE is provided ntn-RACH-LessHO in ReconfigurationWithSync [12, TS 38.331], using a RS resource from an SS/PBCH block with same SS/PBCH block index as the one with same quasi co-location properties as for PDCCH receptions for scheduling an initial PUSCH transmission, as described in Clause 10.1, in controlResourceSetZero provided in ServingCellConfigCommon of ReconfigurationWithSync ------------------------------------------------------unchanged parts are omitted----------------------------------------------------

Conclusion

The following proposals and observation have been made for RACH-less handover.

Observation 1: In current specification, for RACH-less handover, a UE is able to monitor PDCCH according to controlResourceSetZero with a beam index which is configured by either tci-StateID-r18 or dg-beam-r18.

Proposal 1: Support the following the updated text proposals for RACH-less handover.

- Reason for change: Support RACH-less handover for NR NTN.

- Summary of change: Specify 1) the pre-allocated grant is provided with association to SSBs, and 2) the mapping between type-1 CG and SSBs in CG-SDT is baseline of how to configure pre-alocated grant mapped to SSBs

- Consequences if not approved: Incomplete support for RACH-less handover for NR NTN.

22 PUSCH transmission in NTN RACH-less handover A UE indicated to perform PUSCH transmission in RACH-less handover can be provided one or more configurations by respective one or more ConfiguredGrantConfig, for configured grant Type 1 PUSCH transmissions on the initial UL BWP [12, TS 38.331]. A UE can be provided by ntn-SSB-Subset a number of SS/PBCH block indexes NPUSCHSS/PBCH to map to a number of [valid] PUSCH occasions for PUSCH transmissions over an association period. If the UE is not provided ntn-SSB-Subset, the UE determines NPUSCHSS/PBCH from the value of ssb-PositionsInBurst in ServingCellConfigCommon. A PUSCH occasion for a PUSCH transmission is defined by a time resource and a frequency resource and is associated with a DM-RS provided by cg-DMRS-Configuration for the configuration of PUSCH transmissions. An association period, starting from frame with SFN 0, for mapping NPUSCHSS/PBCH SS/PBCH block indexes, from the number of SS/PBCH block indexes, to PUSCH occasions and associated DM-RS resources is the smallest value in the set determined by the PUSCH configuration period provided by periodicity in ConfiguredGrantConfig according to Table 19.1-1 such that NPUSCHSS/PBCH SS/PBCH block indexes are mapped at least once to [valid] PUSCH occasions and associated DM-RS resources within the association period. A UE is provided a number of SS/PBCH block indexes associated with a PUSCH occasion and a DM-RS resource by ntn-SSB-PerCG-PUSCH. If after an integer number of SS/PBCH block indexes to PUSCH occasions and associated DMRS resources mapping cycles within the association period there is a set of PUSCH occasions and associated DMRS resources that are not mapped to NPUSCHSS/PBCH SS/PBCH block indexes, no SS/PBCH block indexes are mapped to the set of PUSCH occasions and associated DMRS resources. An association pattern period includes one or more association periods and is determined so that a pattern between PUSCH occasions with associated DMRS resources and SS/PBCH block indexes repeats at most every 640 msec. PUSCH occasions and associated DMRS resources not associated with SS/PBCH block indexes after an integer number of association periods, if any, are not used for PUSCH transmissions. NPUSCHSS/PBCH SS/PBCH block indexes are mapped to [valid] PUSCH occasions and associated DMRS resources in the following order - first, in increasing order of DMRS resource indexes within a PUSCH occasion, where a DMRS resource index DMRSid is determined first in an ascending order of a DMRS port index and second in an ascending order of a DMRS sequence index [4, TS 38.211] - second, in increasing order of PUSCH configuration period indexes [A PUSCH occasion is valid if it does not overlap with a valid PRACH occasion as described in clause 8.1.] A UE determines a power of a configured grant Type 1 PUSCH transmission as described in clause 7.1.1, where the UE obtains 𝑃𝐿𝑏,𝑓,𝑐(𝑞𝑑) using a RS resource from an SS/PBCH block with index associated with the PUSCH transmission.

Proposal 2: Support the following text propsosal for pathloss determination in case of DG PUSCH in RACH-less handover.

- Reason for change: Provide the same RS resource for dynamic grant PUSCH in RACH-less handover.

- Summary of change: Including other PUSCHs as well as initial PUSCH regarding path-loss determination for RACH-less handover.

- Consequences if not approved: Different RS resources would be applied into initial PUSCH and subsequent PUSCH (or re-transmitted PUSCH).

7 Uplink Power control ------------------------------------------------------unchanged parts are omitted--------------------------------------------------- - PLb,f,c(qd) is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index qd for the active DL BWP, as described in clause 12, of carrier f of serving cell c - If the UE is not provided PUSCH-PathlossReferenceRS and enableDefaultBeamPL-ForSRS, or before the UE is provided dedicated higher layer parameters, the UE calculates PLb,f,c(qd) - using a RS resource from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB - if the UE is provided ntn-RACH-LessHO in ReconfigurationWithSync [12, TS 38.331], using a RS resource from an SS/PBCH block with same SS/PBCH block index as the one with same quasi co-location properties as for PDCCH receptions for scheduling an initial PUSCH transmission, as described in Clause 10.1, in controlResourceSetZero provided in ServingCellConfigCommon of ReconfigurationWithSync ------------------------------------------------------unchanged parts are omitted----------------------------------------------------

Reference

[1]  R1-2312377,	Summary of discussion on remaining issues for RACH-less handover, Moderator (Samsung)

Appendix

Endorsed text proposal in [1]

- Reason for change: Support RACH-less handover for NR NTN.

- Summary of change: Specify 1) the pre-allocated grant is provided with association to SSBs, and 2) the mapping between type-1 CG and SSBs in CG-SDT is baseline of how to configure pre-alocated grant mapped to SSBs

- Consequences if not approved: Incomplete support for RACH-less handover for NR NTN.

22 PUSCH transmission in NTN RACH-less handover A UE indicated to perform PUSCH transmission in RACH-less handover can be provided one or more configurations by respective one or more ConfiguredGrantConfig, for configured grant Type 1 PUSCH transmissions on the initial UL BWP [12, TS 38.331]. A UE can be provided by ntn-SSB-Subset a number of SS/PBCH block indexes NPUSCHSS/PBCH to map to a number of [valid] PUSCH occasions for PUSCH transmissions over an association period. If the UE is not provided ntn-SSB-Subset, the UE determines NPUSCHSS/PBCH from the value of ssb-PositionsInBurst in ServingCellConfigCommon. A PUSCH occasion for a PUSCH transmission is defined by a time resource and a frequency resource and is associated with a DM-RS provided by cg-DMRS-Configuration for the configuration of PUSCH transmissions. An association period, starting from frame with SFN 0, for mapping NPUSCHSS/PBCH SS/PBCH block indexes, from the number of SS/PBCH block indexes, to PUSCH occasions and associated DM-RS resources is the smallest value in the set determined by the PUSCH configuration period provided by periodicity in ConfiguredGrantConfig according to Table 19.1-1 such that NPUSCHSS/PBCH SS/PBCH block indexes are mapped at least once to [valid] PUSCH occasions and associated DM-RS resources within the association period. A UE is provided a number of SS/PBCH block indexes associated with a PUSCH occasion and a DM-RS resource by ntn-SSB-PerCG-PUSCH. If after an integer number of SS/PBCH block indexes to PUSCH occasions and associated DMRS resources mapping cycles within the association period there is a set of PUSCH occasions and associated DMRS resources that are not mapped to NPUSCHSS/PBCH SS/PBCH block indexes, no SS/PBCH block indexes are mapped to the set of PUSCH occasions and associated DMRS resources. An association pattern period includes one or more association periods and is determined so that a pattern between PUSCH occasions with associated DMRS resources and SS/PBCH block indexes repeats at most every 640 msec. PUSCH occasions and associated DMRS resources not associated with SS/PBCH block indexes after an integer number of association periods, if any, are not used for PUSCH transmissions. NPUSCHSS/PBCH SS/PBCH block indexes are mapped to [valid] PUSCH occasions and associated DMRS resources in the following order - first, in increasing order of DMRS resource indexes within a PUSCH occasion, where a DMRS resource index DMRSid is determined first in an ascending order of a DMRS port index and second in an ascending order of a DMRS sequence index [4, TS 38.211] - second, in increasing order of PUSCH configuration period indexes [A PUSCH occasion is valid if it does not overlap with a valid PRACH occasion as described in clause 8.1.]

38.321 v18.0.0

- 5.33 RACH-less initial UL transmission The initial uplink transmission of a RACH-less handover procedure can be performed either using a dynamic uplink grant or a configured uplink grant Type 1 preallocated by RRC, if configured. When rach-LessHO is configured, the MAC entity shall: 1> if cg-RACH-less-Configuration is configured: 2> select a configured uplink grant for initial uplink transmission according to clause 5.8.2; 2> perform initial uplink transmission in the first available CG occasion for RACH-less handover according to clause 5.8.2. 1> else: 2> if tci-StateID is configured in rach-lessHO: 3> indicate to lower layers the TCI state information included in tci-StateID. 2> else if dg-beam is configured in rach-lessHO: 3> indicate to lower layers the SSB index included in dg-beam. 1> monitor the PDCCH as specified in TS 38.213 [6].

38.331 v18.0.0

#### 5.3.5.3 Reception of an RRCReconfiguration by the UE … - if rach-LessHO was included in reconfigurationWithSync included in spCellConfig of an MCG, and upon indication from lower layers that the RACH-less handover has been successfully completed; or, ... 2> if rach-LessHO was included in reconfigurationWithSync and cg-NTN-RACH-Less-Configuration was configured: 3> release the uplink grant configured for RACH-less handover in cg-NTN-RACH-Less-Configuration; … ##### 5.3.5.5.2 Reconfiguration with sync The UE shall perform the following actions to execute a reconfiguration with sync. 2> if NTN-Config is configured for the target cell: … 3> if rach-LessHO is included: 4> configure lower layers in accordance with rach-LessHO for the target SpCell; … CellGroupConfig ::= SEQUENCE { … ReconfigurationWithSync ::= SEQUENCE { spCellConfigCommon ServingCellConfigCommon OPTIONAL, -- Need M newUE-Identity RNTI-Value, t304 ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000}, rach-ConfigDedicated CHOICE { uplink RACH-ConfigDedicated, supplementaryUplink RACH-ConfigDedicated } … rach-LessHO-r18 RACH-LessHO-r18 OPTIONAL -- Need N ]] } … RACH-LessHO-r18 ::= SEQUENCE { targetNTA-r18 ENUMERATED {zero, source} OPTIONAL, -- Need R tci-StateID-r18 TCI-StateId OPTIONAL, -- Cond MobileIAB dg-beam-r18 SSB-Index OPTIONAL, -- Cond DG-RACH-LessHO ... } RACH-LessHO field descriptions. dg-beam This field indicates a beam that the UE should use in the target cell to monitor PDCCH for initial uplink transmission, see TS 38.321 [3].. targetNTA This field refers to the timing adjustment, see TS 38.213 [13] and TS 38.321 [3], indicating the NTA value which the UE shall use for the target PTAG of handover. Only value source is configured by the network in case source cell is a mobile IAB cell.. tci-StateID This field indicates a beam that the UE should use in the target cell to monitor PDCCH for initial uplink transmission. This field is always present in case this cell is a mobile IAB cell.. ConfiguredGrantConfig ::= SEQUENCE { … rrc-ConfiguredUplinkGrant SEQUENCE { … cg-NTN-RACH-Less-Configuration-r18 CG-NTN-RACH-Less-Configuration-r18 OPTIONAL -- Cond RACH-lessHO ]] } … CG-NTN-RACH-Less-Configuration-r18 ::= SEQUENCE { ntn-cg-RACH-less-RetransmissionTimer INTEGER (1..64) OPTIONAL, -- Need R ntn-RSRP-ThresholdSSB-r18 RSRP-Range, ntn-SSB-PerCG-PUSCH-r18 ENUMERATED {oneEighth, oneFourth, half, one, two, four, eight, sixteen}, ntn-SSB-Subset-r18 CHOICE { shortBitmap-r18 BIT STRING (SIZE (4)), mediumBitmap-r18 BIT STRING (SIZE (8)), longBitmap-r18 BIT STRING (SIZE (64)) } OPTIONAL, -- Need R ntn-DMRS-Ports-r18 CHOICE { dmrsType1-r18 BIT STRING (SIZE (8)), dmrsType2-r18 BIT STRING (SIZE (12)) } OPTIONAL, -- Need R ntn-NrofDMRS-Sequences-r18 INTEGER (1..2) OPTIONAL -- Need R } ServingCellConfigCommon ::= SEQUENCE { physCellId PhysCellId OPTIONAL, -- Cond HOAndServCellAdd, downlinkConfigCommon DownlinkConfigCommon OPTIONAL, -- Cond HOAndServCellAdd uplinkConfigCommon UplinkConfigCommon OPTIONAL, -- Need M supplementaryUplinkConfig UplinkConfigCommon OPTIONAL, -- Need S n-TimingAdvanceOffset ENUMERATED { n0, n25600, n39936 } OPTIONAL, -- Need S ssb-PositionsInBurst CHOICE { shortBitmap BIT STRING (SIZE (4)), mediumBitmap BIT STRING (SIZE (8)), longBitmap BIT STRING (SIZE (64)) } OPTIONAL, -- Cond AbsFreqSSB ssb-periodicityServingCell ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 } OPTIONAL, -- Need S dmrs-TypeA-Position ENUMERATED {pos2, pos3}, lte-CRS-ToMatchAround SetupRelease { RateMatchPatternLTE-CRS } OPTIONAL, -- Need M rateMatchPatternToAddModList SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern OPTIONAL, -- Need N rateMatchPatternToReleaseList SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId OPTIONAL, -- Need N ssbSubcarrierSpacing SubcarrierSpacing OPTIONAL, -- Cond HOAndServCellWithSSB tdd-UL-DL-ConfigurationCommon TDD-UL-DL-ConfigCommon OPTIONAL, -- Cond TDD ss-PBCH-BlockPower INTEGER (-60..50), ..., [[ channelAccessMode-r16 CHOICE { dynamic NULL, semiStatic SemiStaticChannelAccessConfig-r16 } OPTIONAL, -- Cond SharedSpectrum discoveryBurstWindowLength-r16 ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5} OPTIONAL, -- Need R ssb-PositionQCL-r16 SSB-PositionQCL-Relation-r16 OPTIONAL, -- Cond SharedSpectrum highSpeedConfig-r16 HighSpeedConfig-r16 OPTIONAL -- Need R ]], [[ highSpeedConfig-v1700 HighSpeedConfig-v1700 OPTIONAL, -- Need R channelAccessMode2-r17 ENUMERATED {enabled} OPTIONAL, -- Cond SharedSpectrum2 discoveryBurstWindowLength-r17 ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms0dot75, ms1, ms1dot25} OPTIONAL, -- Need R ssb-PositionQCL-r17 SSB-PositionQCL-Relation-r17 OPTIONAL, -- Cond SharedSpectrum2 highSpeedConfigFR2-r17 HighSpeedConfigFR2-r17 OPTIONAL, -- Need R uplinkConfigCommon-v1700 UplinkConfigCommon-v1700 OPTIONAL, -- Need R ntn-Config-r17 NTN-Config-r17 OPTIONAL -- Need R ]], [[ featurePriorities-r17 SEQUENCE { redCapPriority-r17 FeaturePriority-r17 OPTIONAL, -- Need R slicingPriority-r17 FeaturePriority-r17 OPTIONAL, -- Need R msg3-Repetitions-Priority-r17 FeaturePriority-r17 OPTIONAL, -- Need R sdt-Priority-r17 FeaturePriority-r17 OPTIONAL -- Need R } OPTIONAL -- Need R ]], [[ ra-ChannelAccess-r17 ENUMERATED {enabled} OPTIONAL -- Cond SharedSpectrum2 ]], [[ featurePriorities-v1800 SEQUENCE { msg1-Repetitions-Priority-r18 FeaturePriority-r17 OPTIONAL, eRedCapPriority-r18 FeaturePriority-r17 OPTIONAL -- Need R } OPTIONAL, -- Need R atg-Config-r18 ATG-Config-r18 OPTIONAL -- Need R ]] } DownlinkConfigCommon ::= SEQUENCE { frequencyInfoDL FrequencyInfoDL OPTIONAL, -- Cond InterFreqHOAndServCellAdd initialDownlinkBWP BWP-DownlinkCommon OPTIONAL, -- Cond ServCellAdd ..., [[ initialDownlinkBWP-RedCap-r17 BWP-DownlinkCommon OPTIONAL -- Need R ]] } BWP-DownlinkCommon ::= SEQUENCE { genericParameters BWP, pdcch-ConfigCommon SetupRelease { PDCCH-ConfigCommon } OPTIONAL, -- Need M pdsch-ConfigCommon SetupRelease { PDSCH-ConfigCommon } OPTIONAL, -- Need M ... } PDCCH-ConfigCommon ::= SEQUENCE { controlResourceSetZero ControlResourceSetZero OPTIONAL, -- Cond InitialBWP-Only commonControlResourceSet ControlResourceSet OPTIONAL, -- Need R searchSpaceZero SearchSpaceZero OPTIONAL, -- Cond InitialBWP-Only commonSearchSpaceList SEQUENCE (SIZE(1..4)) OF SearchSpace OPTIONAL, -- Need R searchSpaceSIB1 SearchSpaceId OPTIONAL, -- Need S searchSpaceOtherSystemInformation SearchSpaceId OPTIONAL, -- Need S pagingSearchSpace SearchSpaceId OPTIONAL, -- Need S ra-SearchSpace SearchSpaceId OPTIONAL, -- Need S PDCCH-ConfigCommon field descriptions. applyIndicatedTCI-State This field indicates, for PDCCH reception in CORESET #0, if UE applies the first, the second, both or none of the "indicated" DL only TCI or joint TCI as specified in TS 38.213 [13], clause 10.1.. commonControlResourceSet An additional common control resource set which may be configured and used for any common or UE-specific search space. If the network configures this field, it uses a ControlResourceSetId other than 0 for this ControlResourceSet. The network configures the commonControlResourceSet in SIB1 so that it is contained in the bandwidth of CORESET#0. If the (e)RedCap-specific initial downlink BWP does not contain the entire CORESET#0, the network configures the commonControlResourceSet in the (e)RedCap-specific initial downlink BWP in SIB1 for (e)RedCap such that it does not have to be contained in the bandwidth of CORESET#0.. commonSearchSpaceList, commonSearchSpaceListExt, commonSearchSpaceListExt2 A list of additional common search spaces. If the network configures this field, it uses the SearchSpaceIds other than 0. If the field is included, it replaces any previous list, i.e. all the entries of the list are replaced and each of the SearchSpace entries is considered to be newly created and the conditions and Need codes for setup of the entry apply. If the network includes commonSearchSpaceListExt/commonSearchSpaceListExt2, it includes the same number of entries, and listed in the same order, as in commonSearchSpaceList.. controlResourceSetZero Parameters of the common CORESET#0 which can be used in any common or UE-specific search spaces. The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. Even though this field is only configured in the initial BWP (BWP#0) controlResourceSetZero can be used in search spaces configured in other DL BWP(s) than the initial DL BWP if the conditions defined in TS 38.213 [13], clause 10 are satisfied..

RAN1 agreements

RAN1#113 For Q1 (pre-allocated grant) Agreement One company thinks that when the network knows the suitable DL beam for RACH-less handover, the pre-allocated grant can be associated with a SSB index of the target cell, and when the network does not know the suitable DL beam, RACH-based HO can be used instead of introducing beam-sweeped pre-allocated grants associated with multiple SSB indexes. Other companies think that the association between the pre-allocated grant for initial transmission and SSB index should be supported without any condition(s), and think that RSRP threshold may be helpful. For Q2 (target cell PDCCH monitoring) Agreement If single beam is indicated, UE will monitor the target cell PDCCH scheduling the first PUSCH based on the indicated beam. RAN1 will further discuss the case where multiple beams are indicated.. RAN1#114 Agreement The following response to Question 2 in RAN2 LS (R1-2304322) is agreed: - To monitor target cell PDCCH for dynamic grant for initial UL transmission, RAN1 think that there is no case where multiple beams are indicated for RACH-less handover. In this case, UE doesn’t expect that multiple beams are indicated from NW. Agreement For pathloss measurement in case of dynamic scheduled initial PUSCH for RACH-less handover, the UE calculates PLb,f,c(qd) using a RS resource from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to monitor PDCCH scheduling dynamic UL grant for initial transmission. Agreement The following response to Question 3 in RAN2 LS (R1-2304322) is agreed: - For the initial UL transmission scheduled by dynamic grant in RACH-less handover, RAN1 thinks that it follows the principle for power control for Msg3 (or MsgA) PUSCH as described in clause 7.1.1 in TS 38.213 except for pathloss determination. For pathloss determination, the UE uses a RS resource from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to monitor PDCCH scheduling dynamic UL grant for initial transmission. - RAN1 may continue further discussion on question 3.. RAN1#114bis RAN1#115

RAN2 agreements

RAN2#121 Agreements - Support RACH-less Handover in Rel-18. - RACH-less Handover in NR NTN is a L3 mobility procedure (FFS if this is combined with the unchanged PCI approach, if supported) and uses the LTE’s RACH-less Handover procedure as a baseline. FFS on TA acquisition - In NTN RACH-less handover, network indicates (implicitly or explicitly) whether NTA in the target cell is identical to the source cell or explicitly provided by the NW. - Support dynamic grant from the target cell for RACH-less PUSCH transmission to reduce random access congestion in the target cell. FFS whether to limit the solution to same feeder link/gateway scenario. RAN2#121bis-e Agreements - In Rel-18 we don’t aim at RACH-less HO for NTN-TN mobility - For initial UL transmission in RACH-less HO, support pre-allocated grant in RACH-less HO command - NTN RACH-less HO is supported for Intra-satellite handover with the same feeder link. i.e., with same gateway/gNB; - NTN RACH-less HO can be supported for intra-satellite handover with different feeder links, i.e., with gateway/gNB switch, inter-satellite handover with gateway/gNB switch, and inter-satellite handover with same gateway/gNB. - RAN2 confirms the general UE procedure for NTN RACH-less HO - receive a RACH-less HO command which can include pre-allocated grant optionally. FFS N_TA is optional. (RRC) - start timer T304 for the target cell (RRC) - perform DL and UL synchronization, and start timer T430. FFS how to perform RACH-less UL synchronization to NTN target cell. (RRC, MAC) - start time alignment timer (MAC) - monitor target cell PDCCH for dynamic grant if pre-allocated grant is not configured in RACH-less HO command (MAC, PHY) - send initial UL transmission including RRCReconfigurationComplete message using the available UL grant (RRC, MAC, PHY) - consider RACH-less HO is completed upon receiving NW confirmation. FFS how to confirm RACH-less HO is successfully completed. (RRC, MAC) - stop timer T304 for the target cell. (RRC) FFS whether to release UL grant if pre-allocated after RACH-less HO completion FFS RACH-less HO failure handling, e.g. whether UE fallback to RACH-based HO to the target cell FFS procedure for RACH-less HO combined with PCI unchanged or CHO if supported - The pre-allocated grant is provided as type-1 CG - At least for pre-allocated grant, for the confirmation of RACH-less HO completion we reuse of LTE approach, i.e., UE Contention Resolution Identity MAC CE is used but UE ignores the content of this field. FFS if anything else is needed for dynamic grant Consider to support combining RACH-less HO with time-based CHO for NTN, taking into account the 1) validity of pre-allocated grant and potential waste of reserved resource; 2) when/how to provide dynamic grant in PDCCH.. RAN2#122 Agreements - In NTN RACH-less handover, NW either indicates NTA in the target cell is identical to the source cell, or the NTA explicitly provided by the NW is 0. RAN2 will not discuss the case where NTA does not equal to 0 2. From RAN2 perspective synchronization among source and target cells is not an issue in NTN RACH-less HO 3. Release pre-allocated UL grant after RACH-less HO completion 4. LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant. FFS any enhancement to the confirmation of RACH-less HO completion, e.g. the NW does not send the UE Contention Resolution Identity MAC CE, and sends PDCCH/PDSCH addressed to C-RNTI 5. Remove “FFS how to perform RACH-less UL synchronization to NTN target cell”, RAN2 assumes the UL sync handling in the target cell is the same in RACH-based HO and RACH-less HO, except how to acquire NTA (FFS on the spec impact, if any). RAN2#123 Agreements - Single beam can be indicated in HO command to monitor target cell PDCCH for dynamic grant for initial UL transmission - The pre-allocated grant is provided with association to SSBs - The mapping between type-1 CG and SSBs in CG-SDT can be the baseline of how to configure pre-allocated grant mapped to SSBs (can rediscuss in case of different input from RAN1) - UE selects an SSB associated to the pre-allocated grant with RSRP above a configured threshold, use the selected SSB and the corresponding UL grant occasions for the initial UL transmission - ta-Report can be included in ServingCellConfigCommon in the RACH-less HO command - RAN2 understands that if pre-allocated grant is not configured and dynamic grant is used for first UL transmission, if UL HARQ mode is configured, HARQ mode A is recommended for the HARQ process (this is anyway up to NW implementation and there is no Stage2 and Stage3 spec impact) - The MAC entity applies the N_TA (value 0 or same as source cell) configured in the RACH-less HO command for the PTAG. FFS on when timerAlignmentTimer associated with this TAG starts If no SSB mapping to pre-allocated grant has RSRP above the threshold, fallback to RACH HO (with new SSB selection), while T304 is running. RAN2#124 Agreements 1. For dynamic grant case, beam information is mandatorily included in the RACH-less HO command. 2. In NTN RACH-less HO, for dynamic grant case, the beam information included in RACH-less HO command is an SSB index (not tci-stateid). 3. Similar to LTE, UE shall not trigger RACH for SR when rach-lessHO is configured. LTE text is used as a baseline 4. UE releases preallocated grant after successful RACH-less HO completion without additional signaling from the network. Nothing is needed to address this issue in MAC. 5. When CG for initial UL transmission is configured, CG occasions mapping to SSB (i.e. ssb position in burst), is optional. If it is not provided, the RACH-less HO configuration is applicable in all SSBs. Adopt similar wording to CG-SDT in the RRC field description. 6. If CG for initial UL transmission is configured, UE starts to monitor PDCCH according to existing DRX behaviour on the selected SSB from RACH-less HO configuration after initial UL transmission. 7. If CG is configured in RACH-less HO, UE uses the earliest available CG occasion associated to the selected SSB for the initial UL transmission. Spec impact is FFS and can use CG-SDT as baseline (if applicable) 8. It is up to NW to configure HARQ mode A or B. RAN2 understands that HARQ mode A should be used the HARQ process of the initial UL transmission using CG 9. We don’t introduce a threshold-based mechanism for Dynamic Grant 10. Check during the final [Post124] review of the joint NTN/mIAB MAC CR for RACH-less HO if the CG-LTM-retransmission timer for the initial UL transmission using CG introduced in LTM can be used for NTN as well (possibly with updates to the value range).