3GPP TSG RAN WG1 #116		R1-2400698

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	5

Source:	Samsung

Title:	Discussion on reply LS on MAC agreements for SL positioning

Document for:	Discussion and decision

# Introduction

In this paper, we discuss issues related to the incoming LS from RAN2 [1].

| R1-2400008 | LS on MAC agreements for SL positioning | RAN2 |
| --- | --- | --- |

The contents are excerpted as follows:

1. Overall Description: During RAN2#124, the following agreements have been achieved in RAN2 related to SL positioning. - Uplink transmission can be considered as prioritized when uplink cannot be transmitted together with sidelink and none of the V2X sidelink communications or NR sidelink communications or sidelink PRS transmissions are prioritized. - The prioritization between SR triggered by UL-SCH and SL-PRS shall follow the same principle as that between UL-SCH and SL-SCH, i.e. based on configured UL/SL prioritization thresholds. - The prioritization between SR triggered by SL-SCH and SL-PRS shall be based on direct comparison between the SL priority for SL-PRS and the SL logical channel that triggered the SR. - Reuse the legacy threshold for SL communications for SL-PRS prioritization. - Implement support for retransmission on dedicated resource pool. - When there are both SL-PRS and SL-SCH data pending for transmission at resource selection, the resource selection should be within the smaller one of the SL-PRS delay budget of the pending SL-PRSs and PDB of the logical channels.. In addition, RAN2 would like to ask RAN1 the following questions: - On the maximum number of parallel SL-PRS transmission - What is the maximum total number of parallel SL-PRS transmission on SL-PRS shared/dedicated resource pool? - What is the maximum number of parallel SL-PRS transmission supported on a SL-PRS shared resource pool and SL-PRS dedicated resource pool, respectively? - When SL-PRS is transmitted on a SL-PRS shared resource pool where PSFCH is configured, if the associated PSSCH transmission is positively acknowledged, should the UE continue to perform SL-PRS retransmission? 2. Actions: To RAN1 group. ACTION: RAN2 respectfully asks RAN1 to take the above information into account. 3. Date of Next TSG RAN WG2 Meetings: TSG RAN WG2 Meeting #125 26th Feb – 01st March 2024 Athens, GR TSG RAN WG2 Meeting #125-bis 15th Apr – 19th Apr 2024 China (TBC)

In this paper, we provide our views on draft replies.

# Discussion

RAN1 considers the necessity of parallel SL-PRS processes at physical layer depends on existence of feedback for SL-PRS transmissions. For shared resource pool, PSFCH can be configured to indicate HARQ-ACK feedback of PSSCH but not for SL-PRS, even if SL-PRS is multiplexed with PSSCH on the same slot and subchannel. For dedicated resource pool, it does not support to configure PSFCH. Therefore, in our understanding, from RAN1 perspective that is up to RAN2 to decide whether and how to use parallel processes for SL-PRS transmission.

When SL-PRS is transmitted on a SL-PRS shared resource pool where PSFCH is configured, receiving Acknowledgement means PSSCH being successfully transmitted, but does not imply any feedback on SL-PRS transmissions. In addition, SL-PRS measurement accuracy depends on number of SL-PRS transmissions, thus receiving single SL-PRS transmission cannot ensure that accuracy requirement being satisfied. Therefore, UE should perform SL-PRS retransmission(s) regardless of received Acknowledgement feedback on PSFCH in a shared resource pool.

# Draft reply LS

1. Overall Description: RAN1 would like to thank RAN2 for informing RAN2 agreements regarding SL-PRS parallel transmissions and SL-PRS retransmission in shared resource pool. RAN1 has discussed this issue, and concluded that It is up to RAN2 to decide maximum number of parallel SL-PRS transmission in a shared resource pool, or a dedicated resource pool, or both. When SL-PRS is transmitted on a SL-PRS shared resource pool where PSFCH is configured, the UE should perform SL-PRS retransmissions regardless of received positive acknowledgement. 2. Actions: To RAN2: ACTION: RAN1 respectfully requests RAN2 to take the above information in the future work.

# References

- R1-2400008(R2-2313848), LS on MAC agreements for SL positioning, RAN2, RAN1#116, Athens, Greece.