3GPP TSG RAN WG1#116			R1-2400707

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	8.5

Source: 	Samsung

Title: 	Maintenance issues for Rel-18 LTM

Document for:	Discussion and Decision

# Introduction

This contribution provides Samsung’s views regarding the maintenance issues for Rel-18 LTM.

# Discussions

## Beam indication

## LTM TCI states during cell switch

There are two scenarios supported for activating and indicating TCI states during cell switch. The first scenario, is to activate LTM TCI states before the cell switch command and indicate one joint or a pair of DL/UL LTM TCI states in the cell switch command from the activated LTM TCI states. The second scenario, is to activate and indicate one joint LTM TCI state or a pair of DL/UL LTM TCI states in the cell switch command.

Agreement RAN1#114

- TCI state activation by MAC CE before cell switch command for one or more than one candidate cells is allowed

Agreement RAN1#113

- For TCI state activation for candidate cell(s) before the cell switch command,

  - MAC CE is used and the details of MAC-CE for TCI state activation for LTM is up to RAN2

  - Further study if PDCCH order for candidate cell(s) can be used

For first scenario, the following agreements have been made in prior meetings:

Activation of LTM TCI states on candidate cells, doesn’t indicate cell switch, and hence the activated TCI states of the serving cell need to be kept for communicating with the UE and potentially indicated to the UE at a future time. Therefore, when LTM TCI states for candidate cells are activated, this doesn’t impact the activated TCI states of the serving cell.

Proposal 1: Activation of LTM TCI states on candidate cells doesn’t impact activated TCI states of the serving cell.

[FL Proposal 5-4-1c-v4]

- The TCI state indicated in the cell switch command is associated with

  - LTM TCI state pool of the target cell, i.e. configured under LTM-Candidate-r18

- UE uses the TCI state provided in the cell switch command after receiving cell switch command until a TCI state [in the new serving cell] is indicated

  - Note: after TCI in the new serving cell is indicated, legacy BM is used.

- FFS: whether and how retain LTM activated TCI states for candidate cell

In RAN1#114b, the following proposal was discussed but not agreement was reached:

The activated TCI states before the cell switch command and the indicated TCI state in the cell switch command should be from the LTM TCI state pool. After indicating one joint or a pair of DL/UL TCI states of the activated TCI states in the cell switch command, UE starts using the TCI states of the candidate cell. At this time, the activated TCI states of the original serving cell, are no longer used and can be deactivated. The candidate cell becomes the new serving cell. The activated LTM TCI states of the candidate cells, become the activated TCI states for candidate cell and can be used for beam indication using the “transmission configuration indicator” field of DCI format 1_1 or DCI format 1_2.

Proposal 2: After indicating one or a pair of TCI state, from the LTM TCI state pool of the target cell, in the cell switch command, the activated TCI states of the serving cell can be deactivated.

Proposal 3: After the cell switch command, the activated LTM TCI states of the candidate cell (the new serving cell) become the activated TCI states of the new serving cell and can be indicated to the UE until a new subset of TCI states is activated.

After cell switch to the candidate cell (new serving cell), a MAC CE activating TCI states from the TCI state list of the new serving cell is sent to the UE. The LTM TCI states of the candidate cells are deactivated. A TCI state indicated to the UE is from the activated TCI states of the new serving cell.

Proposal 4: After the cell switch command and after a MAC CE activating TCI states on the new serving cell, the activated LTM TCI states become deactivated.

Figure 1 illustrates the activation and indication of TCI states before, during and after cell switch.

Figure 1: TCI state activation and indication before, during and after cell switch.

## Beam indication for multiple cells

In the previous meetings the beam indication for multiple cells for CA was mentioned but not discussed in RAN1. Furthermore, RAN2 (in LS R1-230883) made the following agreements

Proposal 2	SCell(s) can be added/modified/released within an LTM candidate cell configuration.

Scell activation state is not in the LTM cell switch MAC CE, but only based on the RRC configuration

Conclusion

No consensus to include simultaneousU-TCI-UpdateList under LTM-Candidate-r18 to activate and indicate TCI states for SCell(s) after cell switch command.

As we are in the maintenance phase, a design for TCI state indication across multiple cells should reuse the existing Rel-17 design as much as possible. Furthermore, there was a conclusion related to this subject matter made in RAN1#115.

Hence, the following aspects can be considered:

- The list(s) of cells (same index) that follow the indicated TCI state for the serving cell also applies to the candidate cell.

- The cell switch command can include a flag indicating whether to apply the indicated TCI state(s) in the cell switch command to the candidate Pcell only or to additional cells as determined by the configured list(s) of cells applying the indicated TCI state.

Proposal 5: For multi-cell operation in LTM consider the following:

- The list(s) of cells (same index) that follow the indicated TCI state for the serving cell also applies to the candidate cell.

- The cell switch command can include a flag indicating whether to apply the indicated TCI state(s) in the cell switch command to the candidate Pcell only or to additional cells as determined by the configured list(s) of cells applying the indicated TCI state.

## Beam application time

For the beam application time, as the TCI state is indicated in the MAC CE of the cell switch command, the beam application time is T slots after the channel conveying the HARQ-ACK feedback (PUCCH or PUSCH). The following can be considered:

- If the TCI state has been activated before the cell switch command, the beam application time T is 3Nslotsubframe,µ slots.

- If the TCI state has not beam activated before the cell switch command, value of T is decided by RAN4. Based on the current RAN4 specifications, when the source RS of the TCI state is SSB, the beam application time T can be given by (see clause 8.10.3 of TS 38.133): 3Nslotsubframe,µ+TOk*Tfirst-SSB+TSSB-procNR slot length. Where,

  - TOk=1, as the target TCI state is not in the active list.

  - Tfirst-SSB is time to the first SSB transmission after the MAC CE with the cell switch command is decoded.

  - TSSB-proc=2 ms

The beam application time is independent of the sub-carrier spacing of the channel used for the cell switch command and of the channels or signals to which the beam indication is being applied.

Proposal 6: When the TCI state has been activated before, the cell switch command the beam application time is 3Nslotsubframe,µ. When the TCI state is activated in the cell switch command. The beam activation time is determined by following RAN4 rules.

Proposal 7: The beam application time is independent of the sub-carrier spacing of the channel used for the cell switch command and of the channels or signals to which the beam indication is being applied.

## Configuration of TRS for candidate cells

Agreement

In R18 LTM, on the QCL source of the TCI state before/during the cell switch command,

- SSB or TRS can be configured in a TCI state for the candidate cell(s) before/during cell switch command

  - Whether the TRS can be used for the candidate cell(s) before/during cell switch command is up to UE capability

In RAN1#114, it has been agreed that the TRS can be configured as a QCL source for the LTM TCI states.

When the UE supports TRS as a source RS of an LTM TCI state for a candidate cell, this would require configuration of NZP CSI-RS resources with a LTM TCI state for quasi-co-location information. The NZP-CSI-RS resource IE

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}                 OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                                     OPTIONAL,   -- Cond Periodic

...

}

The field qcl-InfoPeriodicCSI-RS can refer to a LTM TCI state ID of a candidate cell. We can have a single list of CSI-RS resources with qcl-Info provided by TCI-StateId or LTM-tci-StateId. Alternatively, we can have separate lists of CSI-RS resources, a first list for qcl-Info provided by TCI-StateId, and a second list for qcl-Info provided by LTM-tci-StateId.

A CSI-RS resource with qcl-Info provided by LTM-tci-StateId can be used as a source RS for a LTM TCI state of a candidate cell if supported by the UE.

Proposal 8: If a UE supports TRS as a source RS of an LTM TCI state for a candidate cell, a NZP-CSI-RS-resource can be configured with qcl-InfoPeriodicCSI-RS referring to a LTM TCI state ID of a candidate cell.

## Dynamic cell switch

In RAN1#111, the following agreement was made:

Agreement

- For beam indication timing for Rel-18 LTM,

    - Support Scenario 2: Beam indication together with cell switch command,

      - For Rel-17 unified TCI framework,

        - Beam indication indicates TCI state for each target serving cell

  - FFS: Scenario 1: Beam indication before cell switch command

  - FFS: Scenario 3: Beam indication after cell switch command

- FFS: Activation of TCI state(s) of target serving and/or candidate cell(s).

It is beneficial to have ICBM and LTM integrated in the same framework to allow for seamless transmission from one cell to the next, first using a beam from a target cell having a PCI separate from the PCI of the serving cell, then transitioning to the target cell by a cell switch command. The UE starts communicating using an indicated TCI state with the target cell before the cell switch command. This would be the case of ICBM. A cell switch command is then sent to complete the handover procedure. The advantage of using ICBM beam management for L1/L2 mobility is to reduce latency, signalling overhead and interruption time. We see LTM and ICBM as complimentary solutions, therefore supporting both of them is an important feature. A benefit of having the UE communicate with the target cell first using ICBM, before doing a cell switch using LTM is to minimize communication distribution between the UE and the network. In high mobility scenarios, the link to the source cell could degrade rapidly, making it plausible that the cell switch command from the source serving could be missed by the UE, hence switching beams to the target cell before cell switch and having the cell switch command come from the target cell could improve reliability and reduce disruption.

Proposal 9: Rel-18 LTM and Rel-17 ICBM can operate together. A UE first uses a beam of the target cell, before switching to the target cell.

The UE is first indicated a TCI state(s) for a target cell using the ICBM framework. A UE is then indicated a LTM TCI state(s) in the cell switch command. If the QCL source RS of the TCI state(s) indicated to the UE using the ICBM framework and the QCL source RS of the TCI state(s) in the cell switch command are the same. The UE maintains the same beam after cell switch.

Proposal 10: If the QCL source RS of TCI state(s) indicated to the UE before cell switch and the QCL source RS of TCI state(s) in the cell switch command are the same, the UE maintains the same beam after cell switch.

# Candidate cell TA management

To facilitate the UE-based TA acquisition, a UE can be RRC configured by the network a number of candidate cells on which the UE-based TA acquisition can be performed. We therefore propose the following:

Proposal 11: A UE can be configured by RRC a number of candidate cells on which the UE-based TA acquisition should be performed.

In addition, UE’s behaviors of determining and applying TAs for the candidate cell(s) when both the UE-based TA acquisition and the RACH procedure based TA acquisition are configured/enabled need to be specified. When configured with UE-based TA acquisition, the UE can first calculate and apply the TA – determined based on the DL timing difference; when/after the UE is indicated a TA value by the cell switching command, the UE would apply the indicated TA instead – i.e., the indicated TA value would overwrite the TA value computed by the UE.

Proposal 12: For the same candidate cell, a UE can be configured with both UE-based TA acquisition and RACH resource for TA acquisition

- If a TA value is indicated by the cell switching command, the UE shall apply the indicated TA value rather than the TA value determined by the UE for the candidate cell;

- Otherwise, the UE shall apply their determined TA value for the candidate cell.

# Conclusions

In this contribution, the following observation and proposals are given:

Proposal 1: Activation of LTM TCI states on candidate cells doesn’t impact activated TCI states of the serving cell.

Proposal 2: After indicating one or a pair of TCI state, from the LTM TCI state pool of the target cell, in the cell switch command, the activated TCI states of the serving cell can be deactivated.

Proposal 3: After the cell switch command, the activated LTM TCI states of the candidate cell (the new serving cell) become the activated TCI states of the new serving cell and can be indicated to the UE until a new subset of TCI states is activated.

Proposal 4: After the cell switch command and after a MAC CE activating TCI states on the new serving cell, the activated LTM TCI states become deactivated.

Proposal 5: For multi-cell operation in LTM consider the following:

- The list(s) of cells (same index) that follow the indicated TCI state for the serving cell also applies to the candidate cell.

- The cell switch command can include a flag indicating whether to apply the indicated TCI state(s) in the cell switch command to the candidate Pcell only or to additional cells as determined by the configured list(s) of cells applying the indicated TCI state.

Proposal 6: When the TCI state has been activated before, the cell switch command the beam application time is 3Nslotsubframe,µ. When the TCI state is activated in the cell switch command. The beam activation time is determined by following RAN4 rules.

Proposal 7: The beam application time is independent of the sub-carrier spacing of the channel used for the cell switch command and of the channels or signals to which the beam indication is being applied.

Proposal 8: If a UE supports TRS as a source RS of an LTM TCI state for a candidate cell, a NZP-CSI-RS-resource can be configured with qcl-InfoPeriodicCSI-RS referring to a LTM TCI state ID of a candidate cell.

Proposal 9: Rel-18 LTM and Rel-17 ICBM can operate together. A UE first uses a beam of the target cell, before switching to the target cell.

Proposal 10: If the QCL source RS of TCI state(s) indicated to the UE before cell switch and the QCL source RS of TCI state(s) in the cell switch command are the same, the UE maintains the same beam after cell switch.

Proposal 11: A UE can be configured by RRC a number of candidate cells on which the UE-based TA acquisition should be performed.

Proposal 12: For the same candidate cell, a UE can be configured with both UE-based TA acquisition and RACH resource for TA acquisition

- If a TA value is indicated by the cell switching command, the UE shall apply the indicated TA value rather than the TA value determined by the UE for the candidate cell;

- Otherwise, the UE shall apply their determined TA value for the candidate cell.