3GPP TSG RAN WG1 #116		R1-2400745

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	9.6.3

Source:	Samsung

Title:	LP-WUS operation in CONNECTED modes

Document for:	Discussion and Decision

# Introduction

In WID [1], an objective regarding to support LP-WUS and LP-WUR in RRC CONNECTED mode has been agreed, wherein the details are described as follow:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

This contribution discusses the details on supporting LP-WUS and LP-WUR in RRC CONNECTED mode.

# General design principle

As noted in the WID [1], the optimization of LP-WUS design for RRC_IDLE and RRC_INACTIVE modes is prioritized over the optimization for RRC_CONNECTED mode. Following this general principle, design aspects for LP-WUS operation in RRC_CONNECTED mode shall take the ones for RRC_IDLE and RRC_INACTIVE as a baseline, with further optimization whenever essentially needed. For example, the proposals in [2] and [3] are served our baseline for LP-WUS design in RRC_CONNECTED mode, with further considerations on aspects detailed in Section 3 to 6 in this contribution which are in particular applied to RRC_CONNECTED mode only.

Proposal 1: LP-WUS operation in RRC_CONNECTED mode shall take RRC_IDLE and RRC_INACTIVE modes as a baseline, and is only optimized over aspects exclusively applicable for RRC_CONNECTED mode.

# Information carried by LP-WUS

Similar to RRC_IDLE/INACTIVE mode [2], the amount of information carried by LP-WUS shall consider the trade-off between the amount of resources used for LP-WUS transmission and the detection/decoding performance of the LP-WUS (e.g., false alarm, misdetection, etc.), wherein the maximum number of bits carried by LP-WUS can be determined referring to [3].

To avoid severe false alarm (and the resulting power waste at the MR), the LP-WUS should carry ID information of the target receiver as a baseline, and on top of that, an explicit indication on whether to trigger the PDCCH monitoring can also be included in the LP-WUS. For example, the indication for triggering the PDCCH monitoring can take DCI format 2_6 as a reference.

Proposal 2: For RRC_CONNECTED mode, information carried by LP-WUS can include:

- Identification (UE-group ID, or UE-subgroup ID);

- Indication on whether to trigger the PDCCH monitoring.

# Configuration of LP-WUS

The configuration of LP-WUS at least include the time domain and frequency domain resources for the monitoring occasions of LP-WUS, and for RRC_CONNECTED mode, the configuration can be further provided by dedicated RRC parameter, in addition to the methods supported in RRC_IDLE and RRC_INACTIVE modes.

Proposal 3: In RRC_CONNECTED mode, the configuration of LP-WUS is further provided by dedicated RRC parameter, in addition to the methods supported in RRC_IDLE and RRC_INACTIVE modes.

Detailed configuration of LP-WUS in RRC_CONNECTED mode shall reuse the one for RRC_IDLE and RRC_INACTIVE as much as possible, and proposals in [2] for RRC_IDLE and RRC_INACTIVE modes are repeated here for RRC_CONNECTED mode.

Proposal 4: For configuration of LP-WUS:

- Frequency location of LP-WUS shall be configurable;

- BW of LP-WUS is fixed in a number of PRBs or a number of MHz;

- TDM with other signal(s) and channel(s), and FDM with other signsl(s) and channel(s) in the same numerology shall be supported;

- Sub-grouping for LP-WUS can be supported;

- DRX operation for monitoring LP-WUS is supported.

Moreover, one design aspect exclusively applicable for RRC_CONNECTED mode is the alignment between LP-WUS DRX and C-DRX if C-DRX operation is configured for UE monitoring LP-WUS. For example, as a baseline, the active durations for LP-WUS DRX can be considered as a subset of the active durations for C-DRX, which implies the cycle for LP-WUS DRX is an integer multiple of the C-DRX cycle.

Proposal 5: Alignment between LP-WUS DRX and C-DRX can be supported.

- The cycle for LP-WUS DRX is an integer multiple of the C-DRX cycle.

# Monitoring of LP-WUS

The monitoring of LP-WUS can be triggered by the gNB using either an explicit way or an implicit way, and an application delay can be considered to allow enough time for the UE to turn on the LR for monitoring LP-WUS. Meanwhile, in order to avoid any misalignment between the gNB and UE on which receiver is used, one or two confirmations can be transmitted by the MR to the gNB to confirm the successful reception of the trigger and LP-WUS in RRC_CONNECTED mode. For example, the first confirmation message can be transmitted from the MR to the gNB to confirm the successful reception of the trigger by the MR, and the second confirmation message can be transmitted from the MR to the gNB to confirm the successful reception of the LP-WUS by the LR. After the handshake procedure, the gNB and the UE build up a reliable common understanding on the use of MR or LR to avoid missing transmission or reception between the gNB and the UE. If the gNB misses any of the confirmation messages, the gNB can retransmit the corresponding trigger or LP-WUS until the confirmation message is received. An illustration of this procedure is shown in Figure 1.

Figure 1 Illustration of confirmation procedure for LP-WUS monitoring.

Proposal 6: Support explicit or implicit triggering mechanisms for the LP-WUS monitoring, and potential confirmation message(s) from the UE to the gNB for handshake.

# UE behavior after receiving LP-WUS

For one aspect, RAN1 needs to determine which PDCCH to monitor after receiving a LP-WUS, and the following three options can be considered for potential down selection.

In the first option, the UE can be required to monitor one PDCCH reception occasion, and LP-WUS triggers one-shot PDCCH monitoring.

In the second option, the UE can be required to monitor PDCCH reception occasions within a time duration, wherein the time duration can be either fixed or configured.

In the third option, the UE can be required to monitor PDCCH in the activation of next one or multiple C-DRX cycle, similar to the functionality of DCI format 2_6.

Proposal 7: The PDCCH to monitor after receiving LP-WUS can be according to at least one of the following options:

- Option 1: one-shot PDCCH monitoring;

- Option 2: PDCCH monitoring within a time window;

- Option 3: PDCCH in next one or multiple active periods in C-DRX cycles.

For another aspect, similar to the triggering of LP-WUS monitoring, there can be a delay for preparing to receive the PDCCH, and a confirmation message sent from the UE to the gNB, indicating the reception of LP-WUS. An illustration of this procedure is shown in Figure 2.

Figure 2 Illustration of confirmation procedure for receiving the LP-WUS.

Proposal 8: Consider confirmation message from the UE to the gNB after LP-WUS reception.

# Conclusion

The proposals made in this contribution are summarized below:

Proposal 1: LP-WUS operation in RRC_CONNECTED mode shall take RRC_IDLE and RRC_INACTIVE modes as a baseline, and is only optimized over aspects exclusively applicable for RRC_CONNECTED mode.

Proposal 2: For RRC_CONNECTED mode, information carried by LP-WUS can include:

- Identification (UE-group ID, or UE-subgroup ID);

- Indication on whether to trigger the PDCCH monitoring.

Proposal 3: In RRC_CONNECTED mode, the configuration of LP-WUS is further provided by dedicated RRC parameter, in addition to the methods supported in RRC_IDLE and RRC_INACTIVE modes.

Proposal 4: For configuration of LP-WUS:

- Frequency location of LP-WUS shall be configurable;

- BW of LP-WUS is fixed in a number of PRBs or a number of MHz;

- TDM with other signal(s) and channel(s), and FDM with other signal(s) and channel(s) in the same numerology shall be supported;

- Sub-grouping for LP-WUS can be supported;

- DRX operation for monitoring LP-WUS is supported.

Proposal 5: Alignment between LP-WUS DRX and C-DRX can be supported.

- The cycle for LP-WUS DRX is an integer multiple of the C-DRX cycle.

Proposal 6: Support explicit or implicit triggering mechanisms for the LP-WUS monitoring, and potential confirmation message(s) from the UE to the gNB for handshake.

Proposal 7: The PDCCH to monitor after receiving LP-WUS can be according to at least one of the following options:

- Option 1: one-shot PDCCH monitoring;

- Option 2: PDCCH monitoring within a time window;

- Option 3: PDCCH in next one or multiple active periods in C-DRX cycles.

Proposal 8: Consider confirmation message from the UE to the gNB after LP-WUS reception.

# Appendix

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR).

[2] R1-2400744, LP-WUS operation in IDLE/INACTIVE modes, Samsung.

[3] R1-2400743, LP-WUS and LP-SS design, Samsung.