3GPP TSG RAN WG1 #116        		                           R1-2400748

Athens, Greece, February 26th – March 1st, 2024

Agenda item:		9.10.1

Source:			Samsung

Title:			Discussion on enabling TX/RX for XR during RRM measurements

Document for:		Discussion and decision

- Introduction

In RANP#102, the following Work Item Description (WID) was approved [1]. The motivation behind this is that the gNB can schedule urgent XR traffic without any restrictions of the measurement gap, thereby potentially improving QoS while also minimizing an impact on mobility performance in some scenarios.

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

This contribution considers methods for enabling PDSCH/PUSCH scheduling within the measurement gap.

- Discussion

RRM measurements are associated with scheduling restrictions due to measurement gaps (MGs) for intra-frequency RRM measurements in FR2 or for inter-frequency RRM measurements. The scheduling restrictions occur during the SMTC windows that the UE is configured by RRC as described in TS 38.133. For example, when a UE performs RSRP measurements based on SSBs, the UE does not receive other signaling and does not transmit. Based on the SMTC indication in SIB2, a measurement gap length (MGL) and measurement gap repetition period (MGRP) can be obtained and then the UE has all information to perform measurements. For example, for MGRP of 40 msec and MGL of 6 msec, it is not possible to schedule the UE for 6 msec every 40 msec.

In Rel-16 and Rel-17, RAN2 realized that the UE-common SMTC indication in SIB2 is not appropriate for all UEs in a cell and introduced associated enhancements, including RRC reconfiguration to change MG settings or enable/disable MG. For example, relaxedMeasurement was introduced in Rel-16 and in Rel-17 to address UEs in low mobility such as ones for XR services. For example, s-MeasureConfig was introduced in Rel-16 which is a “threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS”. Then, based on the UE’s RSRP measurement reports, the gNB knows whether or not the UE is required to perform measurements. The UE behavior is undefined in case the UE is not required to perform measurements. For example, a complete UE behavior can be defined, possibly by RRC signaling by the gNB allowing/forbidding the UE to perform measurements when the SpCell RSRP is above the threshold. However, as RAN2 and RAN4 are also responsible groups, RAN1 need not consider any measurement-related enhancements introduced by RAN2 or RAN4.

Observation 1: Unless RAN1 is requested by RAN2/RAN4 to provide support for enhancements to measurement-related features introduced by RAN2/RAN4, RAN1 need not discuss such enhancements.

Discussions in RAN1 can therefore focus on potentials signaling mechanisms for enabling/disabling MGs indicated by higher layers.

How to Enable TX/RX in MG

Considering the SINR/RSRP requirements for supporting XR applications, and the associated low mobility scenarios, it is not even clear whether there is any need/benefit for dynamic indication on top of using s-MeasureConfig and for defining the UE behavior to skip MGs when the RSRP is above the threshold. Therefore, specifications should target support of simple enhancements without over-optimizations. Moreover, as the MGs are relatively sparse and as signaling overhead for a gNB to indicate to a UE to skip an MG is marginal, signaling can be kept simple and indicate only a next MG instead of a series of MGs (e.g. via a bit-map or a counter) as it is also impossible in practice for the gNB to have definitive information for time spans of 100s of msecs in the future.

When a UE is configured with MGs, the UE periodically measures the signal quality of neighboring cells to efficiently enable handover decisions. This process facilitates the UE to detect and switch to the best possible cell, improving overall mobility performance and reducing dropped calls or connection issues. However, use of MGs leads to increased overhead and reduced throughput due to the interruption of data transmission/reception, which can be detrimental for traffic with relatively strict PDB, such as XR traffic. To mitigate such effects, the RRC based configuration is considered to be dynamically enabled/disabled.

The usual of options of DCI and MAC CE can be considered for enabling/disabling the RRC configured MGs to a UE. DCI, can be UE-specific or UE-group common. For a UE-specific DCI, a new DCI field could be introduced. Although the field would be rarely used, typically even less often than A-CSI/SRS triggering, DCI overhead is not an issue for XR applications and UE-specific DCI could be used at any time without even scheduling actual data and still not have any impact on overall spectral efficiency. For example, it would be simple to add a  one-bit field in DCI formats (other than DCI formats 0_0/1_0)  to indicate skipping measurement during the next MGL. For UE-group common DCI, new UE procedures would be required, there is no HARQ-ACK report from the UE, monitoring may not be as frequent as for UE-specific DCI or impact UE power savings, and there won’t be many UEs with simultaneously active XR traffic on a cell. Therefore, UE-group common DCI may not be further considered as there is no apparent benefic over UE-specific DCI while there are disadvantages and a larger specification impact.

MAC CE is another alternative to enable or disable scheduling in an MG. Similar to SPS/CG or SCell activation/deactivation, MAC CE can activate (or deactivate) transmission/reception in the MG. Specifically, if a UE receives a MAC CE activation command for scheduling in the MG, the UE would skip the MG in order for the UE to be scheduled transmissions/receptions within the MG. Conversely, if the UE receives a MAC CE deactivation command indicating to the UE to follow the MG and perform measurements, the UE would follow legacy behavior instead. This method does not require new DCI fields or formats, and the existing MAC CE processing timeline can be used (UE can apply the activation/deactivation command starting from the first slot after slot k + 3ms, where k is the slot in which the UE transmits a PUCCH with HARQ-ACK information related to the PDSCH carrying the MAC CE command). However, there is an additional delay compared to DCI and, as previously discussed, a one-bit indication is sufficient and the corresponding additional DCI overhead is not an issue. Therefore, use of MAC CE may not be considered to support dynamic indication for skipping MGs.

Proposal 1: Consider adding one bit in UE-specific DCI formats (other than DCI format 0_0/1_0) to indicate whether or not a UE skips a next MG and continues receptions/transmissions.

Whether to consider UE-initiated MG skipping

XR traffic for audio/video is periodic and, as network delays are not involved at the UE, jitter is smaller for transmissions by the UE than for receptions by the UE. The UE also reports RSRP measurements and the gNB can know whether the threshold indicated to the UE by s-MeasureConfig by is exceeded or not at the UE. Moreover, leaving to the UE the decision to skip an MG may have adverse effects to the NW operation, as the UE is not aware of the NW layout, traffic ongestion, or possibility for handover, and it is unclear how such functionality can be tested and not lead to unpredictable UE behavior. Also, unlike an indication by the gNB, for which HARQ-ACK feedback is possible and a common gNB/UE understanding can be established, that is not the case for the UE in case the indication is provided by PUCCH or PRACH transmission (which will also complicate gNB implementation) while if the indication is provided by PUSCH, the UE can provide BSR (if not already provided) and the gNB can then decide to indicate whether or not a UE can skip an upcoming MG.

It is again noted that there are already available mechanisms for a UE to relax/skip RRM measurements and that for the mobility scenarios and SINRs required for XR services, even defining the UE behavior to skip MGs when s-MeasureConfig is likely to be sufficient to achieve practically all benefits from skipping MGs in favor of scheduling since the s-MeasureConfig threshold is typically lower than the RSRP required to meet XR latency/reliability requirements. It is actually more likely that XR traffic may arrive at the UE when the UE is not in C-DRX active period (if C-DRX is configured for UE-power savings, although there is some flexibility for the gNB to align C-DRX and MGs) or even when the UE performs measurements during an MG. Therefore, over-optimizations with material impact to the specifications and the gNB implementation, unclear testability, detrimental impact on network operation, and unclear or absent benefits should not be considered.

Proposal 2: Do not consider UE-initiated indication for MG skipping.

Conclusion

This contribution considered how to enable/disable a measurement gap (enable/disable UE transmissions/receptions during an MG) and proposes the following.

Proposal 1: Consider adding one bit in UE-specific DCI formats (other than DCI format 0_0/1_0) to indicate whether or not a UE skips a next MG and continues receptions/transmissions.

Proposal 2: Do not consider UE-initiated indication for MG skipping.

References

[1] RP-234080, New WID: XR (eXtended Reality) for NR Phase 3