3GPP TSG RAN WG1 #116        		                           R1-2400749

Athens, Greece, February 26th – March 1st, 2024

Agenda item:		9.11.1

Source:			Samsung

Title:			Discussion on downlink coverage enhancement for NR-NTN

Document for:		Discussion and decision

- Introduction

At the previous RAN#102, the following Work Item Description (WID) was approved [1]. The motivation behind this is that it needs to study downlink coverage enhancements by considering payload power limitation.

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Notes for this objective: - SSB channel enhancement is not considered - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

This contribution focuses on discussing on evaluation methodologies for the study.

- Discussion

Evaluation methodologies

In Release 18 NTN, there was a similar study phase on evaluating coverage performance for both downlink and uplink. We believe that most of the agreements related to evaluation made in Release 18 NTN can be reused for Release 19 downlink coverage enhancement, for instance, the following agreements and conclusions can be considered as a baseline.

Agreement Coverage performance in NR NTN is evaluated according to the following steps. - Step 1: CNR is calculated as defined in 6.1.3.1 of TR38.821 - For polarization loss, - 3 dB polarization loss is assumed as baseline, and companies are encouraged to report the value and corresponding justification if other value is used - Step 2: Required SNR of target service is evaluated by LLS - Step 3: The CNR and the required SNR are compared Agreement For coverage evaluation of PDSCH in NR NTN, the following table is assumed. Parameter; Value. BLER; For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER.. Waveform; CP-OFDM. Number of UE receive chains; 2 for 2GHz. HARQ configuration; Whether/How HARQ is adopted is reported by companies.. DMRS configuration; 3 DMRS symbols is used for PDSCH of Msg.2. For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies.. PRBs/TBS/MCS for low data rate service; Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead.. PRBs/MCS for VoIP; Any value of PRBs reported by companies will be considered in the discussion. QPSK. PDSCH duration; 12 OS. Payload size for PDSCH of Msg.4; 1040 bits. Other parameters; Reported by companies.. Other parameters; Reported by companies. Agreement For coverage evaluation of PDCCH in NR NTN, the following table is assumed. Parameter; Value. Number of UE receive chains; 2 for 2GHz. Aggregation level; 16. Payload; 40 bits. CORESET size; 2 symbols, 48 PRBs. Tx Diversity; Reported by companies. BLER; 1% BLER optional for 10% BLER. Number of SSB for broadcast PDCCH of Msg.2; Reported by companies. Other parameters; Reported by companies. Conclusion For Rel-18 coverage enhancement in NTN, NLOS environment is deprioritized.

Proposal 1: Consider to reuse some evaluation procedures/assumptions made in Rel-18 NTN study phase for Rel-19 NTN downlink coverage evaluation.

- Comparing CNR and the required SNR

- Prioritize to consider LOS

- PDCCH/PDSCH link level assumptions

Regarding the additional payload assumptions, Table 6.1.1.1-1 in [2] should be considered as a baseline, excluding satellite EIRP density. For instance, 34 dBW/MHz has been assumed for satellite EIRP density in the case of LEO-600 S-band for Set-1 satellite parameters. When determining new potential values for satellite EIRP density that are less than 34 dBW/MHz, it should not affect SSB coverage performance since "SSB channel enhancement" is not within the scope of this WID.

Proposal 2: Consider SSB coverage performance impact when deciding new satellite EIRP density value

For handheld devices, Release 18 NTN has considered FR1-NTN with different RX antenna gains (i.e.,-5 dBi) compared to the value (i.e., 0 dBi) specified in Table 6.1.1.1-3 in [2]. For Release 19 NTN downlink coverage enhancement, -5.5 dBi is considered for RX antenna gain according to the WID. Although the WID considers both FR1-NTN and FR2-NTN, coverage performance evaluations for handheld devices should be based on FR1-NTN, whereas FR2-NTN should only consider VSAT, which is defined in Table 6.1.1.1-3 in [2], without altering RX antenna gain. Note that FR2-NTN includes Ka band for downlink and Ku/K bands for uplink, while Table 6.1.1.1-3 in [2] has considered Ka band for VSAT. However, it is understood that the parameters defined in Table 6.1.1.1-3 in [2] can be reused for Ku/K bands.

Proposal 3: Consider Table 6.1.1.1-3 in TR 38.821 as baseline for UE characteristics

Proposal 4: Handheld device is only considered for FR1-NTN assuming -5.5 dBi for RX antenna gain, while VSAT is only considered for FR2-NTN without modifying RX antenna gain respect to TR 38.821

Conclusion

This contribution discussed on evaluation assumptions for downlink coverage performance evaluation. Followings are proposals in this contribution.

Proposal 1: Consider to reuse some evaluation procedures/assumptions made in Rel-18 NTN study phase for Rel-19 NTN downlink coverage evaluation.

- Comparing CNR and the required SNR

- Prioritize to consider LOS

- PDCCH/PDSCH link level assumptions

Proposal 2: Consider SSB coverage performance impact when deciding new satellite EIRP density value

Proposal 3: Consider Table 6.1.1.1-3 in TR 38.821 as baseline for UE characteristics

Proposal 4: Handheld device is only considered for FR1-NTN assuming -5.5 dBi for RX antenna gain, while VSAT is only considered for FR2-NTN without modifying RX antenna gain respect to TR 38.821

References

[1] RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3

[2] TR 38.821, Solutions for NR to support non-terrestrial networks (NTN)