3GPP TSG RAN WG1 #116		R1-2400829

Athens, Greece, February 26th – March 1st, 2024

Source:	Panasonic

Title: 	Maintenance on Further NR Coverage Enhancements

Agenda Item:		8.6

Document for:	Discussion

# Introduction

Two main objectives of the Rel.18 WID of further NR coverage enhancements are shown as follows [1]

- Specify following PRACH coverage enhancements (RAN1, RAN2)

  - Multiple PRACH transmissions with same beams for 4-step RACH procedure

  - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure.

  - Note 1: The enhancements of PRACH are targeting for FR2 and can also apply to FR1 when applicable.

  - Note 2: The enhancements of PRACH are targeting short PRACH formats and can also apply to other formats when applicable.

- Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

RAN1 has completed fundamental designs to support multiple PRACH transmission with same beams and dynamic waveform switching (DWS) between DFT-s-OFDM and CP-OFDM in RAN1#114. This document provides our view on the remaining essential issues of multiple PRACH transmission and DWS in a maintenance phase.

# Multi-PRACH transmission

## Applicability of PRACH mask

In Rel. 17, a PRACH mask index can be provided to a UE (by ra-ssb-OccasionMaskIndex if configured, or ssb-SharedRO-MaskIndex if configured, or indicated by PDCCH) to restrict a subset of ROs that can be usable for PRACH transmission, where the subset of ROs are associated with the selected SS/PBCH block index. The UE selects/determines a RO from the subset of ROs for sending a single PRACH transmission. In the other words, it can be observed that the PRACH mask index is applied before a selection/determination of a RO for the PRACH transmission. It is applicable to feature combination and CFRA in Rel. 17. The detailed descriptions of TS 38.213 and 38.321 related to the PRACH mask index are copied and highlighted in yellow part in Appendix 1.

In RAN1#115 meeting, RAN1 discussed how to apply PRACH mask index in a multi-PRACH transmission, and there are 4 options are considered in below agreement.

Agreement For multiple PRACH transmissions with indication of PRACH mask index, down-select one of the following options at RAN1#116 - Option 1: UE applies PRACH mask prior to RO group determination. RO group is determined based on the ROs indicated by the PRACH mask index. - Option 2: UE applies PRACH mask after RO group determination. UE transmits PRACH with Npreamblerep preamble repetitions only on a RO group with all the ROs indicated by the mask. - Option 3: UE applies PRACH mask after RO group determination. UE transmits PRACH with Npreamblerep preamble repetitions only on a RO group where at least one the first RO of this RO group is indicated by the mask - Option 4: UE applies PRACH mask after RO group determination. The PRACH mask index indicates one or multiple RO groups for multiple PRACH transmission. - Note: this implies the PRACH mask index indicates the RO group index(es) instead of RO index(es).

We observe that

- Option 1 is based on current specification and has no specification impact,

- Option 2 is different from current specification. It might lead to a case that even when a set of RO groups are determined, but after applying PRACH mask index, there is no suitable RO group that can used by a UE for a multi-PRACH transmission,

- Option 3 is also different from current specification and has no clear technical benefit,

- Option 4 is a totally new solution, where the PRACH mask index is proposed to indicate one or multiple RO groups for a multi PRACH transmission. To do so, it is required to determine RO group index(es) so that the PRACH mask index indicates which RO group index(es) are useable for the multi PRACH transmission. It is foreseen to have heavy RAN1 discussion effort and specification impact.

With the above observations, we propose to take Option 1 as it is workable and simple solution for the maintenance phase without any optimization.

Proposal 1: Support to take Option 1 for applicability of PRACH mask index for multi-PRACH transmission

- UE applies PRACH mask prior to RO group determination. RO group is determined based on the ROs indicated by the PRACH mask index.

## Power ramping counter

For notifying a power ramping counter to higher layers in a single PRACH transmission, 2 UE’s behaviours are specified in current specs as follows

- If UE does not transmit a PRACH in a RO due to reason(s) described in Clause 7.4 in TS 38.213 (shown in below box), Layer 1 notifies higher layers to suspend the corresponding power ramping counter.

- If UE transmits a PRACH with reduced power in a RO due to reason(s) described in Clause 7.4 in TS 38.213 (shown in below box), Layer 1 may notify higher layers to suspend the corresponding power ramping counter.

7.4 Physical random access channel <omitted> If due to power allocation to PUSCH/PUCCH/PRACH/SRS transmissions as described in clause 7.5, or due to power allocation in EN-DC or NE-DC or NR-DC operation, or due to slot format determination as described in clause 11.1, or due to the PUSCH/PUCCH/PRACH/SRS transmission occasions are in the same slot or the gap between a PRACH transmission and PUSCH/PUCCH/SRS transmission is small as described in clause 8.1, or due to HD-UE operation in paired spectrum as described in clause 17.2, the UE does not transmit a PRACH in a transmission occasion, Layer 1 notifies higher layers to suspend the corresponding power ramping counter. If due to power allocation to PUSCH/PUCCH/PRACH/SRS transmissions as described in clause 7.5, or due to power allocation in EN-DC or NEDC or NR-DC operation, the UE transmits a PRACH with reduced power in a transmission occasion, Layer 1 may notify higher layers to suspend the corresponding power ramping counter <omitted>

In RAN1#115 meeting, RAN1 discussed the following options for multiple PRACH transmission in below agreement.

Agreement For multiple PRACH transmissions, down-select one of the following options at RAN1#116: Option 1: - Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are dropped or with reduced transmit power. - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in any of PRACH occasions are dropped or with reduced transmit power. Option 1a: - Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are dropped. - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission on part of PRACH occasions are dropped or when PRACH transmission in any of PRACH occasions is with reduced transmit power. Option 2: - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in at least one PRACH occasion is dropped or with reduced transmit power. Note: this implies it’s up to UE implementation. Option 2a: - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in at least one PRACH occasion is with reduced transmit power. - Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in at least one PRACH occasion is dropped. Option 3: - Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are dropped. - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are with reduced transmit power. Note: whether any of the above options have specification impact is a separate discussion.

Our observations are following.

- We don't see so much difference between "some of PRACH transmission are dropped" and "all or some of PRACH transmission power are reduced" as the importance is, at the end, how much energy can be accumulated at gNB PRACH receiver over the PRACH repetitions. For example, "all PRACH occasions are transmitted with a half of full transmit power" and "only a half of PRACH occasions are transmitted with full transmit power" can be basically equivalent (in spite that the exact performance can be different because of channel estimation performance and time diversity gain and so on). To specify exact condition of the trade-off between the number of transmission occasions and the reduced power would be too complex and not have specific benefit. Therefore, these trade off can be up to UE implementation as the description of "layer 1 may notify higher layer 1".

- On the other hand, at least when all of PRACH occasions are dropped, it would be always reasonable to suspend the corresponding power ramping counter. Therefore, the corresponding description would be "layer 1 notifies higher layers".

With above considerations, option 1 is not reasonable as all of PRACH occasions with the reduced transmission power should be up to UE implementation. Option 2 is not reasonable as all of PRACH occasions are dropped should not be up to UE implementation. Option 2a is not reasonable as one of PRACH occasion is dropped should be up to UE implementation. Option 3 is not reasonable some of PRACH occasion is dropped is not described. Hence, we propose to take Option 1a as follows

Proposal 2: For multi-PRACH transmission, support to take Option 1a as follows

- Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are dropped.

- Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission on part of PRACH occasions are dropped or when PRACH transmission in any of PRACH occasions is with reduced transmit power.

## Transmit power and retransmission

In RAN1#114, it has been concluded to reuse the two transmission power determination equations of Rel-17 NR PRACH to calculate the transmission power of each PRACH transmission in a multi-PRACH transmission in the following.

Conclusion For multiple PRACH transmissions with the same Tx beam, the two transmission power determination equations (just for reference: equation (1) and (2) as shown in the reference) of Rel-17 NR PRACH are reused for calculating the transmission power of each PRACH transmission, i.e., PREAMBLE_RECEIVED_TARGET_POWER = preambleInitialReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep. Note: The following is for reference. For reference: The power control formula of NR PRACH consists of the following two steps: Step 1: Calculate the receive target power of one single transmission. PREAMBLE_RECEIVED_TARGET_POWER=preambleInitialReceivedTargetPower+DELTA_PREAMBLE + (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep (1) Step 2: Calculate the transmission power of single transmission. P_PRACH = min{P_CMAX(i), PREAMBLE_RECEIVED_TARGET_POWER + PL_c} [dBm] (2).

However, it is still not concluded which value(s) is/are to be used in these two equations. In order to do so, it is necessary to conclude whether to apply a multi-PRACH transmission before or after UE’s transmit power reaches a maximum transmit power. The relevant discussion has been started since RAN1#112-bis-e, as shown in the following cases with options and alternatives [2].

- Case 1: Single PRACH transmission is determined for the first RACH attempt based on SSB-RSRP threshold(s), power ramping is applied between RACH attempts. - Option 1: The number of PRACH transmission in RACH re-attempts is not increased, regardless of whether the maximum transmission power is reached or not. - Option 2: The number of multiple PRACH transmissions in RACH re-attempts can be increased based on some condition. - FFS: details. E.g., when the maximum transmission power is reached. - Case 2: Multiple PRACH transmissions are determined for the first RACH attempt based on the SSB-RSRP thresholds. - Option 1: The maximum transmission power is not compulsorily applied for the first RACH attempt. - Alt.1: Power ramping is applied between RACH attempts, the number of multiple PRACH transmissions in RACH re-attempts is the same as that of first RACH attempt. - Alt.2: Power ramping is applied between RACH attempts, the number of multiple PRACH transmissions in RACH re-attempts can be increased based on some condition. - FFS: details. E.g., when the maximum transmission power is reached or the maximum number of attempts for current number of PRACH repetitions is reached. - Option 2: The maximum transmission power is compulsorily applied for the first RACH attempt. - Alt.1: The number of multiple PRACH transmissions in RACH re-attempts is the same as that of first RACH attempt. Power ramping is not needed. - Alt.2: The number of multiple PRACH transmissions in RACH re-attempts can be increased based on some condition. Power ramping is not needed. - FFS: details, e.g., a smaller power headroom based on an increased power ramping counter, or tolerance zone around the SSB-RSRP threshold(s) is defined to determine whether to increase the number of PRACH transmissions.

We think that a multi-PRACH transmission is to be used after UE’s transmit power reaches a maximum transmit power. In this case, power ramping is ONLY applied for single PRACH transmission case, and not applied during the multi-PRACH transmission and between PRACH attempts. Possible UE behaviours could be shown in the following

- UE attempts a single PRACH transmission until UE’s transmit power reaches a maximum transmit power. If it does not receive any response from gNB, it can attempt 2 PRACH transmissions => 4 PRACHs transmissions => 8 PRACH repetitions subsequentially.

- Depending on SSB-RSRP threshold(s), a UE is not required to start from a single PRACH transmission, it can jump to a specific number of PRACH transmissions in a multi-PRACH transmission.

With such understanding, we support Alt. 2 in Option 2 in Case 2.

Alternatively, for a compromised approach, the following FL’s proposal 4-1 has been discussed in RAN1#114 meeting [3]. We also support this proposal.

##### FL’s proposal 4-1 For the first RACH attempt, the UE determines - Whether to perform single PRACH transmission or multiple PRACH transmissions based on SSB-RSRP threshold. - If the SSB-RSRP threshold to determine single PRACH transmission or multiple PRACH transmissions with the smallest configured value of the number of multiple PRACH transmissions is not provided, whether to perform single PRACH transmission or multiple PRACH transmissions is based on whether UE’s calculated transmission power for single PRACH transmission reaches its maximum transmission power. - If the SB-RSRP threshold to determine single PRACH transmission or multiple PRACH transmissions with the smallest configured value of the number of multiple PRACH transmissions is provided, whether to perform single PRACH transmission or multiple PRACH transmissions is based on SSB-RSRP threshold. - If multiple PRACH transmissions are determined, the number of multiple PRACH transmissions is based on the configured SSB-RSRP threshold(s). - Note1: If multiple PRACH transmissions are determined, the power calculation of each PRACH transmission of the multiple PRACH transmissions is a separate discussion.

Proposal 3:  For the first RACH attempt, a UE determines

- Whether to perform single PRACH transmission or multiple PRACH transmissions based on SSB-RSRP threshold.

- If the SSB-RSRP threshold to determine single PRACH transmission or multiple PRACH transmissions with the smallest configured value of the number of multiple PRACH transmissions is not provided, whether to perform single PRACH transmission or multiple PRACH transmissions is based on whether UE’s calculated transmission power for single PRACH transmission reaches its maximum transmission power.

- If the SB-RSRP threshold to determine single PRACH transmission or multiple PRACH transmissions with the smallest configured value of the number of multiple PRACH transmissions is provided, whether to perform single PRACH transmission or multiple PRACH transmissions is based on SSB-RSRP threshold.

- If multiple PRACH transmissions are determined, the number of multiple PRACH transmissions is based on the configured SSB-RSRP threshold(s).

- Note1: If multiple PRACH transmissions are determined, the power calculation of each PRACH transmission of the multiple PRACH transmissions is a separate discussion.

In addition, regarding pathloss aspect, FL has proposed the following proposal 5-3-1 for further discussion.

###### FL’s proposal 5-3-1 For transmission power calculation of multiple PRACH transmissions with the same Tx beam, down select one of the following options: - Option 1: the same pathloss is applied for all the PRACH transmissions within one RACH attempt. - Option 2: the pathloss for each PRACH transmission of the multiple PRACH transmissions within one RACH attempt is separately estimated.

We think that if the multi-PRACH transmission can be finished within relatively short time by configuring sufficient ROs in a frame, the same pathloss measurement is sufficient and better for a combined detection of the multi-PRACH transmission. If the multi-PRACH transmission takes long time by configuring a sparse density of ROs in a frame, not using the latest pathloss measurement can result too high or too low PRACH transmission power. Therefore, gNB can configure how pathloss measurement is taken would be reasonable. Hence, we propose to replace FL’s proposal 5-3-1 as the following.

Proposal 4: For multi-PRACH transmission with Tx same beam in one RACH attempt, based on the density of ROs and so on, gNB can configure either to compensate path loss or not compensate the path loss.

## Determination of number of PRACH transmissions

Regarding candidate values for the number of PRACH transmissions, it has been agreed a set of {2, 4, 8} to compensate the performance shortage of PRACH coverage, and gNB can configure one or multiple values for the number of multiple PRACH transmissions. If multiple values are configured, PRACH resources differentiation between multiple PRACH transmissions with different number of PRACH transmissions is also supported. In this case, a UE needs to determine the number of PRACH transmissions based on SSB-RSRP measurement and SSB-RSRP threshold(s) comparison. Moreover, CFRA with multiple PRACH transmissions is supported for a case of Reconfiguration with sync based on below RAN2 agreements. There is ongoing discussion in RAN2 whether to support CFRA with multiple PRACH transmissions in condition handover, however, this case is not concluded yet. For these cases, a number of PRACH transmissions can be indicated in a DCI.

Agreements (RAN2 #122) RAN2 intends to support CFRA for msg1 repetition for ReconfigurationWithSync case, FFS for other cases. Agreements (RAN2 #123) CFRA with Msg1 repetition for BFR and with PDCCH order are not supported (can be revisited if there is consensus to support this)

Regarding how a UE determines a RO group from the configured RO list for Reconfiguration with sync in CFRA, we think that each RO group (from the configured RO list) can be associated with a number of PRACH transmissions, hence a UE can determine a RO group when it is indicated a number of PRACH transmissions by the DCI. Therefore, we propose the following.

Proposal 5: Support a determination of a number of PRACH transmissions as follows

- For RA without PDCCH order, a UE determines a number of PRACH transmissions based on a comparison between SSB-RSRP measurement and SSB-RSRP threshold.

- For RA with PDCCH order, PDCCH can indicate either of the following.

  - If a number of PRACH transmission is indicated, UE follows the indicated number of PRACH transmissions.

    - UE determines a RO group based on its association with the indicated number of PRACH transmissions.

  - If a number of PRACH transmission is not indicated, the same operation with RA without PDCCH order.

## Interaction between multi-PRACH transmission and Msg3 repetition

In RAN1#111 meeting, it has been agreed that for multiple PRACH transmissions with same Tx beam, at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt. If a multi-PRACH transmission is triggered, the coverage of uplink is poor. It implies that the coverage of Msg3 repetition can also be poor (e.g., Case 3 in Fig. 1) because Msg3 PUSCH is a more coverage bottleneck channel than PRACH as shown in study item phase in Rel. 17. Hence, it is not likely that that a multi-PRACH transmission is required, but Msg3 PUSCH repetition is not required. Furthermore, in Rel. 17, in order to request Msg3 repetition, the separate PRACH resources are used. Having said that multi-PRACH transmission and Msg3 repetition have a strong relationship. The PRACH resource for PRACH repetition and Msg3 repetition should be same when both multi-PRACH and Msg3 repetition are triggered together. Otherwise, the UE is required to transmit two PRACH preambles for a multi-PRACH transmission and a requesting Msg3 repetition, respectively. To avoid this situation, it is required to specify interaction between multi-PRACH transmission and Msg3 repetition, so that the UE can determine a PRACH preamble to be used. This is also required for the provision of PRACH preamble for other purposes (e.g., CFRA, 2-step RACH, 4-step RACH, feature combination etc.). Hence, we propose that

Proposal 6: When a UE requests both multi-PRACH transmission and Msg3 repetition by PRACH resource, the PRACH resource for requesting both multi-PRACH transmission and Msg3 repetition should be supported. The interaction between multi-PRACH transmission and Msg3 repetition needs to be defined.

Moreover, in Fig. 1, as mentioned earlier, the case of having multi-PRACH transmission and not having Msg3 repetition would not be required to be supported. On the other hand, the case of not having multi-PRACH transmission and having Msg3 repetition needs to be supported (e.g., Case 2). A UE with support of Msg 3 repetition only should select either Case 1 or Case 2 only, but it should not select Case 3.

Fig. 1. An example of interaction between multi-PRACH transmission and Msg3 repetition.

Since Rel. 17 specification, when UE requests Msg3 repetition, MCS information field is interpreted based on the following manner.

- 2 MSB bits of the MCS information field are used for selecting one repetition number from a SIB1 configured set with 4 candidate values.

- 2 LSB bits of the MCS information field are used for selecting one MCS index from a SIB1 configured set with 4 candidate MCS indices.

In Case 3, when the multi-PRACH transmission is triggered, it is necessary to support lower coding rate (i.e., more repetitions and/or lower MCS index) of Msg3 PUSCH to improve achievable Msg3 PUSCH coverage performance. In particular, when the number of PRACH transmissions is determined by UE, the proper number of transmissions and/or MCS for Msg3 PUSCH can be different depending on the number of PRACH transmissions. Four repetition factors and four MCS indices are specified/designed without the operation/consideration of the multi-PRACH transmission. For the channel condition, where both the multi-PRACH transmission and Msg3 repetition are required, these configured sets for Msg 3 repetition would not be sufficient. Therefore, to adjust the number Msg3 repetitions and/or MCS depending on PRACH repetition would be necessary. It can be realized by applying a scaling factor to Rel. 17 configured set depending on the multi-PRACH transmission.

Proposal 7: When the multi-PRACH transmission is triggered by UE, the mechanism to enable more repetitions and/or lower MCS index than the Rel.17 configured set for Msg3 repetition should be supported.

# Dynamic waveform switching

## PUSCH scheduled by DCI format 0-1 / 0-2 in PDCCH with CRC scrambled with SP-CSI-RNTI

In RAN1#115, the following was proposed by FL based on the discussion in FL summary [3].

FL proposal:

  - Dynamic waveform switching enhancement is applicable to PUSCH scheduled by DCI format 0-1 or 0-2 in PDCCH with CRC scrambled with SP-CSI-RNTI.

In our understanding, the above proposal intends that the result of dynamic waveform switching is applied not only to the first PUSCH with SP-CSI but also to PUSCH with SP-CSI, but we think it should be clarified. PUSCH with SP-CSI scheduled by DCI format 0-1 or 0-2 in PDCCH with CRC scrambled with SP-CSI-RNTI can be viewed as a similar mechanism to Type 2 CG-PUSCH transmission since it is also activated by DCI. Since there is no consensus to support dynamic waveform switching for Type 2 CG-PUSCH, one of approaches is no need to extend to PDCCH with CRC scrambled with SP-CSI-RNTI. However, in the case of CG-PUSCH, every Type 2 CG-PUSCH configuration could independently choose the waveform. On the other hand, for PUSCH associated with SP-CSI-RNTI, the waveform is governed by the transform precoder parameter listed under PUSCH-Config. This parameter is shared with PUSCH associated with C-RNTI and also shared among multiple SP-CSI configurations. Therefore, to apply dynamic waveform switching to PUSCH scheduled by DCI format 0-1 / 0-2 in PDCCH with CRC scrambled with SP-CSI-RNTI is reasonable.

Proposal 7:

- Dynamic waveform switching enhancement is applicable to PUSCH scheduled by DCI format 0-1 or 0-2 in PDCCH with CRC scrambled with SP-CSI-RNTI.

  - The result of dynamic waveform switching is applied not only to the first PUSCH with SP-CSI but also to PUSCH with SP-CSI.

Proposal 8: Adopt the following TPs to TS 38.212 and TS 38.214.

- For TS 38.212

Reason for change: Introduce dynamic waveform switching for PUSCH scheduled by DCI format 0-1/0-2 in PDCCH with CRC scrambled with SP-CSI-RNTI in TS38.212. Summary of changes: Application of transform precoder indicator to DCI format 0-1/0-2 in PDCCH with CRC scrambled with SP-CSI-RNTI. Consequences if not approved: Specification does not support dynamic waveform switching for PUSCH scheduled by DCI format 0-1/0-2 in PDCCH with CRC scrambled with SP-CSI-RNTI. ##### 7.3.1.1.2 Format 0_1 < No change part is omitted > - Transform precoder indicator - 0 or 1 bit - 1 bit if the higher layer parameter dynamicTransformPrecoderIndicationDCI-0-1 is configured to 'enabled ' and if the UE is configured to monitor DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI or SP-CSI-RNTI, where the bit value of 0 indicates that transform precoder is enabled and the bit value of 1 indicates that transform precoder is disabled. For a DCI format 0_1 with CRC scrambled by CS-RNTI and the value indicated by new data indicator field is 0, or for a DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, the bit is reserved. - 0 bit otherwise. < No change part is omitted > ##### 7.3.1.1.3 Format 0_2 < No change part is omitted > - Transform precoder indicator - 0 or 1 bit - 1 bit if the higher layer parameter dynamicTransformPrecoderIndicationDCI-0-2 is configured to 'enabled ' and if the UE is configured to monitor DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI or SP-CSI-RNTI, where the bit value of 0 indicates that transform precoder is enabled and the bit value of 1 indicates that transform precoder is disabled. For a DCI format 0_2 with CRC scrambled by CS-RNTI and the value indicated by new data indicator field is 0, or for a DCI format 0_2 with CRC scrambled by SP-CSI-RNTI, the bit is reserved. - 0 bit otherwise. < No change part is omitted >

- For TS 38.214

Reason for change: Introduce dynamic waveform switching for PUSCH scheduled by DCI format 0-1/0-2 in PDCCH with CRC scrambled with SP-CSI-RNTI in TS38.214. Summary of changes: Application of transform precoder indicator to DCI format 0-1/0-2 in PDCCH with CRC scrambled with SP-CSI-RNTI. Consequences if not approved: Specification does not support dynamic waveform switching for PUSCH scheduled by DCI format 0-1/0-2 in PDCCH with CRC scrambled with SP-CSI-RNTI. ### 6.1.3 UE procedure for applying transform precoding on PUSCH < No change part is omitted > For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI: - If the DCI with the scheduling grant was received with DCI format 0_0, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder. - If the DCI with the scheduling grant was not received with DCI format 0_0 - If the DCI with the scheduling grant was received with DCI format 0_1 or 0_2 with CRC scrambled by C-RNTI, MCS-RNTI, or CS-RNTI with NDI=1, or SP-CSI-RNTI, and if the UE is configured with a higher layer parameter [dynamicTransformPrecoderIndicationDCI-0-1] in pusch-Config for DCI format 0_1 or [dynamicTransformPrecoderIndicationDCI-0-2] in pusch-Config for DCI format 0_2 and the higher layer parameter is set to 'enabled', - the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the Transform precoder indicator field in the DCI with the scheduling grant. - For pusch-TimeDomainAllocationListForMultiPUSCH in pusch-Config, the UE shall, for all PUSCH transmissions, consider the transform precoding either enabled or disabled according to Transform precoder indicator field in the DCI format 0_1 with the scheduling grant. - If resourceAllocation in pusch-Config for DCI format 0_1 or resourceAllocationDCI-0-2 in pusch-Config for DCI format 0_2 is set to resourceAllocationType0, or if the resource allocation is set to resource allocation type 0 according to the DCI configuration as described in clauses 7.3.1.1.2 and 7.3.1.1.3 of [6, TS 38.212], or if dmrs-Type in DMRS-UplinkConfig is set to ‘type 2’ for this PUSCH transmission, the UE does not expect that the Transform precoder indicator field in the DCI with the scheduling grant indicates that transform precoding is enabled. - If the UE is configured with the higher layer parameter enhanced-dmrs-Type-r18 in DMRS-UplinkConfig, and if the scheduling grant indicates that transform precoding is enabled for the scheduled PUSCH transmission, the UE ignores the higher layer parameters enhanced-dmrs-Type-r18 in DMRS-UplinkConfig, if configured, for the DM-RS transmission of the scheduled PUSCH transmission. - Otherwise, - If the UE is configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter. - If the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder. < No change part is omitted >

## Assistance information for switching waveform

In RAN1#114, RAN1 has the following agreement.

Agreement: - For reporting of power headroom information for assumed PUSCH using target waveform different from waveform of actual PUSCH, support the following. - Power headroom information for assumed PUSCH is based on an actual PUSCH transmission. - In case of no actual PUSCH transmission on a serving cell, power headroom information for assumed PUSCH is not supported. - DWS field needs to be configured for at least one DCI format for the BWP of the actual PUSCH, otherwise power headroom information for assumed PUSCH is not supported. - If actual PUSCH transmission is with DFT-s-OFDM waveform, UE computes power headroom information of an assumed PUSCH with CP-OFDM waveform. If actual PUSCH transmission is with CP-OFDM waveform, UE computes power headroom information of an assumed PUSCH with DFT-s-OFDM waveform. - All parameters that are used for the calculation of PCMAX,f,c(i), except waveform, are the same between assumed PUSCH and actual PUSCH. - In case assumed PUSCH transmission is not supported for the parameters that are used for the calculation of PCMAX,f,c(i), power headroom information for assumed PUSCH is not computed or reported. - Power headroom information for assumed PUSCH contains - PCMAX,f,c(i) of assumed PUSCH - According for applicable MPR, A-MPR, and P-MPR for the assumed PUSCH. - If UE reports power headroom information for assumed PUSCH in a PUSCH transmission, legacy PHR is also reported in the same PUSCH transmission. - No consensus in RAN1 if the following applies or not: If UE reports legacy PHR in a PUSCH transmission, power headroom information for assumed PUSCH is also reported. - Note: RAN endorsed the following at RAN#100: “RAN2 will not work on PHR triggering procedure for dynamic waveform switching in Rel.18 UL Coverage enh WI” [RP-231498]. - Send LS to RAN2 to inform above agreement.

RAN1#115 discussed how to implement highlighted parts in the RAN1 specification. During the discussion on the potential TP, whether PCMAX,f,c(i) for assumed PUSCH can be reported in both DG-PUSCH and CG-PUSCH, or only reported in DG-PUSCH scheduled by DCI in which DWS field is configured was discussed and the following two options were identified.

- Option 1: No conclusion on PUSCH (i.e., PCMAX,f,c(i) for assumed PUSCH can be reported in both DG-PUSCH and CG-PUSCH).

- Option 2: Only PUSCH scheduled by DCI with transform precoding indication.

In our understanding, agreement in RAN1#114 does not include any condition. Condition like Option 2 was considered but decided to go with whether transform precoding indicator is configured or not in BWP instead (as blue highlighted part in the agreement in RAN1#114). In addition, the current specification on PHR processing does not use the information on whether PUSCH is CG or DG. Therefore, PHR processing without distinction of CG or DG should be kept. If distinction is required, there can some UE complexity impact. Since Option 1 can be based on the current agreement and Option 1 will have less specification / implementation impact on the legacy behavior, we prefer Option 1.

Proposal 9:

- PCMAX,f,c(i) for assumed PUSCH can be reported in both DG-PUSCH and CG-PUSCH.

Proposal 10: Adopt the following TPs to TS 38.213.

Reason for change: Introduce power headroom information for assumed PUSCH in TS 38.213. Summary of changes: Description of the provision of Pcmax for an assumed PUSCH and associated conditions. Consequences if not approved: Specification does not support reporting of power headroom information for assumed PUSCH. ### 7.7.1 Type 1 PH report < No change part is omitted > - a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State, and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State, for an actual PUSCH transmission using a spatial domain filter corresponding to the first TCI-State or TCI-UL-State and using a spatial domain filter corresponding to the second TCI-State or TCI-UL-State. If a UE provides a Type 1 power headroom report for an activated serving cell based on an actual PUSCH transmission, is provided assumedPUSCHInfo, and dynamicTransformPrecoderIndicationDCI-0-1 or dynamicTransformPrecoderIndicationDCI-0-2 is set to enabled for the active bandwidth part of the serving cell: the UE provides - PCMAX,f,c(i) based on any applicable maximum output power reduction for an assumed PUSCH with transform precoder enabled, if supported, if transform precoder is disabled for the actual PUSCH; or - PCMAX,f,c(i) based on any applicable maximum output power reduction applicable for an assumed PUSCH with transform precoder disabled, if supported, if the transform precoder is enabled for the actual PUSCH. All other parameters used for the calculation of PCMAX,f,c(i) of the assumed PUSCH are the same as the actual PUSCH; otherwise, UE does not provide a PCMAX,f,c(i) for assumed PUSCH for the serving cell as these are not supported. < End changes >

# Conclusions

In this contribution, we made the following proposals.

Multi-PRACH transmission

Proposal 1: Support to take Option 1 for applicability of PRACH mask index for multi-PRACH transmission

- UE applies PRACH mask prior to RO group determination. RO group is determined based on the ROs indicated by the PRACH mask index.

Proposal 2: For multi-PRACH transmission, support to take Option 1a as follows

- Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are dropped.

- Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission on part of PRACH occasions are dropped or when PRACH transmission in any of PRACH occasions is with reduced transmit power.

Proposal 3:  For the first RACH attempt, a UE determines

- Whether to perform single PRACH transmission or multiple PRACH transmissions based on SSB-RSRP threshold.

- If the SSB-RSRP threshold to determine single PRACH transmission or multiple PRACH transmissions with the smallest configured value of the number of multiple PRACH transmissions is not provided, whether to perform single PRACH transmission or multiple PRACH transmissions is based on whether UE’s calculated transmission power for single PRACH transmission reaches its maximum transmission power.

- If the SB-RSRP threshold to determine single PRACH transmission or multiple PRACH transmissions with the smallest configured value of the number of multiple PRACH transmissions is provided, whether to perform single PRACH transmission or multiple PRACH transmissions is based on SSB-RSRP threshold.

- If multiple PRACH transmissions are determined, the number of multiple PRACH transmissions is based on the configured SSB-RSRP threshold(s).

- Note1: If multiple PRACH transmissions are determined, the power calculation of each PRACH transmission of the multiple PRACH transmissions is a separate discussion.

Proposal 4: For multi-PRACH transmission with Tx same beam in one RACH attempt, based on the density of ROs and so on, gNB can configure either to compensate path loss or not compensate the path loss.

Proposal 5: Support a determination of a number of PRACH transmissions as follows

- For RA without PDCCH order, a UE determines a number of PRACH transmissions based on a comparison between SSB-RSRP measurement and SSB-RSRP threshold.

- For RA with PDCCH order, PDCCH can indicate either of the following.

  - If a number of PRACH transmission is indicated, UE follows the indicated number of PRACH transmissions.

    - UE determines a RO group based on its association with the indicated number of PRACH transmissions.

  - If a number of PRACH transmission is not indicated, the same operation with RA without PDCCH order.

Proposal 6: When a UE requests both multi-PRACH transmission and Msg3 repetition by PRACH resource, the PRACH resource for requesting both multi-PRACH transmission and Msg3 repetition should be supported. The interaction between multi-PRACH transmission and Msg3 repetition needs to be defined.

Proposal 7: When the multi-PRACH transmission is triggered by UE, the mechanism to enable more repetitions and/or lower MCS index than the Rel.17 configured set for Msg3 repetition should be supported.

Dynamic waveform switching

Proposal 7:

- Dynamic waveform switching enhancement is applicable to PUSCH scheduled by DCI format 0-1 or 0-2 in PDCCH with CRC scrambled with SP-CSI-RNTI.

  - The result of dynamic waveform switching is applied not only to the first PUSCH with SP-CSI but also to PUSCH with SP-CSI.

Proposal 8: Adopt the following TPs to TS 38.212 and TS 38.214.

- For TS 38.212

Reason for change: Introduce dynamic waveform switching for PUSCH scheduled by DCI format 0-1/0-2 in PDCCH with CRC scrambled with SP-CSI-RNTI in TS38.212. Summary of changes: Application of transform precoder indicator to DCI format 0-1/0-2 in PDCCH with CRC scrambled with SP-CSI-RNTI. Consequences if not approved: Specification does not support dynamic waveform switching for PUSCH scheduled by DCI format 0-1/0-2 in PDCCH with CRC scrambled with SP-CSI-RNTI. ##### 7.3.1.1.2 Format 0_1 < No change part is omitted > - Transform precoder indicator - 0 or 1 bit - 1 bit if the higher layer parameter dynamicTransformPrecoderIndicationDCI-0-1 is configured to 'enabled ' and if the UE is configured to monitor DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI or SP-CSI-RNTI, where the bit value of 0 indicates that transform precoder is enabled and the bit value of 1 indicates that transform precoder is disabled. For a DCI format 0_1 with CRC scrambled by CS-RNTI and the value indicated by new data indicator field is 0, or for a DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, the bit is reserved. - 0 bit otherwise. < No change part is omitted > ##### 7.3.1.1.3 Format 0_2 < No change part is omitted > - Transform precoder indicator - 0 or 1 bit - 1 bit if the higher layer parameter dynamicTransformPrecoderIndicationDCI-0-2 is configured to 'enabled ' and if the UE is configured to monitor DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI or SP-CSI-RNTI, where the bit value of 0 indicates that transform precoder is enabled and the bit value of 1 indicates that transform precoder is disabled. For a DCI format 0_2 with CRC scrambled by CS-RNTI and the value indicated by new data indicator field is 0, or for a DCI format 0_2 with CRC scrambled by SP-CSI-RNTI, the bit is reserved. - 0 bit otherwise. < No change part is omitted >

- For TS 38.214

Reason for change: Introduce dynamic waveform switching for PUSCH scheduled by DCI format 0-1/0-2 in PDCCH with CRC scrambled with SP-CSI-RNTI in TS38.214. Summary of changes: Application of transform precoder indicator to DCI format 0-1/0-2 in PDCCH with CRC scrambled with SP-CSI-RNTI. Consequences if not approved: Specification does not support dynamic waveform switching for PUSCH scheduled by DCI format 0-1/0-2 in PDCCH with CRC scrambled with SP-CSI-RNTI. ### 6.1.3 UE procedure for applying transform precoding on PUSCH < No change part is omitted > For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI: - If the DCI with the scheduling grant was received with DCI format 0_0, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder. - If the DCI with the scheduling grant was not received with DCI format 0_0 - If the DCI with the scheduling grant was received with DCI format 0_1 or 0_2 with CRC scrambled by C-RNTI, MCS-RNTI, or CS-RNTI with NDI=1, or SP-CSI-RNTI, and if the UE is configured with a higher layer parameter [dynamicTransformPrecoderIndicationDCI-0-1] in pusch-Config for DCI format 0_1 or [dynamicTransformPrecoderIndicationDCI-0-2] in pusch-Config for DCI format 0_2 and the higher layer parameter is set to 'enabled', - the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the Transform precoder indicator field in the DCI with the scheduling grant. - For pusch-TimeDomainAllocationListForMultiPUSCH in pusch-Config, the UE shall, for all PUSCH transmissions, consider the transform precoding either enabled or disabled according to Transform precoder indicator field in the DCI format 0_1 with the scheduling grant. - If resourceAllocation in pusch-Config for DCI format 0_1 or resourceAllocationDCI-0-2 in pusch-Config for DCI format 0_2 is set to resourceAllocationType0, or if the resource allocation is set to resource allocation type 0 according to the DCI configuration as described in clauses 7.3.1.1.2 and 7.3.1.1.3 of [6, TS 38.212], or if dmrs-Type in DMRS-UplinkConfig is set to ‘type 2’ for this PUSCH transmission, the UE does not expect that the Transform precoder indicator field in the DCI with the scheduling grant indicates that transform precoding is enabled. - If the UE is configured with the higher layer parameter enhanced-dmrs-Type-r18 in DMRS-UplinkConfig, and if the scheduling grant indicates that transform precoding is enabled for the scheduled PUSCH transmission, the UE ignores the higher layer parameters enhanced-dmrs-Type-r18 in DMRS-UplinkConfig, if configured, for the DM-RS transmission of the scheduled PUSCH transmission. - Otherwise, - If the UE is configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter. - If the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder. < No change part is omitted >

Proposal 9:

- PCMAX,f,c(i) for assumed PUSCH can be reported in both DG-PUSCH and CG-PUSCH.

Proposal 10: Adopt the following TPs to TS 38.213.

Reason for change: Introduce power headroom information for assumed PUSCH in TS 38.213. Summary of changes: Description of the provision of Pcmax for an assumed PUSCH and associated conditions. Consequences if not approved: Specification does not support reporting of power headroom information for assumed PUSCH. ### 7.7.1 Type 1 PH report < No change part is omitted > - a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State, and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State, for an actual PUSCH transmission using a spatial domain filter corresponding to the first TCI-State or TCI-UL-State and using a spatial domain filter corresponding to the second TCI-State or TCI-UL-State. If a UE provides a Type 1 power headroom report for an activated serving cell based on an actual PUSCH transmission, is provided assumedPUSCHInfo, and dynamicTransformPrecoderIndicationDCI-0-1 or dynamicTransformPrecoderIndicationDCI-0-2 is set to enabled for the active bandwidth part of the serving cell: the UE provides - PCMAX,f,c(i) based on any applicable maximum output power reduction for an assumed PUSCH with transform precoder enabled, if supported, if transform precoder is disabled for the actual PUSCH; or - PCMAX,f,c(i) based on any applicable maximum output power reduction applicable for an assumed PUSCH with transform precoder disabled, if supported, if the transform precoder is enabled for the actual PUSCH. All other parameters used for the calculation of PCMAX,f,c(i) of the assumed PUSCH are the same as the actual PUSCH; otherwise, UE does not provide a PCMAX,f,c(i) for assumed PUSCH for the serving cell as these are not supported. < End changes >

# Appendix 1: PRACH mask

TS 38.213

8.1 Random access preamble For a random access procedure associated with a feature combination indicated by FeatureCombinationPreambles, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB or msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB when provided and a number S of contention based preambles per SS/PBCH block index per valid PRACH occasion by startPreambleForThisPartition and numberOfPreamblesPerSSB-ForThisPartition. The PRACH transmission can be on a subset of PRACH occasions associated with a same SS/PBCH block index within an SSB-RO mapping cycle for a UE provided with a PRACH mask index by ssb-SharedRO-MaskIndex according to [11, TS 38.321]. <omitted> For a PRACH transmission triggered by higher layers, if ssb-ResourceList is provided, the PRACH mask index is indicated by ra-ssb-OccasionMaskIndex which indicates the PRACH occasions for the PRACH transmission where the PRACH occasions are associated with the selected SS/PBCH block index. The PRACH occasions are mapped consecutively per corresponding SS/PBCH block index. The indexing of the PRACH occasion indicated by the mask index value is reset per mapping cycle of consecutive PRACH occasions per SS/PBCH block index. The UE selects for a PRACH transmission the PRACH occasion indicated by PRACH mask index value for the indicated SS/PBCH block index in the first available mapping cycle.

TS 38.321

<omitted> 1> else if an SSB is selected above: 2> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured or indicated by PDCCH (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected SSB). <omitted>

# Reference

[1]	RP-221858, “Revised WID on Further NR coverage enhancements,” China Telecom, RAN#96-e, June 2022.

[2]	R1-2304234, “Final FL Summary on PRACH coverage enhancements”, China Telecom, RAN1#112-bis-e, April 2023.

[3]	R1-2308311, “FL Summary #4 on PRACH coverage enhancements”, China Telecom, RAN1#114, Aug, 2023.

[4]	R1-2312623, “Summary #4 on dynamic switching between DFT-S-OFDM and CP-OFDM,” Moderator (InterDigital, inc.), RAN1#115, Nov. 2023.