3GPP TSG RAN WG1 #114bis			R1-2400868

Athens, Greece, 26th February – 1st March, 2024

Agenda Item: 	9.10.1

Source: 	Sony

Title: 	Considerations on enabling UE Tx/Rx during RRM measurements

Document for: 	Discussion & Decision

# Introduction

In RANP#102, a new work item on eXtended Reality (XR) for NR phase 3 was approved [1]. The WI aims for enhancements addressing system capacity, efficient and effective mechanisms to meet QoS requirements and lower device power consumption, in the context of the demanding scenarios and traffic characteristics requirements of XR, including also multi-modal applications. The WI also aims to realize system capacity gains by enabling transmission/reception in gaps/restrictions that are caused by RRM measurements, while keeping impact to mobility performance limited. The Rel-19 XR ph3 objective that being addressed in this contribution is:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

- Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

This contribution provides our views on enabling transmission/reception in measurement gaps or in the legacy scheduling restrictions period to support XR application.

# Discussion

XR transmission is typically involving video, pose/control, and audio. Video transmission is expected to be transmitted with certain frame rate such as 60 frame/second (fps), 120 fps, etc. Video transmission may have large payload size depending on the required video quality. Pose/control may have even higher frame rate, such as 250 fps [2]. These transmissions shall be performed accordingly. A feature to support XR application in NR, including the above transmission, so called enhanced configured grant (CG) was introduced in Rel-18. However, any interruptions that affecting the above transmission may degrade the XR performance (e.g., capacity) and subsequently reducing the XR user experience.

Apart from performing data transmission and reception, a UE is also required to perform cell measurements. For example, if the signal quality of the serving cell is deteriorating, such as below certain threshold then the UE starts neighbor-cell measurements. A UE shall measure the neighboring cells signal and other carrier components.

The UE can perform measurements on the neighbor signal transmitting on the same frequency while simultaneously transmitting and receiving data from the serving cell (e.g., all signals are within a bandwidth part). However, when the measurement shall be performed in other BWP then the UE has to suspend the communication. A similar situation may happen for measuring on a cell operating at a different frequency (Inter frequency Neighbors) and Other RAT (LTE is other RAT for 5G NR). The UE has to suspend communication (Tx/Rx) with serving cell and needs to tune RF module to configured frequencies (configured Meas Objects) and resume connection with serving cell after some duration. The time duration during which mobile suspends its communication with serving cell to measure inter frequency neighbor or other RAT neighbor is known as Measurement Gap. The measurement operation is illustrated in Figure 1.

Figure 1: Illustration of Legacy Measurement Gap using SSBs

During the measurement gap, the UE can perform cell signal measurement by using SS/PBCH Block (SSB), which is composed of Synchronizations Signal (SS) and Physical Broadcast Channel (PBCH). The number of SSB in one burst depends on the Operating frequency. If Operating frequency (fc) is < 3GHz (FR1) the no. of SSB is 4, for  fc = 3GHz to 6 GHz (FR1) no. of SSB  is 8 and for fc >6 GHz mm-wave no. of SSB is be 64 within one burst. The SSB periodicity can be configured for each cell in the range of 5,10,20,40,80 or 160 ms. However, the UE does not need to measure cell signal with periodicity as frequent as the SSB. The appropriate measurement periodicity can be configured according to the channel condition.

In legacy NR, the UE has scheduling restrictions so that the UE is expected not to transmit and receive during measurement gap. During the measurement gap, the UE is expected to perform RRM operation, such as RSRP measurement of the received SSB. The measurement gap occasions can occur at the same time when the UE has been allocated for data Tx/Rx, such as the downlink configured grant (CR) transmission for XR traffic as shown in Figure 2. The legacy scheduling restrictions may affect XR transmission, making the UE not being able to transmit XR application-related data when needed, which can result in degraded  quality of service (QoS) of XR application. XR application can be sensitive to delay / latency. If the UE/gNB has to postpone or wait with a transmission  of video frame, it may result in that the delayed video frame can be obsolete or unusable. Hence, it can be even discarded at the upper layer.

Figure 2: Collision of Measurement During MGL and Data Tx/Rx

In order to ensure the RRM measurement (e.g., measurement gap) does not affect the XR transmission, we propose a signaling mechanism where the UE can temporarily modify the configured measurement gap, such as to skip the RRM measurement and continue data transmission/reception, particularly for the XR-specific traffic type. In order to support this operation, the UE may also transmit a request to modification the configured measurement gap. The indication for the UE to skip measurement gap can be indicated explicitly or implicitly. An explicit signaling can be provided from gNB to UE, indicating the UE to skip the upcoming measurement gap. This can be provided in layer 1 signaling (e.g., via DCI or MAC CE). The indication from gNB is required so that the UE behavior is still under control by the gNB. This approach is illustrated in Figure 3.

Figure 3: Illustration of Skip / Release Measurement to support data Tx/Rx

Proposal 1: Support an indication from the gNB to UE to temporary modify the configured measurement gap.

The temporary modification of the configured measurement gap can be in a various form, such as to completely skip a measurement gap, and also to shorten the measurement gap (i.e., it becomes a sub-measurement gap).

Proposal 2: Temporary measurement gap modification can be in a form of skipping measurement gap or reducing the measurement gap length.

In case of the RRM measurement is affected by the temporary measurement gap modification, for example when one or more of the measurement gaps has been skipped, then the UE can also indicate that the reported RRM measurement has been affected. Skipping one or more measurement gaps is expected to reduce the quality of the reported RRM measurement from the UE. The details of such indication / reporting can be further developed in other WG(s) (e.g., RAN2).

Proposal 3: Support an indication in the RRM measurement report indicating one or more measurement with temporary measurement gap modification has been performed (and may affect the reported RRM measurement).

# Conclusion

In this contribution, we provide our views on enabling transmission/reception during measurement gap. The proposals are listed below.

Proposal 1: Support an indication from the gNB to UE to temporary modify the configured measurement gap.

Proposal 2: Temporary measurement gap modification can be in a form of skipping measurement gap or reducing the measurement gap length.

Proposal 3: Support an indication in the RRM measurement report indicating one or more measurement with temporary measurement gap modification has been performed (and may affect the reported RRM measurement).

# References

- RP-234080, “New WID: XR (eXtended Reality) for NR Phase 3”, RANP #102, December 2023

- TR 38.838, “Study on XR (Extended Reality) Evaluations for NR”, v17.0.0, December 2021