3GPP TSG RAN WG1 #116 	R1-2400878

Athens, Greece, February 26th – March 1st, 2024

Agenda item:		8.9

Source:	Nokia, Nokia Shanghai Bell

Title:	Maintenance on IoT NTN enhancements

Document for:		Discussion and Decision

# Introduction

After the WI of Rel18, there has been many issues solved with RAN1 consensus on the agreements. While still there are some open issue on discussion for HARQ feedback disabling and GNSS enhancement. In this contribution we provide our view of HARQ-ACK feedback disabling specification in TS 36.213.

# Discussion

## Remaining issue on HARQ feedback disabling

In RAN1 #115 meeting [1], there was agreement for HARQ feedback disabling when multiple TBs are scheduled, as below

Agreement

When multiple TBs are scheduled by a single DCI:

- For Option 1 + Option 3 DCI based overridden mechanism, when DCI indicates HARQ feedback enabled, then the NB-IoT UE always wait for an RTT+3ms (i.e., till subframe n+Kmac+3 in TS36.213 section 16.6) before monitoring NPDCCH.

While for the multiple TBs scheduled, there are two different cases:

Case 1: When multiple TB is enabled/configured while multiple TB is scheduled. For this case, we think it is reasonable to define UE behavior as the RAN1 #115 agreement.

Proposal 1: For case when multiple TB is configured and multiple TB scheduled by one DCI, UE should wait for RTT+3ms before monitoring NPDCCH as RAN1 agreement.

Case 2: When multiple TB is enabled/configured while only single TB scheduled. For this case, we think it should be as the UE behavior for agreeement of single TB scheduled with HARQ feedback disabled in Rel18. I.e. the UE should not wait if Option 1 + Option 3 indicate HARQ feedback disabled for the TB, while UE should only wait if DCI enabled the HARQ feedback for that TB.

Proposal 2: For only single TB scheduled by one DCI, no matter whether multiple TB is configured by RRC, for case with Option 1 + Option 3, UE should only wait for RTT+3ms before monitoring NPDCCH if DCI enabled the HARQ feedback for that TB.

## Remaining issue on GNSS enhancement

### Measurement gap/timer end

At RAN1#113 the following was agreed:

Agreement The UE is not required to transmit or receive any channel / signal within the aperiodic GNSS measurement gap duration before the UE reacquires GNSS successfully. FFS: UE’s behavior within the duration after UE reacquires GNSS successfully to the end of the gap if the UE reacquires GNSS successfully before the end of the gap.

At RAN#114 it was further agreed:

Agreement The UE is not required to monitor N/MPDCCH within the aperiodic GNSS measurement gap, except after a CBRA (PRACH) is sent. - CBRA (PRACH) can be sent at least to request UL resource to report the remaining GNSS validity duration. Note1: The CBRA (PRACH) can only be sent within the duration after UE reacquires GNSS successfully to the end of the gap. Note2: Whether CBRA (PRACH) is sent is up to UE implementation. Note3: no change to existing CBRA procedures FFS: whether other RA procedure is needed.

The Random Access procedure in TS 36.321 defines the Random Access Response window, which also accounts for NTN propagation delays. Thus based on the RAN1 agreement for CBRA it is clear when the UE shall monitor the PDCCH for a response to the PRACH.

Observation 1: Based on the Random Access procedure accounting for NTN propagation delay it is clear when the UE shall monitor the PDCCH for a response to the PRACH.

As noted it is clear that the UE will monitor the PDCCH during relevant parts of the Random Access procedure. However, it is not clear whether the network can expect the UE to start monitoring the PDCCH and perform AS operations after starting the CBRA procedure if the GNSS measurement gap/timer has not ended. In our view, the UE should be schedulable, i.e. monitor PDCCH, when the UE starts the Random Access Response window for the CBRA procedure if the RAR window started before the end of the GNSS measurement gap/timer. This will allow the communication to continue as soon as possible after the UE has acquired the new GNSS position fix within the measurement gap/timer.

Proposal 3: The GNSS measurement gap / autonomous GNSS measurement timer ends after the UE starts the Random Access Response Window for a CBRA procedure if the RAR window started before the end of the original gap/timer.

### Extension of GNSS validity duration / allowed uplink transmission

For additional extension of UL transmission after the original GNSS validity duration expires

The conditions to allow UL transmissions after the expiry of the GNSS validity duration were discussed at RAN1#112 with the following agreement:

Agreement At least for the case when frequency error is within frequency error requirements, study the mechanisms and conditions to allow UL transmission after original GNSS validity duration expires without GNSS re-acquisition for some duration. - FFS: with legacy closed loop time correction or enhanced closed loop time correction - This mechanism is enabled/configured by eNB - FFS: whether such mechanism will be specified depends on the outcome of this study

At RAN1#112bis the following was discussed and recommended for further discussion:

FL recommendation 1-2: Companies are encouraged to evaluate on whether the duration is GNSS validity duration extension, or one value/timer configured by network, then further encouraged to down select: - Alt 1: the duration is based on remaining timeAlignmentTimer - Alt 2: the duration is one value/timer configured by network Note: Frequency error is within frequency error requirements.

At RAN1#113 the following was agreed:

Agreement From RAN1 perspective, at least for the case when frequency error and timing error are within frequency and timing error requirements with legacy closed loop time correction, UL transmission can be allowed in a duration X after original GNSS validity duration expires without GNSS re-acquisition. RAN1 will decide further details of the above.

At RAN1#114 it was further agreed:

Agreement From RAN1 perspective, down select one for the duration X: - Alt-3: when timeAlignmentTimer is not infinity, X is equal to remaining timeAlignmentTimer; - when timeAlignmentTimer is infinity, X is equal to Y; - FFS: whether X can be used to extend the original GNSS validity duration - Y is a configured value. Note 1: The feature can be enabled/disabled by network Note 2 (as already agreed): The duration X is where UL transmission can be allowed after original GNSS validity duration expires without GNSS re-acquisition.

And at RAN1#115 [1] it was agreed:

Agreement From RAN1 perspective, the start time of duration X is at the point where original GNSS validity duration expires ​ - when timeAlignmentTimer is infinity, the end of X is at the point where new timer ULTransmissionExtentionTimer expires and ULTransmissionExtentionTimer is reset with length equal to Y every time when a MAC CE (to be defined by RAN2) is received​ - Note 1: It is up to RAN2 to decide whether the MAC CE is the legacy TAC or a new TAC or a new MAC CE.​ - Note 2: It is up to RAN2 to implement the above behaviour based on new timer, existing timer, or by extending GNSS validity.​ - Send an LS to RAN2

The agreements allow the network to control whether UE can continue uplink transmissions after the expiry of the original GNSS validity duration. The timeAlignmentTimer (RRC configured) has a range from 500 subframes to 10240 subframes and in addition infinity.

In our view, the timeAlignmentTimer is used by the MAC layer to determine whether the UE is uplink time aligned and the timeAlignmentTimer is started/restarted whenever the UE receives a Timing Advance Command (TAC) MAC CE. The network may want to transmit such a TAC shortly before the expiry of the GNSS validity duration, because the UE’s timing is misaligned, e.g. due to the position being outdated. The corresponding restart of the timingAlignmentTimer should not enable the UE to continue transmissions after the GNSS validity duration expiry, because the UE may be in need of the position update. Thus the MAC CE for UL extension should not be the legacy TAC, for the cases with TAT as both finite or infinite.

Observation 2: The eNB may transmit a Timing Advance Command prior to the GNSS validity duration expiry without intending to extend the GNSS validity duration expiry by the corresponding restart of the timeAlignmentTimer.

Proposal 4: The MAC CE that resets ULTransmissionExtentionTimer shall not be the legacy MAC CE TAC, for case with TAT as both finite or infinite.

The agreement of RAN1 #115 enables the ULTransmissionExtentionTimer to be reset and thus the network can choose to reset the ULTransmissionExtentionTimer multiple times and avoid UE GNSS measurements during an extended period (where uplink time and frequency synchronization is sufficiently accurate).

An open issue is whether the extension also applies to extending the original GNSS validity duration. This is an important discussion point, because it also defines whether the UE shall continue to perform uplink pre-compensation based on the old UE location. If the extension does not apply to also reuse the location the UE is not able to perform uplink pre-compensation, because it does not have a location to apply in the calculation of propagation delay and Doppler shift. See e.g. the description in TS 36.300:

#### 23.21.2.2 Timing Advance and Frequency Pre-compensation For the serving cell, the network broadcast ephemeris information and common Timing Advance (common TA) parameters. The UE shall have valid GNSS position as well as the ephemeris and common TA before connecting to an NTN cell. To achieve synchronisation, before and during connection to a cell, the UE shall pre-compensate the Timing Advance (TTA, see TS 36.211 [4] clause 8.1), see Figure 23.21.2.2-1, by considering the common TA, UE position and the NTN payload position through the ephemeris. The UE computes the frequency Doppler shift of the service link, and pre-compensates for it in the uplink transmissions, by considering UE position and the ephemeris. If the UE does not have a valid GNSS position and/or valid ephemeris and Common TA, it shall not transmit until they are regained.

This would require the eNB instead provides Timing Advance Commands to fully control the UE transmission.

Observation 3: If the uplink transmission is allowed in a duration X after expiry of the original GNSS validity duration, the UE cannot perform uplink pre-compensation based on UE location unless the duration X also extends the GNSS validity duration.

If the eNB provides the Timing Advance Commands and notices misaligned uplink transmission the eNB can always trigger a new aperiodic GNSS measurement gap.

Observation 4: The eNB can provide Timing Advance Commands to the UE during the duration X, if the UE is not allowed to perform uplink pre-compensation based on the old UE location. The eNB can trigger a new aperiodic GNSS measurement gap if needed.

In summary, there are two options for ensuring aligned uplink transmissions during the extended duration X:

- UE reuses the old GNSS location for TA updates during the duration X. This is valid for stationary UEs, but will cause uplink interference if the UE is moving.

- UE does not reuse the old GNSS location for further TA updates and assumes the UE specific adjustment N_TA,adj_UE is equal to the value it had at time of expiry of the old GNSS location. The network has to transmit TACs to address the satellite movement’s impact on TA. This is applicable to stationary and moving UEs but implies higher signaling overhead.

Thus the deciding factor is whether the old GNSS location can be used or not. In our view, the eNB shall decide this or specification shall define it, but it shall not be left for UE implementation. One way is that the is eNB to decide whether old GNSS can still be used based on UL reception status, e.g. if the accumulated TA is large it may mean the UE precompensation based on the old GNSS is inaccurate.

Observation 5: The eNB, or alternatively specification, must decide if the UE’s old GNSS location can be used for uplink transmit pre-compensation during the extension X.

Proposal 5: RAN1 to discuss whether option 1 (reuse GNSS) or 2 (rely on TAC) for TA adjustment during extended duration is used or can be configured by the eNB.

There is one aspect of NTN IoT communication, where the reliance on Timing Advance Commands does not work: The use of segments during an uplink repetition period. The UE does not monitor for new Timing Advance Commands during such a repetition period, but may be required to change the Timing Advance between segments due to the satellite movement.

Observation 6: If the UE transmits using segments during the duration X, the UE will not monitor for Timing Advance Commands, to adjust the timing advance per segment, during the segmented transmission.

Therefore, the eNB should either not schedule transmissions that result in use of segments or alternatively, RAN1 needs to define a procedure for the eNB to provide a TAC per segment prior to the first transmission of the repetition period.

Proposal 6: If the UE is not allowed to use the old UE location during duration X, RAN1 needs to define whether 1) eNB cannot schedule segmented transmissions or 2) eNB provides a TAC per segment prior to starting the repetition period.

### Timing Advance after successful GNSS measurement

An open question is how the UE performs uplink transmission after the successful GNSS measurement gap. The UE will have a valid GNSS position and therefore it can determine the UE specific component of the TA (N_TA,adj_UE), which is based on the UE location and the satellite ephemeris. However, it is unclear whether the UE shall reset the N_TA in the N_TA+N_TA,offset+N_TA,adj_common+N_TA,adj_UE definition in TS 36.211.

Observation 7: The value of N_TA is not clear after a UE has completed the GNSS measurement successfully.

The N_TA is updated based on the UE receiving Timing Advance Commands from the network and this could e.g. occur if the UE is not compensating properly the N_TA,adj_UE based on the GNSS being incorrect, when approaching the end of the GNSS validity duration. For example, this may happen if the UE moves more or less than what was expected when reporting the GNSS validity duration.

One option, with high signaling overhead, is to require the UE always performs the Random Access procedure to obtain a new N_TA. Alternatively, RAN1 can discuss whether the UE can reuse the previous N_TA such that the UE can skip the Random Access procedure and instead start monitoring the control channel and potentially transmit Scheduling Request and the like. The previous N_TA may be applicable in scenarios where the UE mobility is limited i.e. the old and new GNSS location will be the same or at least very similar. Therefore the reuse of the previous N_TA could be conditioned on whether the new UE position is different from the previous UE position. If the difference between the new and old UE positions is small the UE could be allowed to reuse the N_TA and skip the Random Access procedure. This would significantly reduce signaling overhead and the delay.

Proposal 7: RAN1 to discuss whether the UE after a successful GNSS measurement gap/autonomous GNSS timer either performs the Random Access procedure or reuses the previous N_TA, when the new UE position is similar to the previous UE position.

### Dependency between aperiodic measurement gap and autonomous measurements

One aspect which needs further clarification is the dependency between the aperiodically triggered GNSS measurement gap and the UE autonomous GNSS measurement. In our view, the UE must always support the aperiodically triggered GNSS measurement gap if the UE supports the UE autonomous GNSS measurement. The logic is that the UE shall perform the GNSS measurement in a gap if triggered by the eNB and only if the UE is not triggered it may perform the autonomous measurement. The use of the autonomous measurement can save signaling, but it is important to maintain the option of the eNB triggered gap if the eNB wants to trigger an "early” measurement (i.e. well in advance of the GNSS validity duration expiry), for example if the eNB has noticed UL unsynchronization issue because of UE unexpected moving.

This logic is also evident from the RAN1#111 agreement below, which defines the UE may re-acquire the GNSS autonomously only if the UE does not receive the trigger from the eNB to perform the GNSS measurement in a gap. Thus the autonomous GNSS procedure is dependent on the eNB aperiodic triggered GNSS measurement procedure.

Agreement For GNSS measurement in RRC connected, if eNB aperiodically triggers connected UE to make GNSS measurement, UE can re-acquire GNSS position fix with a gap - FFS details of gap configuration The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement - FFS based on configured timing

Proposal 8: The UE has to support receiving the aperiodic trigger for a GNSS measurement gap if the UE supports the autonomous GNSS measurement.

# Conclusion

In this contribution, we discussed the remaining issues we have observed for both HARQ feedback disabling and GNSS enhancement for Rel18. Our observations and proposals can be summarized as follows.

Observation 1: Based on the Random Access procedure accounting for NTN propagation delay it is clear when the UE shall monitor the PDCCH for a response to the PRACH.

Observation 2: The eNB may transmit a Timing Advance Command prior to the GNSS validity duration expiry without intending to extend the GNSS validity duration expiry by the corresponding restart of the timeAlignmentTimer.

Observation 3: If the uplink transmission is allowed in a duration X after expiry of the original GNSS validity duration, the UE cannot perform uplink pre-compensation based on UE location unless the duration X also extends the GNSS validity duration.

Observation 4: The eNB can provide Timing Advance Commands to the UE during the duration X, if the UE is not allowed to perform uplink pre-compensation based on the old UE location. The eNB can trigger a new aperiodic GNSS measurement gap if needed.

Observation 5: The eNB, or alternatively specification, must decide if the UE’s old GNSS location can be used for uplink transmit pre-compensation during the extension X.

Observation 6: If the UE transmits using segments during the duration X, the UE will not monitor for Timing Advance Commands, to adjust the timing advance per segment, during the segmented transmission.

Observation 7: The value of N_TA is not clear after a UE has completed the GNSS measurement successfully.

Proposal 1: For case when multiple TB is configured and multiple TB scheduled by one DCI, UE should wait for RTT+3ms before monitoring NPDCCH as RAN1 agreement.

Proposal 2: For only single TB scheduled by one DCI, no matter whether multiple TB is configured by RRC, for case with Option 1 + Option 3, UE should only wait for RTT+3ms before monitoring NPDCCH if DCI enabled the HARQ feedback for that TB.

Proposal 3: The GNSS measurement gap / autonomous GNSS measurement timer ends after the UE starts the Random Access Response Window for a CBRA procedure if the RAR window started before the end of the original gap/timer.

Proposal 4: The MAC CE that resets ULTransmissionExtentionTimer shall not be the legacy MAC CE TAC, for case with TAT as both finite or infinite.

Proposal 5: RAN1 to discuss whether option 1 (reuse GNSS) or 2 (rely on TAC) for TA adjustment during extended duration is used or can be configured by the eNB.

Proposal 6: If the UE is not allowed to use the old UE location during duration X, RAN1 needs to define whether 1) eNB cannot schedule segmented transmissions or 2) eNB provides a TAC per segment prior to starting the repetition period.

Proposal 7: RAN1 to discuss whether the UE after a successful GNSS measurement gap/autonomous GNSS timer either performs the Random Access procedure or reuses the previous N_TA, when the new UE position is similar to the previous UE position.

Proposal 8: The UE has to support receiving the aperiodic trigger for a GNSS measurement gap if the UE supports the autonomous GNSS measurement.

# References

- RAN1 Chairman’s notes, 3GPP TSG RAN WG1 #115, Chicago, USA, November 13th – November 17th, 2023.