3GPP TSG-RAN WG1 Meeting #116		R1-2400892

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	7.2

Source:	Ericsson

Title:	On antenna ports used for transmission of NCD-SSB

Document for:	Discussion, Decision

# Discussion

According to TS 38.211, the antenna port used for transmission of (any) SSB is p = 4000, as shown below [1]:

### 7.4.3 SS/PBCH block #### 7.4.3.1 Time-frequency structure of an SS/PBCH block < Irrelevant parts are omitted > For an SS/PBCH block, the UE shall assume - antenna port p=4000 is used for transmission of PSS, SSS, PBCH and DM-RS for PBCH, - the same cyclic prefix length and subcarrier spacing for the PSS, SSS, PBCH and DM-RS for PBCH, - for SS/PBCH block type A, and with the quantities , and NCRBSSB expressed in terms of 15 kHz subcarrier spacing, and < Irrelevant parts are omitted > ### 8.4.3 S-SS/PSBCH block #### 8.4.3.1 Time-frequency structure of an S-SS/PSBCH block < Irrelevant parts are omitted > For an S-SS/PSBCH block, the UE shall use - antenna port 4000 for transmission of S-PSS, S-SSS, PSBCH and DM-RS for PSBCH; - the same cyclic prefix length and subcarrier spacing for the S-PSS, S-SSS, PSBCH and DM-RS for PSBCH, < Irrelevant parts are omitted >

However, the following text from TS 38.213 [2] indicates that CD-SSB (i.e., the SSB that the UE used to obtain SIB1) and NCD-SSB (the SSB provided by NonCellDefiningSSB) have the same QCL properties if they have the same index, implying that CD-SSB and NCD-SSB are transmitted from different antenna ports.

## 17.1 First procedures for RedCap UE < Irrelevant parts are omitted > If the active DL BWP provided by BWP-DownlinkDedicated, or the initial DL BWP during procedure of PUSCH transmission in RRC_INACTIVE state (as described in clause 19), includes the SS/PBCH blocks provided by NonCellDefiningSSB, these SS/PBCH blocks and the SS/PBCH blocks that the UE used to obtain SIB1 have the same QCL properties, if they have the same index. < Irrelevant parts are omitted >

For reference, the IE NonCellDefiningSSB is defined as follows in TS 38.331 [3]:

#### – NonCellDefiningSSB The IE NonCellDefiningSSB is used to configure a NCD-SSB to be used while the UE operates in an (e)RedCap-specific initial BWP or a dedicated BWP that does not contain the CD-SSB. NonCellDefiningSSB information element -- ASN1START -- TAG-NONCELLDEFININGSSB-START NonCellDefiningSSB-r17 ::= SEQUENCE { absoluteFrequencySSB-r17 ARFCN-ValueNR, ssb-Periodicity-r17 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 } OPTIONAL, -- Need S ssb-TimeOffset-r17 ENUMERATED { ms5, ms10, ms15, ms20, ms40, ms80, spare2, spare1 } OPTIONAL, -- Need S ... } -- TAG-NONCELLDEFININGSSB-STOP -- ASN1STOP NonCellDefiningSSB field descriptions. absoluteFrequencySSB Frequency of the NCD-SSB. The network configures this field so that the SSB is within the bandwidth of the BWP configured in BWP-DownlinkCommon.. ssb-Periodicity The periodicity of this NCD-SSB. The network configures only periodicities that are larger than the periodicity of serving cell's CD-SSB. If the field is absent, the UE applies the SSB periodicity of the CD-SSB (ssb-periodicityServingCell configured in ServingCellConfigCommon or ServingCellConfigCommonSIB).. ssb-TimeOffset The time offset between CD-SSB of the serving cell and this NCD-SSB. Value ms5 means the first burst of NCD-SSB is transmitted 5ms later than the first burst of CD-SSB transmitted after the first symbol of SFN=0 of the serving cell, value ms10 means the first burst of NCD-SSB is transmitted 10ms later than the first burst of CD-SSB transmitted after the first symbol in SFN=0 of the serving cell, and so on. If the field is absent, UE considers that the time offset between the first burst of CD-SSB transmitted in the serving cell and the first burst of this NCD-SSB transmitted is zero. For RedCap UEs in TDD cells, the network configures this time offset to be an integer multiple of the periodicity of the serving cell's CD-SSB..

Furthermore, the field description for NonCellDefiningSSB in the IE BWP-DownlinkDedicated is as follows [3]:

nonCellDefiningSSB If configured, the UE operating in this BWP uses this SSB for the purposes for which it would otherwise have used the CD-SSB of the serving cell (e.g. obtaining sync, measurements, RLM, BFD, beam management). Furthermore, other parts of the BWP configuration that refer to an SSB (e.g. the "SSB" configured in the QCL-Info IE; the "ssb-Index" configured in the RadioLinkMonitoringRS; CFRA-SSB-Resource; PRACH-ResourceDedicatedBFR) refer implicitly to this NCD-SSB. The NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE.

In essence, there is inconsistency between TS 38.211 and TS 38.213 regarding the antenna port used for transmission of NCD-SSB. In our view, it makes more sense to use different antenna ports for the transmission of NCD-SSB and CD-SSB (as implied in TS 38.213) as they are likely to be transmitted from different frequency locations.

# Proposal

Based on the discussion above, we have the following proposal:

- Adopt the following TP for TS 38.211 clause 7.4.3.1:

### 7.4.3 SS/PBCH block #### 7.4.3.1 Time-frequency structure of an SS/PBCH block < Unchanged parts are omitted > For an SS/PBCH block, the UE shall assume - antenna port p=4000 is used for transmission of PSS, SSS, PBCH and DM-RS for PBCH, except if the SS/PBCH block is provided by NonCellDefiningSSB in which case antenna ports p=4001, 4002, 4003, and 4004 are used for the transmission of PSS, SSS, PBCH and DM-RS for PBCH in downlink bandwidth parts with bwp-Id = 1, 2, 3, and 4, respectively, - the same cyclic prefix length and subcarrier spacing for the PSS, SSS, PBCH and DM-RS for PBCH, - for SS/PBCH block type A, and with the quantities , and NCRBSSB expressed in terms of 15 kHz subcarrier spacing, and < Unchanged parts are omitted >

Note that it is possible to configure only one NCD-SSB per DL BWP. The draft Rel-17 CR and the draft Rel-18 CR corresponding to the above TP is available in [4] and [5], respectively. Note that the Rel-17 CR is applicable for NR RedCap (WI code: NR_redcap), whereas the Rel-18 CR is applicable also for ordinary (non-RedCap) UEs supporting BWP without restriction (WI code: NR_BWP_wor).

# References

- TS 38.211 V17.6.0, “NR; Physical channels and modulation (Release 17)”, 3GPP, September 2023.

- TS 38.213 V17.8.0, “NR; Physical layer procedures for control (Release 17)”, 3GPP, January 2024.

- TS 38.331 V17.7.0, “E-UTRA; Radio Resource Control (RRC); Protocol specification (Release 17)”, 3GPP, January 2024.

- R1-2400893, “Draft Rel-17 CR for antenna ports used for transmission of NCD-SSB”, Ericsson, RAN1#116, February 2024.

- R1-2400894, “Draft Rel-18 CR for antenna ports used for transmission of NCD-SSB”, Ericsson, RAN1#116, February 2024.