3GPP TSG RAN WG1 #116			R1-2400901

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.5.1

Source:	Panasonic

Title:	Discussion on on-demand SSB SCell operation

Document for:	Discussion/Decision

# Introduction

In the RAN #102 meeting, new work item for enhancement of network energy saving for NR was approved [1].

In this contribution, we share our preliminary views on the targeted scenarios and corresponding on-demand SSB SCell operation.

# Discussion

## Multi-cell scenarios

As mentioned in the WID as below, the on-demand SSB SCell operation is only confined within CA framework. Depending on the difference on intra-/inter-band CA and the co-location assumption of PCell and SCells , the system operation for network energy saving is different.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

Figure.1 An example of the deployment of CA using multiple TRPs

Figure.1 shows the example of CA deployment. F1 cell in red and F2 cell in green provide with same coverage. F3 cell in blue provides only partial coverage of F1 cell or F2 cell. When F1 cell is configured as PCell for a UE, the F2 and F3 can be configured with on-demand SSB SCell operation. The possible reasons to enable SSB on SCells is different between F2 Cell and F3 Cell.

- PCell and SCell with same coverage: Network may decide whether to switch SSB ON/OFF of F2 Cell based only DL/UL traffic status. As per traffic status, each UE's throughput and multiple UE's cell loads are taken into account at the network. As these traffic are already known at the network from BSR from UE or DL network buffer status, "on-demand" is not required from UE but from system perspective. The UE mobility and cell coverage are not required to be considered in this scenario as F2 SCell coverage is same as the F1 PCell. The mobility related measurement of SSB may be sufficient only F1 frequency. CSI related measurement of SSB may be required in F2 frequency. Or to use CSI-RS for the CSI related measurements. Instead of cross-carrier scheduling from F1 PCell, if PDCCH is located in F2 SCell, SSB ON or TRS is required for the PDCCH monitoring purpose.

- SCell is only part of PCell coverage: In addition to the reason to enable SSB ON of the same coverage of PCell and SCell, UE mobility aspect needs be considered as the coverage of PCell and SCell are different. UE may be within F3 SCell coverage or may not within F3 SCell coverage. Network needs to know whether UE is within F3 SCell coverage or not. One way is based on network implementation method like UE location is estimated from PCell or TRP for F3 SCell has the receiver of SRS or PRACH in F1 frequency. The other way is SSB is sent in F3 SCell and UE measure them. If UE detects SSB of F3 SCell, UE reports the detection of F3 SCell. For such usage, SSB ON is required regardless of DL/UL traffic status. Note that, the final decision of SSB ON/OFF after UE report and whether F3 SCell is detectable is up to the network.

Table 1: summary of the difference for the factor to enable SSB ON/OFF

|  | PCell and SCell are same coverage | SCell is only part of PCell coverage |
| --- | --- | --- |
| SSB ON/OFF based on the DL/UL traffic status | - SSB ON/OFF based on each UE's throughput and multiple UE's cell loads - If TRS is not used, SSB is required for following purpose. - CSI measurement - PDCCH for self-scheduling - PDSCH/PUSCH - SRS | - SSB ON/OFF based on each UE's throughput and multiple UE's cell loads - If TRS is not used, SSB is required for following purpose. - CSI measurement - PDCCH for self-scheduling - PDSCH/PUSCH - SRS |
| SSB ON/OFF based on the UE mobility | Not required to be considered | - SSB ON is required regardless of traffic if UE measure the coverage of SCell. - SSB ON is not required if UE existence of SCell is identified by network implementation. |

Observation 1: PCell and SCell are assumed to be same coverage or not makes the difference on whether the cause of SSB ON/OFF is DL/UL traffic or UE mobility.

Proposal 1: When PCell and SCell are assumed to be same coverage, traffic status is the only reason to enable SSB ON/OFF. "On-demand" is sufficient only from network perspective.

Proposal 2: When SCell is only part of PCell coverage, the mechanism for the detection of UE existence in SCell coverage may be required. In this case, "on-demand" may be from UE request but the final decision of SSB ON/OFF is network.

## Compatibility of the on-demand SSB SCell operation

In this section, we discuss the potential impact to the compatibility and coexistence with legacy UEs by introducing the on-demand SSB feature.

### Compatibility with UEs not supporting on-demand SSB

As the usage of a serving cell frequency is up to network implementation, when an SCell frequency is configured to a Rel.19 UE as an SCell operating on-demand SSB, it may also be configured to a legacy UE not supporting such feature as a PCell or SCell. In spite that this is complicated discussion, this is essential to clarify it, as this influences on the network deployment flexibility.

- On-demand SSB SCell serving a PCell for legacy UE(s): The legacy SSB configured in the SIB1 should be always ON. The on-demand SSB should be additionally configured SSB. If the periodicity of the legacy SSB is short, we do not see the benefits for such usage and deployment of features, as the room for network energy saving is limited. On the other hand, if the periodicity of the legacy SSB is large enough, we see the potential energy saving gain for such deployment. When the on-demand SSB is OFF, the gNB may sleep in the middle of legacy SSB bursts and when on-demand SSB is ON, gNB can serve the new release UEs as SCell. This may be treated as SSB adaptation in time domain in network perspective. We see this may be related and within the scope of the 3rd objective of this work item.

- On-demand SSB SCell serving a SCell for legacy UE(s): If both legacy UEs and Rel.19 UEs are served, the SSB ON/OFF for legacy UEs may be controlled by the MAC-CE based SCell activation/deactivation procedure. While for Rel.19 UEs, more efficient signaling and procedure is also worth to utilize. We see such deployments have energy saving and signaling overhead merits.

Observation 2: When legacy UEs and Rel.19 UEs coexist in a serving cell, it is also possible to operate the on-demand SSB feature for energy saving gain. Whether/how such scenario should be considered would impact

- The signaling/procedure design for on-demand SSB ON/OFF, and

- Whether to define the on-demand SSB as the only SSB in the cell or additional SSB besides the legacy SSB. We are also okay to discuss this in the 3rd objective of this WI.

### Compatibility with Rel.18 SSB-less operation

In the Rel.18, RAN4 discussed and is still specifying the SSB-less operation for SCell. In general, a SCell can be configured with SSB-less operation for intra-band CA, and inter-band CA in FR1/collocated cells. For such SCell, a reference cell (PCell or SCells is not limited) is configured as the synchronization and measurement reference source. In our understanding, the Rel.19 on-demand SSB SCell can operate together with the Rel.18 SSB-less SCell operation for more aggressive energy saving gain. The procedures and signaling design should take this into account.

Proposal 3: The signaling/procedure design for Rel.19 on-demand SSB SCell should consider the coexistence with the Rel.18 SSB-less SCell operation.

# Conclusion

- Based on the discussion, the following proposals are highlighted:

Observation 1: PCell and SCell are assumed to be same coverage or not makes the difference on whether the cause of SSB ON/OFF is DL/UL traffic or UE mobility.

Proposal 1: When PCell and SCell are assumed to be same coverage, traffic status is the only reason to enable SSB ON/OFF. "On-demand" is sufficient only from network perspective.

Proposal 2: When SCell is only part of PCell coverage, the mechanism for the detection of UE existence in SCell coverage may be required. In this case, "on-demand" may be from UE request but the final decision of SSB ON/OFF is network.

Observation 2: When legacy UEs and Rel.19 UEs coexist in a serving cell, it is also possible to operate the on-demand SSB feature for energy saving gain. Whether/how such scenario should be considered would impact

- The signaling/procedure design for on-demand SSB ON/OFF, and

- Whether to define the on-demand SSB as the only SSB in the cell or additional SSB besides the legacy SSB. We are also okay to discuss this in the 3rd objective of this WI.

Proposal 3: The signaling/procedure design for Rel.19 on-demand SSB SCell should consider the coexistence with the Rel.18 SSB-less SCell operation.

# Reference

[1] RP-234065	New WID: Enhancements of network energy savings for NR