3GPP TSG RAN WG1 #116			R1-2400906

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.6.3

Source:	Panasonic

Title:	Discussion on LP-WUS operation in CONNECTED mode

Document for:	Discussion/Decision

# Introduction

In RAN #102 meeting, the new work item of LP-WUS/WUR (low-power wake-up signal and receiver) for NR was approved. The objective of the WI [1] includes the design of LP-WUS operation in CONNECTED mode.

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

In this contribution, we share our views on LP-WUS operation in connected mode.

# Discussion

## The potential power saving gain and priority for LP-WUS operation in CONNECTED mode

As emphasized by the note in the WID, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR. Then the power saving gain from the LP-WUS triggering PDCCH monitoring could be unclear and not that appealing. The application of LP-WUS in connected mode may only be attractive on some specific scenarios that the RRM/RLM/BFD/CSI measurement using MR is in very low-frequent level, e.g., RRM relaxation is assumed, or stationary cases.

Observation 1: The power saving gain of LP-WUS triggering MR PDCCH monitoring is dependent to some specific scenarios and use cases, considering MR needs to perform RRM/RLM/BFD/CSI measurements.

Proposal 1: The applicable scenarios and use cases need to be clarified for better understanding before specifying the procedures.

Considering quite some working load for the LP-WUS/LP-SS physical layer design and idle/inactive mode operation is needed and may fully utilize the planned TUs, the parallel discussion of LP-WUS operation in connected mode may increase the possibility that both idle/inactive and connected mode functionalities may not be able to be finalized in time. Or the specification may not be so optimized for idle/inactive mode, which decreases the merit of the introducing LP-WUS. The scope should be confined.

Proposal 2: The scope of LP-WUS operation in connected mode should be confined to the available TUs.

## The scope for LP-WUS operation in CONNECTED mode

- On UE power saving by adapting the PDCCH monitoring, Rel.16 and Rel.17 already specified below procedures for connected mode:

- DCI format 2_6 may PDCCH monitoring before the active time of C-DRX

- Dormancy SCell indication

- PDCCH skipping

- SSSG (search space set group) switching

- Then for the LP-WUS operation discussion, similar with the situation when specifying Rel.16 WUS/DCP/DCI format 2_6, an essential point is whether C-DRX is always assumed for LP-WUS operation to avoid design duplication.

- For better flexibility of PDCCH monitoring triggering, C-DRX may not be an essential part of the feature. The time domain configuration of LP-WUS can flexibly balance the UE power saving gain and scheduling latency. Such design philosophy is similar with legacy PDCCH skipping.

- On the other hand, using C-DRX procedure has merit of potentially less specification impact as many procedures and features can be reused. Thus, as starting points for discussion, below alternatives for LP-WUS operation combined with C-DRX can be considered:

- Alt.1: LP-WUS is monitored by UE LR and may trigger the UE MR start monitoring DCI format 2_6 (if configured) before C-DRX active time. The merit is better reliability protection as the LP-WUS coverage target is matched to PUSCH for Msg3 and not as good as PDCCH. In this alternative, LP-WUS only triggers DCI format 2_6 monitoring but does not directly trigger PDCCH monitoring in C-DRX active time.

- Alt.2: If DCI format 2_6 is not configured, LP-WUS is monitored by UE LR before C-DRX active time and may trigger the UE MR start monitoring PDCCH during C-DRX active time. This replaces the function of DCI format 2_6. But the reliability may be compromised.

- Proposal 3: PDCCH monitoring triggering by LP-WUS should select only one from the legacy framework of DCI format 2_6 and C-DRX, or PDCCH skipping in order to limit the scope.

# Conclusion

- Based on the discussion, the following proposals are highlighted:

Observation 1: The power saving gain of LP-WUS triggering MR PDCCH monitoring is dependent to some specific scenarios and use cases, considering MR needs to perform RRM/RLM/BFD/CSI measurements.

Proposal 1: The applicable scenarios and use cases need to be clarified for better understanding before specifying the procedures.

Proposal 2: The scope of LP-WUS operation in connected mode should be confined to the available TUs.

- Proposal 3: PDCCH monitoring triggering by LP-WUS should select only one from the legacy framework of DCI format 2_6 and C-DRX, or PDCCH skipping in order to limit the scope.

# Reference

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

# Appendix

From the WID [1], the objectives of the work item are the following:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - This objective is to be further refined in RAN#103