3GPP TSG-RAN WG1 #116	R1-2400908

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.1.3.1

Source:	InterDigital, Inc.

Title:	Discussion on AI/ML-based CSI prediction

Document for:	Discussion and Decision

# Introduction

In RAN #102, normative work on AI/ML for NR Air Interface has been approved with some additional studies which has not been finalized during the study phase [1]. The normative work is based on the previous Rel-18 studies in FS_NR_AIML_Air [2].

For the CSI enhancement use case, the objective of the study is to address some of the outstanding issues identified during the Rel-18 Study Item, as summarized in TR 38.843 [3].

With reference to CSI prediction, the study objectives listed in the Rel-19 WID are shown below [1].

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: …. - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain).

This contribution further discusses aspects of CSI prediction accuracy, model complexity, and LCM aspects related to model monitoring. Additionally, we provide a set of evaluation results for CSI prediction.

# Considerations on AI/ML-based CSI Prediction

## Factors impacting the CSI prediction performance

In Rel-18 SI, CSI prediction performance has been measured by the intermediate KPIs (SGCS and/or NMSE) and the end-to-end KPIs (mean UPT and/or 5% UPT). CSI prediction accuracy can also be measured and reported by the UE, as an intermediate KPI, for efficient model monitoring and operation. Each predicted CSI report may be accompanied with a prediction accuracy computed based on historical predictions and measurements. Such a report can be used to enable adaptive CSI-RS allocation. For example, following the report, the UE may receive updated CSI-RS configuration based on the reported prediction accuracy. The UE could also request changes in CSI-RS configuration depending on the historical prediction accuracy. Reporting the CSI prediction accuracy can improve the resource allocation and end-to-end KPIs.

Proposal 1: Study enhancements to report the prediction accuracy for the UE-sided CSI prediction model.

In Rel-18 SI, the results have shown that the CSI prediction performance depends on channel conditions. Some evaluations in Rel-18 SI have focused on the impact of the two window lengths, observation and prediction windows, under different channel conditions. Most results have shown that an increase in the observation window results in an increase in SGCS for the predicted CSI. Mixed results were observed for the prediction window, where some companies have shown positive gain with increasing windows while some others have shown the opposite. The results have shown that channel conditions, such as Doppler and coherence time, may impact the performance of CSI prediction if the two windows do not align well with these conditions.

Proposal 2: Study enhancements to support variable window length for CSI prediction at inference time.

Data collection for UE-sided CSI prediction is required for both training and monitoring purposes. The collected data timeline for CSI prediction can consist of two time windows, one for historical CSI input (i.e., observation window), and the other for the prediction (i.e., prediction window). The sizes of the observation and prediction windows are important factors in the design, training, and operation of CSI prediction models. For each combination of window lengths, different prediction models may need to be trained and operated. For monitoring purposes, the CSI prediction data collection depends on the length of two windows. The window durations impact the CSI-RS configuration (i.e., CSI-RS need to be transmitted at least during the first window, potentially with higher granularity, while the CSI-RS needs to be available in the observation window for the UE to measure the actual channel). Likewise, the window durations impact the CSI reporting configuration (i.e., the sizes of the windows impact the timing and size of CSI reports). Thus, configurable CSI reporting may be needed depending on the window lengths to support data collection for training and monitoring.

The window durations can be controlled by the gNB. However, their configurations depend on UE feedback. TDCP was specified in Rel-18 and can be used by the gNB to determine appropriate window size. Additional UE feedback, for example based on AI/ML prediction performance under specific channel conditions, should be studied.

Proposal 3: Study CSI-RS configuration enhancements based on variable observation and prediction window sizes for data collection for training and monitoring of UE-side CSI prediction models.

Proposal 4: Study CSI reporting enhancements based on variable observation and prediction window sizes for data collection for training and monitoring of UE-side CSI prediction models.

## Model monitoring for UE-side AI/ML CSI prediction

The findings of the evaluation part of the Rel-18 SI have shown that the intermediate KPIs (such as SGCS and/or NMSE) may not be the best indicators of the overall end-to-end performance. For example, for CSI prediction, a lower SGCS value may not result in a significant throughput loss; moreover, a lower SGCS value may not necessarily indicate poor performance of the model. For example, when the prediction window size is larger than the coherence time of the channel, poor performance is due to incorrect prediction configuration rather than generalized poor model performance. Inaccurate indicators of the model performance may result in increased overhead for model switching, activation/deactivation, or model parameter configuration. To avoid such overhead, it is important to study metrics for monitoring the UE-side CSI prediction model that are more indicative of the end-to-end performance.

Proposal 5: 	Study metrics for monitoring the UE-side CSI prediction model that are more indicative of end-to-end performance.

UE-side performance monitoring based on data distribution may help improve the model performance, where metrics related to the distribution of input data, determined by the UE, could be reported in addition to the intermediate KPI metric. For example, the UE may be configured to perform out-of-distribution detection for the input data, or to provide out-of-distribution metrics. While these metrics may not directly reflect the end-to-end system performance, when used in conjunction with intermediate KPIs and end-to-end metric, they can help improve the model monitoring performance.

Proposal 6:	Study the benefits of using out-of-distribution metrics for UE-side model monitoring.

## Factors impacting the complexity and overhead

For CSI prediction, the generalization evaluations in Rel-18 SI were mostly limited to UE speeds. Majority of the results have shown moderate or significant SGCS degradation when a CSI prediction model was tested with a speed different than the speed used for the model training. Similarly, results by some companies have shown that the end-to-end UPT performance may not generalize between different speeds. The CSI compression sub-use case has covered a wider range of generalization aspects compared to CSI prediction, including deployment scenarios, UE distributions, carrier frequencies, etc. To overcome the issues related to generalization, for example, different models can be used for different speeds, channel conditions, etc. However, this may bring in operational complexity and overhead where frequent model switching may be required. As an alternative, techniques such as pre/post-processing, adaptive quantization, and training with mixed datasets can be used to overcome frequent model switching.

Observation 1: Studies on the generalization aspects for CSI prediction in Rel-18 SI were limited compared to CSI compression.

Proposal 7: Study a wider range of generalization aspects for CSI prediction.

Proposal 8: Study improved model generalization techniques to reduce the need for frequent model switching.

In some scenarios, if the performance of the active and possibly the inactive UE-side prediction models does not meet configured performance thresholds, fallback to legacy CSI reporting may be needed. Considering the CSI prediction reporting scheme is already specified in Rel-18 MIMO, and the configuration between legacy CSI reporting mode and CSI prediction mode is supported, a fallback mechanism which may be commonly used for both AI/ML based CSI prediction scheme and non-AI/ML based CSI prediction scheme can be studied. For example, a UE may report CSI prediction accuracy level (e.g., confidence level of CSI prediction) and gNB may determine fallback decision based on the UE feedback of the CSI prediction accuracy level.

Proposal 9: 	Study a fallback mechanism which can be applicable for both AI/ML based CSI prediction and non-AI/ML based CSI prediction.

# Performance evaluation

## AI/ML Model considerations

### Model architecture

The AI/ML model adopted for CSI prediction has a LSTM backbone with 3 LSTM layers and 1 fully connected layer, with a total of 707 k parameters. Additionally, there is a weighted bypass that connects the input to the output. The input to the model is a (NRx×NTx)MIMO channel associated with a single sub-band and thus the prediction for each sub-band is performed independently. SGCS is used as the loss function for the training.

### Dataset for model training

In the context of CSI prediction, let L denote the number of historical CSI to be fed to the CSI predictor (either the AI/ML model or the baselines), and let N denote the time slot index of the future CSI to be predicted. In the construction of the dataset, Nsamples channel samples are generated, where each two consecutive channel samples are 1 ms a part. The size of channel sample is NRx×NTx×Nsb×NUEs, where NRx, NTx, Nsb, and NUEs denote the number of receive antenna ports, the number of transmit antenna ports, the number of sub-bands, and the number of UEs, respectively. The time samples of each channel element in the 4D tensor are then grouped together to create NRx×NTx×Nsb×NUEs sequences of size Nsamples each. Afterwards, for each channel coefficient, Nseq= Nsamples-L+N+1 pair of sequences are constructed from its initial sequence of size Nsamples time samples. Specifically, for all i ∈ 1, Nseq: the two sequences are defined as follows:

- The first sequence contains the time samples of the channel element from i-1+1 to i-1+L, which is the sequence of features (or sequence of historical CSI) that will be fed to the CSI predictor,

- The second sequence contains the time samples of the same channel element from i-1+L+1 to i-1+N+1, which represents the sequence of labels (or sequence of future CSI) to be predicted by the CSI predictor.

Finally, the dataset consists of the NRx×NTx×Nsb×NUEs×Nseq distinct sequences of features, where each sequence has L features, and their associated NRx×NTx×Nsb×NUEs×Nseq sequences of labels, where each sequence has N labels.

## CSI prediction baseline

### Kalman filter architecture for CSI prediction

The Kalman filter is an algorithm for doing inference in discrete linear dynamic systems in which the state space of the latent variables is discrete and where all latent and observed variables have a Gaussian distribution. In the context of CSI prediction, the CSI evolution over time can be modelled as a noisy dynamic system, and therefore, the Kalman filter can be used to predict the future CSI. Specifically, the Kalman Filter is a two-stage algorithm that assumes there is a smooth trend-line within the data that represents the true value of the CSI before being disturbed by noise. In the first stage, a few historical CSI values are fit to a model, which are then extrapolated to the next time value to generate a CSI prediction along with its error variance. In the second stage, the corresponding CSI measurement is read, and a new CSI trend value is computed as a weighted average of the predicted CSI and the measured CSI. The weights are based on the relative amounts of noise in the data and predictions. The filter then repeats this cycle of prediction and correction as each new CSI value is observed.

Moreover, let hk denote the true (or corrected) CSI state at time k, which includes the historical CSI from k-L+1 to k, hk+N denote the estimated (or observed) CSI state at time k+N, and hk+N denote the predicted CSI state of time k+N. The goal of the Kalman Filter is to predict the CSI state  hk+N of time k+N at time k, using the true CSI state hk at time k, and then correct this prediction using the estimated CSI state hk+N at time k+N.

In this case, the Kalman Filter assumes that the CSI state at time k+N evolves from the CSI state at time k according to

hk+N=Ak+N hk+ qk+N,

Where, Ak+N is the state transition matrix, and qk+N is the process noise with wk+N∼ ℵ 0, Qk+N, in which Qk+N is the covariance matrix of the process noise This equation is called the dynamic (or plant) equation or the channel. Moreover, at time k+N, an estimation of the CSI is made, and the estimated CSI state at time k+N follows

hk+N= hk+N+ vk+N,

Where vk+N is the observation noise with vk+N∼ ℵ 0, Vk+N, in which Vk+N is the covariance matrix of the observation noise. The initial state h0 is generally modelled as a Gaussian random variable, with known mean and covariance. The two noisy sequences and the initial state are mutually independent and the matrix Ak+N depends on the adopted channel model.

The Kalman Filter uses the previous corrected state and the current observed CSI state to predict and correct the current CSI state, hence it is a recursive estimator. Two variables need to be predicted and corrected, which are hk+N, and Pk+N, which is the error covariance matrix or in simple words the measure of the estimated accuracy of the CSI state estimate. There are two stages in the Kalman Filter algorithm, namely, prediction and correction.

Prediction:

The predicted CSI state estimate is given by hk+N= Ak+N hk and the predicted covariance is given by Pk+N= Ak+NPk Ak+NT.

Correction:

After performing the prediction, the measurement residual at time k+N, is given by ek+N= hk+N- hk+N, and its covariance is given by Ek+N= Pk+N+ Vk+N. Hence, the filter gain is given by Gk+N= Pk+NEk+N-1. Based on this, the corrected CSI state is computed as hk+N= hk+N+ Gk+Nek+N, and the corrected covariance is computed as Pk+N= Pk+N- Gk+N Ek+N Gk+NT.

Finally, at each time slot k+N, the CSI state transition matrix Ak+N and the CSI process covariance matrix Qk+N are obtained and updated online along with the prediction and correction procedures.

## CSI prediction simulation configuration

In generating the channel samples, we consider the case of spatial consistency (SC) off (Doppler only); the simulation configuration specific to CSI prediction is shown below.

- Traffic type: FTP Model 1

- Max rank value = 2

- Intermediate KPI#1 = SGCS

- Channel model: UMa

- UE distribution: 100% outdoor UE

- O2I car penetration loss (as per 38.901, section 7.4.3.2 and the evaluation assumption in Table 6.2.1-1 of TR 38.843 [3])

- Number of drops for training: 50

- Number of drops for testing: 10

- Number of UEs per drop: 420 for training, and 126 for testing

- Number of time samples N_samples (5 ms period): 60 for training and 200/400/600 for high RU/med RU/low RU, respectively for SLS testing.

- Observation window: 5/5

- Prediction window: 1/5/5

The common parameters for dataset generation and for performance evaluation are as shown in [3]; for convenience, the common parameters table is also included in the Appendix.

## CSI prediction simulation results

To evaluate the CSI prediction performance, we consider the Kalman Filter approach as the non-AI/ML prediction baseline, and present a revised set of system level simulation results for the 10 km/h and 30 km/h cases, for Doppler (without spatial consistency).

Figure 1 shows the relative performance of the AI/ML model versus the non-AI/ML baseline (the Kalman filter), for the 10 km/h scenario, in terms of mean and 5% throughput, for low, medium and high resource utilization. It can be seen that for the AI/ML approach exhibits a small degradation of less than 1%, compared to the KF benchmark, for the 5%-ile UE throughput. In terms of the mean throughput, the AI/ML performs similar to the KF benchmark, with the relative gain/loss in the (-0.5% to 0.3%) range.

Figure 1 Relative throughput performance of AI/ML vs. Kalman, 10 km/h, Doppler

Observation 2: At speed 10 km/h without spatial consistency, AIML-based CSI prediction achieves roughly the same performance as the non-AIML based CSI prediction (KF) in terms of mean UPT and exhibits less than 1% loss in terms of 5% UPT.

Figure 2 shows the relative performance of the AI/ML model versus the non-AI/ML baseline (the Kalman filter), for the 30 km/h scenario, in terms of mean and 5% throughput, for low, medium and high resource utilization. In terms of the mean throughout, the AI/ML model performs close to the KF benchmark; for the 5%-ile UE throughput, the AI/ML models show an improvement of 7.4% compared to the KF benchmark. As the resource utilization (RU) increases, the relative gain of the AI/ML compared to the KF decreases, and a loss of 2% is seen for the high RU case.

Figure 2 Relative throughput performance of AI/ML vs. Kalman, 30 km/h, Doppler

Observation 3: At speed 30 km/h without spatial consistency, AIML-based CSI prediction achieves roughly the same performance (in the range of -0.4% to 0.4%) as the non-AIML based CSI prediction (KF) in terms of mean UPT. In terms of 5% UPT, the AI/ML model exhibits a 7.4% gain over KF for low RU, and 2% loss for high RU.

# Conclusion

In this contribution, we discussed aspects of CSI prediction accuracy, model complexity, and LCM aspects related to model monitoring, and provided a revised set of evaluation results for CSI prediction.

We provide the following observations and proposals.

Observation 1: Studies on the generalization aspects for CSI prediction in Rel-18 SI were limited compared to CSI compression.

Observation 2: At speed 10 km/h without spatial consistency, AIML-based CSI prediction achieves roughly the same performance as the non-AIML based CSI prediction (KF) in terms of mean UPT and exhibits less than 1% loss in terms of 5% UPT.

Observation 3: At speed 30 km/h without spatial consistency, AIML-based CSI prediction achieves roughly the same performance (in the range of -0.4% to 0.4%) as the non-AIML based CSI prediction (KF) in terms of mean UPT. In terms of 5% UPT, the AI/ML model exhibits a 7.4% gain over KF for low RU, and 2% loss for high RU.

Proposal 1: Study enhancements to report the prediction accuracy for the UE-sided CSI prediction model.

Proposal 2: Study enhancements to support variable window length for CSI prediction at inference time.

Proposal 3: Study CSI-RS configuration enhancements based on variable observation and prediction window sizes for data collection for training and monitoring of UE-side CSI prediction models.

Proposal 4: Study CSI reporting enhancements based on variable observation and prediction window sizes for data collection for training and monitoring of UE-side CSI prediction models.

Proposal 5: Study metrics for monitoring the UE-side CSI prediction model that are more indicative of end-to-end performance.

Proposal 6:	Study the benefits of using out-of-distribution metrics for UE-side model monitoring.

Proposal 7: Study a wider range of generalization aspects for CSI prediction.

Proposal 8: Study improved model generalization techniques to reduce the need for frequent model switching.

Proposal 9:  Study a fallback mechanism which can be applicable for both AI/ML based CSI prediction and non-AI/ML based CSI prediction.

# References

RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Work Item Description.

RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Work Item Description.

3GPP TR38.843 V18.0.0 (2023-12) Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)

C. Wen, W. Shih and S. Jin, "Deep Learning for Massive MIMO CSI Feedback," in IEEE Wireless Communications Letters, vol. 7, no. 5, pp. 748-751, Oct. 2018, doi: 10.1109/LWC.2018.2818160.

W. Liu, W. Tian, H. Xiao, S. Jin, X. Liu and J. Shen, "EVCsiNet: Eigenvector-Based CSI Feedback Under 3GPP Link-Level Channels," in IEEE Wireless Communications Letters, vol. 10, no. 12, pp. 2688-2692, Dec. 2021, doi: 10.1109/LWC.2021.3112747.

A. Vaswani, N. Shazeer, N. Parmar, J. Uszkoreit, L. Jones, A. N. Gomez, Ł. Kaiser, and I. Polosukhin. "Attention is all you need." Advances in neural information processing systems 30 (2017).

# Appendix

The common parameters used for the considered suite of simulations are based on the assumptions listed in [3] and is shown below for convenience.

Table 1 Common parameters used in all Scenarios/Configurations

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Uma, Umi |
| Frequency Range |  | FR1 only, 2GHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | 16 ports ; configuration specific |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15 kHz |
| Simulation bandwidth |  | 10MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | MU-MIMO with rank adaptation |
| Number of users |  | 6 UE per BS |
| Max number of MU layers |  | 12 |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Traffic model |  | Full buffer, Non full buffer (FTP Model 1), packet size 0.5 Mbytes |
| UE distribution |  | CSI prediction: 100% outdoor, including O2I car penetration loss (as per TR 38.901) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | 4ms delay, ideal |
| Channel estimation |  | ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. |
| Baseline for performance evaluation |  | Rel-16 Type II CSI |