3GPP TSG RAN WG1 #116	   	 	  	                	      R1-2400920

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	9.2.4

Source:	LG Electronics

Title:	Discussions on asymmetric DL sTRP/UL mTRP scenarios

Document for:	Discussion and Decision

Introduction

In the WID for phase 5 MIMO[1], objectives for RAN1’s normative work were listed as below.

RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

In this contribution, we discuss enhancements for asymmetric DL sTRP/UL mTRP scenarios. Specifically, we provide our view on pathloss offset for asymmetric DL sTRP/UL mTRP scenarios.

Discussions

  - Pathloss offset for asymmetric DL sTRP/UL mTRP scenarios

In the WID for phase 5 MIMO[1], power control enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios is described, specific to pathloss compensation for micro nodes as UL reception only TRP. From procedure perspective, it would be better to discuss beam management procedure for micro nodes before discussing pathloss offset for UL-only micro nodes. In objective of the WID, it is assumed that Rel-17/18 unified TCI framework is the baseline and the legacy QCL/UL spatial relation rules are fully reused. In that sense, Rel-18 up to two indicated TCI (mainly enhanced for mTRP scenarios) can be easily exploited for this asymmetric DL sTRP/UL mTRP scenarios since one of two UL related indicated TCI can be used for macro gNB and the other UL related indicated TCI can be used for UL-only micro nodes. However, assuming that there is no DL transmissions from micro nodes, DL RS cannot be configured as QCL reference RS for TCI states relevant to micro nodes. Accordingly, SRS-based UL beam management should be the baseline for UL beam determination regarding micro nodes. If NW finds the best UL beam for micro nodes from SRS beam management, SRS for beam management can be configured as spatial relation RS for UL related TCI states toward micro nodes. On top of this procedure, NW can be aware of macro gNB-specific best UL beam and micro nodes-specific best UL beam. So, the remaining issue is how to compensate pathloss from micro nodes to UE. The only way to figure out the exact pathloss offset value for micro nodes is for NW to use the same DL RS (from macro gNB) as PL-RS for both macro gNB-specific best UL beam and micro nodes-specific best UL beam, and to compare the difference between them. The signaling for pathloss offset value for mirco nodes can be achieved by RRC signaling for NW to configure micro nodes-specific pathloss offset value in the unit of dB. This kind of micro nodes-specific pathloss offset value can be configured for TCI state(s), specifically for UL TCI state for micro nodes. Also, considering that pathloss value from micro nodes to UE can change due to UE mobility, the micro nodes-specific pathloss offset value can be updated by MAC CE signaling.

Proposal 1. Specify pathloss offset value for UL-only micro nodes, which is configured in UL TCI state.

    - The pathloss offset value can be updated by MAC CE.

Furthermore, from technical perspective, 2TA enhancements (e.g., by reusing Rel-18 2TA enhancement based on M-DCI based mTRP, or by extending Rel-18 2TA enhancement to S-DCI based mTRP) and enhancements on uplink beam management for UL-only micro nodes can also provide benefit in this DL sTRP/UL mTRP deployment scenarios. In other words, considering different propagation delay between macro gNB and UL-only micro nodes and UE multi-Tx-panel, 2TA enhancements can bring performance benefit from synchronized mTRP uplink transmission perspective in case of DL sTRP/UL mTRP deployment scenarios. In addition, enhancements on SRS-based beam management can be considered for UE to sweep specific range of beam angles especially toward UL-only micro nodes, which is to achieve delay/latency reduction for finding best UL beam for the micro nodes.

Conclusion

In this contribution, we have discussed enhancements for asymmetric DL sTRP/UL mTRP scenarios, and provided the following proposal.

Proposal 1. Specify pathloss offset value for UL-only micro nodes, which is configured in UL TCI state.

    - The pathloss offset value can be updated by MAC CE.

References

[1] RP-234007, “New WID: NR MIMO Phase 5”, Samsung